ALIGN

Version 3.0 for IBM Personal Computers

Program 06/02/89

Minor document update 01/28/90

(c) Copyright John L. Dickinson 1983, 1987, 1989, 1990.  All rights reserved.

DISCLAIMER

This program is provided "as-is" without warranty of any kind,

either expressed or implied, including, but not limited to the

implied warranties of merchantability and fitness for a particular

purpose.  The entire risk as to the quality and performance of the

program is with you.

In no event will I be liable to you for any damages, including any

lost profits, lost savings or other incidental or consequential

damages arising out of the use or inability to use this program

even if I have been advised of the possibility of such damages, or

for any claim by any other party.

COPYING PROVISIONS

This program and documentation may be freely copied and shared

provided it is not modified in any manner and this notice is not

bypassed or removed.  Please register by sending your name,

address and any comments to the author.  There is no charge.

John L. Dickinson

2470 N. Tierra Verde Pl.

Tucson, Arizona 85749-9315

CHANGE HISTORY

11/26/87 Version 2.9

- Initial shareware release.

06/02/89 Version 3.0

- Reset diskette subsystem after drive timeout and display

drive status from BIOS.  This prevents hangs when no diskette

is inserted or when accessing certain tracks on some AAD's.

- Make IN command go to track 1, not 0, to avoid potential

timeout wait on track 0.

- Shorten motor afterrun time so drive stops almost as soon

as key is pressed.

- Allow TPI 48 and TYPE HC to get 360 RPM speed test.

01/28/90

- Document update only.  Updated supplier addresses and phone

numbers.

OVERVIEW

The ALIGN program allows you to move a diskette drive head to

absolute track locations and operate the drive while checks and

adjustments are made.  48, 96, and 135 tracks per inch (TPI)

diskette drives are supported.  These include 40 and 80 track 5

1/4 inch drives as well as 80 track 3 1/2 inch drives.  The PC-AT

High Capacity drive and Personal System/2 High Density drives are

also supported.

You may also be interested in a companion program called DRIVECHK

which is a diskette drive diagnostic program that uses the

Xidex/Dysan Digital Diagnostic Disk (DDD).

ALIGN is one of several tools needed to adjust and service

diskette drives.  The ALIGN program does not attempt to teach you

how to do an alignment, but it isn't difficult if you have what

you need.

In addition to this program, you should have:

The drive manufacturer's alignment and service procedures.

An oscilloscope (dual trace with triggered sweep).

A XIDEX/DYSAN double-sided Analog Alignment Diskette (AAD) for

the drive to be serviced (not required for SPEED test):

Xidex-      Xidex-

Size   TPI Tracks Dysan Model Dysan P/N Current Price Each

------ --- ------ ----------- --------- ------------------

5 1/4"  48   40     224/2A     800180        $50.00

5 1/4"  96   80     206-31     802166        $50.00

3 1/2" 135   80     350/2A     810234        $50.00

The above disks are double-sided.  224/2A is used for the

majority of PC drives which are 40 track.  206-31 can be used

for either the PC-AT's High Capacity drive or a user attached 80

track drive.  350/2A is for 3 1/2 inch drives.  Xidex/Dysan also

makes Analog Alignment Diskettes for other drives, but they

aren't directly supported by ALIGN.  You may still be able to

use ALIGN to step the drive.

Extension or external diskette drive signal and power cables are

very helpful.

See the notes at the end of this document for possible sources for

these items.

An Analog Alignment Diskette contains NO data, just special

waveforms and can't be copied.  (It's really copy protected!) You

can try the program using any diskette.  The ALIGN program does

not write on diskettes.

SYSTEM AND DISKETTE DRIVE REQUIREMENTS

ALIGN runs under IBM PC-DOS version 2.0 or later on a PC, PC-XT,

PC-AT, PC Convertible or Personal System/2 with at least 128K.  An

80 column display, either monochrome or color is required.

Since ALIGN uses standard BIOS interfaces for all I/O, it should

also work on most IBM compatibles.  It does not work properly on

the PCjr.

The diskette drives to be adjusted are connected to the PC in the

usual manner.  The ALIGN program works with standard IBM diskette

drives and should work with others that use the same electrical

interface.  ALIGN has no provisions for changing the BIOS diskette

parameter tables - step time, settle time, etc.

USING THE ALIGN PROGRAM

The ALIGN program consists of the single file ALIGN.EXE.  There is

no installation other than copying it to your program diskette.

Boot DOS and insert the ALIGN program disk in any working drive.

After the DOS prompt, enter ALIGN (preceded by a drive specifier,

if necessary) and press Enter.  Once ALIGN is loaded, the program

disk is no longer needed in the system.

Press any key to get past the initial screen to the main command

menu.

Commands are entered one at a time in ALIGN's command area to

control the diskette drive under test.  All commands may be

abbreviated and the first letter is sufficient for most commands.

(INFO, TYPE and TPI require more.)

If you are testing the "A" drive, ALIGN's default of DRIVE 0 is

correct.  If you are testing the "B" drive enter DRIVE 1 to select

it.  See the DRIVE command description for more information or if

you are testing an external diskette drive.

Chances are, you'll be able to use the program without reading any

more of this documentation.

Some commands require operands as listed on the menu.  Numeric

operands, such as track number or seconds, are decimal.  The first

track is track 0, not track 1.

If the diskette drive under test is running, pressing (almost) any

key causes it to stop.  This key may be the first letter of the

next command or any key such as the Enter key.  The drive stops by

itself after 60 seconds.

To increment or decrement the current track number, use the cursor

up and cursor down keys on the numeric keypad.  If the drive is

not running, it will start.

If the drive is stopped and you want to continue the previous

motion command, just press the Enter key.  This enters a null

command which is interpreted just like a GO command.

Commonly used commands may also be entered using the program

function keys as shown on the command menu.

When finished, enter Q or QUIT to return to DOS.

STATUS DISPLAY AND "TIME OUT"

When the drive is running, ALIGN is continually trying to read the

diskette using BIOS diskette INT 13 services.  The status response

from the BIOS is displayed in the lower right hand corner of the

display when the drive is running.  The status display includes

the status in English, followed by the hexadecimal status value

returned by the BIOS.

Usually the status display can be ignored completely.  Indeed,

earlier versions of ALIGN did not display status.  "Adr mark not

fnd" is the normal response when reading an Analog Alignment Disk.

"Zero status" is the normal response when reading a normal DOS

formatted diskette.

You may find the status useful in unusual situations.  "Time out"

is indicated when there is no diskette in the drive, it is not

latched or as explained below.

"Time out" also occurs on some combinations of particular Analog

Alignment Diskettes, drives and tracks.  This appears to be caused

by the diskette controller becoming confused by the Analog

Alignment Diskette's special waveforms.  Whenever a "Time out"

error occurs, ALIGN does a diskette subsystem reset before

proceeding to avoid controller and PC hangs.  If you are not

reading track 0, this causes a recalibrate to track 0 after every

timeout.  This recalibration drive motion is not desired and can

interfere with reading the oscilloscope.  See the RESET command

description for an explanation of recalibration.

You may not experience the "Time out" problem at all.  If you do,

the problem and the recalibration seeks it causes can be avoided

with a temporary hardware change of disconnecting pin 30, Read

Data, on the 34 pin interface to the drive.  (This may not be the

correct pin number on newer CMOS interfaces as used on IBM PS/2

systems.)

The connector has even numbered pins, 1 to 33, on one side and odd

numbered pins, 2 to 34 on the other.  The odd numbered pins are

usually tied together and connected to ground on the drive.  Pin

30 is the third pin in from the end on the side with the even

numbered pins.  It is on the end furthest from the keying slot

which is between pins 4 and 6.  One way to disconnect it

temporarily during adjustments is to put a small piece of tape

over the pin's foil on the drive circuit card.

ALIGN COMMAND MENU

|-----------| |----------------------------------------------------------------|

|     |     | | Altseek trk trk     Alternate seek between two tracks          |

| IN  | OUT | | Cycle               Cycle tracks 3,2,1,0,3,2,1,0,3,2,1...      |

| (1) | (2) | | Drive   <0,1,2,3>   Selects a drive number  (default 0)        |

|-----|-----| | Go or (null)        Start drive motion                         |

|     |     | | Head    0..1        Set head                                   |

| T 0 | H 1 | | In                  Move to track 0, then to cat's eye.        |

| (3) | (4) | | INFo                Display alignment disk track information   |

|-----|-----| | Max     39..255     Set maximum track allowed.                 |

|     |     | | Out                 Move toward center hub, then to cat's eye. |

| T 1 | H 0 | | Pause   0..180      Set seconds to pause at second ALTSEEK trk |

| (5) | (6) | | Quit                Quit and return to DOS                     |

|-----|-----| | Reset               Reset diskette system                      |

|     |     | | Seconds 3..999      Set length of time to run drive            |

|T 34 |CYCLE| | Speed               Display spindle speed. (Use DOS diskette)  |

| (7) | (8) | | TPi     <48,96,135> Specifies track density of diskette        |

|-----|-----| | Track   0..max      Set track and run drive                    |

|STOP/|     | | TYpe    <REG,HC,X2> 'regular', High Capacity or double-step    |

| GO  |RESET| |----------------------------------------------------------------|

| (9) |(10) |       DRIVE  TPI  TYPE  HEAD  TRACK  MAX  SECONDS  PAUSE

|-----------|         0     48   REG    0     16    39     60       0

Function Keys

-------------  Enter command:

up= Incr. Trk

dn= Decr. Trk

(up and dn are the cursor keys)

DETAILED COMMAND DESCRIPTIONS (in alphabetical order)

ALTSEEK trk trk

The ALTSEEK (alternate seek) command causes the drive to start

and do a head seek back and forth between the two specified

tracks.

Since ALIGN provides the CYCLE command to test and adjust the

track 0 switch, there really isn't any particular reason for

using the ALTSEEK command.  It might be useful if you have some

special tests or just enjoy watching the head move.

Normally, the head moves back and forth as fast as ALIGN, the

system BIOS and the disk drive hardware can go.  The PAUSE

command can be used before the ALTSEEK command to cause the head

to pause at the second specified track.  If you want the pause

to be at the first specified track, reverse the two tracks in

the ALTSEEK command line.

Examples:

ALTSEEK 5 15

A 0 1

A 26 16

CYCLE

The CYCLE command starts the drive and cycles the head through

tracks 0 through 3 in the following order 3,2,1,0,3,2,1,0,3,2...

and so on for checking the track 0 switch.  The drive changes

tracks every few seconds so you have time to see if the track 0

switch has switched.  Between each track (0,1,2 or 3) the CYCLE

command moves the head to track 16 so there is a seek similar to

that when a drive is first used after power-on.

Believe it or not, on Tandon TM-100 series drives, the track 0

switch should switch between tracks 1 and 2.  (It's NOT supposed

to switch at track 0!)  This is a position in the middle of the

first four tracks.  The actual track 0 signal on the drive

interface is a combination of the track "0" switch and other

indications in the drive electronics.

Switch here

|

V

0     1     2     3

While the CYCLE command is active, the PC's speaker sounds rapid

beeps equal to the current track number, 1 to 3, each time it

changes tracks.  Track 0 is a single longer beep.  These beeps

allow you to know what track the drive is on without trying to

watch a meter or test light and the PC's display at the same

time.  A higher pitch sounds in addition to the track beeps when

the switch should switch.  After a little practice, the sound

eliminates the need to watch the screen.  As you watch your

meter or light, don't be confused by the very brief switch

changes when the head moves to track 16 and then back toward

track 0.

While it's always nice to set adjustments "right on," the track

0 switch adjustment isn't critical.  The drive operates fine

even when it switches a whole track off from the ideal point

between tracks 1 and 2.

DRIVE  <0,1,2,3>

The DRIVE command specifies which physical drive is under test

using a drive address number of 0, 1, 2 or 3.  The default is

drive 0 which is the first internal diskette drive.  The DRIVE

command does not cause the drive to start.  Examples:

DRIVE 0         (Set drive 0)

D 2             (Set drive 2)

The drive number designates a physical drive address and not a

DOS drive letter or a drive letter associated with a special

device driver such as DOS 3.2's DRIVER.SYS.  (DRIVER.SYS may

be installed for any diskette drive, but it is not used or

needed by the ALIGN program.)

Drive address numbers are:

0  First internal diskette drive.  Identified by DOS as

"A" (or "A" and "B" when only one internal diskette

drive is installed.)

1  Second internal diskette drive.  Identified by DOS as "B".

2  First external diskette drive.

3  Second external diskette drive.

The drive letter that DOS assigns to external drives varies and

depends on the number of fixed disks, system board switch

settings and device drivers.  ALIGN doesn't care what letter DOS

uses.

GO

If the drive is stopped, the GO command starts the drive at

whatever track it happens to be on.  A null entry is the same,

so you can simply press the Enter key with nothing on the

command line to start the drive.

HEAD  0..1

The HEAD command selects the lower or upper head (diskette side)

and starts the drive.  HEAD 0 is the lower head and the only

head on a single-sided drive.  HEAD 1 specifies the upper head

on a double-sided drive.

It may seem strange that the HEAD command starts the drive.

This allows rapid switching to compare heads by alternating the

two HEAD command program function keys.

Examples:

HEAD 0          (Set head 0, the lower head)

HEAD 1          (Set head 1, the upper head)

H 1             (Set head 1, the upper head)

IN

OUT

The IN command moves the head first to track 1 and then "in"

towards the center.  It stops on the radial alignment (cat's

eye) track.

The OUT command is similar, but moves the head first toward the

center spindle and then immediately moves it "out" away from the

center.  It stops on the radial alignment track.

IN and OUT each have the same effect as entering two TRACK

commands.

Both commands end up on the radial alignment track with the

drive running.  The ending track depends on the Analog Alignment

Diskette specified with the TPI command:

TPI   Radial alignment track

---   ----------------------

48           16

96           32

135           40

After making a head adjustment it's a good idea to re-seek to

the radial alignment track and check the cat's eye pattern

again.  Both the IN and OUT commands cause a re-seek to this

track.  The two commands also allow checking the accuracy of

seeks (and the resulting head alignment) in both directions.  If

there is a difference in the cat's eye patterns in the two

directions, you may want to re-adjust the alignment to split the

difference.  A difference indicates play in the mechanism or an

imprecise head stepper motor.

INFO

The INFO command causes another screen to be displayed with a

brief description of each track on the Xidex/Dysan Alignment

Disk.  It's just for reference.

The information displayed depends on the Analog Alignment

Diskette specified with the TPI command.

Pressing any key returns you to the ALIGN command menu screen.

MAX 39..255

The MAX command sets the maximum track allowed with the TRACK

command.

Normally, the TRACK and ALTSEEK commands accept track numbers

from 0 through 39 or 79 depending on the current value specified

with the TPI command.  The MAX command allows you to change the

high track limit all the way up to 255.  The MAX command does

not cause the drive to start.

There really isn't much occasion to use the MAX command.  Maybe

you're looking for the track 39 signal on the XIDEX/DYSAN

alignment disk.  If it doesn't appear on what the system thinks

is track 39, maybe it will show up at what the system thinks is

track 40, or 41 or....

Actually, there is a limit to how far the head can go past track

39.  You may want to use the MAX command to allow you to

investigate just how much further the head will go.  (A copy

protection scheme might use one track past the "end" of the

disk.)

However, before you go charging off into the stratosphere of

track addressing, realize that when the head hits the upper

limit, it does so with the full force of the stepper motor.

This alone could knock a drive out of alignment or possibly

cause permanent damage.  Use the MAX command at your own risk.

OUT

See IN command description.

PAUSE  0..180

The PAUSE command can be used before the ALTSEEK command to

cause the head to pause at the second track specified on the

ALTSEEK command line.  The PAUSE operand specifies the number of

seconds to wait.  The PAUSE command does not cause the drive to

start.

The pause is approximate and may vary by up to about a second

from that specified.  Note too that the SECONDS command value

should be larger than the PAUSE value for PAUSE to be useful.

Otherwise the drive may stop during the PAUSE period.

See the ALTSEEK command description.

Examples:

P 5                        (5 seconds)

PAUSE 60                   (1 minute)

QUIT

The QUIT command ends the ALIGN program and returns to DOS.

RESET

The RESET command causes a reset of the diskette system (via

BIOS INT 13 with AH=0).  RESET does not cause the drive to

start.

After entering RESET, the next time any drive is accessed it

is then be "recalibrated."  This means that the diskette

controller makes no assumptions about which track the head is

on, moves the head until track 0 is located as indicated by the

drive, and then proceeds with the requested operation.  Once

this recalibration process is complete, the diskette controller

keeps it's own head position register and merely pulses the head

forward or back, one step at a time to get to a desired track.

If one pulse is missed or the head is moved manually with

power-on, then the controller's track register won't agree with

the physical position of the head.

"Recalibration" is not an adjustment, but rather a normal

operation done by the diskette controller so it can get in sync

with the physical head position.  Every drive is "calibrated" by

the controller the first time you use it after power-on or boot.

The RESET command allows you to recalibrate during the drive

adjustment procedure.  For instance, assume the drive is on

track 16 (or what you think is track 16).  If you issue a RESET

command and then simply start the drive motion (null entry), you

will hear the drive move to track 0 and then back out to track

16 as the drive is recalibrated.

The RESET command is useful when servicing drives and you think

the drive is not seeking to the correct track.  This could

happen while adjusting the track 0 switch.  Also, the drive

won't seek to the correct track anytime the head has been moved

against the stepper's electrical hold.  This could occur when

the head assembly hits it's travel limit at either end.  This

isn't normal operation, but when the adjustments are wrong it

can (and does) happen.

If you don't find an alignment disk signal where you think it

should be, try the RESET command.  It is also a good idea to use

the RESET command just before the final checks on radial head

alignment to be sure you're not exactly 1 or 2 tracks off.

ALIGN does an automatic RESET between every CYCLE command seek

to make track 0 switch adjustment easier.

Without the RESET command, it would otherwise be necessary to

re-boot or turn the machine off-and-on to force a recalibrate

operation.

SECONDS  3..999

Normally, each time you start the drive using ALIGN, the drive

runs for 60 seconds if not stopped by pressing any key.  This is

usually enough time to make a check or adjustment and prevents

the drive from running "forever."

The SECONDS command allows you to change this 60 second limit

anywhere from 1 to 998 seconds.  A value of 999 means there is

no limit and the drive will run continuously until stopped by

pressing a key.

The SECONDS command does not cause the drive to start.

Examples:

SECONDS 120                (2 minutes)

S 10

S 999                      (no limit)

SPEED command

The SPEED command measures diskette drive spindle speed which is

the speed the diskette rotates.  It also gives and indication of

the Instantaneous Speed Variation or ISV.  ISV is a short term

speed variation which doesn't show in the average speed.

Possible causes of high ISV include incorrect belt adjustment,

binding, motor cogging or poor brush contact.

SPEED requires a regular DOS formatted diskette and does not work

with the Xidex/Dysan AAD, Analog Alignment Diskette used with

most other ALIGN commands.  The SPEED command does not write on

the diskette (nor does any ALIGN program command.)

The SPEED command has no operands, but the drive should be

properly specified with the DRIVE, TPI and TYPE commands first,

if necessary.  SPEED also uses current values as displayed for

the track and head, either by default or from prior ALIGN

commands.  The track and head should not affect the measured

speed, although SPEED must find a valid sector on the indicated

track and head.  ALIGN determines the speed by measuring the time

between successive reads of sector 1.

To use the SPEED command, insert a DOS formatted diskette and

enter SPEED.  Let it run for a few seconds, stopping it with any

key.  It you don't stop it, it runs for 60 seconds or whatever

time has been set with the SECONDS command.

Nearly all diskette drives have a nominal speed of 300 RPM.  The

only IBM exception now is the PC-AT's High Capacity drive which

has a nominal speed of 360 RPM.  The PC-AT High Capacity drive

runs at 360 RPM regardless of the type of media it is using.

Spindle speed accuracy is not as critical as the displayed

numbers might suggest.  Most drives allow a speed tolerance of 1

or 1.5 percent which is about plus or minus 3 or 4 RPM.  The

actual speed also varies a bit due to the drag of different

diskettes.  Some people over-emphasize spindle speed because

it's about the only adjustment they can make without extra

equipment.  The SPEED command dumps out all those numbers more

for entertainment than necessity.

The measured speed is always within the displayed range of

approximately 280 to 320 RPM for 300 RPM drives and 330 to 390

RPM for 360 RPM drives.  This is true even if the actual speed

is outside that range and is the result of the measurement

method used by the program.  In practice, a drive whose speed is

outside the range probably won't be working anyhow and may also

show an erratic speed.  If there's any question, a look at the

strobe on most drives will tell you very quickly if the drive is

anywhere near the correct speed.

There is another way to measure the speed if you think the

actual speed is outside the displayed range and the drive reads

without error.  Check to see that SECONDS had the default value

of 60.  If not, enter SECONDS 60.  Then enter SPEED and let the

drive run for the full minute.  The displayed "Measurement

Number" at the end of the minute is the speed in RPM.  This is

only accurate if there are no read errors.

SPEED example:

Current  Average  Maximum  Minimum  Percent  Measurement

RPM      RPM      RPM      RPM      ISV      Number

298.40   298.40   298.40   298.40    0.00       1

298.37   298.38   298.40   298.37    0.01       2

298.38   298.38   298.40   298.37    0.01       3

298.40   298.39   298.40   298.37    0.01       4

298.37   298.39   298.40   298.37    0.01       5

298.38   298.38   298.40   298.37    0.01       6

298.40   298.39   298.40   298.37    0.01       7

298.37   298.38   298.40   298.37    0.01       8

298.39   298.39   298.40   298.37    0.01       9

298.41   298.39   298.41   298.37    0.01      10

298.39   298.39   298.44   298.34    0.03      38

280  285  290  295  300  305  310  315  320

���������� |....|....|....|....|....|....|....|....| ����������

^

The easiest part of the display to read is the speed meter on the

bottom.  It indicates the current RPM and changes whenever the

actual drive speed changes.  (It is not the average drive speed.

However, if it doesn't move much, then the current and average

are the same.)  The arrow on the meter moves in one RPM

increments, displaying the "Current RPM" rounded to the nearest

whole RPM value, which is close enough.

You can probably ignore the rest of the SPEED display.  The first

10 lines show individual statistics for the first 10 measured

revolutions.  (SPEED lets the drive come up to operating speed

and stabilize before starting measurement.)  "Current RPM" is for

just one revolution.  All the other statistics are cumulative.

The last line of statistics, separated with a blank line, is the

data for the last revolution measured.  It is updated every

revolution until the SPEED command is stopped or times out.  As

all the values except "Current RPM" are cumulative, they tend to

stabilize as the test progresses.

"Current RPM" is the measured spindle speed for one revolution.

"Average RPM" is the average of the spindle speed RPM

measurements so far and is the best overall indication of spindle

speed.

"Maximum RPM" and "Minimum RPM" are the highest and lowest

spindle speeds encountered during the test.

"Percent ISV" is an indication of Instantaneous Speed Variation.

It is calculated by taking the difference between the maximum and

minimum speeds, dividing by the average speed and multiplying by

100 to get a percentage value.  Drive factors are usually the

cause of ISV problems.  However, keep in mind that the diskette

and its jacket are part of the system which contributes to ISV,

possibly by binding, buckling or other irregularities.

The "Measurement Number" is the revolution number since the drive

came up to speed and measurements started.

TPI     <48,96,135>

The TPI command specifies the track density of the Analog

Alignment Disk (AAD) you are using.  48 is the default.  You may

prefer to think in terms of the number of tracks instead.

TPI   Size    Tracks

---   ------   ------

48   5 1/4"     40

96   5 1/4"     80

135   3 1/2"     80

The TPI command does not cause the drive to start.

Normally, you set TPI to match the physical drive to be tested.

You can test the High Capacity drive on the PC-AT with either a

96 TPI AAD or a 48 TPI AAD.  Testing it at 80 tracks with the

model 206-31 96 TPI AAD certainly gives the best results.  If

you only have the model 224/2A AAD for 48 TPI drives, you can

still get useful displays by using it and specifying TPI 48 and

TYPE REG.  The PC-AT then double-steps the High Capacity drive

as it always does with 40 track diskettes.

As with a High Capacity drive, a "regular" 80 track drive can be

tested with either a 96 TPI AAD or 48 TPI AAD.  To use a 48 TPI

AAD on an 80 track drive, specify TPI 48 and TYPE X2.  ALIGN

then double-steps, multiplying all track numbers by 2 before

all seeks.

Don't do the reverse and specify TPI 96 or 135 on a 40 track

drive.  ALIGN tries to use tracks that aren't there and bangs

the head assembly against its travel limit.

Examples:

TPI 48               (Set 48 tracks per inch AAD)

TP 96                (Set 96 tracks per inch AAD)

TRACK  0..max

The TRACK command moves the head to a specified track and starts

drive motion.  The drive continues to run until a key is pressed

or for 60 seconds.  (The 60 second run time may be changed using

the SECONDS command.)

The track number that may be specified is in the range 0 through

39 for 48 TPI drives and 0 through 79 for 96 TPI or 135 TPI

drives.  The upper track limit may be extended using the MAX

command.  Examples:

TRACK 0      (Move to track 0 and run drive)

T 34         (Move to track 34 and run drive)

T 16         (Move to track 16 and run drive)

The track number can be incremented or decremented by one track

using the cursor up and cursor down keys on the numeric keypad.

This is easier than specifying the tracks one at a time with the

TRACK command.  Use the TRACK command to move to the first

desired track and use the cursor keys from that point.  Or, hold

the cursor key until the desired track is reached.

TYPE     <REG,HC,X2>

The TYPE command specifies a "regular" diskette drive, the

PC-AT's High Capacity drive or a "regular" drive that is to be

double-stepped.  The Personal System/2 High Density drive is

also considered a "regular" drive.  TYPE REG is the default.

REG           Specifies a "regular" drive      (default)

HC            Specifies a PC-AT High Capacity drive

X2            Have ALIGN double-step a "regular" drive

The TYPE command does not cause the drive to start.

TYPE HC is only valid on the PC-AT.  (It attempts to run the

drive at a 300 KBS data transfer rate, not the 500 KBS rate used

with the special high capacity media.)

While it's best to test the High Capacity drive as TYPE HC and

TPI 96, if you only have a 48 TPI AAD diskette, you can use TYPE

REG and TPI 48.  Similarly, an 80 track "regular" drive is best

tested as TYPE REG and TPI 96, but you can use a 48 TPI AAD

diskette by specifying TYPE X2 and TPI 48.  See the TPI command

description.

TYPE X2 is really only useful with TPI 48 and an 80 track

regular drive.  However, ALIGN does no checking and gladly

attempts to double-step with TPI 96 or TPI 135 and bangs the

head carriage at its travel limit.  Be sure you understand X2

and what it means before you try it.

Examples:

TYPE REG           (Specify "regular" drive)

TY HC              (Specify High Capacity drive)

TYPE X2            (Double-step a "regular" drive)

INSTANTANEOUS SPEED VARIATION TESTING

ALIGN's SPEED command gives an indication of Instantaneous Speed

Variation, ISV.  Since the SPEED command only checks the speed

once per revolution, it is not a true ISV test.

To test ISV with an oscilloscope, the Tandon manual says to first

"write a 2F (all ones) pattern on any track."  The ALIGN program

won't write this or anything else on your diskettes.  A totally

satisfactory substitute is to use the DOS FORMAT command to format

a work diskette in the drive under test.  This puts a pattern on

every track.  Admittedly, it's not "all ones" but you'll hardly

know the difference as you observe the pulse jitter on the

oscilloscope.  Also, while Tandon says to use "any" track, the

jitter is much worse on the inside (high) tracks.  Use a middle

track, such as the radial alignment track, to check the

specification.  Get to know what normal jitter looks like on track

0 and in towards the hub too (39 or 79).

MESSAGES

(operand name) operand is too large.  Range=(min)...(max)

Explanation:  The value you specified as a parameter is too

large for the command you entered.

Action:  Choose a value within the specified range and retry the

command.

(operand name) operand is too small.  Range=(min)...(max)

Explanation:  The value you specified as a parameter is too

small for the command you entered.

Action:  Choose a value within the specified range and retry the

command.

(operand name) operand must be a numeric integer

Explanation:  The value you specified as a parameter is not a

valid numeric constant.

Action:  Check the characters you entered and retry the command

using numeric characters only for the operand value.  Decimal

points, plus and minus signs are not permitted.

Can't process.  HC is not valid with 135 TPI

Explanation: A High Capacity drive has been specified with the

TYPE command, but the current TPI (tracks per inch) is incorrect

for a High Capacity drive.

Action:  Either enter TPI 96 or 48 or change the drive type with

the TYPE command.

Diskette system has been reset

Explanation:  This is an informational message displayed every

time the RESET command is executed.  It indicates that the next

time the drive is started, the diskette controller will

re-calibrate the drive by seeking to track 0 before seeking to

the current track.

Action: None.

Drive operand is not 0, 1, 2 or 3

Explanation:  The specified diskette drive number must be 0, 1,

2, or 3.

Action:  Retry the command using the correct drive number.

Error reading diskette

Explanation:  The program encountered a disk error while reading

the diskette for spindle speed testing.  Testing and measurement

continues, although some of the calculated results may lose

accuracy.  If you just get a single error, you can restart the

speed test.  If you get this error continuously, then you are

either using the wrong diskette or may have a solid diskette

drive failure.

Action:  Make sure you are using a regular DOS formatted

diskette and that the DRIVE, TYPE and TPI values are properly

set.  The ALIGN speed test only works with ordinary DOS

formatted diskettes.

Missing operand(s)

Explanation:  Not enough operands were entered on the command

line for the specified command.

Action:  Re-enter the command with the proper number of operands

as indicated in the command description.

Running continuously

Explanation:  The selected drive is running and will run until

stopped.  This is the result of specifying SECONDS 999

previously.

Action: Press any key to stop the drive when you want it to

stop.

Running:  nnn seconds left

Explanation:  The selected drive is running and will run for the

specified number of seconds.

Action:  Either let the drive run until the timer is up or press

any key to stop the drive when you want it to stop.  If the

drive isn't running long enough for you, increase the time it

runs with the SECONDS command.

Too many operands

Explanation:  More operands were entered on the command line

than the specified command can accept.

Action:  Re-enter the command with the proper number of operands

as indicated in the command description.  Many commands do not

have any operands.

TPI operand is not 48, 96 or 135

Explanation:  The specified TPI (tracks per inch) must be

48, 96 or 135.

Action:  Retry the command using the correct track density.

TYPE operand is not REG, HC or X2

Explanation:  The specified TYPE value must be REG, HC or X2.

See the TYPE command description for an explanation.

Action:  Retry the command using the correct TYPE value.

Unknown command

Explanation:  The specified command is not recognized by ALIGN.

Action:  Check the command spelling and re-enter it.

Warning: TYPE HC is intended for PC-AT

Explanation:  You specified a High Capacity drive and ALIGN is

not running on a PC-AT which is the only IBM Personal Computer

that supports a High Capacity drive.

Action:  If you are not testing a High Capacity drive, then

re-enter the TYPE command specifying a type that is appropriate

for your Personal Computer model and diskette drive type.  If

you are testing a High Capacity drive on a machine other than a

PC-AT, then you may continue.  However, High Capacity support in

ALIGN has only been tested on a PC-AT.  Operation with other

machines and non-IBM hardware may not be successful.

TANDON TM-100 SERIES REPAIR TIPS

LOOSE SCREWS AND NUTS - HEAD MISALIGNMENT

In my experience, the most frequent cause of head misalignment and

drive failure on Tandon TM-100 series drives is a loose Allen head

screw on the stepper motor shaft.  This allows the motor shaft to

slip where it connects to the split band positioner pulley.

Tighten this Allen head screw before doing an alignment.  It's not

unusual to tighten it one-half turn holding the short end of an

Allen wrench with just your fingers.  Don't over-do it, however,

as you could strip the threads.

The stepper motor shaft is smooth and has no flats or keys.  As

long as you are doing an alignment and the Allen screw is tight,

the exact pulley position on the shaft usually doesn't matter.

Sometimes it can be out of the head alignment cam screw adjuster

range.  If this happens, loosen the Allen screw, rotate the pulley

a bit, tighten it, and try again.

While you're at it, check the two nuts that hold the track stepper

motor to the casting.  Also, remove the clear plastic cover on the

bottom of the head assembly and carefully tighten the little

Phillips screw that holds the split band on the pulley.

If you're not going to do an alignment, don't tighten these screws

since just tightening them might change the alignment some.

TRACK 00 SWITCH PROBLEMS - BOOT FAILURE

The track 0 microswitch frequently becomes intermittent or fails.

You can easily check this by powering-off the system and removing

plug 11 on the back of the drive.  Push a few pieces of bare wire

into the connector and check the switch contacts with an analog

ohm meter.  The switch is single pole, double throw (SPDT).

|     |  |

|     |  |  <- wires to switch

------------

|          |         COM - common center contact

| Plug     |         NC  - normally closed

|          |         NO  - normally open

|          |

Pin # ->  |4  3  2  1|

------------

COM     NC NO  <- Switch contacts

Operate the switch manually.  It is a little hard to get to and

you may want to remove the logic board first.  Check to see that

contact closure is positive, occurs when the switch clicks,

and without any wavering around.

If the switch is not operating properly, the problem is usually

just a very small amount of corrosion or oxidation on the switch

contacts.  Rather than replacing the switch, the easy and

effective repair is to pass some reasonable current through the

switch and operate the switch a few times.  Be sure that plug 11

is disconnected from the logic board and that you make connections

to just the plug.  Use an external CURRENT LIMITED low voltage

supply to burn off the little bit of oxidation.  I don't know how

much current to recommend.  I use just enough to cause a little

spark when the power supply leads are touched, probably 0.5 or

1.0 amp.  Be sure to use a current limited supply or put an

appropriate current limiting resistor in series.  If you don't

know how to do this, then find someone who does rather than risk

any harm or damage.

POSSIBLE SOURCES FOR SUPPLIES

For the type of drive used in the original PC, the alignment and

service procedures are in the Tandon "OEM Operating and Service

Manual TM-100-1 and -2 Disk Drives, 48 Tracks per Inch."

Priority One Electronics has sold this as part number

07TND179022001, for $36.00.

Priority One Electronics

9161 Deering Ave.

Chatsworth, CA 91311

Telephone (800) 423-5922

(818) 709-5111

It is also Tandon part number 179022-001, listed at $25.00 on a

December 1982 Tandon price list.  I don't know if they sell it

directly to end users.

Tandon Corporation

20320 Prairie Street

Chatsworth, CA 91311

Telephone (213) 993-6644

Analog Alignment Disks (AAD's) are not available directly from

Xidex/Dysan Corporation in small quantities.  Their regional sales

offices or main office can give you the name of a dealer who

stocks them or can get them for you.  Any Xidex/Dysan dealer can

order them.

Xidex/Dysan Corporation, CE Division

5100 Patrick Henry Drive

Santa Clara, CA 95050

Telephone (800) 233-5099 Xidex

(408) 988-3472

(800) 422-3455 Xidex CE Division

One mail order dealer with a range of Xidex/Dysan products is

Supply Line, Inc.

1420 N. Claremont Blvd. Suite 205B

Claremont, California 91711

Telephone (800) 422-3392  outside of California

(800) 648-7376  inside California

(714) 621-1984

Extension or external diskette drive signal and power cables may

be difficult to locate or you may have to make your own.