DRIVECHK

Version 4.4 for IBM Personal Computers

Program 05/30/89

Minor document update 01/28/90

(c) Copyright John L. Dickinson 1983, 1987, 1989, 1990.  All rights reserved.

DISCLAIMER

This program is provided "as-is" without warranty of any kind,

either expressed or implied, including, but not limited to the

implied warranties of merchantability and fitness for a particular

purpose.  The entire risk as to the quality and performance of the

program is with you.

In no event will I be liable to you for any damages, including any

lost profits, lost savings or other incidental or consequential

damages arising out of the use or inability to use this program

even if I have been advised of the possibility of such damages, or

for any claim by any other party.

COPYING PROVISIONS

This program and documentation may be freely copied and shared

provided it is not modified in any manner and this notice is not

bypassed or removed.  Please register by sending your name,

address and any comments to the author.  There is no charge.

John L. Dickinson

2470 N. Tierra Verde Pl.

Tucson, Arizona 85749-9315

CHANGE HISTORY

11/26/87 Version 4.3

- Initial shareware release.

05/30/89 Version 4.4

- Reset diskette subsystem after drive timeout.  This prevents

hangs when no diskette is inserted.

- Add NOZAP and ZAP commands.

- Minor updates to STATUS messages.

01/28/90

- Document update only.  Updated supplier addresses and phone

numbers.

OVERVIEW

DRIVECHK is a diskette drive diagnostic program that graphically

displays alignment and other drive characteristics of 48, 96 and

135 tracks per inch (TPI) diskette drives without the need for an

oscilloscope.  These include 40 and 80 track 5 1/4 inch drives as

well as 80 track 3 1/2 inch drives.  The PC-AT High Capacity drive

and Personal System/2 High Density drive can also be tested.  A

drive can be checked in just a few minutes without removing the

PC's cover.

You may also be interested in a companion program called ALIGN

which uses the Xidex/Dysan Analog Alignment Disk (AAD) and an

oscilloscope.  It is more suited to drive repair than diagnosis.

DRIVECHK can be used to check a drive's operation periodically,

prove a suspected defective drive is bad, or as a final check

after repairs or adjustments.  DRIVECHK is not designed for making

drive adjustments, although it can be used to set the radial head

alignment which is only one of several diskette drive adjustments.

DRIVECHK works with a XIDEX/DYSAN Digital Diagnostic Disk (DDD).

The Digital Diagnostic Disk is specially formatted and precisely

recorded with varying degrees of intentionally misaligned

information.  The Xidex/Dysan DDD disk has no programs on it and

cannot be copied since it is made on special equipment that

records "between the grooves."  It is not a PC-DOS formatted disk

and its tracks have 16 sectors (except the 96 TPI High Capacity

DDD for the PC-AT which has 26 sectors).  You select a different

DDD depending on the size and track density of the drive you want

to test.  You can also select a High Capacity diskette to test the

PC-AT high capacity drive.

Xidex/       Xidex/   Price

Size     TPI   Tracks  Dysan Model  Dysan P/N  Each

------   ---   ------  -----------  ---------  ------

5 1/4"    48     40      508-400     802045    $40.00

5 1/4"    96     80      506-400     802010    $40.00  (normal media)

5 1/4"    96     80      516-400     810253    $40.00  (high capacity)

3 1/2"   135     80      305-400     810244    $40.00  (normal media)

3 1/2"   135     80 -Use 305-400 normal media version- (high density)

All of the above disks are double density and double-sided.

508-400 is used for the majority of PC drives which are 40 track.

506-400 can be used either to test the PC-AT's High Capacity drive

or a user attached 80 track drive.  It tests the PC-AT High

Capacity at a 300 KBS data rate.  516-400 tests the PC-AT High

Capacity drive in it's high capacity mode at a 500 KBS data rate.

305-400 is for normal media 3 1/2 inch drives and is also used to

test the Personal System/2 High Density (1.44M) drive at its lower

capacity data rate.

Xidex/Dysan also makes DDD's for other sizes and densities, but

they aren't supported by DRIVECHK.

Using DRIVECHK and a DDD disk you can observe:

* Radial head alignment of each head

* Operating Margin  (How much error can the drive

tolerate?)

* Head positioner hysteresis (backlash)

* Seek variation or head positioner linearity  (How does the

radial head alignment vary at different tracks?)

* Margin variation: Relative performance at different tracks

* Head azimuth

* Diskette eccentricity (clamping error or spindle wobble)

* Drive spindle speed

* Instantaneous speed variation

The program does NOT display:

* Index timing

* Track 0 stop adjustment

* Track 0 switch adjustment

I don't think there is any program that can test these last items

with the PC's covers in place.  Index timing is not a usual cause

of drive failure unless the drive has been disassembled (such as

removing the cone lever assembly to fix a door latch!)

DRIVECHK runs under IBM PC-DOS version 2.0 or later on a PC,

PC-XT, PC-AT, PCjr, PC Convertible, Personal System/2 or

compatibles with at least 128K.  An 80 column display, either

monochrome or color is required.

DRIVECHK works with standard IBM drives any many others.  However,

DRIVECHK has no provisions for changing the BIOS diskette

parameter tables - step time, settle time, etc.

USING THE DRIVECHK PROGRAM

The DRIVECHK program consists of the single file DRIVECHK.EXE.

There is no installation other than copying it to your program

diskette.

It is possible that other memory resident DOS programs could

interfere with DRIVECHK or cause DRIVECHK to give incorrect

results.  If you suspect improper operation, boot DOS from your

master DOS diskette without loading other programs or options

before running DRIVECHK.  This is not a likely cause of problems,

but is something to keep in mind.

The DRIVECHK program reads the Digital Diagnostic Disk and

produces three graphic displays on a monochrome (or color) display

using character graphics.  Personal System/2 users with monochrome

displays may need to adjust display controls to make the display

brighter than normal to see everything.

There are no pass-fail answers and the results are subject to

interpretation.  While every drive appears different, defective

drives are usually rather obvious.

In the discussions that follow, all three displays use the same

notation to indicate success or failure to read various data

sectors on the DDD.

*  An asterisk indicates that the sector was successfully

read after seeking to the track from both directions.

+  A plus sign indicates that the sector was successfully

read after seeking from one direction only.  (From track

0)

-  A minus sign indicates a successful sector read after

seeking from one direction only.  (Opposite of +, a

successful seek from the center out.)

blank  A blank space indicates an unsuccessful read operation

after a seek from either direction.

X  An X is like the "Free" space in Bingo.  It shows up in

the displays for a couple of sectors that DRIVECHK isn't

equipped to retrieve.  (Either they are not recorded on

the DDD disk or else DRIVECHK simply can't handle them.

I'm not sure which.)

Since the purpose of DRIVECHK and the DDD disk is to test the

drive to the limits of it's capability, many blanks (read

failures) are to be expected even on the best drives in perfect

adjustment.  An occasional + or - is normal too, but usually at

only at the limit of a drive's capability.  That is, you might

see a + or - at the end of a row of asterisks.

During the tests, the status returned from each read attempt is

displayed on the left of the screen.  You can generally ignore it.

Status messages here may appear to be "errors" when DRIVECHK tests

your drive beyond its capability.  This is normal and occurs when

a blank will displayed instead of an asterisk, + or -.

No printing capability in built into the program, so the PrtSc

(Print Screen) key is used to get hard copy.  Graphic characters

are used, making the IBM Graphics Printer or equivalent helpful,

although the results should be readable on any printer.

DRIVECHK COMMAND OVERVIEW

Commands are entered in DRIVECHK's command area to specify the

drive, choose a diagnostic display, reset the diskette subsystem

or quit the program.  Only enough letters to define the command

uniquely must be entered.  Any command that starts a diagnostic

test can be stopped by pressing any key during the test.

Command summary:

Drive specification -

DRIVE   <0,1,2,3>     Selects a drive number           (default 0)

TPI     48            Specifies  48 TPI DDD            (default)

96            Specifies  96 TPI DDD on regular media

96HC          Specifies  96 TPI DDD on high capacity media

135           Specifies 135 TPI DDD

TYPE    REG           Specifies a "regular" drive      (default)

HC            Specifies a PC-AT High Capacity drive

SIDES   <SS,DS>       Specifies single or double-sided (default DS)

LIMIT   END           Limits seeks to mfr's specified last track (dflt)

PLUS          Allows a seek one track past the "end"

Choosing a diagnostic display -

CENTERING             Display eccentricity (misclamp or wobble)

CHART                 Display alignment and several other factors

AZIMUTH               Display head azimuth (rotation)

SPEED                 Display spindle speed

ALIGN                 Repeating display for alignment

Other -

HELP or ?             Display this command summary

RESET                 Reset the diskette subsystem

NOZAP                 Rarely needed.  See command description.

ZAP                   Rarely needed.  See command description.

QUIT                  Exit DRIVECHK and return to DOS

TYPICAL OPERATION OVERVIEW

Boot DOS and insert the DRIVECHK program disk in any working

drive.

After the DOS prompt, enter DRIVECHK (preceded by a drive

specifier, if necessary) and press Enter.  Once DRIVECHK is

loaded, the program disk is no longer needed in the system.

Press any key to get past the initial screen to the main command

menu.

If you are testing the "A" drive, DRIVECHK's default of DRIVE 0 is

correct.  If you are testing the "B" drive enter DRIVE 1 to select

it.  See the DRIVE command description for more information or if

you are testing an external diskette drive.

If necessary, specify drive and Digital Diagnostic Disk

characteristics using the TPI, TYPE, SIDES and LIMIT commands as

needed.

Insert the Xidex/Dysan Digital Diagnostic Disk in the drive to be

tested.

The next three commands take a little time to do the analysis.

The results are displayed when the commands complete.  If hard

copy is needed, use the PrtSc key.

Enter the CENTERING command.  If the display indicates off

center operation, then reclamp the diskette.

Enter the CHART command.

If testing with a 48 TPI DDD or a PC-AT High Capacity drive

with the high capacity 515-400 DDD, enter the AZIMUTH command.

Enter the SPEED command.  After a few seconds, press any key

to stop the drive and display.

If desired, select another drive with the DRIVE command.

When finished, enter Q or QUIT to return to DOS.

DETAILED COMMAND DESCRIPTIONS (in alphabetical order)

ALIGN command

The ALIGN command can assist adjusting the radial head alignment.

DRIVECHK really isn't intended for doing any adjustments, but

ALIGN is provided to get radial track alignment done in a pinch.

The CHART command gives a much better overall picture of

alignment, but takes a minute or two, making adjustments tedious,

if not impossible.  ALIGN shows less information, but is much

faster and repeats until stopped by pressing any key.

ALIGN produces the same diagnostic display as the CHART command,

but only for one track.  This is track 16 for 48 TPI, track 32 for

96 TPI and track 32 for 135 TPI.  These tracks are the same ones

used with the "cat's eye" pattern on the analog alignment

diskettes for 48 and 96 TPI.  For 135 TPI, ALIGN uses track 32,

not track 40 as on the 135 TPI analog alignment disk.

The ALIGN command seeks to the track from both directions as do

CHART and the other diagnostic displays.  ALIGN takes about 10

seconds to refresh the display as you make adjustments.

There's more to a complete alignment job than just radial head

alignment.  Realize that making this one adjustment is no

substitute for a complete checkup with an oscilloscope and analog

alignment disk.

AZIMUTH command

AZIMUTH measures the amount of head rotation around a vertical

axis.  Azimuth can be measured when using 48 TPI or High Capacity

96 TPI DDDs.  Azimuth tracks are not recorded on all DDDs since

azimuth is much less critical on the narrower tracks of 96 and 135

TPI diskettes.

The DDD tracks for measuring azimuth are all too "easy" in my

opinion.  Therefore, small azimuth errors cannot be detected.

Fortunately, small azimuth errors are of little consequence.  An

Analog Alignment Disk and an oscilloscope provide a much more

critical azimuth test.

Azimuth in minutes

--------  ++++++++

43333222  22233334

29630741  14703692

|--------||--------|

|********||X*******|   Head 1 (Upper)

|--------||--------|

|********||X*******|   Head 0 (Lower)

|--------||--------|

If the display shows anything but all asterisks, then the drive

definitely has a problem.  Note, however, that inability to read

the azimuth data can also be caused by incorrect radial head

alignment.  When using the High Capacity 96 TPI DDD, the display

is wider and has more resolution due to the greater number of

sectors.

CENTERING command

The CENTERING command displays information on diskette

eccentricity.  This could be caused by clamping error or spindle

wobble.  To get a better idea of the effect being measured, think

how a record would rotate with its hole punched off center.  If

the display shows eccentricity, remove the DDD disk, inspect the

center hole, and reclamp it.

48 TPI example:           Head 0     Head 1

Sector->   Sector->

|--------| |--------|

27 +9.0 |**-*****| |***-****|

27 -9.0 |********| |********|

|--------| |--------|

Track 24 +8.0 |********| |********|

24 -8.0 |********| |********|

|--------| |--------|

21 +7.0 |********| |********|

21 -7.0 |********| |********|

|--------| |--------|

The CENTERING display shows three sets of data for each head.  The

top set is the hardest to read and the bottom set is the easiest

to read.  The actual offsets differ depending on the track density

(TPI) of the DDD you are using.  When using the High Capacity 96

TPI DDD, the display is wider and has more resolution due to the

greater number of sectors.

To help understand what's happening in this test, picture the head

traveling between the two rails of a circular railroad track.  If

it can "see" both the rails, then an asterisk is displayed for

each "rail."  If the head strays from a point halfway between the

"rails" then it may not be able to "see" one of the rails and a +,

- or blank is displayed.  The circular tracks are straightened out

for this display and 8 data points (13 for High Capacity 96 TPI

DDD) are shown for one revolution of each "rail."  If your

CENTERING display shows successful reads and read failures

alternating from one side of center to the other, then it

indicates bad disk centering or spindle wobble.

An occasional read failure such as the two "-" characters in the

example, are not necessarily an indication of a problem.  In the

example above, the read failures were on the most severe test, and

there the other "rail" was read perfectly.

Here's an example of what a centering problem might look like:

|--------|

|*-  -***|

|****+  +|

|--------|

The CENTERING command should be used first to check the DDD

diskette's clamping.  Incorrect clamping invalidates the results

of other displays.  If poor results are obtained repeatedly, then

continue with other tests.  Radial alignment errors may be

affecting the CENTERING results.

Hopefully, you will never clamp your DDD disk off-center since

such damage no doubt affects its accuracy.

CHART command

CHART displays the most interesting and useful of the three

displays which looks about like this for a fairly normal drive:

96 TPI example:

-------------------------------------------------------------------------------

Radial          Hyste-   Seek   Margin

Out     In        Track Align.  Margin  resis    Vari.   Vari.

|--------|--------|   ----- ------  ------  ------  ------  ------

|  ******|***     |     79   -1.5     9.5     0.0

|  ******|**+     |     71   -2.0     9.0     0.3

Head 1| *******|****    |     41   -1.5    10.5     0.0

Upper | *******|****    |     32   -1.5    10.5     0.0

|+*******|****    |      6   -1.5    10.5     0.3

|********|X***-   |      0   -2.0    11.0     0.3

|--------|--------|    Hd.1  -1.7    10.2     0.1     0.2     0.6

| *******|X****+  |      0   -1.0    11.0     0.3

| +******|*****-  |      6   -0.5    10.5     0.5

Head 0| +******|******  |     32   +0.0    11.0     0.3

Lower | *******|*****   |     41   -1.0    11.0     0.0

|  ******|****-   |     71   -1.0    10.0     0.3

|  ******|X****   |     79   -0.5    10.5     0.0

|--------|--------|    Hd.0  -0.7    10.7     0.2     0.3     0.3

Drive 0       Overall  -1.2    10.4     0.2     0.3     0.5

-------------------------------------------------------------------------------

This chart shows the results of reading "Progressive Offset"

tracks on the DDD disk.  Each horizontal line indicates the

results of reading one track.  Each head attempts to read 6

progressive offset tracks spread across the disk.  There are 16

sectors on each track (26 for High Capacity 96 TPI DDD).  Half are

displaced from the track center line in increasing amounts toward

the hub and are shown on the right half of the display.  The other

half are displaced in increasing amounts toward the outside of the

disk and are shown on the left half of the display.  The small

(easy) displacements are adjacent to the vertical centerline of

the display.  The larger (difficult) displacements are at the left

and right edges of the display.

The displacements depend on the DDD you are using.

TPI  Displacements in milli-inches                               Increment

---  ----------------------------------------------------------  ---------

48   6.0  7.0  8.0  9.0 10.0 11.0 12.0 13.0                       1.0 mil.

96   3.0  3.5  4.0  4.5  5.0  5.5  6.0  6.5                        .5 mil.

96HC 1.0  1.5  2.0  2.5  3.0  3.5  4.0  4.5 5.0 5.5 6.0 6.5 7.0    .5 mil.

135  1.0  1.5  2.0  2.5  3.0  3.5  4.0  4.5                        .5 mil.

The displayed numeric results are all in milli-inches, regardless

of the track density (TPI) of the drive under test.  Generally,

the higher the track density, the smaller the numbers for normal

drives.  The CHART command's "picture" is scaled so it's image

size is about the same, regardless of the track density.

There are no Pass/Fail limits built into DRIVECHK.  You can

establish your own thresholds based on experience.

Note: The CHART command generally displays all the progressive

offset tracks available on the DDD.  However at 135 TPI, there are

7 progressive offset tracks available, not 6 as at 48 and 96 TPI.

In this case, the CHART command ignores track 69 (but it does

track 67 which is pretty close).  Displaying track 69 would not

add much value to the display.

Confused?  Never mind, it's still easy to read the display.

The chart indicates:

* Radial head alignment of each head

In general, the pattern should be centered from left to right.

That is, it should be centered about the vertical centerline of

the chart.  The ideal numeric result is 0.  When most people

talk about a drive's "alignment," this is usually what they are

talking about.

Positive values indicate a head adjusted towards the drive

spindle.  Negative values indicate away from the spindle.

The top half of the chart is the upper head and its pattern

should be directly over that of the lower head.  If not, then

the two heads are not aligned relative to each other.  You can

check this visually even if the overall pattern is off to the

left or right.  Numerically, it's the difference between the

"Radial Align." for "Hd.0" and "Hd.1".

The results with a 96 TPI High Capacity DDD appear wider than

the other's results since it has more sectors.  The 96 TPI High

Capacity CHART display adds four "easy" offsets (columns) on

both sides of the vertical center line and one more difficult

offset (column) at the outer edges.  Keep this in mind when

comparing the High Capacity CHART display with others.  The

equivalent offsets (columns) are shown with a double line over

them.

* Operating Margin

Operating Margin is a term for a measurement of the drive's

ability to read weak signals.  Weak signals can be caused by a

faulty or weak disk, poor head to media contact, bad electronics

or many other factors..  Operating Margin answers the question

"How much cumulative error can the drive tolerate?"  This may

also be called "Read Sensitivity."

The wider the pattern and the larger the number, the better.

Since results with a 96 TPI High Capacity DDD appear wider than

the others, keep this in mind when comparing CHART displays from

other DDDs.

* Head positioner hysteresis (backlash)

Play in the head positioner mechanism shows up as + signs down

one edge of the pattern and - signs down the other.  A few here

and there at the edges are of no consequence.  An ideal numeric

result is 0.

You won't see significant hysteresis in most modern drives

because they use a split band positioner that is very effective

at keeping backlash to a minimum.  Occasionally a head stepper

motor can contribute to this effect if it doesn't step with

precision.

* Seek variation or head positioner linearity

The test for seek variation answers the question "How does the

radial head alignment vary at different tracks?"  While the

horizontal pattern lines for each track may have different

widths, they should all be about a common vertical center line.

That is, they shouldn't wander, from left to right.  A drive

only has one radial alignment adjustment and it is set near the

middle of the disk.  This measurement shows the radial alignment

at other track locations.

The numeric result is the average radial alignment difference

from the nominal radial alignment for all tracks.  An ideal

result is 0.

* Margin variation: Relative performance at different tracks

The width of the pattern lines shouldn't vary too much from

track to track, although it's common for the Operating Margin to

be somewhat worse toward the inner tracks where the linear

recording density is actually higher.  This can give the overall

display a somewhat rounded appearance much like two parenthesis:

().

Be sure, however, not to accept too much of this rounded

appearance.  In general, any good drive should show at least one

asterisk on either side of the vertical line at the inner

(highest) track.  Note that if it doesn't, it may still be able

to read a good disk since those "easy" points adjacent to the

centerline are already recorded somewhat off track center.

The numeric result is the average margin difference from the

nominal margin for all tracks.  An ideal result is 0.

DRIVE  <0,1,2,3>

The DRIVE command specifies which physical drive is to be tested

using a drive address number of 0, 1, 2 or 3.  The default is

drive 0, the first internal diskette drive.  Examples:

DRIVE 0         (Set drive 0)

D 2             (Set drive 2)

The drive number designates a physical drive address and not a DOS

drive letter or a drive letter associated with a special device

driver such as DOS 3.2's DRIVER.SYS.  (DRIVER.SYS may be installed

for any diskette drive, but it is not used or needed by the

DRIVECHK program.)

Drive address numbers are:

0  First internal diskette drive.  Identified by DOS as

"A" (or "A" and "B" when only one internal diskette

drive is installed.)

1  Second internal diskette drive.  Identified by DOS as "B".

2  First external diskette drive.

3  Second external diskette drive.

The drive letter that DOS assigns to external drives varies and

depends on the number of fixed disks, system board switch settings

and device drivers.  DRIVECHK doesn't care what letter DOS uses.

HELP command

The HELP command causes another screen to be displayed with a

brief DRIVECHK command summary.  Pressing any key returns you to

the main DRIVECHK screen.

A question mark in the command line or pressing function key F1

also displays the help screen.  Examples:

HELP

H

?

LIMIT command

LIMIT END         Limits seeks to mfr's specified last track (default)

LIMIT PLUS        Allows a seek one track past the "end"

The limit command specifies the highest track number that will be

used for directional seeks from the center (near the hub) out.

Most drives don't have a high track stop and can handle one or

more tracks past the end of the advertised track capacity.

DRIVECHK can take advantage of this capability so tests can

perform a seek from both directions to the normal "last" track.

For instance, on a 40 track drive which has tracks numbered from 0

to 39, DRIVECHK seeks to track 39 both from a lower track and

track 40, the ENDing track PLUS one.  This is LIMIT PLUS and must

be specified if you want this feature.  LIMIT END is the default.

Some drives have upper track limit stops and can't go past track

39 (or 79).  With these drives it is impossible to seek to the

ENDing track from both directions.  If you are testing one of

these drives, specify LIMIT END or use the default.  On a 40 track

drive with tracks from 0 to 39, DRIVECHK will then seek to track

39 only from a lower track, avoiding the track limiter.

This LIMITing only affects the results for the last track in the

CHART display.  However, the LIMIT command also controls the last

track used for directional seeks for all data points on all

diagnostic displays.  If your drive can't handle the extra track,

and you have specified LIMIT PLUS, then the head mechanism will

hit the stop.  This probably won't hurt anything, but it's likely

to rattle your nerves, destroy the displayed results and require a

RESET command or re-boot to get back on track.

If you test a drive with LIMIT PLUS and the drive limits seeks to

the ending track, then the DRIVECHK displays may appear with all

plus signs (+) instead of the usual asterisks.

You can ignore the LIMIT command if you wish.

NOZAP

NOZAP is an obscure command that you can probably ignore.  It

doesn't even appear on the DRIVECHK help screen.

If:

- You specify TPI 96   (regular media DDD, not high capacity)

and

- You specify TYPE HC  (high capacity drive)

and

- You do not have an IBM PC-AT (i.e. you have a clone

or a non-IBM high capacity controller and drive in a

PC or XT etc.)

then, you may need to enter the NOZAP command.  Otherwise, ignore

it.  If NOZAP is needed, enter it before any commands that run the

drive, otherwise the zap is applied the first time the drive is

run.  If this happens, then re-boot.

NOZAP avoids a fixup "zap" of the BIOS data area required by a

regular media, 96 TPI (80 track) disk in the IBM PC-AT's high

capacity drive.  The zap is correct for the IBM PC-AT.  If you

don't have an IBM brand PC-AT, then the zap may be harmful as the

BIOS data area may not be exactly like IBM's.  Enter NOZAP before

any commands that run the drive to bypass the normal IBM PC-AT

fixup.

If you have this combination and enter NOZAP, then DRIVECHK

attempts to run.  It's not supported as DRIVECHK can't possibly

tell how to program your particular hardware for what would

otherwise be a nonsupported 720K diskette in a high capacity

drive.

When NOZAP is in effect, "NOZAP" appears in the lower left of the

screen while the disk is running.

QUIT command

The QUIT command ends the program and returns to DOS.

RESET command

The RESET command causes a reset of the diskette system (via BIOS

INT 13 with AH=0).  You probably won't need to use RESET except

possibly if you are doing radial head alignment or the head has

hit a limit stop.  (See the LIMIT command.)

After entering RESET, the next time any drive is accessed, it will

then be "recalibrated."  This means that the diskette controller

makes no assumptions about which track the head is on, moves the

head until track 0 is located as indicated by the drive, and then

proceeds with the requested operation.  Once this recalibration

process is complete, the diskette controller keeps it's own head

position register and merely pulses the head forward or back, one

step at a time to get to a desired track.  If one pulse is missed

or the head is moved manually with power-on, then the controller's

track register won't agree with the physical position of the head.

"Recalibration" is not an adjustment.  It is a normal operation

done by the diskette controller so it can get in sync with the

physical head position.  Every drive is "calibrated" by the

controller the first time you use it after power-on or boot.

The RESET command allows you to recalibrate during drive

adjustment or test.  For instance, assume you are using the ALIGN

command to display track 16 results (or what you think is track

16).  If you believe the drive is actually on a different

track, then issue RESET before ALIGN.

It is also a good idea to use the RESET command just before the

final checks on radial head alignment to be sure you're not

exactly 1 or 2 tracks off.

Without the RESET command, it would otherwise be necessary to

re-boot or turn the machine off-and-on to force a recalibrate

operation.

SIDES command

SIDES   <SS,DS>

The SIDES command specifies either single or double-sided testing.

Double-sided is the default.  If a single-sided drive is tested

while DRIVECHK thinks it is a double-sided drive, no harm is done.

It just takes more than twice as long as it needs to.  Since

specifying SIDES SS cuts the testing in half, it speeds up all

diagnostic displays, but gives no results for the second head.

You may want to use SIDES SS to make the ALIGN display even

faster.  Examples:

SIDES SS              (Set single-sided)

S DS                  (Set double-sided)

SPEED command

The SPEED command measures diskette drive spindle speed which is

the speed the diskette rotates.  It also gives and indication of

the Instantaneous Speed Variation or ISV.  ISV is a short term

speed variation which doesn't show in the average speed.  Possible

causes of high ISV include incorrect belt adjustment, binding,

motor cogging or poor brush contact.

The SPEED command has no operands, but the drive should be

properly specified with the DRIVE, TPI and TYPE commands first, if

necessary.  SPEED requires a DDD diskette and does not work with

standard DOS diskettes.  (If you don't have a DDD, use the ALIGN

program and a standard DOS diskette to measure the speed.)

To use the SPEED command, enter SPEED and let it run for a few

seconds, stopping it with any key.  It you don't stop it, it stops

on its own after testing 150 revolutions.

Nearly all diskette drives have a nominal speed of 300 RPM.  The

only IBM exception now is the PC-AT's High Capacity drive which

has a nominal speed of 360 RPM.  The PC-AT High Capacity drive

runs at 360 RPM regardless of the type of media it is using.

Spindle speed accuracy is not as critical as the displayed numbers

might suggest.  Most drives allow a speed tolerance of 1 or 1.5

percent which is about plus or minus 3 or 4 RPM.  The actual speed

also varies a bit due to the drag of different diskettes.  Some

people over-emphasize spindle speed because it's about the only

adjustment they can adjust without extra equipment.  The SPEED

command dumps out all those numbers more for entertainment than

necessity.

The measured speed is always within the displayed range of

approximately 280 to 320 RPM for 300 RPM drives and 330 to 390 RPM

for 360 RPM drives.  This is true even if the actual speed is

outside that range and is the result of the measurement method

used by the program.  In practice, a drive whose speed is outside

that range probably won't be working anyhow and may also show an

erratic speed.  If there's any question, a look at the strobe on

most drives will tell you very quickly if the drive is anywhere

near the correct speed.

SPEED example:

Current  Average  Maximum  Minimum  Percent  Measurement

RPM      RPM      RPM      RPM      ISV      Number

298.40   298.40   298.40   298.40    0.00       1

298.37   298.38   298.40   298.37    0.01       2

298.38   298.38   298.40   298.37    0.01       3

298.40   298.39   298.40   298.37    0.01       4

298.37   298.39   298.40   298.37    0.01       5

298.38   298.38   298.40   298.37    0.01       6

298.40   298.39   298.40   298.37    0.01       7

298.37   298.38   298.40   298.37    0.01       8

298.39   298.39   298.40   298.37    0.01       9

298.41   298.39   298.41   298.37    0.01      10

298.39   298.39   298.44   298.34    0.03      38

280  285  290  295  300  305  310  315  320

���������� |....|....|....|....|....|....|....|....| ����������

^

The easiest part of the display to read is the speed meter on the

bottom.  It indicates the current RPM and changes whenever the

actual drive speed changes.  (It is not the average drive speed.

However, if it doesn't move much, then the current and average

will be the same.)  The arrow on the meter moves in one RPM

increments, displaying the "Current RPM" rounded to the nearest

whole RPM value, which is close enough.

You can probably ignore the rest of the SPEED display.  The first

10 lines show individual statistics for the first 10 measured

revolutions.  (SPEED lets the drive come up to operating speed and

stabilize before starting measurement.) "Current RPM" is for just

one revolution.  All the other statistics are cumulative.  The

last line of statistics, separated with a blank line, is the data

for the last revolution measured.  It is updated every revolution

until the SPEED command is stopped or "times out" at 150

revolutions.  As all the values except "Current RPM" are

cumulative, they tend to stabilize as the test progresses.

"Current RPM" is the measured spindle speed for one revolution.

"Average RPM" is the average of the spindle speed RPM measurements

so far and is the best overall indication of spindle speed.

"Maximum RPM" and "Minimum RPM" are the highest and lowest spindle

speeds encountered during the test.

"Percent ISV" is an indication of Instantaneous Speed Variation.

It is calculated by taking the difference between the maximum and

minimum speeds, dividing by the average speed and multiplying by

100 to get a percentage value.

The "Measurement Number" is the revolution number since the drive

came up to speed and measurements started.

TPI command

TPI     <48,96,96HC,135>

The TPI command specifies the track density (tracks per inch) of

the DDD you are using.  For 96 TPI, the TPI command also indicates

whether the DDD is on regular media or high capacity media for the

PC-AT High Capacity drive.  48 is the default.  You may prefer to

think in terms of the number of tracks instead.

TPI    Size    Tracks

-----   ------   ------

48     5 1/4"     40

96     5 1/4"     80 (regular media)

96HC   5 1/4"     80 (high capacity media)

135     3 1/2"     80 (regular media)

Normally, you set TPI to match the physical drive to be tested.

You can test the High Capacity drive on the PC-AT with one of

three DDDs:

PC-AT High Capacity Drive test options

TPI         TYPE

Digital Diagnostic Disk         command     command

----------------------------    --------    -------

48 TPI                          TPI 48      TYPE HC

96 TPI (regular media)          TPI 96      TYPE HC

96 TPI (high capacity media)    TPI 96HC    TYPE HC

Testing it at 80 tracks with the model 506-400 96 TPI or 516-400

96 TPI High Capacity DDD certainly gives the best results.  But,

if you only have the model 508-400 DDD for 48 TPI drives, you can

still get useful diagnostic displays by using it and specifying

TPI 48 and TYPE HC.  The PC-AT then double-steps the High Capacity

drive as it always does with 40 track diskettes.  Note that the

CHART pattern is about half as wide and the resolution half of

what it would be with the correct 96 TPI DDD.  The CENTERING

display is also much more critical.

If you are specifying TPI 96 and TYPE HC (a regular 96 TPI DDD and

a high capacity drive) and you do not have an IBM PC-AT (i.e. you

have a clone or a non-IBM high capacity controller and drive in a

PC or XT), then you may need to specify NOZAP.  See the NOZAP

command.

Don't specify TPI 96, 96HC or 135 on a 40 track drive.  DRIVECHK

tries to use tracks that aren't there and bangs the head assembly

against its travel limit.

Examples:

TPI 48               (Set 48 tracks per inch DDD)

TP 96                (Set 96 tracks per inch DDD)

TPI 96HC             (Set 96 tracks per inch DDD

on High Capacity media.)

TYPE command

TYPE     <REG,HC>

The TYPE command specifies a "regular" diskette drive or the

PC-AT's High Capacity drive.  TYPE REG is the default.  The

Personal System/2 High Density (1.44M) drive is tested with TYPE

REG.

REG           Specifies a "regular" drive      (default)

HC            Specifies a PC-AT High Capacity drive

High Capacity support in DRIVECHK has only been tested on a PC-AT.

Operation with other machines and non-IBM hardware may not be

successful.  TYPE HC tests the drive at a 300 KBS data transfer

rate with TPI 48 or TPI 96.  It tests at the 500 KBS rate when

used with the special high capacity DDD and TPI 96HC.  Results

with the 300 KBS test are probably a good indication of

performance at 500 KBS as well as the 40 track compatibility mode.

While it's best to test the High Capacity drive as TYPE HC and TPI

96 or TPI 96HC, if you only have a 48 TPI DDD diskette, you can

use TYPE HC and TPI 48.  See the TPI command description.

If you are specifying TPI 96 and TYPE HC (a regular 96 TPI DDD and

a high capacity drive) and you do not have an IBM PC-AT (i.e. you

have a clone or a non-IBM high capacity controller and drive in a

PC or XT), then you may need to specify NOZAP.  See the NOZAP

command.

Examples:

TYPE REG           (Specify "regular" drive)

TY HC              (Specify High Capacity drive)

ZAP

ZAP is an obscure command that you can probably ignore.  It

doesn't even appear on the DRIVECHK help screen.

ZAP reverses a prior NOZAP command.  It restores the default for

the special case IBM PC-AT, TPI 96, TYPE HC operation.  See the

NOZAP command for an explanation.

REPEATABILITY

If you run DRIVECHK more than once on the same drive, chances are

you will not get exactly the same result.  Close, but not exact.

The reason is that the drive is tested to the point of read

failure and this point may not be exactly the same from trial to

trial.  Variations in clamping, temperature, head contact and

chance effects all contribute to minor variations in results.  No

two Digital Diagnostic Disks are exactly equal either.  (Remember,

the extent of intentional misalignment on the disk is an ANALOG

quantity.)

Take extra care when handling and storing your Digital Diagnostic

Diskettes.  They are precision tools.  Handling and damage that

would not cause read or write failures on your data diskettes

could cause DDD's to give different results.  DDD's do not

come with hub reinforcing rings.  Don't put them on as it may

affect accuracy.  Also, don't insert a DDD in a drive that has

known mechanical or electrical problems that could cause damage.

One "Oops!" and you can kiss it good-bye.  While your DDD's can

last a long time, you may want to order extras, just in case....

MESSAGES

(operand name) operand is too large.  Range=(min)...(max)

Explanation:  The value you specified as a parameter is too

large for the command you entered.

Action:  Choose a value within the specified range and retry the

command.

(operand name) operand is too small.  Range=(min)...(max)

Explanation:  The value you specified as a parameter is too

small for the command you entered.

Action:  Choose a value within the specified range and retry the

command.

(operand name) operand must be a numeric integer

Explanation:  The value you specified as a parameter is not a

valid numeric constant.

Action:  Check the characters you entered and retry the command

using numeric characters only for the operand value.  Decimal

points, plus and minus signs are not permitted.

Azimuth can only be checked with TPI 48 or 96HC

Explanation:  You requested an azimuth display with the AZIMUTH

command, but the TPI (tracks per inch) is not set at 48 or 96HC.

The 48 TPI and 96 TPI High Capacity DDD are the only ones with

azimuth information recorded on it.  96 TPI regular media and

135 TPI DDDs do not have azimuth tracks.

Action:  If TPI is set wrong and you are using a 48 TPI DDD or

96 TPI High Capacity DDD, then enter TPI 48 or TPI 96HC as

appropriate and then retry the AZIMUTH command.  Otherwise,

azimuth cannot be checked.

Can't process.  HC is not valid with TPI 135

Explanation: A High Capacity drive has been specified with the

TYPE command, but the current TPI (tracks per inch) is incorrect

for a High Capacity drive.

Action:  Either change the drive type with the TYPE command to a

type other than HC or change the TPI with the TPI command to 48,

96 or 96HC as appropriate.

Can't process.  96HC is only correct with TYPE HC

Explanation:  A TPI (tracks per inch) of 96 with a High Capacity

media DDD has been specified with the TPI command, but the

current drive type is incorrect for a High Capacity media DDD.

Action:  Either change the drive type with the TYPE command to

type HC or change the TPI with the TPI command to something

other than 96HC.

Diskette system has been reset

Explanation:  This is an informational message displayed every

time the RESET command is executed.  It indicates that the next

time the drive is started, the diskette controller will

re-calibrate the drive by seeking to track 0 before seeking to

the current track.

Action: None.

Drive operand is not 0, 1, 2 or 3

Explanation:  The specified diskette drive number must be 0, 1,

2, or 3.

Action:  Retry the command using the correct drive number.

Error reading diskette

Explanation:  The program encountered a disk error while reading

the diskette for spindle speed testing.  Testing and measurement

continues, although some of the calculated results may lose

accuracy.  If you just get a single error, you can restart the

speed test.  If you get this error continuously, then you are

either using the wrong diskette or may have a solid diskette

drive failure.

Action:  Make sure you are using the correct Digital Diagnostic

Diskette and that the DRIVE, TYPE and TPI values are properly

set.  The DRIVECHK speed test does not work with ordinary DOS

formatted diskettes.  If you don't have a Digital Diagnostic

Diskette, use the ALIGN program and any DOS formatted diskette

to check the drive speed.

LIMIT operand is not PLUS or END

Explanation:  The specified LIMIT operand must be PLUS or

END.

Action:  Retry the command using a correct LIMIT parameter.

Missing operand(s)

Explanation:  Not enough operands were entered on the command

line for the specified command.

Action:  Re-enter the command with the proper number of operands

as indicated in the command description.

Read error verifying DDD diskette

Explanation:  DRIVECHK was unable to read the DDD model number

that is recorded on every DDD diskette.  Either you do not have

a DDD in the drive under test or the identifying sector cannot

be read.

Action:  DRIVECHK continues with the requested diagnostic test

on the assumption that the drive is out of adjustment and some

useful results may be obtained.  If you do not have a DDD in the

drive under test, then press any key to stop the test.

SIDES operand is not DS or SS

Explanation:  The specified SIDES operand must be DS or SS.

Action:  Retry the command using a correct SIDES parameter.

Test ended

Explanation:  You have stopped a diagnostic test by pressing any

key during the test.  No results will be displayed.

Action:  If you still want to run the test, then re-enter the

command to run it.

Too many operands

Explanation:  More operands were entered on the command line

than the specified command can accept.

Action:  Re-enter the command with the proper number of operands

as indicated in the command description.  Many commands do not

have any operands.

TPI operand is not 48, 96, 96HC or 135

Explanation:  The specified TPI (tracks per inch) must be

48, 96, 96HC or 135.

Action:  Retry the command using the correct track density

specification.

TYPE operand is not REG or HC

Explanation:  The specified TYPE value must be REG or HC.

See the TYPE command description for an explanation.

Action:  Retry the command using the correct TYPE value.

Unknown command

Explanation:  The specified command is not recognized by

DRIVECHK.

Action:  Check the command spelling and re-enter it.

Warning: TYPE HC is intended for PC-AT

Explanation:  You specified a High Capacity drive and DRIVECHK

is not running on a PC-AT which is the only IBM Personal

Computer that supports a High Capacity drive.

Action:  If you are not testing a High Capacity drive, then

re-enter the TYPE command specifying a type that is appropriate

for your Personal Computer model and diskette drive type.  If

you are testing a High Capacity drive on a machine other than a

PC-AT, then you may continue.  However, High Capacity support in

DRIVECHK has only been tested on a PC-AT.  Operation with other

machines and non-IBM hardware may not be successful.

Your model (model) DDD is not for (tpi) TPI

Explanation:  DRIVECHK read the DDD diskette and found it was

for a different TPI (tracks per inch) than that specified with

the TPI command.  The track density of the DDD you use must

match the value entered with the TPI command.  Furthermore, if

you have a 96 TPI High Capacity DDD, you must use TPI 96HC.

Action:  Either insert the correct DDD or use the TPI command so

it agrees with the DDD you are using.

COMPETITIVE PROGRAMS

There are several diskette drive diagnostic programs and disks on

the market.  While I have never used any of them and have only

seen sales literature for some, I'd say that DRIVECHK has the

following advantages:

- Graphic results with a somewhat unique presentation that

lets you judge the drive's performance for yourself.

(Like a gauge in a car compared to "idiot lights.")

- Packs more information into each display, making it

easier to see the overall results and easier to use

the program.  The user doesn't have to do much to

see all the data.

- Supports 96 and 135 TPI drives as well as 48 TPI

- Supports the PC-AT High Capacity drive with 3 different

types of DDDs.

- Allows testing the PC-AT High Capacity drive with a

48 TPI DDD (which gives reduced resolution).

- Displays an indication of instantaneous speed variation.

And disadvantages:

- Slower

- No pass-fail results.

You may wish to investigate other programs on the market that have

a similar purpose.  The first three listed here use the some of

the same Xidex/Dysan DDD diskettes that DRIVECHK uses.  (I don't

know what disk Cross-Chex uses.)

- Interrogator (Xidex/Dysan)

- Memory Minder / Disk Drive Analyzer (J&M Systems)

- ReadiScope (ReadiWare Systems)

- Cross-Chex (Award Software)

- Datalife Disk Drive Analyzer (Verbatim)

- Recording Interchange Diagnostic (Dymek)

ORDERING DIGITAL DIAGNOSTIC DISKETTES

Digital Diagnostic Disks (DDD's) are not available directly from

Xidex/Dysan Corporation in small quantities.  Their regional sales

offices or main office can give you the name of a dealer who

stocks them or can get them for you.  Any Xidex/Dysan dealer can

order them.

Xidex/Dysan Corporation, CE Division

5100 Patrick Henry Drive

Santa Clara, CA 95050

Telephone (800) 233-5099 Xidex

(408) 988-3472

(800) 422-3455 Xidex CE Division

One mail order dealer with a range of Xidex/Dysan products is

Supply Line, Inc.

1420 N. Claremont Blvd. Suite 205B

Claremont, California 91711

Telephone (800) 422-3392  outside of California

(800) 648-7376  inside California

(714) 621-1984