========= BUILDING THE ELECTRONICS BOARD FOR THE ORGANEZE SYSTEM ===========

Apart from the Latches, making up the circuit is just a matter of following

the wiring and component list. The wiring of the Latches is to some extent

variable depending upon the configuration of your organ therefore it is

suggested that before reading this document you should read through the

ORGANEZE.DOC file and run the software so as to understand the overall

layout of the ORGANEZE SYTEM especially the part about configuring the

software to suit your organ.

======= BRIEFLY HOW THE ORGANEZE CIRCUIT WORKS ============

The organ is played by the opening of solenoid valves to admit air to pipes

in respose to data supplied either from a computer running ORGANEZE software,

or from an EPROM which has been programmed from files prepared by ORGANEZE.

The computer can be connected to the circuit by it's parallel printer

cable. A single pole switch is used to select between computer/Eprom modes.

In either mode a stream of data bytes is supplied by computer or Eprom.

To describe the stream it is assumed that you have run the software and red

ORGANEZE.DOC.  Suppose that your organ has a keyframe of 56, then the status

of these keys (whether they are to be on or off) can be represented by seven

bytes of data, each byte having 8 bits and each bit can be either 1 or 0

(ie on or off). If therefore those 7 bytes of data are supplied together with

one more byte containing a number representing a time duration then the organ

will play a combination of pipes for a specified time. At the end of that time

another set of bytes is supplied.

If instead of 56 keys you use 60 keys then ORGANEZE will use 8 data bytes

and four bits of the first byte would be unused. Up to 120 keys can be used.

When in computer mode the software supplies this data as a continuous stream

the circuit interfaces with the computer, allows the bytes to enter and directs

the first one (which is the time byte) to a load a Counter and each following

byte to a particular 8 bit Latch which copies the byte. When all latches are

filled a Busy is passed to the computer which makes it wait.

Now each bit which is 1 (ie ON) of the Latches will be operating a solenoid

(via a Driver) so the organ is now playing some motes, meanwhile the timing

number which was loaded into the Counter is being down-counted by a clock

oscillator.

When the Count reaches zero the Busy is removed and another batch of bytes is

accepted.

When in EPROM mode the circuit works more or less the same. The EPROM is

addressed by a 17 bit counter (the EPROM holds 128 kilo bytes). As the counter

counts up the Eprom presents successive bytes at it's output for the circuit

to intake as above. When all Latches are filled the Counter is stopped

similarly to the computer busy. The process will repeat after the time lapse.

One EPROM plays for about 30 to 40 minutes then restarts at the beginning.

=================== THE CIRCUIT ==========================

These are the IC's etc......

A,B,C,D,E,U,V,S  are 74LS193 (8 bit counters)

F a 32 pin zero-insertion-force socket for EPROMS.

K,M are 74LS14  (schmitt trigger inverters)

X  74LS260  (dual 5 input nor gates)

G,N  74LS240  (octal inverting buffers)

L  74LS74  (dual flip-flop)

W  74LS00  (quad nand gate)

P,Q  74LS123  (dual multivibrators)

T  74LS154  (4 to 16 way decoder)

Latches 74LS273  (octal D type latches)

Solenoid Drivers  ULN2803  (octal Darlingtons)

EPROM  M27C1001

Diodes, two very small signal diodes are needed.

Assorted resistors and capacitors as shown in the wiring list.

One multiturn variable 100k resistor.

Two single pole miniature switches.

In addition to the wiring list, eight small 0.1mF capacitors

should be scattered about the board connected between +5V

and Earth as interference decouplers.

A convenient layout on 0.1 inch copper strip board.......

(It will be necessary to use 2 pieces if only 4" wide is available       )

(The board will be about 6" wide x 9" depending on number of latches used)

A   B   C   l   d   o

switches   F    a   r   u

E   D   t   i   t

X   U   V   c   v   l

M   K   G   h   e   e

Cent   L   N   e   r   t

Socket  W   Q   s   s   s

here   P

S   T

NOTES

=====

1. Abreviation CENT means the Centronics socket which accepts the computer

printer cable. Pin numbers are usually embossed in the molding.

2. In the wiring list the single letters followed by a number refer to

IC's and the number is the pin.

3. An IC package has a notch at the left end. With the notch at the left the

pins are numbered starting 1 at bottom left and continuing to the bottom

right, then continuing the numbers from top right along to top left.

4. In the following wiring list a comma means that the pins etc shown

at each side of the comma are to be connected together.

5. HIGH means that the pin must be connected via a 1k resistor to +5v but

you can connect up to about ten highs together to the same 1k resistor.

6. When a resistor capacitor or diode is involved to indicate that the words

following the component are where its other end should go.

TIPS

======

Mark out IC locations, wire up and fully check connection/non-connection

before fitting IC's.

Use bare thin wire under IC's to link pins of the same IC.

You can get a 9 pin SingeInLine package of 8 resistors (all commoned to

the nineth pin. These are in compact form and very handy for all those

resistors going to +5v.

====== WIRING LIST ======

A3,F2                         A4,B4,C4,D4,E4,F31,V5,U5,HIGH

M4,G6

A5,B12                        M6,G4

A1,A8,A9,A10,A14,A15,EARTH    K2,G2

A16,+5V                       G10,EARTH

A11,B11,C11,D11,E11,P10,L9    M2,G8

B2,F28                        M12,G11

B3,F4                         M10,G13

M8,G15

B5,C12                        K12,G17

B6,F29                        G20,+5V

B7,F3                         N2,P5

B1,B8,B9,B10,B14,B15,EARTH    N10,EARTH

B16,+5V                       N20,+5V

C1,C8,C9,C10,C14,C15,EARTH    U2,X11,X12,X13

C2,F26                        U3,X10

C3,F27

U6,X9

C5,E12                        U7,X8

C6,F23                        U8,U14,EARTH

C7,F25                        U13,V4

C16,+5V                       U16,+5V

D1,D8,D9,D10,D14,D15,EARTH    V2,X4

D2,F11                        V3,X3

D3,F12                        V6,X2

V7,X1

D5,P12                        V8,V14,EARTH

D6,F10                        V16,+5V

D7,F9                         X7,EARTH

D12,E5                        X14,+5V

D16,+5V

E1,E8,E9,E10,E14,E15,EARTH    F13,G12,U15,ALL LATCHES 3 (DATABUS 0)

E2,F7                         F14,G14,U1,ALL LATHES 4 (DATABUS 1)

E3,F8                         F15,G16,U10,ALL LATHES 7 (DATABUS 2)

F17,G18,U9,ALL LATCHES 8 (DATABUS 3)

E6,F6                         F18,G9,V15,ALL LATCHES 13 (DATABUS 4)

E7,F5                         F19,G7,V1,ALL LATCHES 14 (DATABUS 5)

E16,+5V                       F20,G5,V10,ALL LATCHES 17 (DATABUS 6)

F1,F32,+5V                    F21,G3,V9,ALL LATCHES 18 (DATABUS 7)

F16,F22,EARTH

F24,K6,N1                     SPST SWITCH,G1,G19,N19,K5 and via 1Kohm to +5v

OTHER SIDE OF SPST SWITCH,EARTH

CENT 2,M1,1Kohm TO +5V            (Computer mode/Eprom mode)

CENT 3,M3,1Kohm TO +5V

CENT 4,M5,1Kohm TO +5V        CENTRE OF SPChangeOver SWITCH,EARTH (Stop/GO)

CENT 5,K1,1Kohm TO +5V        CLOSED SIDE OF SWITCH,L13 and via,1Kohm to +5v

CENT 6,M13,1Kohm TO +5        OPEN SIDE OF SWITCH,L10 and via 1Kohm to +5v

CENT 7,M11,1Kohm TO +5V              (Stop/Go switch)

CENT 8,M9,1Kohm TO +5V

CENT 9,K13,1Kohm TO +5V       W8 is Stop/Clear, goes to some Latches pin 1

K7,EARTH                        (see Configuring later in this document)

K14,+5V

M7,EARTH                                �� via 2K2ohm TO +5V

K4,N17                        CENT 11 ���� via >Diode> TO W11,P9

M14,+5V                                 �� via 100pF TO EARTH

CENT 1,K3,1Kohm TO +5V                  �� via 2K2ohm TO +5V

CENT 10 ���� via >Diode> TO Q13

L14,+5V                                 �� via 100pF TO EARTH

L11,L12,L7,EARTH              Note > > indicates direction that diode conducts

L3,MAX (see configuring later)

L6,W13                          Q6��� via 1Kohm TO +5V

L8,W9,W10                          �� via 0.01mF TO Q7

W7,EARTH

W14,+5V                         Q15��� via 2K2ohm TO +5V

W1,W2,W6,P3,L1                      �� 0.1mF TO Q14

W4,X6

W5,X5                           P15 ��� via 50Kohm via 100K variable TO +5V

�� via 0.1mF TO P14

W12,Q4

N3,N18,Q10,T19                 T2, pin 11 of all Latches which are to load

W3,S14                             the first data byte after the time byte.

ALL LATCHES 10,EARTH           T3, pin 11 of all Latches which are to load

ALL LATCHES 20,+5V                 the second data byte after the time byte.

P2,P4                          T4, etc

P11,L2,L4,Q3,Q11,S4,S11,HIGH   T5, etc

P7,1Kohm TO +5V                    to limit of T17 but omitting T12

P1,P8,EARTH                        (see configuring later in this document)

P13,U4

P16,+5V                       Latch pin 2, ULN2803 pin 1    (usual connection)

Q1,Q8,Q9,EARTH                  "    "  5,    "     "  2    (but see         )

Q2,Q5                           "    "  6,    "     "  3    (configuring     )

Q12,S5                          "    "  9,    "     "  4    (later           )

Q16,+5V                         "    " 12,    "     "  5

S2,T22                          "    " 15,    "     "  6

S3,T23                          "    " 16,    "     "  7

S6,T21                          "    " 19,    "     "  8

S7,T20

S8,EARTH                      ULN2803 pin 9,EARTH (to carry about 2 amps)

S16,+5V                       ULN2803 pin 10, +24V

T1,U11,V11                    (EARTH means the common negative connection)

T12,T18,EARTH                 (of the 5 Volt and 24 Volt supplies        )

T24,+5V

======== HOW THE CIRCUIT IS CONFIGURED TO SUIT YOUR ORGAN =========

1.  Most of the circuit is standard the only parts to be configured are the

pins 1 and pins 11 of the latches, also sometimes the connections from latches

to drivers.

To understand these variations you need to know the following......

Data comes either from the computer or the EPROM in batches of bytes. Each

batch sets the organ to play certain notes etc for a certain length of time.

When that time has elapsed another batch comes along.

The first byte in the batch is the timing byte which need not concern us

here. The next byte contains the state of control keys and the note keys

from the bottom of your computer screen layout. The next byte is the keys

for the next higher notes and so on. The last byte is your highest notes,

the top of your screen layout.

All the data bytes are available to all the latches but depending upon

the connection of a Latches pin 11 (top right pin) it will copy only one

particular byte of each batch.

These are the connections to the decoder T......

T2 .......  2nd. byte  (remember the first byte was the timing byte)

T3 .......  3rd. byte

etc. up to

T11.......  11th. byte

then

T13.......  12th. byte

etc. up to

T17.......  16th. byte

More than one latch can copy the same byte.

The number of keys in your organ determins the number of bytes in a batch.

The last of these T pins which is used to connect a latch must also be

connected to L3 (called the MAX connection, used to stop the data flow

when all latches are filled).

The connection of the bottom left pin (pin 1) determins how a latch will

be cleared to all zeros. They can be cleared by connecting to W8 so that

only when the stop/go switch is set to stop will they be cleared (this is

refered to as the Stop/Clear connection).

Alternatively they can be connected to one of the control keys you are

using to bring into play another rank of pipes. This means that although

the latch will be told by its pin 11 to copy a particular data byte it will

be held in the clear state if its pin 1 is kept at 0 by the control key.

2. Now cosider this diagram. Each latch is numbered for reference purposes

as 1 if it copies the first byte, 2 if it copies the second and so on.

If more than one latch copy the same byte they are lettered a,b,c etc.

At the top right of each is shown the decoder pin to which the top right pin

is connected.

Similarly against the bottom left corner is shown the clearing connection

for the Bottom Left pin, either SC for StopClear or the number of a

control bit.

Latch 1 having the control bits is         �����������Ŀ-de4  �������Ŀ

cleared by switching to Stop,note that     �  latch 3b ��all�>�drivers�

its bottom left pin is connected to SC  c1-�������������      ���������

(StopClear). Its cotrol bits are           �����������Ŀ-de4  �������Ŀ

represented by c1 c2 c3 its other bits     �  latch 3a ��all�>�drivers�

might be a drum a cymbal and the three  SC-�������������      ���������

lowest notes. Note only 5 go to driver.    �����������Ŀ-de3  �������Ŀ

Latch 2 copies the next 8 lowest notes     �  latch 2  ��all�>�drivers�

and it is also connected to SC.         SC-�������������      ���������

Latch 3a has the next 8 notes going        �����������Ŀ-de2  �������Ŀ

up and is also connected to SC.            �  latch 1  ���5��>�drivers�

Latch 3b has the same 8 notes as 3a,    SC-��|��|��|����      ���������

but its bottom left pin goes to control      c1 c2 c3

c1 so that if c1 is 1 the latch will

copy byte 3 but if it is O it will be

held in the cleared state.

So the bottom 19 notes are played on say bourdons and are always in play.

The upper 8 of those 19 notes can also be played on say trombones.

Note that three of the drivers in the bottom driver IC are unused as the

control latch bits are not connected to them.

If you can design your organ so that the ranks begin and end at latch

ends then that is all there is to it and no more.

3. What happens if we want a pipe rank to begin or end at a note part way

through a latch ? For that Latch you could use And gates in between the Latch

and it's Drivers and use the control key to open the appropriate gates but

is easier to connect two latches to copy the byte which is to be split,

use part of each and use appropriate cotrol key or stop/clear (W8) on their

pins 1 as shown in the next diagram.........

The lower half of this diagram             �����������Ŀ-de7    �������Ŀ

shows latch 1 playing the bottom 3         �  latch 6b ���all��>�drivers�

notes as before then a group of         c2-�������������        ���������

uncontrolled latches 2,3a,4a,5a,6a      �  �����������Ŀ-de6    �������Ŀ

playing the next 40 notes up using say  �  �  latch 5c ���top6�>�drivers�

bourdons. 43 bourdons in all.           c2-�������������        ���������

Then there are 3b 4b and 5b controlled     �����������Ŀ-de5    �������Ŀ

by c1. The top 4 notes of 3b are           �  latch 4b ���all��>�drivers�

used, all of 4b are used and the        c1-�������������        ���������

bottom 2 notes of 5b, making a rank     �  �����������Ŀ-de6    �������Ŀ

of 14 pipes of say trombones. The rank  �  �  latch 5b ���bot2� �omitted�

ends are deliberately placed in         c1-�������������      � ���������

awkward places in mid latches.          �  �����������Ŀ-de4  �>�������Ŀ

At the top are 5c & 6b controlled by    �  �  latch 3b ��top4��>�drivers�

c2. The top 6 notes of 5c & all of 6b   c1-�������������        ���������

play a rank of say violins whose bottom    �����������Ŀ-de7    �������Ŀ

note is next above the top of the          �  latch 6a ���all��>�drivers�

trombones.                              SC-�������������        ���������

Note that the driver IC opposite 3b     �  �����������Ŀ-de6    �������Ŀ

will drive the two top pipes of the     �  �  latch 5a ���all��>�drivers�

rank (the bottom notes of latch 5b)     SC-�������������        ���������

and the bottom 4 pipes of the rank      �  �����������Ŀ-de5    �������Ŀ

(the top notes of latch 3b). Two        �  �  latch 4a ���all��>�drivers�

drivers will be unused.                 SC-�������������        ���������

�  �����������Ŀ-de4    �������Ŀ

All together there are four             �  �  latch 3a ���all��>�drivers�

individual drivers not used which       SC-�������������        ���������

can be used by any other part           �  �����������Ŀ-de3    �������Ŀ

of the circuit provided their input     �  �  latch 2  ���all��>�drivers�

pins are not also connected to their    SC-�������������        ���������

adjacent latch.                         �  �����������Ŀ-de2    �������Ŀ

�  �  latch 1  ���5����>�drivers�

SC-��|��|��|����        ���������

c1 c2 c3

IMPORTANT NOTE. If you have understood the above you will see that the

latch or latches containing rank control keys cannot also contain notes

which are played by controlled ranks; their bottom left pin can only be

connected either to SC or to a control key not both. This must be

remembered when configuring ORGANEZE.

4. TO CONTROL THE KEYFRAME OF EXISTING ORGANS.

Traditional organs already contain all the 'And logic' in pneumatic

form so they simply need a simple row of latches and gates.

5. PRINTED CIRCUIT BOARDS have been designed but as yet have not been put into

production. It is necessary to get some idea of likely demand before an

economic sized batch can be madeso I am waiting to assess response.

The circuit is on a pair of double sided boards.

One board has the System including the computer interface and room for

up to three EPROMS. The other board has twelve latches and ten drivers.

For a sizeable organ you can have more than one driver board and to allow

any number of boards to be used each driver board has an IC to boost the

drive power of the data bus. The boards plug in using DIN 41612 connectors.

The sytem board is standard and needs no configuring.

All of the linking up of the top right and bottom left latch pins is

done on the back of the connectors rather than on the driver boards.

When latches are split the cross linking of latches to the drivers adjacent

to other latches has to be wired on the driver board and circuit board

tracks broken.

Another circuit board version being considered has all the lot on one

board enough for any traditional organ and enough for a reasonable sized

new-make organ.

The board can be constructed from the above wiring list but an enlarged A3

diagram of a hand wired board on 0.1" stripboard, from which you can work is

available from the author (free to Registered Users).

It is suggested that you print out this document.

--------------- END ----------------