Cyrix 486SLC/486DLC

Cache Controller

The SETCACHE.EXE program initializes the Cyrix 486 chips internal Cache

to the desired mode. When the Cyrix chips are reset, cache is disabled.

This program allows the user to setup cache for maximum operation speed.

The SETCACHE program may be executed in the AUTOEXEC.BAT file to enable

cache operation at startup, or may be executed at any time from the

command line. The default values have proven to be the fastest operation,

however each user can establish their own setup. It may be that the

't' option can have a beneficial affect.

Command Line options are as follows:

Cache Control program for Cyrix 486SLC/486DLC Microprocessors.

COPYRIGHT 1993 BY JOHN HICKEY of J. JENKS ENTERPRISES.

}SETCACHE [options]

Defaults to 2-way assoc., all mem. cached except A0000-FFFFF.

Options as follows:

t - DOS memory area A0000-FFFFF is cached.

l - Cache inhibit 64K at 1 meg boundaries.

m - Cache mode is direct-mapped.

h - Flush Cache on CPU Hold.

r#aaaaas - Inhibit cache at region #, starting

at address aaaaa X 1000h, for s size, where:

#     - 1 to 4

aaaaa - 00000-FFFFF hex = address bits 31-12 of inhibited memory.

s     - 0-F size code as follows:

size-code     Range  size-code     Range  size-code     Range

0 - none       N/A   1 - 4KBytes   aaaaa  2 - 8KBytes   aaaaj

3 - 16KBytes  aaaak  4 - 32KBytes  aaaal  5 - 64KBytes  aaaa0

6 - 128KBytes aaaj0  7 - 256KBytes aaak0  8 - 512KBytes aaal0

9 - 1MBytes   aaa00  A - 2MBytes   aaj00  B - 4MBytes   aak00

C - 8MBytes   aal00  D - 16MBytes  aa000  E - 32MBytes  aj000

F - 4GBytes  00000(Cache is totally inhibited)

a = 0-F Hex  j = 0, 2, 4, 6, 8, A, C or E Hex  k = 0, 4, 8 or C Hex

l = 0 or 8 Hex     Spaces unnecessary between options.

02 00 000000 000000 000000 000000

SETCACHE with no options will initialize the Cache to the default values

of 2-way associative cache and all memory cached except A0000 to FFFFF,

and displays the above help screen.

To setup the Cache to the default values without printing the Help screen,

follow the SETCACHE command with a space.

Option

t      - Cache DOS memory area A0000-FFFFF.

l      - Inhibit Caching of 64k bytes at each 1meg boundary.

m      - Cache Mode is direct mapped.

h      - Flush cache on CPU HOLD.

r#aaaaas - Inhibit Cache in specific memory regions.

#     - Non-Cacheable Region number 1 to 4.

aaaaa - Region beginning boundary address in 4k segments (1000 hex).

The boundary address must start on specific power of 2

address, depending upon the size code. Valid addresses

will be rounded down to the nearest appropriate value.

s     - Size code 0-F hex specifying region granularity.

Size           Boundary   Boundary

code           addr range ending digit(s)

0 - not        N/A

1 - 4KBytes   aaaaa(000) 0-F

2 - 8KBytes   aaaaj  "   0,2,4,6,8,A,C,E

3 - 16KBytes  aaaak  "   0,4,8,C

4 - 32KBytes  aaaal  "   0,8

5 - 64KBytes  aaaa0  "   00-F0

6 - 128KBytes aaaj0  "   00,20,40,60,80,A0,C0,E0

7 - 256KBytes aaak0  "   00,40,80,C0

8 - 512KBytes aaal0  "   00,80

9 - 1MBytes   aaa00  "   000-F00

A - 2MBytes   aaj00  "   000,200,400,600,800,A00,C00,E00

B - 4MBytes   aak00  "   000,400,800,C00

C - 8MBytes   aal00  "   000,800

D - 16MBytes  aa000  "   0000-F000

E - 32MBytes  aj000  "   0000,2000,4000,6000,8000,A000,C000,E000

*F - 4GBytes   00000  "   Cache is totally inhibited.

After the SETCACHE program executes, the values currently in the

Cache Controller registers are read back and are displayed as follows:

RX RY area-1 area-2 area-3 area-4

where

RX     = Cache Control Register CCR0

RY     = Cache Control Register CCR1

area-n = aaaaas for Non-Cacheable Region n.

The Cache Controller settings will remain unchanged until a System Reset

is done or SETCACHE is rerun with new parameters.

!!CAUTION!!!

Do not attempt to run this program on any CPU other than a 80486.

It uses 486 unique instructions and will cause illegal instruction

faults on lesser CPU's!!

SETCACHE.EXE is the Copyrighted work of John Hickey and J. JENKS Enterprises.

Copyright June, 1993

All Rights Reserved

Author         : John C. Hickey

Date of release: 06/04/93

Please let me know if you have any problems or find the program to be

useful. Registered users may call if they have any problems, and will

be notified of any upgrades.

John Hickey

J. Jenks Enterprises

2155 Philippe Pkwy

Safety Harbor, Fl, 34695

(813) 726-0648