XIRCOM CREDITCARD ETHERNET ADAPTER

README DOC

Release 2.00

December 16, 1993

========================================================================

Thank you for purchasing the Xircom CreditCard Ethernet Adapter.  Xircom

has done everything to ensure that our CreditCard Ethernet Adapter is

compatible with notebooks that support the Personal Computer Memory Card

International Association's (PCMCIA) 2.0 to 2.1 I/O slot configuration.

Xircom is committed to making all of its networking products compatible

with the widest range of notebook and laptop PCs possible.  As a company,

we are equally committed to customer satisfaction and timely response to

the needs and concerns of our customers.

As this technology evolves and more PCMCIA 2.0 and 2.1 PCs become

available, you can be assured Xircom will continue to provide the

highest level of hardware and software compatibility in the industry.

If you have compatibility issues related to your specific environment,

or you have general questions, please contact Xircom Technical Support.

Information on Xircom Technical Support is located at the end of this

file.

Remember to register your Xircom CreditCard Ethernet Adapter

to receive information about future software upgrades and

new products.

CONTENTS:

---------

1)  CreditCard Ethernet Adapter Characteristics

2)  New features in this release

3)  CECFG.EXE notes

4)  Microsoft Windows for Workgroups v3.11 with Netware Configuration

5)  IBM OS/2 LAN Requester installation (for IBM OS/2 LAN Server

version 3.0)

6)  Special software notes

7)  Machine specific notes

8)  Xircom Technical Support

CREDITCARD ETHERNET ADAPTER CHARACTERISTICS

==================================================

This release of Xircom's CreditCard Ethernet Adapter has the following

characteristics:

- Card and Socket Services set versions 2.0 to 2.1 compatibility.

Card and Socket Services is a software specification defined by the

PCMCIA organization.  Its purpose is to serve as an intermediary

between the network device driver and the system resources required by

the PCMCIA Adapter.

- Intended for use with Intel 82365 PCIC, Sharp PHIC, Databook TCIC,

Cirrus Logic CL-PD6710 or CL-PD6720 or Toshiba ICCNT PCMCIA

interface chips.

- Variable interrupts, 3 to 15, default interrupt 5.

- Variable I/O window, 16 bytes needed, default base at 300h.

- Requires a 8Kbyte memory window on a 8Kbyte boundary, within segment

C000h to E800h, default is D000h.

- Keyword support to disable link integrity.

- ODI, NDIS for OS/2 2.1 and DOS support, and PACKET drivers.

FEATURES OF NEW RELEASE

=======================

Features of release 2.0 of the Xircom CreditCard Ethernet

Adapter Network drivers from previous releases:

-  Advanced Power Management for suspend/resume through the use of Card

and Socket Services

-  Increased performance from Memory Mapped I/O mode

-  Support of Cirrus Logic PCMCIA controller in operating the

CreditCard adapter in both 8 and 16 bit mode

-  OS/2 version 2.0 and above support for DOS and NDIS

-  Hot swapping through the use of Card and Socket Services

-  Artisoft LANtastic 5.0 support

CECFG.EXE CONFIGURATION UTILITY

===============================

Supplied on this disk is a utility called CECFG.EXE.  The utility can be

run from the DOS command line or included in the AUTOEXEC.BAT file or the

CONFIG.SYS file.  It is recommended that CECFG.EXE be run prior to

loading any DOS Xircom network driver.  This utility is intended for DOS

usage only.

The CECFG.EXE utility must be informed about which type of network

driver your network uses: NDIS, ODI, or Packet driver.  The CECFG.EXE

file must be located in the same directory as the network driver.

The CECFG.EXE allows the Xircom Credit Card to become a client of

Card Services.  This is mandatory to support the functions of Card and

Socket Services.  CECFG.EXE must be loaded prior to loading a network

driver when using Card and Socket Services.

MICROSOFT WINDOWS FOR WORKGROUPS V3.11 WITH NETWARE CONFIGURATIONS

==================================================================

The following installation instructions are not available in the

manual.

Begin by copying the Xircom ODI drivers as outlined in the

Xircom Users Manual in the Novell NetWare section.  Copy

them into the Windows directory of the machine, run those

drivers at the top of the AUTOEXEC.BAT and log in to the

NOVELL server in the AUTOEXEC.BAT.

After arriving at a DOS prompt, install Windows For

Workgroups 3.11 while you are simultaneously logged in to

the NOVELL server.  When the time comes to configure the

network portion of Windows For Workgroups, choose a Xircom

Pocket Ethernet Adapter II (ODI/NDIS3).

When choosing drivers, make IPX/SPX the default protocol.

Choose setup and choose ODI interface.  Windows For

Workgroups will modify your CONFIG.SYS, AUTOEXEC.BAT,

SYSTEM.INI, PROTOCOL.INI and NET.CFG files.

Edit the SYSTEM.INI and remove the [netcard] and [transport]

lines in the NETWORK DRIVERS section (denoted by Brackets

[]).  When finished, it should match the stub in the

SYSTEM.INI below.  Modify the PROTOCOL.INI, NET.CFG and

AUTOEXEC.BAT replacing any reference to PE2ODI with CEODI.

Reboot the machine, log in to the NOVELL server and then

launch Windows.  After Windows launches, log in to Windows

For Workgroups.

EXAMPLES:

************************************************************

*                    CONFIG.SYS                            *

************************************************************

device=C:\WINDOWS\HIMEM.SYS

device=C:\WINDOWS\EMM386.EXE noems x=d000-d1ff

FILES=50

BUFFERS=32

FCBS=16,8

STACKS=9,256

DEVICE=C:\WINDOWS\IFSHLP.SYS

************************************************************

*                     AUTOEXEC.BAT                         *

************************************************************

C:\WINDOWS\net start

cd \windows

cecfg driver=odi

lsl

ceodi

ipxodi

C:\WINDOWS\odihlp.exe

netx

f:

login

C:\WINDOWS\SMARTDRV.EXE /X

doskey

path C:\WINDOWS;c:\dos;c:\

SET TEMP=C:\WFW\TEMP

prompt $p$g

************************************************************

*                     SYSTEM.INI                           *

************************************************************

[Network]

winnet=wfwnet/00025100

multinet=netware3

FileSharing=Yes

PrintSharing=Yes

LogonDisconnected=yes

EnableSharing=Yes

UserName=PATTI

Workgroup=XIRCOM

ComputerName=PATTI

Comment=patti

logonvalidated=no

reconnect=yes

[NetWare]

NWShareHandles=FALSE

RestoreDrives=TRUE

[network drivers]

devdir=C:\WINDOWS

LoadRMDrivers=No

[Password Lists]

*Shares=C:\WINDOWS\Share000.PWL

PATTI=C:\WINDOWS\PATTI.PWL

[NWNBLINK]

LANABASE=0

**********************************************************

*                    PROTOCOL.INI                        *

**********************************************************

[network.setup]

version=0x3110

netcard=ms$xirc,1,MS$XIRC,4

transport=ms$nwlinknb,NWLINK

transport=ms$netbeui,NETBEUI

lana0=ms$xirc,1,ms$nwlinknb

lana1=ms$xirc,1,ms$netbeui

[MS$XIRC]

[NWLINK]

BINDINGS=CEODI

[NETBEUI]

BINDINGS=CEODI

LANABASE=1

[net.cfg]

PATH=C:\WINDOWS\NET.CFG

[Link Driver CEODI]

data=   Frame Ethernet_802.3

data=   Frame Ethernet_SNAP

data=   Frame Ethernet_802.2

data=   Frame Ethernet_II

************************************************************

*                           NET.CFG                        *

************************************************************

link driver ceodi

frame ethernet_802.3

frame ethernet_802

====== End of Windows for Workgroups instructions======

LAN REQUESTER INSTALLATION (for IBM OS/2 LAN Server version 3.0)

================================================================

The following notes covers installation of the CENDIS.OS2 driver into

the above mentioned NOS.  This information is not provided in the

manual.

1. Follow the instructions in your OS/2 documentation for an ADVANCED

LAN Requester installation.

2. When installing and configuring the Requester component, under

Select Requester Services, choose "Neither of the above" for

messaging services, unless you know you need them and have the

information required for configuring these services.

3. When you reach the Configure window, select LAN Adapter and

Protocol Support, then under the OPTIONS window choose Copy

additional network adapter drivers.

4. On the Copy Additional CreditCard Drivers window, type in the path

A:\NDIS, then insert the Xircom CreditCard Drivers diskette in drive

A: and follow the prompts.

5. When the Options window is displayed again, select Configure

Workstation and choose the Xircom CrediCard Ethernet Adapter with

IBM OS/2 NETBIOS protocol.

6. Follow the prompts to complete the installation.

7. Shut down OS/2 and reboot the PC.

SPECIAL SOFTWARE NOTES

======================

***                     MEMORY MANAGERS                           ***

*** When using a memory manager be certain to exclude the memory  ***

*** range being used by the CreditCard adapter.  An 8k range will ***

*** need to be excluded with in C000 to E800.                     ***

example:   device=c:\dos\himem.sys

device=c:\dos\emm386.exe x=D000-D1FF

*** This example excludes the D000 segment that is used by the    ***

*** CreditCard in a default configuration.  If the CreditCard is  ***

*** configured for a memory address different from the default be ***

*** certain to exclude that memory address range in use.          ***

- The OS/2 drivers available on the disk are intended to support OS/2

versions 2.0 and above.  These drivers do not support Microsoft OS/2

version 1.3.

- When using the CEODI.OS2 driver in a OS/2 2.1 workstation environment

for attaching to a Netware 4.01 server, we have observed in certain

situations trying to login will generate an error "can not load a

segment of the program login".  If this occurs we recommend using

NWTOOLS.EXE in the netware subdirectory, on the workstation, to login

to the NW 4.01 server.

- The drivers contained on this disk support promiscuous mode which

allows the drivers to be used with Lan Analyzer packages.  However,

the drivers only capture valid packets, the drivers will not report

those packet that are short, runt or have CRC errors.

- During installation of Netware Requester version 2.01 for OS/2, when

prompted for the source drive, which has default A:, insert the

Xircom CreditCard disk and hit return following prompted instructions.

The CEODI.OS2 driver is located in the root of this diskette as well

in the ODI subdirectory for an easier installation.

MACHINE SPECIFIC NOTES

======================

NEC Ultralite VERSA

-------------------

If you are using Card and Socket services on the machine we recommend

setting the Xircom's CreditCard memory address to 'DE000'.  See the

Xircom manual appendix 'C' for configuration of memory keyword.

Sharp 6781 and Dell 320 Sli

---------------------------

If you are experiencing problems try the following.

The above laptops may have an early version of Socket Services, built

into the BIOS, that is not compatible with newer PCMCIA software

specifications.  In this case, either the NET.CFG file must be edited to

include the keyword string 'socketservices NO', or the PROTOCOL.INI

file must include the keyword string 'socketservices = NO'.  To use the

Packet driver load the drivers in the following manner:

cecfg driver=packet socketservices=no

cepd  socketservices=no

** Sharp 6781 only **

We recommend using memory segment D000 for the PCMCIA Card on the Sharp

6781, this is the default memory assignment.

The Sharp BIOS should also be configured.  On page 2-of-4 in the setup,

underneath the section heading "Floppy Drives & IC Card," configure

the third entry as follows:

Device 3 (Other):   IC Card COM Port:   COM3 I/O 3E8H, IRQ 5

Also on page 3-of-4 in the setup have the heading EMS Pageframe: DISABLED

Toshiba T3300SL and Grid Convertible

------------------------------------

The above laptops must be run in 8-bit mode.  This requires that the

'iowordsize' keyword be set to '8' in either the NET.CFG or PROTOCOL.INI

file, or on the command line for Packet driver.

Packet driver example:

cecfg driver=packet iowordsize=8

cepd iowordsize=8

** Toshiba T3300SL only **

The BIOS also must be set up to accommodate I/O cards.  To set up the

PCMCIA slot, select the SETUP option in the program TEST3.EXE.  Change

the option on the 'Card Modem' entry to 'Not Used'.  This prevents

BIOS from enabling the PCMCIA slot for a proprietary Toshiba modem card.

Toshiba 1950, 4500, and 4600 series

-----------------------------------

The Toshiba laptop may have a version of Card and Socket Services that is

not compatible with the newer specifications.  When using socket services

1.0, card services release 2.0 version 1.0 from Toshiba do the following.

Use the Keyword 'SOCKETSERVICES' and 'CARDSERVICES' set to 'NO' to allow

the Xircom CreditCard to bypass card and socket services, see Xircom

manual 'Appendix C' for configurations of these keywords for the various

drivers.  Or remove card and socket services from the config.sys of the

laptop.

CompuAdd 425tx

-------------------------------------------------------------------------

To support the CompuAdd notebook using a Databook "TCIC" PCMCIA

controller use the following configuration.

- CEODI.COM "ODI" driver support:

Append the following lines to the end of the net.cfg file.

link driver cecfg

TCIC 2C0

- CENDIS.EXE

Contact Xircom Technical Support for an update or specific

instructions for using this driver on a CompuAdd 425tx.

XIRCOM TECHNICAL SUPPORT

========================

To obtain technical support for your Xircom product, please call or send

a facsimile to the appropriate number listed below.  Access the Xircom

INFOSYS BBS at up to 14400 bps, No parity, 8 data bits, 1 stop bit

(N-8-1).

Corporate Offices        Technical Support and Information

-----------------------  ---------------------------------------------

Xircom, Inc.             North and Latin America... TEL:  800-874-4428

Corporate Headquarters   International ............ TEL:  818-878-6280

26025 Mureau Road        24-Hour Facsimile......... FAX:  818-878-7175

Calabasas, CA 91302      24-Hour Data Lines ....... BBS:  818-878-7618

TEL: (818) 878-7600   24-Hour Internet.......... INT: cs@xircom.com

FAX: (818) 878-6630   CompuServe                 CIS:  GO XIRCOM

HOURS:  6:00 a.m. to 4:30 p.m., Monday through Friday (Pacific)

Xircom Europe NV         Europe, Mid-East, Africa.. TEL:  32 3 326 3494

Bisschoppenhoflaan 82-84 24-Hour Facsimile......... FAX:  32 3 326 3150

2100 Antwerp, Belgium    24-Hour Data Lines........ BBS:  32 3 326 2368

TEL: 32 3 326 34 94

FAX: 32 3 326 31 50

HOURS:  8:30 a.m. to 5:30 p.m., Monday through Friday

8:30 a.m. to 4:30 p.m., Friday

Xircom Asia LTD.         Asia Pacific.............. TEL:  852-525-2078

1604 Centry Square       24-Hour Facsimile......... FAX:  852-525-8955

1-13 D'Aguilar Street    24-Hour Data Lines........ BBS:  852-537-6048

Hong Kong

TEL: 852-525-2078

FAX: 852-525-8955

HOURS: 9:00 A.M. TO 5:00 P.M., Monday through Friday

---------------------------------------------------------------

Thank you for making Xircom a part of your network.

------------------------------------------------------------------------

Xircom acknowledges all tradenames and trademarks used in this document

as the property of their respective owners.

===============================[  END  ]================================