WeatherMan:

A Radiofacsimile Reception Program

for

IBM PCs and Compatibles

Table of Contents

(Find topics by searching for the topic title.  Be sure to include the

brackets in the search string.)

[Notice]

[I]...........What Is Wefax?

[II]..........How WeatherMan Works

[III].........Connecting a Radio to a Sound Card

[IV]..........Creating a Configuration File

[V]...........Running WeatherMan

[VI]..........WeatherMan Keyboard Commands

[VII].........The SHOWPIC Program

[VIII]........The RAW2PCX Program

[IX]..........WXDEMO.EXE

[Appendix 1]..Weather Facsimile Frequencies

[Appendix 2]..Troubleshooting Q&A

[Appendix 3]..VOC Driver Files and Sound Cards

[Appendix 4]..About the Sample Pictures

[Notice]

*****

Notice, Part 1-- The Plea for Cold, Hard Cash:

The computer programs and documentation described in this document are

neither free, nor are they in the public domain: they are shareware.

In this particular case, the term "shareware" means that you are welcomed

and encouraged to use the programs for a trial period of not more than 30

days.  If, after or during the trial period, you decide to continue

using the programs, you should send a check (drawn on a U.S. bank) or

a U.S. Postal Money Order in the amount of $20.00 (US currency) to the

program's author:

Paul Hitchcock

2309 Blake Street, #308

Berkeley, CA 94704

*****

Notice, Part 2-- WeaselSpeak, or The Ugly Legal Stuff:

The programs and documentation in this package are distributed "as-is",

with no warranty expressed or implied, including, but not limited to,

merchantability or fitness for a particular purpose.  Any and all

damages arising out of the use of the programs and/or documentation

are the responsibility of the purchaser and/or user of the programs

and documentation.

All programs and documentation in this package are Copyright (C) 1994

by Paul Hitchcock

"Sound Blaster" is a registered trademark of Creative Labs, Inc.

"PAS 16", "ThunderBoard", and "ProAudio Spectrum" are registered

trademarks of Mediavision, Inc.

*****

[Introduction]

WeatherMan is a computer program for IBM PCs and compatibles which,

when used with a Sound Blaster-compatible sound card, lets you receive,

display, and store to disk weather photos and charts of the type

broadcast over the high-frequency (HF) shortwave bands.

To use the program you must have the following items:

1) A shortwave receiver, equipped with a beat-frequency

oscillator (BFO)

2) A 100% Sound Blaster-compatible sound card

3) A VGA video card and monitor capable of displaying

640x480 16-color graphics

4) An IBM PC or compatible 386/486 computer, running at

33 MHz or faster

5) A VOC file driver program (usually supplied with the

sound card).  See Appendix 3 of this document.

6) A patch cord for connecting the shortwave receiver to the

sound card

[I]  What Is Wefax?

Wefax, or weather facsimile, is the name for a process by which

photographs and other graphical material are distributed via radio.

At the transmitter, an object is scanned line-by-line at a constant

rate by an optical sensor; the output of the sensor is used to

control the frequency of an oscillator in such a way that the

oscillator's output is linearly proportional to the amount of light

refected from corresponding areas of the scanned object.  The

frequency of the oscillator ranges from 1500 Hz for pure-black areas

to 2300 Hz for white regions.  This "frequency-modulated subcarrier"

is then used to frequency-modulate a radio wave which is then

broadcast.

At the receiver the process is reversed: the wefax signal is

demodulated, resulting in a set of tones whose frequencies

are again linearly proportional to the light and dark areas of the

original scanned object.  If an FM receiver is employed to detect

the signal, the original frequencies are faithfully reproduced.

More commonly, however, the signal is demodulated using an AM

receiver equipped with a beat-frequency oscillator (BFO).  While

FM reception generally results in better noise rejection, the BFO

method gives the listener the ability to choose (within limits,

of course) the center frequency of the 800 Hz audio band; this

capability can be very useful when trying to receive a fax in

the presence of an interfering signal.  BFO-based reception is

assumed throughout the remainder of the discussion.

For the purpose of this discussion, a wefax broadcast can be

defined by two parameters:  (1) its transmission rate (or "drum

speed")  in lines per minute (lpm) and (2) its index of correlation

(IOC).  The IOC is essentially the ratio of the transmitted

picture's line width to its line height.  At the present time,

nearly all U.S. (and many foreign) wefax broadcasts are transmitted

with a drum speed of 120 lpm and an IOC of 576. There are a number

of other radiofacsimile formats, but the present version of WeatherMan

can only receive faxes broadcast at 120 lpm.

A more complete description of weather facsimile broadcasting can

be found in the 1990 edition of the ARRL Handbook.

(Note: The term "wefax" is not really appropriate for describing

HF weather facsimile broadcasts, but since "everybody" uses it in

this context, I'll follow the same convention.)

[II] How WeatherMan Works

Sound Blaster-compatible sound cards are usually supplied with a

"VOC file driver" program which-- thanks to the miracle of

DMA-interrupts-- lets you play "VOC" files that have been loaded

into memory while your computer is running another completely

independent program.  A basic VOC file is nothing more than a set

of numbers representing analog voltage levels that have been

sampled from, say, your soundcard's mic input.

Just as the VOC driver is capable of playing sounds in the

background, it's also able to record sounds as a background

task as well.  WeatherMan uses this facility to record an incoming

line of fax material at the same time it's decoding and displaying

the last received line.

WeatherMan measures the time interval between consecutive zero-point

crossings of the recorded samples to calculate an average audio

frequency over a given number of samples.  Once the frequency has

been determined, the program assigns a proportional color number

from 0 through 15 to the frequency, plots the point, then goes

back for more samples.  This process continues until the program's

user takes some intervening action.

In it's current incarnation, WeatherMan samples the mic input at a

rate of 10,000 samples per second and uses, on average, 8 samples

per pixel.  From these numbers, one would assume this would yield a

picture 625 pixels wide (5000 samples per half-second line divided

by 8).  Unfortunately, it's not quite that simple:  Due to variations

in the sampling rate of individual sound card/computer combinations

and due also to the need to start the next line in a timely fashion,

the actual width of the picture is 623 pixels.  This means that two

pixels of every line are lost.

Now for the good news-- although two pixels per line are not plotted,

if the picture being received is properly aligned, no useful data is

lost.  The nature of fax transmission requires that useful information

can't be transmitted over the entire line: for example, with a

mechanical fax receiver, time must be allowed for the pen or plotting

stylus to return to the margin to begin drawing the next line.

Now for the bad news-- compromises had to be made.  In order for

WeatherMan to run on the greatest number of machines, I decided

that the screen resolution should be no greater than 640x480 in

16 colors.  Unfortunately, since a vertical resolution of 480 pixels

would result in displaying only a small portion of a weather photo,

I also decided that the program would plot only one line for every

two received. Lines are not skipped, however: a smoothing algorithm

is used on every two received lines to produce a smooth-appearing

single line, which is then plotted.  Using this method, about

two-thirds of a complete weather photo can be displayed on a

640/480 screen.

Now for the completely impartial(but interesting) news-- forget what

was said about white being the highest frequency and black being the

lowest.  In commercial fax receivers, those conditions undoubtedly

obtain.  However, I found that the best results were achieved when

receiving faxes on the other sideband; hence, WeatherMan employs a

reversed palette in which darker colors represent higher frequencies.

As will be seen later on, WeatherMan's configuration file can be

modified to comply with the "standard" if that's what you need.

[III] Connecting a Radio to a Sound Card

To use WeatherMan, you must connect the audio output of your radio

receiver to the microphone input of your sound card.  But first:

*****************************************

*                                       *

*     Warning!  Warning!   Warning!     *

*     Warning!  Warning!   Warning!     *

*     Warning!  Warning!   Warning!     *

*                                       *

*****************************************

Got your attention?  Good, because this is very important:  unless

you completely understand what you are doing when you connect any

two pieces of electrical equipment together :

THERE IS AN INHERENT RISK OF PERMANENT DAMAGE TO EITHER OR BOTH

DEVICES OR TO YOURSELF!!!!  Fear not, but read on...

In most cases, connecting a shortwave receiver to a sound card is

simple:  just run a patch cord from the microphone output of the

shortwave receiver to the microphone input of the sound card.

I have an old, battery-operated Radio Shack DX-440 which I use

for wefax (yes-- it does have a mic output, albeit an undocumented

one) and this method works just fine.  But what if A) your receiver

has a headphone jack, but no mic output, or B) your receiver is NOT

battery-powered and must be powered from the AC line, or C) your

receiver has neither a mic output nor a headphone jack?

Case A: headphone jack, no mic output

A sound card's mic input typically requires a signal level in the

10-20 millivolt range, but a headphone jack output can supply 1 or

more volts.  Since you don't want to risk damaging your sound card,

you must reduce the level of the signal.  The simplest solution is

to use a resistive voltage divider between the receiver and the

sound card; better yet, use a step-down audio transformer.   For the

DX-440's headphone jack, I've found the circuit below to give

acceptable results

10000 ohm resistor

-----------

O--|           |--------*-----------------O S

-----------         |                   O

---                  U

R                            |   |                 N

A                            |   | 1000 ohm        D

D                            |   | resistor

I                             ---                  C

O                              |                   A

|                   R

O-----------------------*-----------------O D

Case B: NOT a battery-powered receiver

If your receiver has an internal power transformer that isolates

its circuitry from the power line, it's probably OK to connect its

mic output to the sound card.  If you're not sure, use an audio

transformer between the radio and the sound card.  No, you can't

just use the resistive divider above.  I have an old Hallicrafters

AC/DC shortwave receiver and several years ago I got an unpleasant

surprise when I absent-mindedly connected the ground lead of an

oscilloscope to the Hallicrafter's chassis (yes, I did know better,

but my brain was taking an unscheduled holiday).  Spark city.  Can

you say "ozone"?  Please believe me: there is no better way to turn

your computer into a smoking hunk of silicon and plastic than to

apply 120 volts AC to its innards.

Case C: neither headphone jack nor mic output exists

If you're not comfortable and familiar with modifying and/or repairing

electronic equipment, get a new radio before trying to use WeatherMan.

If you possess the needed skills, you already know what to do.

I suspect most of the people who will use WeatherMan are technically

knowledgeable folks who will have absolutely no problem connecting

their radios to their sound cards.  If you are in that class, let me

apologize for the simplistic admonitions.  But if you are not 100%

certain you know what you're doing, find someone who does know and

ask for help before trying to use the program.

[IV] Using WeatherMan : Creating a Configuration File

Now that your radio and sound card are connected together, there's

one more thing you have to do before running WeatherMan: edit the

program's "configuration file".  This file contains details about

your hardware setup and display preferences.  A sample configuration

file is given below (it's a copy of the one supplied with the program

set):

1)    812   <----------------- sweep delay

2)    1050  <----------------- lowest audio frequency

3)    1800  <----------------- highest audio frequency

4)    $220  <----------------- I/O port used by sound card

5)    5     <----------------- interrupt used by sound card

6)    C:\CT-VOICE.DRV    <---- path and name of VOC driver

7)    0      63      63      63  <- <color no.> <R> <G> <B>

8)    1      59      59      59     each RGB value must be

9)    2      55      55      55     greater than 0 and less

10)    3      50      50      50     than 63

11)    4      46      46      46

12)    5      42      42      42

13)    6      38      38      38

14)    7      34      34      34

15)    8      29      29      29

16)    9      25      25      25

17)    10     21      21      21

18)    11     17      17      17

19)    12     13      13      13

20)    13      8       8       8

21)    14      5       5       5

22)    15      0       0       0

Let's go through the list (the line numbers, e.g., "20)", are for

reference only and should not appear in the actual configuration file):

Line 1 : "sweep delay"- Since I can't think of a technical-sounding

name for this entry, I'll call it what it is: a "fudge factor".

Since Weatherman samples slightly less than a full line, the sweep

delay number is used in an empty counting loop inside the program to

ensure the received picture won't be skewed as it's being received

and displayed.  How do you know what sweep delay number to use?

You don't-- you have to determine it by trial and error.  Fortunately,

I can offer some guidelines:

Suppose you are receiving a fax composed of vertical lines that is

supposed to look like this:

|       |       |

|       |       |

|       |       |

|       |       |

|       |       |

but instead it looks like this:

\       \       \

\       \       \

\       \       \

\       \       \

\       \       \

In this case, the sweep delay number should be INCREASED.

Conversely, if the picture looks like this,

/       /       /

/       /       /

/       /       /

/       /       /

/       /       /

the sweep delay number should be DECREASED.

Lines 2 and 3: "lowest audio frequency" and "highest audio frequency"--

These define the white-to-black frequency range and have two principal

uses: 1) by widening/narrowing the range, you can stretch/compress the

contrast of the received picture, and 2) by  shifting the center of

the range up or down, you can possibly reduce interference caused by

transmissions on adjacent frequencies. Experiment with various

combinations to achieve the best results.  The numbers given are

in cycles per second (Hz).

Line 4: Sound card I/O port- If necessary, change this number in

accordance with your particular sound card installation.  Consult

your sound card's reference manual for details.  Also, note that

the prefacing "$" denotes a hexadecimal number.

Line 5: Sound card interrupt number- Change if necessary, as in Line 4.

Line 6: Path and name of VOC driver- Insert the DOS path/file name

combination for your own configuration here.  For example, if you

have a Sound Blaster and your driver is in directory "SB" on drive

D:, then Line 6 would read

D:\SB\CT-VOICE.DRV

If the program can't find the driver, it will stop and tell you so.

Lines 7 through 22: color palette- This is pretty self-explanatory.

The default palette is for a 16-level gray scale.

The format of the configuration file requires that comments be

separated from the file elements by at least one space and the file

should not contain any empty lines.

[V] Running WeatherMan

Once you have created a configuration file, WeatherMan is run using

the following syntax:

WXMAN

or

WXMAN <configuration file name>

If you choose the first option, "WXMAN", you must have a configuration

file with the name "WXMAN.CFG" located in the same directory as the

program file.  The second option lets you choose the name and path of

a different configuration file.  Suppose, for example, you have a

reversed-palette configuration file named "WXMANREV.CFG" in directory

"CONFIGS" located on drive C:.  You then run the program with the

command "WXMAN C:\CONFIGS\WXMANREV.CFG".

With your shortwave receiver turned on and connected to the sound card,

activate the receiver's BFO, then tune in a wefax station.  Adjust the

BFO up or down for the best picture.  When tuning the BFO, it's usually

easiest to start at a high-frequency audio output and work back to the

proper frequency.

[VI] WeatherMan Keyboard Commands

Once WeatherMan is up and running, you have several commands at your

disposal

<G>rab a Picture

<C>lear Screen

<1>..<9> Delay Picture Reception

<P>osition Graphics Cursor

<H>elp

<ESC> Exit WeatherMan

<+/-> increase/decrease sweep delay

Each of the commands is activated by pressing the letter key within

the "<>" symbols. Case is unimportant.

<G>rab a picture- Pressing "G" saves the currently-displayed picture

to disk.  The saved picture is time- and date-stamped

at the bottom of the screen.  Saved picture files are

given the name "GRABxxx.RAW", where "xxx" denotes the

number of the saved picture.  When WeatherMan starts

up, it searches the current directory for previously-

saved pictures and sets its first file name to the

next highest number to avoid overwriting old files.

<C>lear Screen- Pressing "C" clears the screen and the current

image is lost forever.

<1>..<9> Delay Picture Reception- Pressing "1" through "9" causes

sampling of the incoming fax to be

delayed an amount proportional to

the value of the selected key.

These commands let you "walk" the

true edge of the incoming fax to the

left side of the screen.

<P>osition Graphics Cursor-  After positioning a picture with the "1"

through "9" keys, pressing "P" moves the

graphics cursor to the top line of the

screen, without clearing the screen.

<H>elp- Pressing "H" displays a command summary.  No fax data is

received or displayed while the "H" command is in effect.

<ESC> Exit WeatherMan- Pressing the escape key unconditionally exits

the program; you are not prompted to save the picture first.

<+/-> Increase/Decrease sweep delay- use these two keys whenever

your picture is slightly skewed and you don't want to exit

and create a new configuration file.

WeatherMan's RAW picture file format is exceedingly simple, consisting

of a file of 640*480=307,200 bytes, with each byte representing a

color number in the 0 to 15 range. The file is a record of consecutive

pixel values starting at the upper left-hand side of the screen (0,0)

and extending to the lower right-hand side (639,479).  This format

should make it easy for you to write your own programs for manipulating

captured picture data.

The utility program RAW2PCX.EXE (see section [VIII]) can be used to

convert the RAW format into the PCX format used by many PC

paint/image-processing programs.

[VII] The SHOWPIC program

Having saved a bunch of pictures to disk, it would be nice to see what

they looked like again, wouldn't it?  That's the purpose of SHOWPIC

and its syntax is

SHOWPIC [-p] <picture file> <picture file>...

where the optional -p switch loads a palette from the file SHOWPIC.PAL.

The format of SHOWPIC.PAL is the same as in lines 7 through 22 of the

configuration file.  See the sample palette file for details.

Example 1:

SHOWPIC GRAB0.RAW GRAB11.RAW GRAB2.RAW

displays the pictures GRAB0.RAW, GRAB11.RAW, and GRAB2.RAW consecutively

using the built-in gray-scale palette.

Example 2: SHOWPIC -p GRAB1.RAW TEST.RAW

displays the two given pictures using the palette contained in SHOWPIC.PAL.

You can move to the next picture in the set at any time by pressing any key.

[VIII] The RAW2PCX program

To convert a picture file to a PCX graphics file use either of the commands

RAW2PCX  <input file name>, or

RAW2PCX  <input file name>  <output file name>

Example 1:

RAW2PCX GRAB0.RAW

This command creates a new file "GRAB0.PCX" in the PCX format.

Example 2:

RAW2PCX GRAB0.RAW  TEST.PCX

Same result as in Example 1, but the output file is named "TEST.PCX"

instead.

Warning: RAW2PCX will overwrite any existing file having the same

name as the output file.

*****

[IX] WXDEMO.EXE

Wefax broadcasts have a very distinctive sound and once you've heard

one, you'll probably never forget what it sounds like; unfortunately,

the sound is difficult to describe in words.  That's where the WXDEMO

program comes in.

Syntax:

WXDEMO <picture file>

WXDEMO takes a single argument, the name of a picture file, then

simulates the reception of the picture through your sound card and on

your monitor.  If you've never heard a fax transmission before, WXDEMO

should help you learn what to listen for as you tune through the

shortwave bands.

[Appendix 1] Weather Facsimile Frequencies.

The following is a very small list of stations that transmit weather

photos and charts during the day.  It is not at all inclusive: there

are literally dozens of other stations across the HF bands.

(All frequencies are in kilohertz.)

Frequency (KHz)     Location

---------          ---------

6453             California

8682             California

9090             California

11090             Hawaii

16135             Hawaii

8080             Virginia

15959             Virginia

20015             Virginia

[Appendix 2] Troubleshooting Q&A

Q: Why isn't anything being displayed?

Make certain your radio is turned on and the BFO is activated.

Check the connection between the radio output and sound card mic

input. If you have a MediaVision PAS16 card, use the command

"PAS SET MIC TO 100" before running WeatherMan.

Q: Why does the picture look like a photo negative?

Either you have loaded a reversed palette in the configuration file

or you are tuned to the wrong sideband.  Adjust the file or the BFO

accordingly.

Q: Why is the picture grainy?

The signal may be weak, or there may be a source of interference nearby,

such as an electric motor, fluorescent light, or the like.  Use a good

antenna and/or increase the mic input level, and/or remove the source

of the noise.

Q: Why is the picture "ghosty"?

Just like television, if a signal is received along two or more paths

having different effective lengths, you'll get ghosts.  There's no

cure, except perhaps to re-orient the receiving antenna.

Q: Why are there 30 wavy vertical lines across the picture?

You're picking up 60 Hz (power line) interference from somewhere,

possibly from a ground-fault loop.  Try transformer-coupling the

receiver to the sound card and make sure you're using a good quality

shielded cable.

Q: I've set the "sweep delay" constant.  Why do my pictures still come

out slightly skewed at times?

Unfortunately, a sweep delay constant that works well one day may not

work as well the next and may have to be changed.  The speed at which

WeatherMan runs depends on many factors including the memory manager

you use, the kind and number of TSRs you have installed, and even the

temperature.  Generally speaking, however, a sweep delay constant

which works OK at the start of a session will continue to work fine

for at least several hours.  For minor corrections to the sweep delay

constant, use the "+" and "-" keys while the program is running.

Q: I get great reception until I turn on the computer.  What gives?

The computer is a wonderful source of radio waves.  Keep the antenna

as far as possible from the computer and connect it to your radio

with coax.  The coax shield should be connected to a good ground.

Q: When my radio is on, why do I hear Skippy the Clown urging me to

do bad things?

Uh huh... That's kind of out of my area of expertise...

[Appendix 3] VOC Driver Files and Sound Cards

WeatherMan has been tested with two sound cards: Sound Blaster (v1.5) and

the Sound Blaster emulation mode of the ProAudio Spectrum 16 (PAS16).

While both cards work, the PAS16 card resulted in significantly better

performance.  This is not a criticism of the Sound Blaster (I have

and use both cards), but simply a statement of fact.

Regarding VOC drivers, WeatherMan has been tested only with

CT-VOICE.DRV, version 1.0 available from Creative Labs, Inc and

with CTV-TB.DRV, a ThunderBoard driver from MediaVision, Inc.

MediaVision PAS16 owners should use the "CTV-TB.DRV" ThunderBoard

program instead of the PAS16 driver.  This driver program can be

obtained from the MediaVision BBS (510-770-0968) by downloading the

file "CTV.ZIP".  Incidentally, this program was found to work with

the Sound Blaster, as well.

[Appendix 4] About the Sample Pictures

The sample pictures were received using the following extremely

non-ideal setup:

Radio      : Radio Shack DX-440

Sound Card : ProAudio Spectrum 16, Sound Blaster Emulation

Antenna    : 25-ft. random wire antenna, located indoors

Location   : Third floor of an apartment building

Problems   : 1) antenna only 6 ft. from computer

2) power company transformer about 50 ft away

from computer.

Even with the setup above, I think the pictures are pretty good; I'll

leave it up to you to imagine what they might have been like using an

excellent shortwave receiver and a good antenna.

***************

Thanks for trying WeatherMan!