This file is a reprint from a message taken off the InterNet.

From: John Baum <jbaum@freenet.columbus.oh.us>

Newsgroups: mail.list.tapr.aprs

Subject: [APRSSIG:171] Documentaion for APRS

Date: Tue, 14 Feb 1995 20:27:43 -0600

Sender: news@ke6et.clark.net

Reply-To: aprssig@tapr.org

There is no "real" first time step by step user guide in the docs

so I wrote one and posted on my BBS in the same directory where the program

is located, and have "attached" it to this message.

I developed this to help others avoid the same mistake that I

had, as well as the problems that others ran into on getting the program

going.

Bob is right, in that writing documentaion for the program is hard.

It is an ever growing and changing beast. I don't think that there is a

single person out there who knows about all of the hidden treasures in the

program, (Including WB4APR).

For my own use, I printed out all the docs that I found to effect

my operations and put them in a 3 ring binder (Hey Bob, how about a 70 char.

wide line instead of 80 for use who print them out, this way we won't have to

reformat them to keep from punching holes in text). The first thing I do when

a new version arrives, is look to see what is new in the readme's, then I

print out the new ones for the binders.

There is a lot of information there, and it is scattered (like PHG

tables in the DF section). You would be surpised how much you can learn by

just scanning the files. BUT, until the program stops evolving (fat chance)

or someone steps up to either write the docs, our prints a "DUMMY'S GUIDE TO"

let us muddle thru.

I think that the best idea would be to have users who are

knowledgeable in the different aspects of the program, write a guide to that

part of the program. Face it, we are all genius's in our own little areas, so

lets share that knowledge with the rest of the world.

Content-Description: First time user guide

APRS is a different type of packet radio program. Once it is up

and running, it's easy to use. The main problem is getting it up and running.

The following will walk the first time user through the "Unpacking" of the

program from it's distribution form, and the initial start up of the program.

It is in no ways a comprehensive guide to using the program, the dozen or so

Readme files that come with the program do that.

This program was written by Bob Brininga, WB4APR.  It is a

shareware program, which means if you like, the author would appreciate it if

you were to register it (still cheap at $19).

UNZIPPING THE PROGRAM

APRS is normally found in two forms. The first form, is the full

size program called APRSxxx.ZIP, where xxx is the version number. This has

everything in it. The next form is called xxxTOyyy.zip, where xxx is one

version number, and yyy is a second version number (example 403to503.zip).

This is an upgrade, which will update program, and add any new files added

sense xxx.  It is normally 1/2 the size of the full blown program.  This is

handy if you are downloading the program from a BBS. (B.T.W., if you are using

a later version than xxx, and want to upgrade to yyy, the upgrade will still

work.)

To unzip the program, you must have at least version 2.0 of

PKUNZIP. Any earlier versions will not be able to unzip the program. Because

the program was zipped up with the directories intact, it must be unzipped

with the "-d: switch. (i.e.: PKUNZIP -d a:\APRS504.ZIP). This creates the

needed directories, and places the files in them. If you don't use the "-d",

everything will end up in the same directory, and nothing will work. Ask any

of us "Old Timers", we've forgotten the -d on more than one occasions.

HOW TO START THE PROGRAM FOR THE FIRST TIME

To start the program, Just type "APRSxxx", where xxx is the version

number that you just unzipped.

Next you will be asked to enter your call sign

Next you will need to enter the Comm Port number that your TNC is

connected to. If you do not have a TNC, but want to look at the program,

enter "N".

Next it will ask what speed does your Computer talk to your TNC at.

Normally it will be 9600 baud. If you have it set for a different speed

(1200, 2400, 4800, etc.) type that it. THIS IS NOT THE SPEED OF THE TNC.

Next it will ask you what Band you want to work, HF of VHF.

Normally you will be on VHF. (Columbus operating Frequency is 145.79 MHz.)

Next it will ask you what type of TNC you are using. The MFJ and

PacComm are TAPR clones.

Next you will enter in your UTC time offset. For Eastern Daylight

Savings time, it is -4.

Next you will need to enter in the Comm Port number that your

GPS/LORAN receiver, Weather Station, or Dopler Direction finder is located.

These will work only if you have the proper registration numbers for them.

Otherwise type "N".

Now the program will draw a Map of Ohio. Move your cursor to where

you are located, using the arrow keys, and push the page down key to zoom in

on your location. New maps may pop up on the screen as you are zooming down.

Keep moving your cursor to your general location. If it moves off the

screen, hit the home key, and the screen will redraw to where the cursor is.

Once you have the cursor on top of your location, write down the

LAT & LON numbers located in the upper left hand of the screen. Once you have

done that, hit the "INSERT" key. Type "B" for Building, then "Q" for

QTH/home.

At this time it will then ask for a course, just hit enter, a

speed, again just hit enter, a comment, type in and short message about

yourself if you like, then hit enter, then DAY-TIME, again hit enter, and

finally it will ask if this is accurate. This is for your location. If it is,

type "Y", if not, type "N" and you will start over again with the cursor

location.

Once you are in the program, you can bring up the help screen by

hitting "F1" key. This brings up a menu along the bottom of the screen. Hit

the appropriate key for the help that you need.

To quit out of the program, hit "Q" then follow the instructions.

If you are not registered, you can save some time locating your qth

when the program first comes up, by putting your QTH's LAT & LON numbers in

the first two lines of the MAPLIST.APR file. (You will have to convert these

numbers to Decimal Degrees by dividing the Minutes and decimal minutes by 60.)

This is a simple ASCII Text file, and you can use most any word processor to do it.

If you have any questions about the program or the maps, I can be

reached at KB8JXO @ W8CQK.#CMH.OH.USA.

73 - John Baum - KB8JXO

Sysop - APRS OHIO BBS -  614-443-4249

The Ohio Mapmaker - KB8JXO - Sysop APRS OHIO LLBBS (614) 443-4249

Packet address KB8JXO@W8CQK.#CMH.OH.USA

The 137th most knowledgable person about A.P.R.S.