STANDARD MICROSYSTEMS CORPORATION

NetWare 2.15 or 2.2 Server &

Netware 286 Shell

For Ethernet LAN Adapters

RELEASE INFORMATION

Version 6.10             April 19, 1994

VERSION HISTORY

-- Verison 6.10 (Current Version) --

Fixes miscompare bug for IPX Shell.

-- Version 6.09  --

Fixed load problem when warm booting.

-- Version 6.08 --

Updates to Ethernet Lower Mac to support 795 based adapters

-- Version 6.07 --

Fixes bug in IPX Shell when running with PCAGENT, workstation

hangs or is not seen by PCAGENT.

No change to 2.2 Server.

-- Version 6.06 (July 12, 1993)

- Fixes Multicast Address bug in 2.2 Server

-- Version 6.05 (July 9, 1993)

- Fixes problem where Macintosh machines using Appletalk Phase

2 could not attach or logon after the 286 Server's media

protocol was change to E2 type with the ECONFIG utility.

- Fixes problems with address aliasing and transmit underrun in

8216 boards.

-- Version 6.04 (June 17, 1993)

- New version of Server & Shell for 8216 boards that is

compatable with more machines.

- Initialization code in Shell is released after loading, thus

the Shell takes up less system memory than previous versions.

-- Version 6.03 (6/10/93)

- Fixes data miscompare bug in Compaq Pro Signia machines

-- Version 6.02 (6/9/93)

- Fixes data miscompare bug with 8216 boards for both 2.2 Server

and IPX Shell.

-- Version 6.01 (5/21/93)

- Fix IPX Shell bug where driver will not attach to server

after system warm-start

-- Version 6.00 (5/14/93)

- New implementation of both 286 Server and IPX Shell designed

to support 8216 (Ultra) Family of LAN Adapters.

- New implementation of Appletalk Phase II in 286 server

-- Version 5.00 (11/13/92)

New IPX driver utilizing new design implementation.  Also,

supports SNMP processing.

-- Version 4.03

Fixed General Protection Interrupt errors when using Jumpers

config option.

-- Version 4.02 --

- Fixed false conflict between disk driver and jumpers config. (This

bug was present in both server and shell, however, there was no

comparable symptom in the shell driver).

- Fixed GPI errors when using fixed config (server)

- Fixed "Adapter not found" message that was reported when using MCA

adapters (server).

-- Version 4.00 --

Changed Signon string to SMC. Put software configured adapters

first in configuration table.

-- Version 3.12 --

- Fixed no NIC test failure when garbage was printed on a Micro

Channel machine if no adapter was found.

- Fixed problem with workstation driver to make it work with 32K

of RAM on the 8003EBT and 8003WT adapters.

- Fixed interrupt problem with driver that caused it to hang when

running on 16 bit AT bus adapters in 16 bit mode.

- Fixed a problem with the server driver that caused it to not be

linkable on certain Micro Channel machines.

-- Version 3.11 --

Modified server code so that four (4) drivers will now

be able to load on the server.  Added support for 8013EP/A

and 8013WP/A Micro Channel adapter boards.

-- Version 3.10 --

Added version string information so that Novell VERSION

program will be able to print out the signon string upon

execution.

-- Version 3.09 --

Changed overflow code to follow the June 1989 specification.

Changed file names and sign-on strings for AppleTalk Phase 1

and Phase 2 drivers.  The RAM base address of the following

non-software jumper configured adapters must be on a 32 Kbyte

boundary (i.e., C8000, D0000, D8000):

- 8003E

- 8003EBT

- 8013EBT

- 8003S

-- Version 3.08 --

Adds full 16-bit compatibility when using 16-bit Ethernet

adapters in conjunction with 16-bit video adapters.

This version contains modifications for the WD83C690 Network

Interface Controller.  This version or later versions must

be used with adapters which contain the WD83C690.  It is

recommended that the latest version of the driver be used

in all applications.

NOTE:  Use of adapters which contain the WD83C690 with

previous versions of this driver may lead to data

integrity problems.

There is a known problem during connection retry in situations

where the cable has been disconnected from a workstation LAN

adapter.  If the cable is disconnected and the user attempts to

access the network, the following error message will display:

"Error sending on network, Abort, Retry, Ignore?"

The cable must be reconnected, then "R" must be pressed twice

(rather than once) to retry connection and resume normal

network communication.

-- Version 3.07 --

The server driver was modified to work with an AppleTalk Phase I

Network.  To use this new capability, the AppleTalk VAPS must be

purchased from Novell. The server is built in the usual manner.

The workstation driver has been modified to share the LAN adapter

with the new 8003PKDR.EXE driver which conforms to the FTP

Software Packet Driver Specification. The 8003PKDR driver can

interface to TCP/IP software from FTP Software and other vendors

which support the FTP Packet Driver Specification. With these new

drivers, a workstation can communicate with Novell/IPX and TCP

nodes concurrently through a single 8003/8013 family LAN adapter.

For installation information, refer to the INSTALL.DOC file

included with the drivers.

The server and workstation drivers were both modified to work

with the new JUMPERS v2.00 utility supplied by Novell. With this

utility, both the shell and server can be configured to match

any jumper or software configuration that can be set on the

adapter.  The changes to the drivers reduce the amount of

configuration options available through the configuration

table.  If other configurations are desired, use the JUMPERS

utility to modify the linked IPX.COM or NET$OS.EXE files.

See the JUMPERS documentation furnished by Novell, and the

INSTALL.DOC file furnished with this driver.

The default packet format for StarLAN Networks was changed in

this release.  The current default is the 802.3 packet format

which is the same as the Ethernet format. This makes the default

incompatible with the previous driver's default packet format

for StarLAN. For information on changing the default packet

format for StarLAN refer to the INSTALL.DOC file.

-- Version 3.06  --

IBM PS/2-Models 25 and 30286 are supported in this version.

The Netware 386 MAP ROOT command is supported in this version.

This version is compatible with the Novell utility JUMPERS v1.00.

This utility may be used to reconfigure the IPX.COM file and

create configurations not contained in the driver configuration

table.  JUMPERS map may also be used to set the StarLAN packet

protocol. Refer to the INSTALL.DOC file for more information on

StarLAN packet protocols.

-- Version 3.05 --

Changed to work with the PS/2 Model 55 and the PS/2 Model 30.

-- Version 3.04 --

New functions have been added to increase the driver's

flexibility and some old problems have been fixed with this

release.  This driver supports the entire 8003/8013 family of

LAN adapters.

NOTE:  Do not use the NetWare 386 version 3.0 MAP ROOT

function with version 3.04 or earlier versions

of this driver.

In this release, the driver will read the configuration in-

formation directly from registers in software configured

XT/AT bus adapters (currently 8003EB and 8003W) in the same

way that Micro Channel adapters have been configured in the

past. This "Auto Configuration" feature eliminates the need

to reconfigure the driver to match different adapter config-

urations.

When using StarLAN adapters, this driver can be configured

to use either the Novell StarLAN driver compatible packet

format (AT&T StarLAN compatible) or the standard IEEE 802.3

packet format.

More information about these features as well as tips on how

to best use them can be found in the INSTALL.DOC file.

This driver contains fixes for two problems which existed in

previous drivers, including the Novell certified v2.11.3 driver.

Problem 1: (A workstation problem)

-  Regarding the IPX call to Cancel Request and occurred only

in certain NetBIOS applications which call this IPX function.

Problem 2:

-  Occurred only on certain adapter revisions when under high

load situations. This problem caused the server to "lock up"

or get a GPI interrupt.

-- Version 3.02 --

This release provides support for the 8013EBT adapter.  It

contains a completely new driver configuration table to support

adapters with 8, 16, and 32 Kbytes of RAM. The default con-

figuration has changed to IRQ 3, Base I/O 280, RAM at CA00:0

for 8 Kbytes.

There is one known problem with this driver, Configuration 12

(IRQ=2, I/O Base=220, RAM at C000:0 for 32 Kbytes) will NOT

function with an 8013EBT adapter in the file server.

-- Version 2.11.3 --

The driver files were moved to the root directory to comply

with new NetWare v2.15 installation procedures, and the

8003.LAN file was changed to specify floppy disk volume label

LAN_DRV_200 instead of LAN_DRV_002.  No software changes to

the actual driver were made.

EtherCard PLUS, TokenCard, and SuperDisk are trademarks, and SMC and

Standard Microsystems are registered trademarks of Standard

Microsystems Corporation. Other trademarks mentioned herein belong to

their respective companies.