Watch -- Multi-Function MIDI Analyzer

Copyright 1990,  Kevin Weiner

All rights reserved

Hardware

--------

Watch needs a minimum of 162K of available ram for the program code,

though 280K or more is recommended to permit large enough data areas for

processing incoming data.  Additional space is required to work with

standard MIDI files.

The following MIDI interfaces are supported: all Roland MPU-401

compatibles, IBM Music Feature card, Key Electronice MIDIator,

Creative Labs SOUND BLASTER (with MIDI adapter).

Directories

-----------

For ease of use, it is recommended that WATCH.EXE and its associated

files be kept in a subdirectory in the DOS path.  The help files are

assumed to be in the current directory, or else in the path.  When Watch

exits it updates the file WATCH.STP with any setup changes that were

made.  The first time the program is run, that file is created in the

current directory if it was not found.  Watch will also search the path

for the setup file, so if only one setup file is desired, it should be

moved to a path directory.

Device setup

------------

To select special MIDI device options, include the -d option on the

Watch command line.  The -d option identifies the MIDI interface(s) in

use, and optionally, the IRQ and base I/O address:

-dmfc      selects IBM Music Feature

-dmpu      selects MPU-401 compatible

-dmidiator selects MIDIator

-dsbmidi   selects SOUND BLASTER MIDI

(The first 3 letters of the device name are sufficient.)

The MIDIator cannot be detected automatically and the -dmid option

must always be specified if one is in use.

Optional IRQ and I/O port address information can be added following

the device id if necessary:

-dxxx:<irq>:<ioaddr>

For example, to define an MPU-compatible interface using IRQ 5 and I/O

address 332 (hex), use:

-dMPU:5:332

If only the I/O address is required but the default IRQ is alright,

use -dMPU::332.

The irq and ioaddr are not relevant to the MIDIator, though the first

option number can be given to indicate use of the com port 2 instead of

the default com 1.  (ex: -dmid:2)

To set up a device before hand, define the "opt1" environment variable

to be the -d option that would be included on the command line.  For

example:

set opt1=-dmpu:5:332

This line would generally be added to the autoexec.bat file to

select the option permanently.

Help

----

All data entry within Watch is self-prompting.  More complete usage

information is contained in the help files, WATCH1.HLP and WATCH2.HLP.

The help files may be viewed within the program via the F1 help option

on any screen.  They may also be viewed as normal text files, or

printed.  If using an editor to view them, be careful not to modify

them, since they are specially formatted and may not be viewable within

the program if modified.

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

Public (software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  PsL cannot debug pro-

programs over the telephone, though we can answer questions.

Disks in the PsL are updated  monthly,  so if you did not get

this disk directly from the PsL, you should be aware that the

files in this set may no longer be the current versions. Also,

if you got this disk from another vendor and are having prob-

lems,  be aware that  some files may have become corrupted or

lost by that vendor. Get a current, working disk from PsL.

For a copy of the latest monthly software library newsletter

and a list of the 2,000+ disks in the library, call or write

Public (software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

Orders only:

1-800-2424-PSL

MC/Visa/AmEx/Discover

Outside of U.S. or in Texas

or for general information,

Call 1-713-524-6394

PsL also has an outstanding

catalog for the Macintosh.