TeleShare DOS    REMOTE ACCESS  TSDIAL     Page 1

_______

____|__     |               (R)

--|       |    |-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

-----|   |   |---------------------

|___|___|    Member

TeleShare/DIAL   the TeleShare dialer program

==============================================

Updated: February, 1994  Version 1.50

TSDIAL  is the the Telephone dialer that comes with TeleShare.

TSdial's simple script language  and  command  line  switches,

will do  a  good  job  for  most  TeleShare  users.  TSDIAL is

designed to minimally and momentarily intrude into DOS  memory

space  while  it does its job, then quickly quit leaving other

software to continue.  TSDIAL is shareware.  If  you  are  not

familiar  with  shareware please read the SHR_WARE.DOC and the

REGISTER.DOC distributed with TeleShare.

Its not a good idea to use the dialer  with  a  direct  serial

connection.   It whines and thrashes around with the low level

DOS insides  of  your  computer  looking  for  a  non-existent

modem.   It  is  a good idea to run RS232OFF if you accidently

run the dialer without a modem.

Command line switches:

First, if you just enter the dialer's name or add a -H it will

display  its  Help  or  usage  screen,  which   provides   the

information repeated below.

Dialing:   TSdial [-Fyourfile -D433-5189 -T -B2400 -P1 -W -H]

-F  for the File switch.  The manual lists WAIT,

BEEP, etc. commands.  You can put commands in

the specified file for the dialer to follow.

-D  for the Dialing switch.

Command line dialing only uses the TSCONFIG.CFG file.

-T  for the Terminal switch.

-B9600 Baud override.

-P2    Port override.

-W[99] to Wait for a phone call.  [Optionally specify minutes].

TeleShare DOS    REMOTE ACCESS  TSDIAL     Page 2

-H  for this help.

The  -F file switch uses a text file of instructions that will

normally include a range of simple script commands.  It is the

most general purpose switch.   The  script  commands  will  be

provided after the command line switches.

The  -D  Dial  a  number switch is useful for simply dialing a

number or hanging up and the like.  For example,  for  hanging

up you  can  use  "TSDIAL  -DATH0".   The dialer automatically

reads in the general teleshare setup file and uses it to write

to the correct communications port.

The -T Terminal  switch  sets  up  the  dialer  as  a  minimal

terminal,  most  useful  for learning, testing, and diagnostic

activities.  Use the ESCape key to step through  the  terminal

commands.

The  -B  Baud  switch allows you to bypass the setup file with

the baud setting.

The -P Port number switch allows you to bypass the setup  file

with the Port number.

The  -W  Wait  for  a call switch simply sets the dialer up to

wait for an incoming call.  It normally will then wait  up  to

about  23  hours (to zero minutes in the same hour in the next

day).  Adding 30 will tell it to wait up to 30 minutes  for  a

call.

You  may  add  a  D,  as  in -TD or -WD, for the demonstration

setup.  The -F and -D switches cannot use  the  D  because  it

might conflict with a filename or dialing instructions, so you

may add  a  secondary  -D  as  in "-Fscript_file -D".  See the

DEMONSTRATION command below.

-H /H -?  ?  The command line help or usage.

The simple script commands:

They are shown below just like they might be used.  Its a good

idea to  check  out  the  included  sample  files.    If   you

miss-spell a script command, that line is sent to the modem as

is.   This feature is designed to allow you to send any string

to your modem.

Some of the commands are "modem specific" and will  be  listed

first below.    These  commands should also be placed first or

early in the file so the dialer can  setup  the  modem  before

processing the  rest  of  the  file.    You cannot 'reset' the

dialer by sending them later.

TeleShare DOS    REMOTE ACCESS  TSDIAL     Page 3

Modem Specific commands:

NAME its_name Your description for the modem.  The dialer uses

this for its internal database.

INIT The modem  initialization  string.    The  initialization

string sets the modem up for reliable communications.

FAIL The  modem  fail  strings.    The  dialer  uses word fail

commands, such as "BUSY" instead of numbers so you can  easily

understand them.

COMPRESS The  modem  compression  string, if any.  Compression

strings are used by modems as they negotiate  a  setup  for  a

particular communications   session.    If  both  modems  have

similar compression capability  they  can  setup  much  faster

effective data transfers for you.

DEMONSTRATION  To  allow the dialer to continue operating even

when its setup routines fail to gain control of the  specified

communications port.    This  allows  you  to practise or test

drive without a modem or COM port setup.

PROTOCOL The modem protocol string, if any.  Protocol  strings

are  like  a  form  of  compression string, in that modems can

negotiate a common protocol for enhanced  behaviour  during  a

communications session.

INITBAUD The  initial  modem baud rate.  Note that 'INIT BAUD'

could be mistaken as 'INIT' by the dialer, so avoid using it.

LOCKBAUD 1 to lock the baud  rate,  or  0  to  avoid  locking.

Locked   baud   rates   are   used   with  higher  performance

communications sessions.

HANDSHAKE 1 for handshaking or 0 for no handshaking.

ENDMODEM The switch or signal for the final modem  command  in

the  setup  file  so  the  dialer  does not have to search the

entire file for setup commands.  It speeds  up  processing  to

tell the dialer where to stop hunting.

Other Dialer commands:

# This  #  symbol is used for comments in the file.  Put the #

at the far left column to tell the dialer to ignore  the  rest

of the  line.   This symbol follows standard M4 or Make script

language.  You will notice that these comments are  echoed  to

the screen when ECHO is ON.

ABORT  To  tell  the  dialer  to  immediately quit leaving the

machine and  the  communications  settings  unchanged.    This

allows other communications software to take over and continue

TeleShare DOS    REMOTE ACCESS  TSDIAL     Page 4

a session.

BEEP 5.   To beep five times.  You can tell the dialer to beep

from 1 to 99 times.

BLOCK To block or stop strings  that  are  not  recognised  as

modem commands from being sent to the modem.

CLEAR To simply clear the screen.

DIAL some_thing.    To dial a number or whatever follows it on

the same line.  See also REDIAL below.

ECHO To display a single line when ECHO is off.

ECHO ON   To switch display of the script commands on

ECHO OFF  or off.  Useful for script reviews/debugging,

especially when used with DEMONSTRATION.

HANGUP To send the modem a hangup command.    You  could  also

just  send a command string like "ATH0", but with this command

the dialer checks and reminds wonky modems when necessary.

LOGFILE some_file_name To  instruct  the  dialer  to  echo  or

repeat the script commands into the disk file specified on the

same line.    It  opens  and appends to the bottom of existing

files.

PULSE To  tell  the  dialer  to  change  from  TONE  to  PULSE

dialing.  The dialer defaults to tone.

QUIT  To shut down gracefully after closing the communications

session .

REDIAL 5.  To tell the dialer to redial 5  times  when  it  is

given a  DIAL  command.  You can set it to any value between 1

and 99 times.  (Go ahead and try something like 934  times  if

you like).    Put  the  REDIAL  command  above the target DIAL

command in the script file.

WAIT 12:00 To tell the dialer to wait until a  specified  time

in 24 hour format.  If you do not include a time, it will wait

indefinitely, like a non-TSR screen saver.

WAITFORCALL To  wait  up  to  about  23 hours for a call.  The

dialer normally uses the previous  hour  as  the  target  time

unless otherwise  instructed.   Note that "WAIT FOR CALL" will

be interpreted as a WAIT command.

WAITFORCALL 22:35 To wait until the given time for a call.

WAITFORSTRING HI THERE To wait for a  specific  string  to  be

send  'from  the  other  end' once a connection has been made.

The string will be case  insensitively  compared  to  incoming

TeleShare DOS    REMOTE ACCESS  TSDIAL     Page 5

strings  for  three  times  the  carrier timeout (normally 180

seconds), then will quit as an error.

Any other text strings found in the  script  file  are  simply

sent  to  the modem/port, just in case you have a use for such

an occurrence.  Passwords might be one example.

Support and the ASP

Please consider reporting  the  circumstances,  preferably  in

written  form, to technical support when you discover problems

with the software.

For technical support, either call (604) 433-5189, or write to

Suite 606 - 6455 Willingdon Avenue, Burnaby, British Columbia,

Canada, V5H 4E4.  Both registered and soon  to  be  registered

users  currently  get  the  same support without any arbitrary

time limit.  You may also try leaving a Compuserve message  at

75020,  2664,  but be aware that messages rapidly 'scroll off'

Compuserve.  The best electronic message area (also  available

from     Compuserve)     is     via     the    internet    at:

INTERNET:vic_williams@mindlink.bc.ca.

P. C. Softsmith

Suite 606 6455 Willingdon Ave

Burnaby, B. C., Canada

V5H 4E4

ASP Introduction:

Please  also  carefully  read  the  accompanying  SHR_WARE.DOC

document.

"This  program  is  produced by a member of the Association of

Shareware Professionals (ASP). ASP wants to make sure that the

shareware principle works  for  you.  If  you  are  unable  to

resolve  a  shareware-related  problem  with  an ASP member by

contacting the member directly, ASP may be able to  help.  The

ASP  Ombudsman  can help you resolve a dispute or problem with

an ASP member, but does  not  provide  technical  support  for

members'  products.  Please  write to the ASP Ombudsman at 545

Grover Road, Muskegon, MI 49442 or send a  CompuServe  message

via CompuServe Mail to ASP Ombudsman 70007,3536."

This  document and associated TeleShare documents and software

are Copyright (C)1993-4 P. C. Softsmith All Rights Reserved.

THE DOCUMENTATION AND ASSOCIATED SOFTWARE ARE DISTRIBUTED  AND

TeleShare DOS    REMOTE ACCESS  TSDIAL     Page 6

SOLD  WITH NO WARRANTIES, EITHER EXPRESS OR IMPLIED, REGARDING

ITS MERCHANTABILITY OR FITNESS  FOR  ANY  PARTICULAR  PURPOSE.

THE  INFORMATION  IN  THIS  DOCUMENTATION IS SUBJECT TO CHANGE

WITHOUT NOTICE AND DOES NOT REPRESENT  A  COMMITTMENT  ON  THE

PART OF THE VENDOR.

OTHER  PRODUCT  AND  BRAND  NAMES  MENTIONED ARE TRADEMARKS OR

REGISTERED TRADEMARKS OF THEIR RESPECTIVE HOLDERS.