An Assembly Language Primer

(c) 1983 by David Whitman

TABLE OF CONTENTS

Introduction.......................................2

The Computer As A Bit Pattern Manipulator..........3

Digression: A Notation System for Bit Patterns.....5

Addressing Memory..................................7

The Contents of Memory: Data and Programs..........8

The Dawn of Assembly Language......................9

The 8088..........................................11

Assembly Language Syntax..........................14

The Stack.........................................17

Software Interrupts...............................19

Pseudo-Operations.................................21

Tutorial..........................................23

2

>>INTRODUCTION<<

Many people requesting CHASM have indicated that they are

interested in *learning* assembly language.  They are beginners,

and have little idea just where to start.  This primer is

directed to those users.  Experienced users will probably find

little here that they do not already know.

Being a primer, this text will not teach you everything there is

to know about assembly language programming.  It's purpose is to

give you some of the vocabulary and general ideas which will help

you on your way.

I must make a small caveat: I consider myself a relative beginner

in assembly language programming.  A big part of the reason for

writing CHASM was to try and learn this branch of programming

from the inside out.  I think I've learned quite a bit, but it's

quite possible that some of the ideas I relate here may have some

small, or even large, flaws in them.  Nonetheless, I have

produced a number of working assembly language programs by

following the ideas presented here.

3

>>THE COMPUTER AS A BIT PATTERN MANIPULATOR.<<

We all have some conception about what a computer does.  On one

level, it may be thought of as a machine which can execute BASIC

programs.  Another idea is that the computer is a number

crunching device.  As I write this primer, I'm using my computer

as a word processor.

I'd like to introduce a more general concept of just what sort of

machine a computer is: a bit pattern manipulator.

I'm certain that everyone has been introduced to the idea of a

*bit*.  (Note: Throughout this primer, a word enclosed in

*asterisks* is to be read as if it were in italics.)  A bit has

two states: on and off, typically represented with the symbols

"1"  and "0".  In this context, DON'T think of 1 and 0 as

numbers.  They are merely convenient shorthand labels for the

state of a bit.

The memory of your computer consists of a huge collection of

bits, each of which could be in either the 1 or 0 (on or off)

state.

At the heart of your computer is an 8088 microprocessor chip,

made by Intel.  What this chip can do is manipulate the bits

which make up the memory of the computer.

The 8088 likes to handle bits in chunks, and so we'll introduce

special names for the two sizes of bit chunks the 8088 is most

happy with.  A *byte* will refer to a collection of eight bits.

A *word* consists of two bytes, or equivalently, sixteen bits.

A collection of bits holds a pattern, determined by the state of

its individual bits.  Here are some typical byte long patterns:

10101010         11111111         00001111

If you've had a course in probability, it's quite easy to work

out that there are 256 possible patterns that a byte could hold.

similarly, a word can hold 65,536 different patterns.

4

All right, now for the single most important idea in assembly

language programming.  Are you sitting down?  These bit patterns

can be used to represent other sets of things, by mapping each

pattern onto a member of the other set.  Doesn't sound like much,

but IBM has made *BILLIONS* off this idea.

For example, by mapping the patterns a word can hold onto the set

of integers, you can represent either the numbers from 0 to 65535

or -32768 to 32767, depending on the exact mapping you use.  You

might recognize these as the BASIC's range of possible line

numbers, and possible values for integer variables.  This

explains these somewhat arbitrary seeming limits: BASIC uses

words of memory to hold line numbers and integer variables.

As another example, you could map the patterns a byte can hold

onto a series of arbitrarily chosen little pictures which might

be displayed on a video screen.  If you look at the table in

the back of your BASIC manual, you'll notice that there are

*exactly* 256 different characters that can be displayed on your

screen.  Your computer uses a byte of memory to tell it what

character to display at each location of the video screen.

Without getting too far ahead of myself, I'll just casually

mention that there are about 256 fundamental ways the 8088 can

manipulate the bit patterns stored in memory.  This suggests

another mapping which we'll discuss in more detail later.

The point of this discussion is that we can use bit patterns to

represent anything we want, and by manipulating the patterns in

different ways, we can produce results which have significance in

terms of what we're choosing to represent.

5

>>DIGRESSION: A NOTATION SYSTEM FOR BIT PATTERNS<<

Because of their importance, it would be nice to have a

convenient way to represent the various bit patterns we'll be

talking about.  We already have one way, by listing the states of

the individual bits as a series of 1's and 0's.  However, this

system is somewhat clumsy, and error prone.  Are the following

word patterns identical or different?

1111111011111111                         1111111101111111

You probably had trouble telling them apart.  It's easier to tell

that they're different by breaking them down into more manageable

pieces, and comparing the pieces.  Here are the same two patterns

broken down into four bit chunks:

1111 1110 1111 1111                      1111 1111 0111 1111

Some clown has given the name *nybble* to a chunk of 4 bits,

presumably because 4 bits are half a byte.  A nybble is fairly

easy to handle.  There are only 16 possible nybble long patterns,

and most people can distinguish between the patterns quite

easily.

Each nybble pattern has been given a unique symbol agreed upon by

computer scientists.  The first 10 patterns were given symbols

"0" through "9", and when they ran out of digit style symbols,

they used the letters "A" through "F" for the last six patterns.

Below is the "nybble pattern code":

0000 = 0    0001 = 1    0010 = 2    0011 = 3

0100 = 4    0101 = 5    0110 = 6    0111 = 7

1000 = 8    1001 = 9    1010 = A    1011 = B

1100 = C    1101 = D    1110 = E    1111 = F

Using the nybble code, we can represent the two similar word

patterns given above, with the following more manageable

shorthand versions:

FEFF       FF7F

6

Of course, the assignment of the symbols for the various nybble

patterns was not so arbitrary as I've tried to make it appear.  A

perceptive reader who has been exposed to binary numbers will

have noticed an underlying system to the assignments.  If the 1's

and 0's of the patterns are interpreted as actual *numbers*,

rather than mere symbols for bit states, the first 10 patterns

correspond to binary numbers whose decimal representation is the

symbol assigned to the pattern.

The last six patterns receive the symbols "A" through "F", and

taken together, the symbols 0 through F constitute the digits of

the *hexadecimal* number system.  Thus, the symbols assigned to

the different nybble patterns were born out of historical

prejudice in thinking of the computer as strictly a number

handling machine.  Although this is an important interpretation

of these symbols, for the time being it's enough to merely think

of them as a shorthand way to write down bit patterns.

Because some nybble patterns can look just like a number, it's

often necessary to somehow indicate that we're talking about a

pattern.  In BASIC, you do this by adding the characters &H to

the beginning of the pattern: &H1234.  A more common convention

is to just add the letter H to the end of the pattern: 1234H.  In

both conventions, the H is referring to hexadecimal.

Eventually you'll want to learn about using the hexadecimal

number system, since it is an important way to use bit patterns.

I'm not going to discuss it in this primer, because a number of

books have much better treatments of this topic than I could

produce.  Consider this an advanced topic you'll want to fill in

later.

7

>>ADDRESSING MEMORY<<

As stated before, the 8088 chip inside your computer can

manipulate the bit patterns which make up the computer's memory.

Some of the possible manipulations are copying patterns from one

place to another, turning on or turning off certain bits, or

interpreting the patterns as numbers and performing arithmetic

operations on them.  To perform any of these actions, the 8088

has to know what part of memory is to be worked on.  A specific

location in memory is identified by its *address*.

An address is a pointer into memory.  Each address points to the

beginning of a byte long chunk of memory.  The 8088 has the

capability to distinguish 1,048,576 different bytes of memory.

By this point, it probably comes as no surprise to hear that

addresses are represented as patterns of bits.  It takes 20 bits

to get a total of 1,048,576 different patterns, and thus an

address may be written down as a series of 20 bits (or 5 nybble

codes).  For example, DOS stores a pattern which encodes

information about what equipment is installed on your IBM PC in

the word which begins at location 00410.  Interpreting the

address as a hex number, the second byte of this word has an

address 1 greater than 00410, or 00411.

The 8088 isn't very happy handling 20 bits at a time.  The

biggest chunk that's convenient for it to use is a 16 bit word.

The 8088 actually calculates 20 bit addresses as the combination

of two words, a segment word and an offset word.  The combination

process involves interpreting the two patterns as hexadecimal

numbers and adding them.  The way that two 16 bit patterns can be

combined to give one 20 bit pattern is that the two patterns are

added out of alignment by one nybble:

0040      4 nybble segment

0010     4 nybble offset

--------

00410     5 nybble address

Because of this mechanism for calculating addresses, they will

often be written down in what's called segment:offset form.

Thus, the address in above calculation could be written:

0040:0010

8

>>MEMORY CONTENTS: DATA AND PROGRAMS<<

The contents of memory may be broken down into two broad classes.

The first is *data*, just raw patterns of bits for the 8088 to

work on.  The significance of the patterns is determined by what

the computer is being used for at any given time.

The second class of memory contents are *instructions*.  The 8088

can look at memory and interpret a pattern it sees there as

specifying one of the 200 some fundamental operations it knows

how to do.  This mapping of patterns onto operations is called

the *machine language* of the 8088.  A machine language *program*

consists of a series of patterns located in consecutive memory

locations, whose corresponding operations perform some useful

process.

Note that there is no way for the 8088 to know whether a given

pattern is meant to be an instruction, or a piece of data to

operate on.  It is quite possible for the chip to accidentally

begin reading what was intended to be data, and interpret it as a

program.  Some pretty bizarre things can occur when this happens.

In assembly language programming circles, this is known as

"crashing the system".

9

>>THE DAWN OF ASSEMBLY LANGUAGE<<

Unless you happen to be an 8088 chip, the patterns which make up

a machine language program can be rather incomprehensible.  For

example, the pattern which tells the 8088 to flip all the bits in

the byte at address 5555 is:

F6 16 55 55

which is not very informative, although you can see the 5555

address in there.  In ancient history, the old wood-burning and

vacuum tube computers were programmed by laboriously figuring out

bit patterns which represented the series of instructions

desired.  Needless to say, this technique was incredibly tedious,

and very prone to making errors.  It finally occurred to these

ancestral programmers that they could give the task of figuring

out the proper patterns to the computer itself, and assembly

language programming was born.

Assembly language represents each of the many operations that the

computer can do with a *mnemonic*, a short, easy to remember

series of letters.  For example, in boolean algebra, the logical

operation which inverts the state of a bit is called "not", and

hence the assembly language equivalent of the preceding machine

language pattern is:

NOTB [5555]

The brackets around the 5555 roughly mean "the memory location

addressed by".  The "B" at the end of "NOTB" indicates that we

want to operate on a byte of memory, not a word.

Unfortunately, the 8088 can't make head nor tail of the string of

characters "NOTB".  What's needed is a special program to run on

the 8088 which converts the string "NOTB" into the pattern F6 16.

This program is called an assembler.  A good analogy is that an

assembler program is like a meat grinder which takes in assembly

language and gives out machine language.

Typically, an assembler reads a file of assembly language and

translates it one line at a time, outputting a file of machine

language.  Often times the input file is called the *source file*

and the output file is called the *object file*.  The machine

language patterns produced are called the *object code*.

10

Also produced during the assembly process is a *listing*, which

summarizes the results of the assembly process.  The listing

shows each line from the source file, along with the shorthand

"nybble code" representation of the object code produced.  In the

event that the assembler was unable to understand any of the

source lines, it inserts error messages in the listing, pointing

out the problem.

The primeval assembly language programmers had to write their

assembler programs in machine language, because they had no other

choice.  Not being a masochist, I wrote CHASM in Turbo Pascal.

When you think about it, there's a sort of circular logic in

action here. Some programmers at Borland wrote the Turbo Pascal

in assembly language, and I used Turbo to write an assembler.

Someday, I hope to use the present version of CHASM to produce a

machine language version, which will run about a ten times

faster, and at the same time bring this crazy process full

circle.

11

>>THE 8088<<

The preceding discussions have (I hope) given you some very

general background, a world view if you will, about assembly and

machine language programming.  At this point, I'd like to get

into a little more detail, beginning by examining the internal

structure of the 8088 microprocessor from the programmer's point

of view.  This discussion is a condensation of information which

I obtained from "The 8086 Book" which was written by Russell

Rector and George Alexy, and published by Osborne/McGraw-Hill.

Once you've digested this, I'd recommend going to The 8086 Book

for a deeper treatment.

Inside the 8088 are a number of *registers* each of which can

hold a 16 bit pattern.  In assembly language, each of the

registers has a two letter mnemonic name.  There are 14

registers, and their mnemonics are:

AX BX CX DX     SP BP    SI DI     CS DS SS ES    PC ST

Each of the registers are a little different and have different

intended uses, but they can be grouped into some broad classes.

The *general purpose* registers (AX BX CX DX) are just that.

These are registers which hold patterns pulled in from memory

which are to be worked on within the 8088.  You can use these

registers for just about anything you want.

Each of the general purpose registers can be broken down into two

8 bit registers, which have names of their own.  Thus, the CX

register is broken down into the CH and CL registers.  The "H"

and "L" stand for high and low respectively.  Each general

purpose register breaks down into a high/low pair.

The AX register, and its 8 bit low half, the AL register, are

somewhat special.  Mainly for historical reasons, these registers

are referred to as the 16 bit and 8 bit *accumulators*.  Some

operations of the 8088 can only be carried out on the contents of

the accumulators, and many others are faster when used in

conjunction with these registers.

12

Another group of registers are the *segment* registers (CS DS SS

ES).  These registers hold segment values for use in calculating

memory addresses.  The CS, or code segment register, is used

every  time the 8088 accesses memory to read an instruction

pattern.  The  DS, or data segment register, is used for bringing

data patterns in.  The SS register is used to access the stack

(more about the stack later).  The ES is the extra segment

register.  A very few special instructions use the ES register to

access memory, plus you can override use of the DS register and

substitute the ES register if you need to maintain two separate

data areas.

The *pointer* (SP BP) and *index* (DI SI) registers are used to

provide indirect addressing, which is an very powerful technique

for accessing memory.  Indirect addressing is beyond the scope of

this little primer, but is discussed in The 8086 Book.  The SP

register is used to implement a stack in memory. (again, more

about the stack later)  Besides their special function, the BP,

DI and SI registers can be used as additional general purpose

registers.  Although it's possible to directly manipulate the

value in the SP register, it's best to leave it alone, since you

could wipe out the stack.

Finally, there are two registers which are relatively

inaccessible to direct manipulation.  The first is the *program

counter*, PC.  This register always contains the offset part of

the address of the next instruction to be executed.  Although

you're not allowed to just move values into this register, you

*can* indirectly affect its contents, and hence the next

instruction to be executed, using operations which are equivalent

to BASIC's GOTO and GOSUB instructions.  Occasionally, you will

see the PC referred to as the *IP*, which stands for instruction

pointer.

The last register is also relatively inaccessible.  This is the

*status* register, ST.  This one has a *two* alternate names, so

watch for FL (flag register) and PSW (program status word).  The

latter is somewhat steeped in history, since this was the name

given to a special location in memory which served a similar

function on the antique IBM 360 mainframe.

13

The status register consists of a series of one bit *flags* which

can affect how the 8088 works.  There are special instructions

which allow you to set or clear some of the flags.  In addition,

many instructions affect the state of the flags, depending on

their outcome.  For example, one of the bits of the status

register is called the Zero flag.  Any operation which ends up

generating a bit pattern of all 0's automatically sets the Zero

flag on.

Setting the flags doesn't seem to do much, until you know that

there a whole set of conditional branching instructions which

cause the equivalent to a BASIC GOTO if the particular flag

pattern they look for is set.  In assembly language, the only way

to make a decision and branch accordingly is via this flag

testing mechanism.

Although some instructions implicitly affect the flags, there are

a series of instructions whose *only* effect is to set the flags,

based on some test or comparison.  It's very common to see one

of these comparison operations used to set the flags just before

a conditional branch.  Taken together, the two instructions are

exactly equivalent to BASIC's:

IF (comparison) THEN GOTO (linenumber)

14

>>ASSEMBLY LANGUAGE SYNTAX<<

In general, each line of an assembly language program translates

to a set of patterns which specify one fundamental operation for

the 8088 to carry out.

Each line may consist of one or more of the following parts:

First, a label, which is just a marker for the assembler to use.

If you want to branch to an instruction from some other part of

the program, you put a label on the instruction.  When you want

to branch, you refer to the label.  In general, the label can be

any string of characters you want.  A good practice is to use a

name which reminds you what that particular part of the program

does. CHASM will assume that any string of characters which

starts in the first column of a line is intended to be a label.

After the label, or if the text of the line starts to the right

of the first column, at the beginning of the text, comes an

instruction mnemonic.  This specifies the operation that the line

is asking for.  For a list of the 200-odd mnemonics, along with

the instructions they stand for, see The 8086 Book.

Most of the 8088 instructions require that you specify one or

more *operands*.  The operands are what the operation is to work

on, and are listed after the instruction mnemonic.

There are a number of possible operands.  Probably the most

common are registers, specified by their two letter mnemonics.

Another operand type is *immediate data*, a pattern of bits to be

put somewhere or compared or combined with some other pattern.

Generally immediate data is specified by its nybble code

representation, marked as such by following it with the letter

"H".  Most assemblers allow alternate ways to specify immediate

data which emphasize the pattern's intended use.  CHASM

recognizes ten different ways of representing immediate data.

15

A memory location can be used as an operand.  We've seen one way

to do this, by enclosing its address in brackets.  (You can now

see why the brackets are needed.  Without them, the assembler

couldn't distinguish between an address and immediate data.)  If

you've asked the assembler to set aside a section of memory for

data (more on this latter), and put a label on the request, you

can specify that point in memory by using the label.  Finally,

there are a number of indirect ways to address memory locations,

which you can read about in The 8086 Book.

The last major type of operands are labels.  Branching

instructions require an operand to tell them where to branch

*to*.  In assembly language, you specify locations which may be

branched to by putting a label on them.  You can then use the

label as an operand on branches.

Often times, the order in which the operands are listed can be

important.  For example, when moving a pattern from one place to

another, you need to specify where the pattern is to come from,

and where it's going.  The convention in general use is that the

first operand is the *destination* and the second is the

*source*.  Thus, to move the pattern in the DX register into the

AX register, you would write:

MOV AX,DX

This may take some getting used to, since when reading from left

to right it seems reasonable to assume that the transfer goes in

this direction as well.  However, since this convention is pretty

well entrenched in the assembly language community, CHASM goes

along with it.

The last part of an assembly language line is a *comment*.  The

comment is totally ignored by the assembler, but is *vital* for

humans who are attempting to understand the program.  Assembly

language programs tend to be very hard to follow, and so it's

particularly important to put in lots of comments so that you'll

remember just what it was you were trying to do with a given

piece of code.  Professional assembly language programmers put a

comment on *every* line of code, explaining what it does, plus

devoting many entire lines for additional explanations.  For an

example, you should examine the BIOS source listing given in the

IBM Technical Reference manual.  Over *half* the text consists of

comments!

16

Since the assembler ignores the comments, they cost you nothing

in terms of size or speed of execution in the resulting machine

language program.  This is in sharp contrast to BASIC, where each

remark slows your program down and eats up precious memory.

Generally, a character is set aside to indicate to the assembler

the beginning of a comment, so that it knows to skip over.  CHASM

follows a common convention of reserving the semi-colon (;) for

marking comments.

17

>>THE STACK<<

I've been dropping the name *stack* from time to time.  The stack

is just a portion of memory which has been temporarily set aside

to be used in a special way.

To get a picture of how the stack works, think of the spring

loaded contraptions you sometimes see holding trays in a

cafeteria.  As each tray is washed, the busboy puts it on top of

the stack in the contraption.  Because the thing is spring

loaded, the whole stack sinks down from the weight of the new

tray, and the top of the stack ends up always being the same

height off the floor.  When a customer takes a tray off the

stack, the next one rises up to take its place.

In the computer, the stack is used to hold data patterns, which

are generally being passed from one program or subroutine to

another.  By putting things on the stack, the receiving routine

doesn't need to know a particular address to look for the

information it needs, it just pulls them off the top of the

stack.

There is some jargon associated with use of the stack.  Patterns

are *pushed* onto the stack, and *popped* off.  Accordingly,

there are a set of PUSH and POP instructions in the 8088's

repertoire.

Because you don't need to keep track of where the patterns are

actually being kept, the stack is often used as a scratch pad

area, patterns being pushed when the register they're in is

needed for some other purpose, then popped out when the register

is free.  It's very common for the first few instructions of a

subroutine to be a series of pushes to save the patterns which

are occupying the registers it's about to use.   This is referred

to as *saving the state* of the registers.  The last thing the

subroutine will do is pop the patterns back into the registers

they came from, thus *restoring the state* of the registers.

Following the analogy of the cafeteria contraption, when you pop

the stack, the pattern you get is the last one which was pushed.

When you pop a pattern off, the next-to-last thing pushed

automatically moves to the top, just as the trays rise up when a

customer removes one.  Everything comes off the stack in the

reverse order of which they went on.  Sometimes you'll see the

phrase "last in, first out" or *LIFO stack*.

18

Of course, there are no special spring loaded memory locations

inside the computer.  The stack is implemented using a register

which keeps track of where the top of the stack is currently

located.  When you push something, the pointer is moved to the

next available memory location, and the pattern is put in that

spot.  When something is popped, it is copied from the location

pointed at, then the pointer is moved back.  You don't have to

worry about moving the pointer because it's all done

automatically with the push and pop instructions.

The register set aside to hold the pointer is SP, and that's why

SP shouldn't be monkeyed with.  You'll recall that to form an

address, two words are needed, an offset and a segment.  The

segment word for the stack is kept in the SS register, so you

should leave SS alone as well.  When you run the type of machine

language program that CHASM produces, DOS will automatically set

the SP and SS registers to reserve a stack capable of holding 128

words.

19

>>SOFTWARE INTERRUPTS<<

I have been religiously avoiding talking about the various

individual instructions the 8088 can carry out, because if I

didn't, this little primer would soon grow into a rather long

book.  However, there's one very important instruction, which

when you read about it in The 8088 Book, won't seem particularly

useful.  This section will discuss the *software interrupt*

instruction, INT, and why it's so important.

The 8088 reserves the first 1024 bytes of memory for a series of

256 *interrupt vectors*.  Each of these two word long interrupt

vectors is used to store the segment:offset address of a location

in memory.  When you execute a software interrupt instruction,

the the 8088 pushes the location of the next instruction of your

program onto the stack, then branches to the memory location

pointed at by the vector specified in the interrupt.

This probably seems like a rather awkward way to branch around in

memory, and chances are you'd never use this method to get from

one part of your program to another.  The way these instructions

become important is that IBM has pre-loaded a whole series of

useful little (and not so little) machine language routines into

your computer, and set the interrupt vectors to point to them.

All of these routines are set up so that after doing their thing,

they use the location pushed on the stack by the interrupt

instruction to branch back to your program.

Some of these routines are a part of DOS, and documentation for

them can be found in the DOS Technical Reference manual.  The

rest of them are stored in ROM (read only memory) and comprise

the *BIOS*, or basic input/output system of the computer.

Details of the BIOS routines can be found in the Hardware

Technical Reference Manual.  DOS and BIOS interrupt data is also

available in Peter Norton's book "Programmer's Guide to the IBM

PC".

The routines do all kinds of useful things, such as run the disk

drive for you, print characters on the screen, or read data from

the keyboard.  In effect, the software interrupts add a whole

series of very powerful operations to the 8088 instruction set.

20

A final point is that if you don't like the way that DOS or the

BIOS does something, the vectored interrupt system makes it very

easy to substitute your own program to handle that function.  You

just load your program and reset the appropriate interrupt vector

to point at your program rather than the resident routine.  This

is how all those RAM disk and print spooler programs work.  The

programs change the vector for disk drive or printer support to

point to themselves, and carry out the operations in their own

special way.

To make things easy for you, one of the DOS interrupt routines

has the function of resetting interrupt vectors to point at new

code.  Still another DOS interrupt routine is used to graft new

code onto DOS, so that it doesn't accidentally get wiped out by

other programs.  The whole thing is really quite elegant and easy

to use, and IBM is to be complimented for setting things up this

way.

21

>>PSEUDO-OPERATIONS<<

Up to this point, I've implied that each line of an assembly

language program gets translated into a machine language

instruction.  In fact, this is not the case.  Most assemblers

recognize a series of *pseudo-operations* which are handled as

embedded commands to the assembler itself, not as an instruction

in the machine language program being built.  Almost invariably

you'll see the phrase "pseudo-operation" abbreviated down to

*pseudo-op*. Sometimes you'll see *assembler directive*, which

means the same thing, but just doesn't seem to roll off the

tongue as well.

One very common pseudo-op is the *equate*, usually given mnemonic

*EQU*.  What this allows you to do is assign a name to a

frequently used constant.  Thereafter, anywhere you use that

name, the assembler automatically substitutes the constant.  This

process makes your program easier to read, since in place of the

somewhat meaningless looking pattern, you see a name which tells

you what the pattern is for.  It also makes your program easier

to modify, since if you decide to change the constant, you only

need to do it once, rather than all over the program.

The only other type of pseudo-op I'll talk about here are those

for setting aside memory locations for data.  These pseudo-ops

tend to be quite idiosyncratic with each assembler.  CHASM

implements three such pseudo-ops: DB (declare bytes), DW (declare

words) and DS (declare storage).  DB is used to set aside small

data areas, which can be initialized to any pattern, one byte at

a time.  DW is an analogous pseudo-op for setting asides whole

words of memory, one at a time.  DS sets up relatively large

areas, but all the locations are filled with the same initial

pattern.

If you put a label on a pseudo-op which sets aside data areas,

most assemblers allow you to use the label as an operand, in

place of the actual address of the location.  The assembler

automatically substitutes the address for the name during the

translation process.

Some assemblers have a great number of pseudo-ops.  CHASM

implements a few more, which aren't discussed here.

22

>>TUTORIAL<<

To conclude this primer, this section will walk through the

process of writing, assembling, and running a very simple

program.

Our program will just print a message on the video screen, and

then return to DOS.  Although very simple, this program will

demonstrate a number of points, including a DOS function call,

setting aside memory for storage, and good programming form.

The DOS technical reference manual (or Norton's book) discusses

the various DOS functions and interrupts available to the

assembly language programmer.   To print a text string to the

video screen, we'll use function 9.  You should read the

documentation for this function at this time.

Did it make any sense?  To use this function, we have to load the

DX register with the address of a string in memory, specify

function 9 by loading a "9" into the AH register, then ask DOS to

do the printing by executing interrupt 21H.  Basically, we just

set things up, and DOS does all the real work.

Here's the code to do this:

MOV AH, 9                 ;specify DOS function 9

MOV DX, OFFSET(MESSAGE)   ;get address of string

INT 21H                   ;call DOS

Note that none of the lines starts at the left margin  (column

one). If they did, CHASM would think that the instruction

mnemonic was meant to be a label, and would get very confused.

Also note that each line has a comment explaining what's going

on.

The second line needs a little explaining.  CHASM's OFFSET

function returns the address of whatever is included in the

parentheses, in this case, MESSAGE.  The assumption made here is

that later in the program we will set aside a memory location

containing our string to be printed, and give it the name

"MESSAGE".

23

Once we've printed our string, we want to return to DOS.  If we

don't explicitly transfer control back to DOS, the 8088 will

happily continue to execute whatever random patterns are in

memory after our stuff.  Remember "crashing the system"?  One of

DOS's reserved interrupts handles program termination, returning

you to DOS.  The proper instruction is:

INT 20H        ;return to DOS

All that's left at this point is to set aside a chunk of memory

containing a string to be printed.  We'll use CHASM's DB (Declare

Bytes) pseudo-op to do this:

MESSAGE DB  'Hello, World!$'     ;message to be printed

The memory location is given the name "MESSAGE" because the line

started with a "MESSAGE" as a label.  Now CHASM will know that

the preceding OFFSET was talking about this memory location.  You

don't need to worry about what the actual address of MESSAGE is,

CHASM takes care of that.

Fourteen bytes of memory get set aside here, containing the ASCII

codes for the characters in "Hello, World!$".  Note that the

string ends in the character "$".  DOS function 9 prints

characters until it encounters a "$", at which point it stops.

If you forget to put the "$" at the end of your string, you'll

have the less-than-amusing experience of watching DOS attempt to

print out the entire contents of memory.

24

Bringing everything together, and adding a few comments at the

beginning, here's our complete program:

;=====================================;

; HELLO    Version 1.00               ;

;          1984 by David Whitman      ;

;                                     ;

; Sample source file for CHASM.       ;

; Prints a greeting on the console.   ;

;=====================================;

MOV AH, 9                 ;specify DOS function 9

MOV DX, OFFSET(MESSAGE)   ;get address of string

INT 21H                   ;call DOS

INT 20H                   ;return to DOS

MESSAGE DB  'Hello, World!$'      ;message to be printed

After writing all this, we need to create a text file which

contains the lines of our program.  You do this with a text

editor or word processor.  (Of course, in real life you write the

program using the editor in the first place.)

CHASM likes its source files in "standard DOS" format, what some

word processors call "document" or "ASCII mode".  Most word

processors, and all straight text editors work in this format

automatically.  Wordstar and Easywriter (and probably a few other

packages) have their own special formats, but their manual should

tell you how to make standard DOS files.

At this point, make a standard DOS file named HELLO.ASM which

contains the above program lines.  If you're feeling lazy, or if

you run into problems, the file EXAMPLE.ASM on your CHASM disk

has these lines already entered for you.  Just copy EXAMPLE.ASM

into a new file called HELLO.ASM and you're in business.

It's now time to assemble the program.  If it's not already

there, copy HELLO.ASM onto your CHASM disk. Put the CHASM disk

into your default drive, and start up CHASM by typing its name:

A> CHASM

CHASM will respond by printing a hello screen, and ask you to

press a key when you're done reading it.  When you do so, CHASM

will ask you some questions:

25

Source code file name? [.asm]

Type in the HELLO.ASM, or just HELLO, then hit return.  (If you

don't enter the file extension, CHASM assumes that it's .ASM)

Direct listing to Printer (P), Screen (S), or Disk (D)?

CHASM wants to know where to send the listing produced during the

assembly process.  If you have a printer, turn it on, and then

press P.  If you don't have a printer, press S.

The last question is:

Name for object file? [hello.com]

CHASM is asking for the name you'd like to give to the machine

language program which is about to be produced.  Just press enter

here.  CHASM will name the program HELLO.COM

At this point CHASM will start accessing the disk drive, reading

in your program a line at a time.  A status line will appear at

the bottom of your screen, telling you how far along the

translation has gotten.  CHASM will make two passes over your

file, outputing the translated program on the second pass.

If the listing went to your printer, CHASM automatically returns

you to DOS when finished.  If the listing went to the screen,

CHASM waits for you to press a key to indicate that you're done

reading.  Near the bottom of the listing will be the message:

XXX Diagnostics Offered

YYY Errors Detected

If both numbers are 0, everything went fine.  If not, look up on

the listing for error messages, which will point out the

offending lines.  At this point, don't worry too much about what

the error messages say, just fix the line in your input file to

look like the text developed above.  Once you manage to get an

assembly with no errors, you're ready to go on.

Your disk will now contain a machine language program named

HELLO.COM.  Confirm this by typing DIR to get a directory

listing.  You should see the new program file listed.

26

To run the machine language program, you just type its name,

without the .COM extension.  (Note: even though you don't *enter*

the "COM", the file has to have this extension for DOS to

recognize it as a machine language program.)  Try it out now.

From the DOS prompt, type: HELLO.  Your disk drive will whir for

a second, then the message "Hello World!" will appear.

For a further exercise, you might try printing a carriage return

and then a line feed before the message, to space it down the

screen a little.  Carriage return has ASCII code 13, and line

feed is 10.  Read the CHASM User's Manual about the DB pseudo-op,

and add these two characters at the beginning of STRING using

their decimal representations.

Try writing a new program called BEEP which writes the "bell"

character to the screen.  You can use BEEP to signal the end of

long batch files, or to annoy your co-workers.  Resist the urge

to program a loop into BEEP.

An even more advanced exercise would be to clear the screen

before printing a message.  The easiest way to do this is to

use a BIOS function, VIDEO_IO, documented on in the Hardware

Technical Reference Manual.  The comments in the BIOS listing

tell you what values to load into which registers to get VIDEO_IO

to monkey with the screen for you.  Load the registers and

execute INT 10H, once to blank the screen, and again to move the

cursor to the upper left hand corner.

If you've read all of this primer and run the above program,

maybe modifying it a little, you're no longer a rank beginner.

At this point you should have enough of a start to be able to

digest the CHASM User's Manual and The 8086 Book, then begin to

write your own programs.  Good Luck!

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

The Public (Software) Library

Disks in the library are updated monthly.

For a copy of the latest monthly software library newsletter

and a list of the 800+ disks in the library, call or write

The Public (Software) Library

P.O.Box 35705, Rm. F

Houston, TX 77235-5705

(713) 721-6104

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions.