WWIV Modem Documentation

By William Daystrom

Copyright (c) 1989 White Star Software

WWIV Modem Documentation

One of the more tedious aspects to configuring your bulletin board is

modem setup. There are countless brands and variations of modems on the

market, and despite standards, most have slight differences in the way

they will work best with WWIV. However, once you have your modem

functioning correctly with WWIV, you probably will never have to bother

with modem init functions again until you acquire a different modem.

If you are intending to be a sysop, it is generally assumed that you

know enough about your modem to at least have it operating correctly

with your computer. Therefore, this part of the documentation will not

detail how to install your modem. It will show you how to configure your

modem to work with the BBS, as running a bulletin board requires special

settings.

A word about modem cables. It is possible for your modem and cable to

work perfectly well with your terminal program, yet not work with the

BBS. This is because the BBS requires certain information to be carried

between the modem and the computer that is ordinarily not required by

normal use. The vast majority of modem cables manufactured today will be

fine with WWIV. If you constructed your own cable, however, you must be

sure that the following pins are connected (on the DB-25): 2, 3, 7, 8,

and 20.

Now on to modem settings. Your modem will need to tell the BBS

information to enable it to answer and terminate calls correctly. The

first important setting is the carrier detect, or CD. Most modems have

two possible settings for this parameter, either always on, or carrier

is monitored. The second setting is required to enable the BBS to detect

whether or not a caller is on-line. Most modems default to the first

state, however, so you will usually have to change this setting so that

carrier is monitored.

Modems vary in how they may be configured. More modern modems utilitze

software switching to alter default states. In simple language, this

means that your modem will alter its configuration based on commands

given to it from a computer program (such as a terminal program). For

example, with Hayes compatible modems, the command "ATZ" tells the modem

to reset to default configuration. Other modems use physical switches to

alter the default characteristics of the modem. If your modem has such

switches, your modem manual will describe what each of them controls,

and how to change them. In either case, the first goal is to alter your

modem's configuration so that it will monitor carrier. With physical

switch modems, locate the proper switch and make sure it is in the

position for CD monitoring. With software-switch modems, your task is

easier. Using the INIT program, you can alter the default initialization

command issued to the modem each time you run the BBS. You can do this

in Section 4 of INIT. Any commands you add to the command line must be

entered just before the {. Your modem manual will tell you what the

correct command is to monitor carrier. With many modems of this type,

&C1 is the command needed to track carrier. Check your modem manual

anyway to verify.

- 1 -

WWIV Modem Documentation

The second important modem setting is Data Terminal Ready, or DTR. It

is used by your computer to control carrier. When DTR is turned off,

the modem will drop carrier and hang up on the caller. As with CD, most

modems are configured incorrectly for BBS use. There are two common

settings for DTR: controlled and ignored. When DTR is monitored and

controlled, your computer is capable of instructing the modem to drop

carrier. When DTR is ignored, the computer cannot do this. Obviously,

for bulletin board operation, DTR will need to be controlled. You can

change this default by using the same procedure you used to alter the CD

default. With most modems of the software-switch type, &D2, added to the

modem init string in INIT, is the correct command.

Special Modem Requirements

There are a wide variety of modems available in the marketplace today,

and many of the newer ones require special configuration to work with

WWIV. It is beyond the scope of this documentation to detail the steps

needed to make all modems fully compatible with WWIV. Therefore, try

to install your modem following the guidelines already presented. If

you still have difficulty, contact the sysop of a WWIV Support BBS.

See the text file, SUPPORT.LST, for a listing of support systems.

- 2 -