Slicware is Copy(c)right 1989,1992 by Gary M. Raymond, New Orleans, La.

Port Modem Register Test Program V1.1 by Gary M. Raymond

HOME of SlicWare

Gary M. Raymond

P.O.Box 8184

New Orleans, La.  70182

(504) 288-6550

Compuserve 70613,3165

Files Provided:

====================================================================

TESTPORT.COM   Serial Port Test Utility.

TESTPORT.DOC   This document.

TestPort will run on just about any version of DOS and with any

amount of RAM. If nothing else, TestPort will assist you in learning

more about serial communications and understanding your modem. Having

written and supported several terminal programs over the past seven

years, I have long ago realized that most problems involving terminal

software are actually modem or modem configuration problems. This

utility should take some of the mystery away from configuring,

understanding, and diagnosing serial communications hardware. If for

no other reason than curiosity, you owe it to your modem to use

TestPort at least once and read through this file. TestPort was

written in ASIC, and, I will gladly supply the source code with your

registration, if requested.

====================================================================

TestPort is a basic, easy to use, modem diagnostic tool. Although the

information TestPort provides will at first be seemingly cryptic to

the novice user, it can easily be learned and put to good use.  Bit

mapping the modem registers provides a wealth of information about

the state of your modem. Your modem is both configured and monitored

through these registers or electronic switches. Everything from baud

rate to data parity is determined by the one (on) or zero (off) bit

settings in these registers. Particular register bits are also used

to inform the terminal program that a character has arrived or is

ready to be transmitted along with numerous other useful information.

According to the IBM scheme of things, you could have as many as

65,536 port addresses. These ports have an address in the hardware

set up according to IBM's scheme. Each address points to a one byte

location called a base address. Since we can both set and retrieve

bits in these bytes we refer to them as maskable base registers. Each

of the next subsequent nine bytes (BASE+1, BASE+2 etc) form that

ports general register set as outlined in the text below. Serial

communication ports begin with COM Port number one (COM1) with an

address assigned by IBM as decimal 1016 or 3F8 hex. Hence, the

address for the COM1 port register is 3F8. This is the place where a

byte of data is received (3F8 - Received Data Register) or sent (3F8

- Transmitter Holding Register) depending on requirements at the

moment. Therefore the base address allows indirect access to both the

THR and RDR (plus BRDL when bit 7 of LCR is set). The next register

(the IER) is located at this base address plus one, (3F8 + 1 = 3F9)

and so on.

There are two general methods of obtaining bytes of data from a

serial port after it has been set up. The first involves stoping

every so often and looking to see if a byte has arrived, and if so,

taking it. This method of polling the modem registers is ok when

working at very slow baud rates. Polling is the method used by the

PC's Basic Input Output Service (BIOS firmware.) Unfortunately, if a

byte of data arrives before we have had a change to remove the

previous one, information can be lost. To overcome this drawback of

polling we can force the PC to react to the arrival of a byte by

removing and saving it in a special storage area until we have time

to use it. This is done by designing a special Interrupt Service

Routine (ISR) that will be activated instantly (no matter what else

the PC may be doing) to collect and store the byte of data. This is

referred to as an "interrupt on receive" terminal program, such as

our product, SLICK terminal.

There are many things that can go wrong when installing a modem

on your computer. Usually, problems are evenly divided between

hardware and software configuration errors. In general, the modem

itself must be mechanically set, via jumper pins or dip switches,

to address whatever available serial port is usable on your PC.

The selected port must also be paired with an interrupt request line

(IRQ). Note, no two serial devices (examp: modem and mouse) may share

actively the same serial port IRQ address. Some modems force, or

allow you to force, certain register bits either high (1) or low (0)

in lieu of preferred software control. TestPort is useful in

EXAMINING all of the register settings that occur as a results of

these various configurations. Improper configuration of software is

another problem. Here, there are generally three areas of trouble.

Improper comm port, interrupt and modem command (initialization)

string settings. So, to review, One: the serial services must first

be available on the PC. Two: both the modem and terminal software

must agree in settings with the PC hardware you are attempting to

access. And finally, the modem initialization command string must

correctly complete the job of setting any other registers that

require attention, such as setting FAX mode on or off, hardware error

correction enabled or disabled, etc. etc. For what its worth, just

one wrong AT command in the string will cause the entire string to

FAIL. So, if your particular brand modem does not use &D1 (set dtr)

as an example, and, it is inserted into your complete string, such as

ATM1S0=0V1X4&D1S7=45, then the entire string will fail returning an

error message, leaving your modem still waiting to be configured.

====================================================================

USING TestPort's register mapping ability:

To run TestPort with mapping action, you must pass the active port

number via a command parameter. TestPort will work on the four

standard IBM default ports of 1 (3F8), 2 (2F8), 3 (3E8), and 4 (2E8).

Let's assume your modem is configured to operate using COM2. Type

TESTPORT 2 at the DOS prompt. In most cases, you will really want to

do this while your terminal program is active. You do this by

dropping to DOS from within your terminal program, then executing

TestPort. Most all Terminal programs have a drop to DOS feature,

although it might be called something else. TestPort does a passive

read only on these registers and therefore will in no way alter or

change their bit settings. By using the drop to dos feature, which

allows the modem code to remain loaded in ram, you eliminate the

possibility that any modem registers will be inadvertently altered,

by your terminal program, before you get to read the registers. Some

terminal programs also have features that allow the naming and

execution of any external dos utility via a combination of trigger

hot keys. This is a MUCH better way of quickly running TestPort.

Don't forget in either case to pass the active port number in use.

(i.e. TESTPORT 2.)  If TestPort is executed as an external utility,

screen output data will remain until a key is pressed. This will give

you opportunity to hit Shft PrtSc and save a copy of the screen to

your printer for evaluation and analysis. If you do not have a

printer, you should make a long hand record on paper.

USING TestPort's UART finder:

Running TestPort without a port parameter will cause TestPort to

cycle through the four common UART addresses looking for an active

(U)niversal (A)synchronous (C)ommunications (C)ontroller. (8250 IC)

If active port addresses are located, TestPort will notify you of

those that appeared active. Notice, I said appeared active. It is

possible to have two serial ports but only one terminated to a

modem. The other may be just a jack on the rear of your PC, there

for some later expansion. A simple serial card with its open jack

will still show as an active port.

Screen Output:

------------------------------------------------------------------------

The Correct Syntax is TESTPORT #, (# to test), i.e. TESTPORT 4,

however, TestPort will now try to determine what if any serial

port is responding to TestPort's Inquiries....................

COM1 appears to have an active UART!

COM2 appears to be inactive.

COM3 appears to be inactive.

COM4 appears to have an active UART!

Enter Port# to test or Esc to exit ->

In five seconds type AT <Enter>, active port will echo OK or 0, else Esc!

------------------------------------------------------------------------

Although this kind of information is by far un-conclusive, it does

give you enough insights to continue experimenting. In the case of

the information given above, the modem must be attached to either

COM1 or COM4. To determine which it is, try entering 1 first. If

there is no echoing of your keystroke or an OK reply to AT (the modem

attention command) the probability is COM1 is not the port connected

to your modem. Now try 4. This should test ok! More than likely, COM1

is an extra, as of yet unused, serial adapter.

Although this testing facility is only a raw polling terminal, set to

1200 baud, N81, it can easily be used for simple task, like dialing

out to test other modems etc. Try typing ATH1 and see if you hear

your modem take the line off hook....ATH0 to replace it. Fun hun?

====================================================================

SCREEN GENERATED BY TESTPORT when mapping registers:

When TestPort is run, it produces the following screen, indicating

the various one byte registers that have been read, their decimal,

hex, and (binary) bit map values. It's those bit settings you are

most interested in. The two indirect registers (THR and RDR) read

via the base register are not useful here. However, those two plus

the eight indicated account for the 10 registers of a UART chip.

*******************************************************

Modem Register Bit Map by Gary M. Raymond SSC

Phone (504) 288-6550

Modem Registers     Dec Value    Hex Value     Bit Map

76543210

IER (Int Enable)             0           00    00000000

IIR (Int ID)                 1           01    00000001

LCR (Line Control)           3           03    00000011

MCR (Modem Cont)             8           08    00001000

LSR (Line Status)           97           61    01100001

MSR (Modem Status)          48           30    00110000

BRDL (Low Register)          6           06    00000110

BRDH (Hi  Register)          0           00    00000000

*******************************************************

Hit Any Key to Exit!

NOTE: Bit positions are read left to right, starting with position

0 (zero) and ending with 7. This accounts for a full byte of

eight bits.

====================================================================

SAMPLE USES for TESTPORT:

Q) My modem wont hang up, what do I look for?

A) Look at bits 0 and 1 of MCR after starting your terminal program

but before dialing a number. They should both be set to 1. If not

check to see if your modem requires external dip switch settings

to force DTR, RTS or software AT commands like &D2 and &C1.

Q) How can I tell if my terminal program is interrupt driven or

a polling BIOS type?

A) Look at bit 0 of IER, it will be set to 1 if the terminal is

interrupt driven. Also, bit 3 of MCR is usually set to 1 as

well. These bits are set to 0 (zero) in a polling terminal.

Q) My modem wont respond on COM2.

A) Run TESTPORT 2. If If you see each modem register returning FF hex

(dec 255 or 11111111 binary) odds are this port is not activated

correctly via pin or dip settings on the modem card. Check your

modem manual for correct pin or dip switch settings.

Q) How can I tell what Baud rate my modem is locked at?

A) First, run TestPort and obtain the BRDL and BRDH register values.

Then cross reference them with the Baud Rate Value chart in this

document file.

Q) Can I use TestPort to place a call to a BBS?

A) Yes, but it's VERY limited as a polling terminal and not intended

for that kind of application. No protocols, no dial directory, no

no nothing, just a bare foot terminal!

Q) My modem and software configured for COM3 is brain dead, can

TestPort help?

A) You bet! Some serial communications software, especially tsr type,

use BIOS services for communicating with the serial ports. BIOS

then depends on DOS to insert the correct port addresses into the

BIOS area of memory. Some versions of DOS fail to place the

default addresses for COM3 and COM4 in the required BIOS area of

memory. TestPort overcomes this limitation by poking ALL of the

IBM default addresses (3F8 - 2F8 - 3E8 - 2E8) into the correct

area of bios memory starting at segment 0040. This feature is

automatic anytime you run TestPort in any mode. To cancel out

these values, you only need reboot the PC. I will also provide a

seperate utility (PCA.COM) that can be used via your autoexec.bat

to insure these addresses are always available after bootup.

Q) My terminal program has a host mode that allows remote access to

DOS via a redirection statement in a batch file. Only guess what?

It locks up my computer. Can TestPort help me?

A) Well, maybe. There are many computers (clones or otherwise) that

just do not take kindly to re-direction. (CTTY COM1 or CTTY > AUX1

type calls) If the reason is due to the problem outlined in the

answer above, TestPort or PCA will solve it. If not, there is

a device driver in the public domain called GateWay that usually

will accept re-direction. It can be found on most BBS'es. One

nice thing about GateWay is it still allows access to the host

keyboard and monitor; simple re-direction does not.

Q) Can I use TestPort to see the default and or active settings

of all registers in my modem?

A) Yes, actually, you can do that with any terminal program. To use

TestPort, type TESTPORT at the dos prompt. Select the active port

number and wait five seconds for the terminal to respond. Type

AT&V then hit enter. On some older modems you cannot dump the

entire register set and you must poll each individually. Say

you want to see the present value in S7, type ATS7? and so on.

Q) My old modem had a volume control on the card. My new one does

not. Can TestPort save my marriage and turn down the volume?

A) Yes, and again, it can also be done with your present terminal.

By default most modems allow the speaker on at full volume. You

can control this by resetting the M and L registers. ATM1 will

turn off the sound after a connect and ATL0 will reduce the volume

to the lowest setting. Of cource, these switches belong together

in your initialization command string. (ATM1L1etc.etc.etc.)

====================================================================

MODEM REGISTER BIT MAP DATA:

For general purpose information, the port addresses of

these registers are as follows:

BASE = COMPORT ADDRESS  (COM1 = 3F8h)

Therefore, MCR for COM1 equals 3F8h + 4 or 3FCh

Ten Modem Registers

---------------------------------------------

THR,RDR, *BRDL =  BASE

IER,     *BRDH =  BASE  + 1  ;  Interrupt Enable Register

IIR            =  BASE  + 2  ;  Interrupt Identification Register

LCR            =  BASE  + 3  ;  Line Control Register

MCR            =  BASE  + 4  ;  Modem Control Register

LSR            =  BASE  + 5  ;  Line Status Register

MSR            =  BASE  + 6  ;  Modem Satus Register

---------------------------------------------

*requires bit 7 set to 1 in LCR for access

====================================================================

IER  BIT MAPPING   (Interrupt Enable Register)

---------------------------------------------

BIT     Activates             Action

0    =  Data Received         Read RDR

1    =  THR empty             Output to THR

2    =  Data error/break      Read LSR

3    =  MSR change            Read MSR

4    =  zero

5    =  zero

6    =  zero

7    =  zero

Although the 8250 Uart has 10 one byte registers, only seven have

direct access, and of the seven, three of these are used

redundantly to gain access to the three non directly accessible

registers. This indirect access is achieved by setting bit 7 of

LSR to 1. Then, the BRDL and BRDH registers are accesses at BASE

and BASE + 1. These two registers are used to lock the port baud

rate. The BRD (Baud Rate Divisor) is a 16 bit number (two bytes)

and therefore requires two (one byte) register access through

BASE and BASE + 1. Interesting enough, the high byte is only

necessary for baud rates of 300 or below. At 1200 or higher BRDH

is set at zero. The formula for calculating the BRD value

demonstrates why.

UART CLOCK SPEED (1.8432mhz)

BRD =   ----------------------------

16 x bit transfer rate (not really baud)

1,843,200

BRD =   ---------   =   256 or    110000000  or 100h

16 x 300

B   B

R   R

D   D

breaking the result into  H   L

two bytes produces =      1 & 1000000

where BRDH = 1hex and BRDL = 80h (128 dec)

FOR 1200 BAUD:

1,843,200

BRD =   ---------   =   96 or    1100000  or 60h

16 x 1200

B   B

R   R

D   D

breaking the result into  H   L

two bytes produces =      0 & 1100000

where BRDH = 0hex and BRDL = 60h (96 dec)

The THR (Transmit Holding Register) and RDR (Received Data Register)

are accessed via the same physical address as the port base address.

Since you cannot in theory send and receive at the same instant, this

little scheme seems ok.

IIR  BIT MAPPING   (Interrupt Identification Register)

---------------------------------------------

BIT

0    =  more than 1 intr has occurred

12                       12

1-2  =  00  change in MSR        10    data received

01  THR empty            11    recv error/break

3    =  zero

4    =  zero

5    =  zero

6    =  zero

7    =  zero

LCR  BIT MAPPING   (Line Control Register)

---------------------------------------------

BIT                      01                 01

01   =  Data Length      00=5bits           10=7bits

01=6bits           11=8bits

2    =  Stop Bits        0=1bit             1=2bits

345                345

345  =  Parity           000=NONE           101=MARK

100=ODD            111=SPACE

110=EVEN

6    =  Break Condition  0=disabled         1=enabled

7    =  Port Toggle      0=Use THR/RDR/IER  1=Use BRDL/BRDH

SAMPLE: Using N parity, 8 data bits and 1 stop bit requires

the LCR register of 00000000 to be then set as follows:

76543210 = bit pattern

11 = 8 bits

0   = 1 stop bit

000    = no parity

0       = break disabled

0        = toggle inactive

----------

00000011 = 3h to send to LCR

Bit 7 of LCR controls the dual function of bits 0 and 1 in the

IER Register and should be set to zero in the above example.

MCR  BIT MAPPING   (Modem Control Register)

---------------------------------------------

BIT

0    =  Set DTR line active

1    =  Set RTS line active

2    =  USER BIT #1

3    =  USER BIT #2

4    =  UART loopback

5    =  zero

6    =  zero

7    =  zero

Bit 3 is normally set to 1 for interrupt driven terminal

programs. Most programs force the modem to hang up by setting

bits 0 and 1 to 0. Modem software setup for these first two bits

therefore is usually 1 and 1.

LSR  BIT MAPPING   (Line Status Register)

---------------------------------------------

BIT

0    =  byte in RDR (received)

1    =  overrun in RDR

2    =  Parity error detected

3    =  Framing error (out of sync, no stop bit received)

4    =  Break Detect

5    =  THR empty

6    =  TSR empty (removes char from THR)

7    =  Time Out

A communications program usually checks bit 5 here before sending

another character out the base port.

MSR  BIT MAPPING   (Modem Status Register)

---------------------------------------------

BIT

0    =  change in [C]lear [T]o [S]end  CTS

1    =  change in [D]ata [S]et [R]eady DSR

2    =  change in ring indicator RI

3    =  change in [D]ata [C]arrier [D]etect  DCD

4    =  Clear to send ..... [CTS] set high

5    =  Data set ready .... [DSR] set high

6    =  Ring Indicator .... [RI]  set high

7    =  Data Carrier detect [DCD] set high

Most BBS programs monitor bit 7 of the MSR to determine

if carrier is lost during a logon.

===================================================================

BAUD RATE DIVISOR (BRD Register) CHART

Bit Rate            BRDH           BRDL

---------------------------------------

50                   09h            00h

110                  04h            17h

300                  01h            80h

---------------------------------------

1200                 00h            60h   From 1200 up, BRDH is

2400                 00h            30h   always set to zero!

4800                 00h            18h

9600                 00h            0Ch

19200                00h            06h

===================================================================

Registering your copy will help continue the competitive advantages

of providing economical shareware. As an additional bonus, I will

ship you several other useful modem and dos utilities free!  If

you have any comments or suggestions about this utility, feel free

to write or call.

Name_____________________________________________________________

Mailing Address__________________________________________________

City & State ___________________________________________________

ZIP _____________________________ Phone _________________________

Send $4 + $1 (ship & hand) check or money order to:

Gary M. Raymond

P.O.Box 8184

New Orleans, La. 70182

504-288-6550

Compuserve 70613,3165

====================================================================

EOF