SANDWICH PANEL LOAD-SPAN TABLES

As a COPAN/II user, you have access to the use of a program that

generates look-up tables for sandwich panel applications.   If

you are interested in this service, fill out the attached request

form and up to 20 pages of output will be run for you at no

charge.

Load-Span produces look-up tables that provide approximate load

carrying ability of specific core/face combinations over varying

core thicknesses and spans.   The output can be in KG/M^2, PSF or

PSI.

Here is a portion of a page taken from a Load-Span run:

#Deflection Limit: L/ 100#

SPAN    D       0.50IN    0.75IN    1.00IN    1.25IN

30  0.30        0.58      1.39      2.39      3.52

34  0.34        0.41      1.02      1.79      2.68

38  0.38        0.30      0.76      1.37      2.08

42  0.42        0.22      0.58      1.06      1.63

46  0.46        0.17      0.45      0.84      1.30

50  0.50        0.13      0.36      0.67      1.05

54  0.54        0.10      0.29      0.54      0.86

58  0.58        0.08      0.23      0.45      0.71

62  0.62        0.06      0.19      0.37      0.59

66  0.66        0.05      0.16      0.31      0.50

LBS/FT^2        1.98      2.08      2.17      2.27

I  (in^4)   .0228498  .0448021  .0741391  .1108609

Z  (in^3)   .0620655  .0908475  .1199358  .1491757

Loads are based on a predefined deflection criteria expressed as

SPAN over any integer between 10 and 1000.    In essence, the

report is saying that a core having a some thickness over a span

will be able to carry this load while bending that amount.

The above table would be read as follows:

A 1.00" core, when loaded over a 50" span will deflect .500"

while carrying .67 PSI.

The core type, in this case, was a 5 pound PVC with .118" facings

of MAT/ROVING at 35% resin.  The end fixity was Simple-Fixed.

A single page of output shows the same table three times each one

with a different deflection limit.   The following page shows a

complete page of output from which the above was taken.

#

This page is blank

#

Which Span/Core Combinations Are Acceptable?

Another way that the report can be run is when you are looking

for a specific load performance.  Let's say that a structure

needs to take a 1psi load.   By locating all of the loads that

are within a certain range, say 1.00 to 1.25, the corresponding

core thickness and span is found.

The report will flag the loads that are within this range with a

< mark to the immediate right of the applicable value.

Ordering Information

If you are interested in obtaining some tables for your own use,

fill out the request form and mail it to us.  You are entitled to

20 pages of output at no charge.   The only catch is that a

maximum of only two face materials will be accepted.  For

instance, you could not run 20 different face materials on the

same core.  You could run 10 thickness of two different materials

or 20 thicknesses of the same.  If you have any questions, just

give us a call.

All output will be sent to you on an MS/DOS formatted 5-1/4"

floppy disk.  The file(s) will be in ASCII so printing should be

very uncomplicated.  Any word processor/text editor will work

just fine.

If you would like us to print them on presentation quality paper,

the charge is $.50 per page.

Additional pages of output can be obtained from us for $1.00 per

page for orders up to 50 pages.   50 - 100 pages are $.75/pg

while over 100 are $.60.    Complete look-up manuals based on

your materials are available.  Call for pricing.

#

LOAD-SPAN REQUEST FORM

This form is used to generate load/span tables for sandwich

panels.   Each page of output contains 360 beam equation results.

Name    _________________________________________

Company _________________________________________

Address _________________________________________

City    ___________________________State______Zip_________

Phone#

================================================================

MATERIALS           (Only one core and one face per sheet)

Core Description>

Shear Modulus   >         psi      Strength>         psi

Density         >         PCF

The mechanical properties are not necessary if the manufacturers

advertised properties are to be used but make sure that the

description is complete.

Face Description>

Bending Modulus >          psi     Strength>         psi

Density         >          PCF     Poisson >

All mechanicals must be given for face material.  Sandwich will

be considered symmetrical.   (Same properties/thicknesses for

each face)

================================================================

CALCULATION PARAMETERS                      #Output Units  PSI[ ]  PSF[ ]#

#Parameter             Start    Increment   End        Default#

Length of span                             XXXXX   24 to 144 by 12

------------------------------------------------------------------

Core thickness                             XXXXX .5 to 3.25 by .25

------------------------------------------------------------------

Deflection Limits    L/____  L/____  L/____        100, 150, 200

------------------------------------------------------------------

Facing thickness                                        None

------------------------------------------------------------------

End Fixity    Simple[ ]  Fixed[ ]  Simple-Fixed[ ]     Simple

Each page produces output for a single face thickness.  Because

no default exists for facing thickness, all 3 values must be

given.   If the default values are to be used for the others,

enter nothing.

#              Parados Technical Services   912 N. Main Street   Ann Arbor, MI  48104  (313) 668-8111#