After the <CR> is pressed,  a smaller sub-menu will appear in the

same window.

- - - - - - - - - - - - - - - - -

(( Ply Override Options ))

View Current Costs

Override

Exit

- - - - - - - - - - - - - - - - -

Ply Override Sub-Menu

When this menu is displayed, the reverse video pointer will be

positioned on top of the first choice.   To display the current

costs of the various plies within the laminate, press the <CR>.

[View Current Costs]

The communication window (#4) will be used to display a list  of

what plies exist within the laminate.  Each ply will be listed

only once regardless of how many times it was called out in the

LAMCON cross-section.

- - - - - - - - - - - - - - - - -

#  Ply Id              Cost

1  A130-Uni            1.74

2  XPR-240             1.57

3  C238-E/BI           1.25

4  CSM 3/4oz            .98

- - - - - - - - - - - - - - - - -

You will not be able to tell what the actual laminate looks like

from this display.  If you are not sure of whether or not you

wish to change the cost values, return to the main menu and do a

[Scan laminate].

The core ply will not be displayed in this list as it is treated

separately.  If you wish to verify or change the core cost, you

must move to the [Core options] feature via the main menu.

#

[Override]

If the values shown in window #4 are not correct, then it will be

necessary to override those that are in need of change.

To do so, move the menu pointer down one position to [Override]

and press the <CR>.   A Ref #> prompt will appear in window #3

just below the [Exit] option.

- - - - - - - - - - - - - - - - -

(( Ply Override Options ))

View Current Costs

Override

Exit

Ref #> _

- - - - - - - - - - - - - - - - -

If we use the example given on the previous page, COSAN would be

waiting for a response of 1 to 4.   By entering a valid ply

reference value, COSAN will then ask for the new cost which will

be assigned to all plies of that type within the laminate.

The units of entry will also be shown on the 25th row of the

screen so that the unit base remains consistent.

After the cost has been accepted, the new cost values will be

relisted in window #4.   This will allow you to verify that the

value you have entered is in fact the one you intended.

Exiting the [Override]

COSAN will continue to display Ref# prompts after each cost is

entered.  This will allow you to change as many costs as you wish

without having to reselect the [override] option.   When you do

not wish to change any additional costs, enter a null string at

the Ref #.   (This means press <CR> with nothing entered.)

When the null string has been detected,  the Ref# will be removed

from the screen.  At this time, the sub-menu will again be active

and the menu pointer can be moved once again using the number

keypad.  Move the pointer to [exit] and press the <CR> to return

back to the main menu.

#

Using [scan laminate]

As mentioned earlier, it is possible to get an overall view of

the laminate file before calculations take place.  To accomplish

this, move the pointer in the main menu down to [Scan laminate]

and press the <CR>.

- - - - - - - - - - - - - - - - -

(( Scan Options ))

Scan Laminate in Memory

Load and Scan

Exit

- - - - - - - - - - - - - - - - -

This sub-menu will appear in window #3 with the menu pointer

resting on top of the first choice.   To scan the laminate that

has been loaded from disk, press the <CR>.

Full Screen or Partial?

Two small prompts will appear in the communication window which

will be used to determine what kind of screen output you desire.

No printer output is available for this option.

- - - - - - - - - - - - - - -

Full Screen

Partial

- - - - - - - - - - - - - - -

The first letter of each of these will be highlighted in a

brighter color.   This means that either a [F] or [P] will be

required to continue the scan process.

#

Partial Screen  [P]

The [P] key will produce the laminate schedule in the

communication window and the listing will be automatic.  This

means that the plies will be scrolled for you at a comfortable

reading pace.   If they move too quickly, scroll them again and

use the [Ctrl] [S] sequence to stop the action.  The only

information given with the partial screen is ply # and id.   When

the output is complete, the total thickness is shown.

Core plies are always highlighted in bright white.

Full Screen  [F]

The [F] key will use the status window to display the current

laminate.   The listing will be fully manual which means that

each ply will be displayed one at a time.   To cause the next ply

to be output, press the [space bar].  When the output is complete

the total thickness is shown.

The following information is provided with a full scan:

Ply #--Identification--Angle--Resin%--Thickness--Cost--Weight

Core plies are always highlighted in bright white.

As soon as the total thickness of the laminate is shown, the

control is passed back to the sub-menu in window #3.   The menu

pointer is then reactivated.

[Load and Scan]

This option is not yet functional but will be ready for Version

2.0  of COSAN.   This function will allow you to interrogate any

LAMCON file on disk while not distrubing the current laminate

file in memory.  Full and partial listings will apply.

Exiting from the scan option...

Move the menu pointer to the [Exit] option within the sub-menu

and press the <CR>.   Remember that this can be done only after

the laminate  has been fully listed for either full or partial

screens.   A completion indicator is the display of the total

thickness.

#

Changing Drives Within COSAN

If you are operating COSAN from a two floppy system or should you

wish to read LAMCON files from other hard disk sub-directories,

it is possible to change the drive/path default while operating

COSAN.

To do this, move the pointer to the [Change drive] option within

the main menu.  Press the <CR>.

A small prompt will appear on the 25th row of the screen.

Enter a new drive-path or <CR> to abort:

- - - - - - - - - - - - - - - - - - - - - - - - - - - -

This prompt indicates that, like most other COPAN/II prompts, a

null string <CR> will be construed as a request to terminate.  If

this occurs, no action will be taken and the current drive/path

will remain unchanged.

If you do wish to change the drive/path, you must enter a

legal drive-path string.  The following is a few examples of what

constitutes an acceptable string:

String       Action taken by COSAN

------       ----------------------

A:         Changes drive to A

B:         Changes drive to B

C:         Changes drive to C

C:\COPAN      Changes drive to C and sets directory to COPAN

A:         Makes a low pitched sound- no action taken

When a drive/path string is correct, the new drive/path is

displayed in the 25th row.    If COSAN makes a short low-pitched

sound and nothing is displayed in the 25th row, it means that the

string was not understood and the current drive/path remains

unchanged.

Reading the Disk Directory

Many times it is necessary to peek at the directory of a disk

while running COSAN.  It is almost inevitable that sooner or

later you will forget the name of a laminate file.

#

To look at the various files on disk, move the pointer to the

[read directory] option in the main menu.  Press the <CR>.

A small sub-menu appears to help you select the file group that

you wish to review.

- - - - - - - - - - - - - - - - - - -

(( Directory Options ))

LAMCON            Batch List

Resin             Syntactic Foam

Status            c2base

All               exit

- - - - - - - - - - - - - - - - - - -

As soon as this one appears, the reverse video pointer will be

waiting for you on top of the [LAMCON] choice.  By pressing the

<CR>, all LAMCON files, those ending with extension .LCF, will be

displayed in the status window.

The total file count will be shown after all files have been

listed.  The information will be limited to filename only.  The

time and date of file creation will not be included.

Checking for remaining disk space

Just prior to control being returned to the sub-menu, the number

of disk bytes remaining will be displayed in the communication

window.

- - - - - - - - - - - - - - - - - - - - - - -

(( Input/Update Screen ))

Bytes available:  ###,###

- - - - - - - - - - - - - - - - - - - - - - -

This is to help you manage disk space, especially for those

operating with a dual floppy system.   Each time a group of files

is displayed, the disk space will be given.

When you have seen enough, move the pointer to the [Exit] option

in the sub-menu and press the <CR>.   The screen will clear and

the windows will be redrawn putting you back in the now familiar

primary screen.

#

Using Tolerances

Cost and weight tolerances are built-in to COSAN to provide

results that more closely approximate real life situations.  When

a laminate is actually constructed, the final weight is a very

finite number however subsequent production of the same part will

not reproduce the exact same weight.  As weight varies, so do the

costs related to a composite part.   For this reason, COSAN can

produce cost and weight results with high, low and nominal values.

[Weight Tolerances]

By activating this function from the main menu, the following

sub-menu will be diplayed in the communication window.

- - - - - - - - - - - - - - - - -

(( Weight Tolerances ))

+        -

Core density    00.0     00.0

Fabric

Wet-out

Core take-up

Overall

Clear/reset

Exit

- - - - - - - - - - - - - - - - -

Weight Tolerance Menu

Each catagory be may given a separate percentage value according

to whatever tolerance you wish to apply.   Each entry is to be a

value expressed as a percentage.   ie. 5 is five percent while an

entry of .5 would be half of one percent.

The reverse video pointer will situated on top of the first menu

choice.    To make an entry for this catagory, press the <CR>.

The cursor will then be positioned underneath the + column.

Enter the value you desire and follow it with a <CR>.  The cursor

then move over one spot to the right and the minus value may then

be entered.  Do not indicate 5 percent as -5 in the minus column.

When the minus value is entered,  the menu pointer will be

repositioned and the number keypad will again be operational.

The first four menu items all function in the same manner.   Move

the menu pointer,  press the <CR> and then enter the value.  Just

follow the cursor.

#

[Overall]

This function is used to apply a given value to all catagories

at the same time.   If 5 is entered here while all others are set

to 0, the result will be 5 percent for all catagories.  In

another example, if core density is set to 5 percent with the

others at zero, an overall of 5 will increase core density to 10

and the others to 5.

[Clear/Reset]

If you commit an entry error, the values can be set to zero using

this function.   Simply move the pointer to the option and press

the <CR>.   COSAN will not stop to confirm your request.  The old

values will be erased immediately.   COSAN will sound off with an

unmistakable auido emission to let you know that everything has

been reset to zero.

To exit the weight tolerance menu, move the pointer to [exit].

- - - - - - - - - - - - - - - - -

(( Cost Tolerances ))

+        -

Core            00.0     00.0

Fabric

Resin

Overall

Clear/Reset

Exit

- - - - - - - - - - - - - - - - -

Cost Tolerance Menu

The cost tolerances are handled in exactly the same manner as

weight.   If you have already forgotten,  flip back a page and

refresh your memory.   There is one paragraph that I will

replicate so to remind you of an important item.

Each catagory be may given a separate percentage value

according to whatever tolerance you wish to apply.  Each

entry is to be a value expressed as a percentage.   ie. 5

is five percent while an entry of .5 would be half of one

percent.

When the cost tolerances have been completed, move the pointer

down to the [exit] choice and hit the <CR>.   The  communication

screen will be cleared and the main menu will be revitalized.

The status board will post the current tolerance settings each

time that they are changed.

#

How to [Exit] from COSAN

When all has been completed, you may wish to exit from COSAN.

Move the pointer down to the [Exit] and press the <CR>.

Here's what you'll receive:

- - - - - - - - - - - - - - - - - - - - - - - -

I wish to have nothing more to do with COSAN

Yes            No

- - - - - - - - - - - - - - - - - - - - - - - -

Well, it may not be exactly like this, but the message will be

equally clear.