ResisPop 1.0 Documentation

revised for version 1.01, June 14, 1989

revised for version 1.02, December 30, 1989

revised for version 1.03, January 27, 1990

Description:

ResisPop 1.0  is a simple resistor selection program that allows

you to convert arbitrary resistance values between 1.0 ohm and 10 Mohm

to standard values for 0.1, 0.2, 0.5, 1, 2, 5, and 10 percent tolerances.

The standard values are not always commercially available values

(especially below 10 ohms and above 100k ohms) so you need to consult

manufacturers data before finalizing your selections.

ResisPop selects the nearest standard value to the value you enter.

When the value you enter is exactly between two standard values, Resispop

selects the smaller value.

Since ResisPop is a TSR (Terminate and Stay Resident) program, there are

certain restrictions to its use.

1. If you use SideKick, SideKick must be loaded after ResisPop.

If ResisPop senses that SideKick is installed, ResisPop will

not install itself.

2. ResisPop will not pop-up if your video display is in a

graphics mode. This is to prevent garbling your screen on

exit. If you are using a graphics oriented program and the

program will allow you to go to DOS without exiting the program,

ResisPop will be able to pop-up in that environment.

3. ResisPop takes about 40k of memory in excess of the application

you are running.

Getting Started:

To load ResisPop, type 'Resispop' at the DOS prompt with a copy of the

ResisPop disk in the default drive for a floppy based system or a copy

of ResisPop on the default directory of your hard disk for a hard disk

based system.

To start ResisPop, press Ctrl-Alt-R (the <Ctrl>, <Alt>, and the <R> keys

simultaneously).

Removing Resispop From Memory:

To remove ResisPop from memory, start Resispop, type 'q'

or 'Q' in the data entry window and then hit the <Enter> key and

ResisPop will deactivate itself if it is safe to do so (i.e. if no

system crash is likely to occur as a result).

What If I Don't Like TSR's?:

Included with ResisPop is a non-TSR version, Resistor.exe, for those of

you who would rather not use memory for a TSR or are running DesqView or

other DOS compatible multitasking software and don't need the pop-up

capability.

Copyright and Distribution Restrictions:

ResisPop and Resistor are Copyright (c) Notor Engineering Services, 1989

1990, all rights reserved. You are free to use this program for your personal

work, or to give a copy to a friend or colleague provided the original

files are not modified in any way whatsoever. You may also upload the

archive file, Rpop.com, to any bulletin board system or information service.

This software may not be published for commercial purposes as part of a

collection or as a stand alone product without the express written consent of

Notor Engineering Services.

If you desire to distribute Resispop or Resistor for commercial purposes or

as a "give away" (i.e. as the equivalent of a coffee cup or other freebie)

with your company logo on the label, license agreements and duplication

services are available from Notor Engineering Services.

Support:

If you have any questions or comments about this software package you may

address them to

John Notor

Notor Engineering Services

1548 Arata Court

San Jose, CA 95125

(408) 947-8848

CompuServe ID: 75075,467