����������������������������������������������������������Ŀ

� ������������������������������������������������������Ŀ �

� �                 Six Sigma Simulator                  � �

� �              SSSWIN.EXE...Version 9404               � �

� ������������������������������������������������������Ĵ �

� �          A Quality Assurance Training Tool:          � �

� �  Statistics Committee of the QA Section of the PMA   � �

� ������������������������������������������������������Ĵ �

� �      Bob Obenchain, CompuServe User [72007,467]      � �

� �     5261 Woodfield Drive North, Carmel, IN 46033     � �

� �������������������������������������������������������� �

������������������������������������������������������������

The Six Sigma Simulator, SSSWIN, is a  Microsoft  Windows  application

with   several   potential   uses   in   computer-based  training  for

"statistical thinking."  For example, SSSWIN software can be  used  in

hands-on  workshops  as  well  as in self-paced learning to illustrate

basic concepts related to...

variation in histograms   and the   normal distribution.

=======================             ===================

Specifics...

SSSWIN displays colorful and  "dynamic"  histograms  of  pseudo-random

normal  deviates  with  mean  zero and variance one using 12, 24 or 48

cells covering the range from -6.0 to +6.0.

[1] SSSWIN trains users to anticipate sampling variability in

histograms and illustrates how this variability changes with

sample size and cell width.

[2] SSSWIN illustrates chi-squared goodness-of-fit tests.  Chi-

square tests of normality are the main focus, but a chi-square

test for uniformity of the resulting observed significance levels

is also illustrated.  [Only "null" cases where the distribution

being sampled is normal or uniform are treated.]

[3] SSSWIN illustrates difficulties involved in using Monte-Carlo

simulation techniques to estimate small (tail-area) probabilities,

such as deviations from the mean by more than 4, 5 or even 6

standard deviations.

SSSWIN.DOC Table of Contents:

=============================

(i) Introduction: the Normal Distribution & Variation in Histograms.

(ii) SSSWIN Component Files and Main-Menu Items.

(iii) SSSWIN Instructional Objectives / Learning Points.

Six-Sigma-Simulator for Microsoft Windows         Page 2 of 14

(iv) Numerical Examples / Advanced Topics illustrated by SSSWIN.

(v) References and Software Update History.

============

<<< INTRODUCTION >>>

============

A "normal" distribution is commonly characterized as the familiar "bell-

shaped curve" with...

68.27% of its total area inside +/-1 standard deviation  of the mean,

95.45% of its total area inside +/-2 standard deviations of the mean, &

99.73% of its total area inside +/-3 standard deviations of the mean.

� � �

� � � � �

� � � � � � �

� � � � � � � � �

� � � � � � � � �

� � � � � � � � � � �

_ � � � � � � � � � � � � � � � _

-3    -2    -1     0     1     2     3

�     �  Standard Deviations  �     �

�     �     �<��68.27%�>�     �     �

�     �<��������95.45%�������>�     �

�<��������������99.73%�������������>�

A primary objective of SSSWIN is to illustrate  what  samples  from  a

normal  distribution  typically "look like" when they are displayed in

HISTOGRAMS, like the one above.   Specifically,  relative  frequencies

are calculated to describe how a sample of observed values fall into a

series of adjacent cells (or bins) of equal  width.   A  histogram  is

simply  a  "bar  chart"  in  which  the height of each bar depicts the

relative frequency with which observations fell into that cell.

NOTE: One of the things that becomes immediately quite "obvious" to

users of SSSWIN is that histograms of normal deviates tend to NOT

be anywhere as nearly "smooth" and symmetrical as in their TRUE

distribution...at least when the sample size is less than

SEVERAL THOUSANDS and/or cells are less than ONE STANDARD

DEVIATION wide.

We assume here that some potential users of SSSWIN may feel a need  to

be  convinced  that  they  can  "trust" the insights they will develop

using SSSWIN.  Therefore, we now discuss some  technical   issues  and

provide literature references.

Six-Sigma-Simulator for Microsoft Windows         Page 3 of 14

How do I know that values displayed by SSSWIN have a normal distribution?

=========================================================================

See Park and Miller(1988) or Hoaglin(1982) for an introduction

to pseudo-random number generation concepts.

SSSWIN uses state-of-the-art algorithms to generate deviates

that are, as close as is possible, independent and identically

distributed (i.i.d.) normal deviates with mean zero and variance

one.  The user of SSSWIN may choose between three state-of-the-

art algorithms [L'Ecuyer(1988), Wichman and Hill(1982), or

Knuth(1981)] and two types of Box-Muller transformation [a

rejection method from Press, et. al.(1988) or a brute-force,

trignometric approach.]

Functions compiled from source code copyrighted by Numerical

Recipes Software, [Press, et. al. (1988)], are bound into

SSSWIN.EXE with permission of the authors.

What are chi-square goodness-of-fit tests for normality or uniformity?

=====================================================================

Lancaster(1982) describes the following early history...

Karl Pearson (1900) introduced test statistics, applicable to

histograms, that sum terms of the form: the squared difference

between observed and expected frequencies divided by the

expected frequency.  The underlying probability model is that

of a multinomial distribution, but Pearson demonstrated that

the asymptotic distribution of the test statistic follows a

chi-square distribution.

R. A. Fisher (1922, 1925) demonstrated that the degrees-of-

freedom for chi-square test statistics would be n-p-1 (where n

is the number of cells in the histogram) when asymptotically

efficient estimates of p parameters are derived using the data

at hand.  However, p = 0 in SSSWIN since expected frequencies

are those for a normal distribution with mean zero and

variance one [rather than for the normal distribution with

mean and variance matched to those of the observed sample.]

Roscoe and Byars(1971) and Moore(1986) describe currently

recommended rules-of-thumb for chi-square goodness-of-fit

testing.  It is generally agreed that choosing cell widths to

yield equal cell expectancies is best.

In SSSWIN chi-square tests of normality, all of the interior

cells are of equal width instead of equal expectancy.  In

cases like this, the commonly accepted guideline is that

tail-cells should be merged until there is an expectancy of

at least 2 counts in each tail.

Six-Sigma-Simulator for Microsoft Windows         Page 4 of 14

In the special case of SSSWIN's chi-square test for uniformity

of observed significance levels, equal cell widths do yield

equal expectancy.  The commonly accepted guideline for this

case would be to accumulate a per-cell expectancy of at least

1 before computing chi-square statistics.

What is the observed significance level of a test statistic?

============================================================

The "significance level" of a observed chi-square test statistic

is the probability that a chi-square distributed random variable

(with the appropriate number of degrees-of-freedom) would exceed

that observed numerical value.  SSSWIN computes these observed

significance levels as values of incomplete gamma functions,

Press, et.al.(1988).

When the hypothesis being tested is true (and it always is in

SSSWIN tests for normality), observed significance levels should

be uniformly distributed on [0,1].

On the other hand, the user of SSSWIN may change the "exptmin"

parameter that controls the number of tail-cells to be combined

to achieve a minimum expectancy.  Instead of the commonly

recommended value (exptmin = 2), the user of SSSWIN may reduce

exptmin to as low as 0.1 or increase it to as high as 10.0

simply to see any resulting effects on the null distribution of

observed significance levels.

Why are the very small  probabilities  associated  with  normal-theory

deviations  from  the  mean by 4 to 6 standard deviations of interest?

======================================================================

The concept of "Six-Sigma Capability," Harry(1987), is rapidly

becoming a catch-phrase, at least here in the United States, for

"world-class quality."  This criterion is defined as follows:

The distance from the target to the spec limits should be 6 *

sigma, and the process mean should be no more than 1.5 *

sigma from the target...where sigma is the standard deviation

of the process distribution.

In other words, the distance to the CLOSER spec limit from

the process mean should be at least 4.5 * sigma.

A process operating either at or better than this capability

level is expected to produce nonconforming units at a rate of

at most 3.4 parts per million.  (Note: all of these "defects"

are expected to occur as violations of the closer spec

limit.)

Thus, to visualize a process operating at "Six-Sigma Capability"

using SSSWIN, one simply pretends that the process specification

limit closest to the process mean (of 0.0) is at -4.5 or +4.5.

Six-Sigma-Simulator for Microsoft Windows         Page 5 of 14

============================================

<<< SSSWIN Component Files and "MainMenu" ITEMS: >>>

============================================

Files associated with the Six-Sigma-Simulator are:

SSSWIN.EXE ...Main Windows Executable Module

SSSWIN.DOC ...This ASCII Documentation File

SSSWIN.PAR ...Default Parameter Settings File

SSSWIN.SAV ...Detailed Simulation Results File

SSS-RAND.C ...Example C-language Source Code for Random

Number Generation Algorithms

SSSWIN.EXE may be copied to the sub-directory of your choice  on  your

personal  computer's  hard  disk.   In  the  installation instructions

detailed below, we assume that  the  Drive:\Path  of  your  choice  is

C:\WINDOWS.   You  should then use the "File/New.../Program" menu item

from within Microsoft Windows to define file properties...

Description:         SixSigSim

Command Line:        ssswin.exe

Working Directory:   c:\windows

Once installed in this way, you double-click  on  the  histogram  ICON

labelled "SixSigSim" to launch the SSSWIN application.

<<< SSSWIN Main Menu ITEMS: >>>

Histograms

=---------

True Distribution       ...Display histogram showing the LONG-RANGE

AVERAGE expected shape.

Cumulative Counts       ...Display histogram of all accumulated

simulation results since the last reset.

Incremental Counts      ...Display histogram of results in the most

recent batch (of SampleSize = reppbat).

Chi-Square Sig. Levels  ...Display a 20-cell histogram of observed

significance levels of incremental chi-

square tests for normality.

Tail-cells are merged together to  accumulate  a  minimum  chi-squared

expectancy.   The resulting degrees-of-freedom for the goodness-of-fit

test of normality  will  never  change  on  an  "Incremental"  display

because   each   such  display  corresponds  to  the  same  number  of

observations.  But the number  of  merged  cells  will  diminish  (and

degrees-of-freedom  will increase) on "Cumulative" displays as results

accumulate over time.

Cell Width

=---------

One Sigma => Max        ...Use "widest" histogram cell widths.

Half Sigma => Mid       ...Use "intermediate" histogram cell widths.

Quarter Sigma => Min    ...Use "narrow" histogram cell widths.

Six-Sigma-Simulator for Microsoft Windows         Page 6 of 14

NOTE: In addition to their primary purpose of determining histogram

"Cell Widths", each of these menu sub-items also discards all

previously accumulated results (without saving them in the

SSSWIN.SAV file) and resets the simulation to zero.

Monte-Carlo

=----------

Pseudo-Random Algorithm:    [ Le'Cuyer, Wichman-Hill or Knuth ]

=-----------------------

rntype=1: The 32-bit L'Ecuyer(1988) algorithm.

rntype=2: The Wichman-Hill(1982) algorithm; see also remarks

by McLeod(1985) and Zeisel(1986).

rntype=3: A floating point version of the fast "subtractive"

algorithm due to Knuth ...from Press, Flannery,

Teukolsky, and Vetterling(1988).

Box-Muller Transformation:   [ Trig-Accurate or Reject-Fast ]

=-------------------------

bmtype=0: The gasdev() function of Press, Flannery, Teukolsky,

and Vetterling (1988) rescales uniform variates to the

range (-1.0,1.0) and rejects any pair that falls outside

of the unit circle.

bmtype=1: This is the brute force trignometric approach which

requires computation of a sine and a cosine as well as

the logarithm.

NOTE: In addition to their primary purpose of determining which pseudo-

random number generation algorithms to use, each of these Monte-

Carlo menu sub-items also discards all previously accumulated

results (without saving them in the SSSWIN.SAV file) and resets

the simulation to zero.

Other

=----

Set Sample Size          10 <= reppbat <= 1,000

Delay Refresh            0 <= pause <= 100  (0 <= clock ticks <= 3,000)

Initial Seed             0 <= iduminit <= 32,767

Tail Area Expectancy     0.1 <= exptmin <= 10.0

Max. Total Replications  1,000 <= maxreps <= 2,000,000,000

About SSSWIN...          Display author/address/version

NOTE: The "Set Sample Size" sub-item on the "Other" menu controls the

value of <<< reppbat >>>, the number of samples displayed in each

"incremental" histogram.  When reppbat is very small (say,

between 10 and 25), the incremental histogram display may change

so quickly that it will become difficult to see the shapes of

these histograms!  In these cases, the user may use the "Delay

Refresh" menu sub-item to force the simulation to slow down

considerably.

HINT: Don't slow the simulation down any more than absolutely necessary

because the resulting "pauses" can severely increase mouse and

keyboard response times.  [One pause unit = 30 clock ticks.]

Six-Sigma-Simulator for Microsoft Windows         Page 7 of 14

Besides, the rate at which observations are accumulated into

the "Cumulative Counts" and "Chi-Square Sig. Levels" displays

is also slowed when pause > 0.

NOTE: The "Initial Seed" menu sub-item controls the starting point of

the simulation that will follow the current simulation.  Any

change you make here does not become the permanent default seed

until you select the "Save Parameter Settings" sub-item on the

SSSWIN "File" menu.  And "iduminit=0" actually means that SSSWIN

will use your system's clock to get a DYNANIC seed for each

reset rather than use any FIXED seed value (1 to 32767).

NOTE: The "Tail Area Expectancy" menu sub-item controls the degrees-

of-freedom of computed chi-square statistics.  Decreasing

"exptmin" tends to increase degrees-of-freedom, and vice versa.

NOTE: None of the sub-items on the "Other" menu automatically triggers

a total reset of the simulation currently being performed.

However, changing reppbat or exptmin (using the "Set Sample

Size" or "Tail Area Expectancy" items) does reset the count of

observed significance levels of incremental chi-squares to zero.

And reducing maxreps below the currently accumulated total

(using the "Maximum Total Replications" item) would trigger a

total reset as well as an automatic append to SSSWIN.SAV.

File

=---

Save Parameter Settings ...Write parameter values to SSSWIN.PAR file.

Write Detailed Output   ...Append results to the SSSWIN.SAV file.

Exit                    ...Leave SSSWIN and return to MS Windows.

Example Contents for the SSSWIN.PAR file...

=======================================================================

200  ;number of replications per batch

1  ;Box-Muller type [0,1]

1  ;random number type [1,2,3]

0  ;random number seed

3.0  ;minimum expectancy in tail cells

2000000000  ;maximum total replications

300  ;pause before screen refresh ( pause=10 => 300 clock_ticks)

=======================================================================

WARNING: The SSSWIN.SAV file grows in size each time the user clicks on

the "Write Detailed Output" menu item.  Results are also

appended to this file any time a simulation of specified

"maxreps" size is completed and a new simulation starts.

WARNING: Using the ASCII text editor or word process of his/her choice,

the user should periodically edit the SSSWIN.SAV file to

print results or save them to a different filename.ext.  It

is highly recommended that all out-dated text be removed from

the SSSWIN.SAV file during this manual editing process.  Of

course, the user may also use the DOS "del ssswin.sav"

Six-Sigma-Simulator for Microsoft Windows         Page 8 of 14

command or the Windows File Manager to simply delete a

SSSWIN.SAV file that has become too large and/or outdated.

In SSSWIN, results are accumulated using "long"  integer  values  that

cannot  exceed  2,147,483,647.   SSSWIN.EXE  automatically  stops (and

writes out summary results in a file named SSSWIN.SAV) after "maxreps"

replications.  On the other hand, that many replications would require

about 70 hours on a 33Mhz 486-machine!  Of course, you can always stop

SSSWIN.EXE whenever you want via the "File/eXit" menu item.

==========================================

<<< INSTRUCTIONAL OBJECTIVES / LEARNING POINTS >>>

==========================================

[1] Sampling variability in histograms is considerably higher than

statistical novices expect it to be.

*****************************************************************

Sampling variability does decrease as the number of values

displayed increases and/or cell widths increase.

...literally thousands of values need to be accumulated before

a histogram can be expected to be smooth and symmetrical like

its parent, normal distribution.

...if a small sample, like 25 to 50 values, is to be displayed

in a histogram, it can be quite misleading to use cell widths

of less than about 1 standard deviation.  Only six 1-sigma

cells are commonly needed to display a small sample, but any

attempt at higher "resolution" is folly.

*****************************************************************

[2] Chi-squared goodness-of-fit tests for normality are fairly

robust to choice of tail-cell minimum expectancy as long as this

minimum expectancy exceeds 1.

*****************************************************************

When a very low tail-cell minimum expectancy like 0.1 is used,

the null distribution of observed significance levels develops

a "peak" in its [0.00,0.05] cell, indicating a tendency for the

test to become too large in size (false rejection of normality.)

*****************************************************************

Six-Sigma-Simulator for Microsoft Windows         Page 9 of 14

[3] Values outside of [-4,+4] are seldom seen in SSSWIN histogram

displays.

*****************************************************************

A small part of this phenomenon is due to the limited resolution

of Microsoft Windows graphics mode; any cell with a relative

frequency less than about 0.005 will not be displayed at all!

===================================

...with wide (1-sigma) cells, a relative frequency of 0.005 will

display as a thin, horizontal, black line.

...a relative frequency of 0.005 is most easily spotted using

0.25-sigma or 0.50-sigma cells in an "incremental" histogram

for a sample size of at most 200.  Any single count falling

within [-6,+6] is then certain to be displayed, if only very

briefly.

HINT: Don't try to watch both the lower tail, [-6,-3], and the

upper tail, [+3,+6], at the same time.  By using the "Delay

Refresh" item on the "Other" menu to slow the simulation down

considerably, one person might be able to watch both tails at

once.  But any such delay would be counter-productive when you

are waiting to see relatively "rare" events to occur!

HINT: When using 0.25-sigma or 0.50-sigma cells, note the COLOR of

any short (single count) bar that appears in a tail cell.  This

color-code will be easier for you to "see" and "remember" than

exactly where a small bar suddenly appeared and disappeared!

Colors and Locations of 0.25-sigma Tail Cells...

================================================

COLOR:    Lower (Left-Hand) Tail          Upper (Right-Hand) Tail

-------  -----------------------------   -----------------------------

White    [-4.75,-4.50)                   [+3.00,+3.25) & [+5.00,+5.25)

Black    [-4.50,-4.25)                   [+3.25,+3.50) & [+5.25,+5.50)

Blue     [-4.25,-4.00)                   [+3.50,+3.75) & [+5.50,+5.75)

Green    [-4.00,-3.75) & [-6.00,-5.75)   [+3.75,+4.00) & [+5.75,+6.00)

Cyan     [-3.75,-3.50) & [-5.75,-5.50)   [+4.00,+4.25)

Red      [-3.50,-3.25) & [-5.50,-5.25)   [+4.25.+4.50)

Magenta  [-3.25,-3.00) & [-5.25,-5.00)   [+4.50,+4.75)

Yellow   [-5.00,-4.75)                   [+4.75,+5.00)

Six-Sigma-Simulator for Microsoft Windows         Page 10 of 14

Colors and Locations of 0.50-sigma Tail Cells...

================================================

COLOR:    Lower (Left-Hand) Tail          Upper (Right-Hand) Tail

-------  -----------------------------   -----------------------------

White    [-3.50,-3.00)                   [+4.50,+5.00)

Black    [-4.00,-3.50)                   [+5.00,+5.50)

Blue     [-4.50,-4.00)                   [+5.50,+6.00)

Green    [-5.00,-4.50)

Cyan     [-5.50,-5.00)

Red      [-6.00,-5.50)                   [+3.00.+3.50)

Magenta                                  [+3.50,+4.00)

Yellow                                   [+4.00,+4.50)

*****************************************************************

If you were to watch only one tail of SSSWIN histograms for

unusually large deviations (either positive or negative), how

long would you expect to have to wait to see a value strictly

larger [in absolute value] than each of the following?

Probability    Probability   Expected "waiting time"

Abs.  in a single    of at least   when viewing incremental SSSWIN

Value  trial...       1 in 200...   histograms of size 200 each...

=====  ============   ===========   ===============================

3.0   0.00135        0.23676       about 4

3.5   0.000233       0.04554       about 22

4.0   0.0000317      0.00632       about 158          Motorola

4.5   0.00000340     0.000680      about 1,471    <-- Capability

5.0   0.000000287    0.0000574     about 17,422       Benchmark

5.5   0.000000019    0.00000380    about 263,158

6.0   0.000000001    0.00000020    about 5 million*

* Unfortunately, cells outside of [-6,+6] are NOT actually

displayed by SSSWIN!

*****************************************************************

You may also use the "Write Detailed Output" item on the "File"

menu to write detailed results, including accumulated frequencies

in individual cells, to the SSSWIN.SAV output file.  An example

is given next...

*****************************************************************

====================================

<<< NUMERICAL EXAMPLES / ADVANCED TOPICS >>>

====================================

The first six columns of a SSSWIN.SAV output file are listed below for

a run in which 2 billion pseudo-random normal deviates were generated:

Six-Sigma-Simulator for Microsoft Windows         Page 11 of 14

Random Number Seed  = -12345

Elapsed Simulation Time = 75.9 hours on a 33Mhz 80486

SSSWIN Parameter Settings...

1  ;Box-Muller type [0,1]             <== brute-force trig.

2  ;random number type [1,2,3]        <== exceptionally smooth

500  ;replications per batch                (Wichman-Hill)

SSSWIN Summary Statistics...

Total Number of Replications = 2,000,000,000

Minimum Expectancy in Tail Cells =  3.0

Cum. Chi Sq. =    31.53 with d.f.= 47

Inc. Chi Sq. =    15.47 with d.f.= 21

Row  CutVal    ExpFract     ExpTail    CumFract   ChiSq

0   6.0000 0.000000001 0.000000001 0.000000001    0.00

1   5.7500 0.000000003 0.000000004 0.000000003    0.10

2   5.5000 0.000000015 0.000000019 0.000000016    0.30

3   5.2500 0.000000057 0.000000076 0.000000051    1.29

4   5.0000 0.000000211 0.000000287 0.000000215    0.14

5   4.7500 0.000000730 0.000001017 0.000000720    0.30

6   4.5000 0.000002381 0.000003398 0.000002367    0.17

7   4.2500 0.000007291 0.000010689 0.000007301    0.03

8   4.0000 0.000020983 0.000031671 0.000021163    3.08

9   3.7500 0.000056746 0.000088417 0.000056807    0.13

10   3.5000 0.000144212 0.000232629 0.000144274    0.05

11   3.2500 0.000344396 0.000577025 0.000344469    0.03

12   3.0000 0.000772873 0.001349898 0.000773634    1.50

13   2.7500 0.001629865 0.002979763 0.001628786    1.43

14   2.5000 0.003229902 0.006209665 0.003231321    1.25

15   2.2500 0.006014807 0.012224472 0.006014945    0.01

16   2.0000 0.010525658 0.022750130 0.010529525    2.84

17   1.7500 0.017309025 0.040059155 0.017307328    0.33

18   1.5000 0.026748047 0.066807202 0.026744732    0.82

19   1.2500 0.038842578 0.105649780 0.038837542    1.31

20   1.0000 0.053005481 0.158655261 0.053008091    0.26

21   0.7500 0.067972080 0.226627342 0.067971260    0.02

22   0.5000 0.081910175 0.308537517 0.081910796    0.01

23   0.2500 0.092756175 0.401293692 0.092750980    0.58

24   0.0000 0.098706323 0.500000015 0.098710573    0.37

25  -0.2500 0.098706293 0.499999985 0.098712368    0.75

26  -0.5000 0.092756175 0.401293692 0.092752522    0.29

27  -0.7500 0.081910175 0.308537517 0.081907382    0.19

28  -1.0000 0.067972080 0.226627342 0.067969859    0.15

29  -1.2500 0.053005481 0.158655261 0.053005838    0.00

30  -1.5000 0.038842578 0.105649780 0.038845790    0.53

31  -1.7500 0.026748047 0.066807202 0.026750819    0.57

32  -2.0000 0.017309025 0.040059155 0.017308901    0.00

33  -2.2500 0.010525658 0.022750130 0.010521947    2.62

34  -2.5000 0.006014807 0.012224472 0.006016818    1.35

35  -2.7500 0.003229902 0.006209665 0.003229841    0.00

36  -3.0000 0.001629865 0.002979763 0.001631015    1.62

37  -3.2500 0.000772873 0.001349898 0.000773311    0.50

Six-Sigma-Simulator for Microsoft Windows         Page 12 of 14

38  -3.5000 0.000344396 0.000577025 0.000344350    0.01

39  -3.7500 0.000144212 0.000232629 0.000143954    0.92

40  -4.0000 0.000056746 0.000088417 0.000056830    0.25

41  -4.2500 0.000020983 0.000031671 0.000020895    0.73

42  -4.5000 0.000007291 0.000010689 0.000007312    0.12

43  -4.7500 0.000002381 0.000003398 0.000002366    0.19

44  -5.0000 0.000000730 0.000001017 0.000000707    1.50

45  -5.2500 0.000000211 0.000000287 0.000000216    0.23

46  -5.5000 0.000000057 0.000000076 0.000000054    0.44

47  -5.7500 0.000000015 0.000000019 0.000000011    2.23

48  -6.0000 0.000000003 0.000000004 0.000000005    0.13

49 -99.9900 0.000000001 0.000000001 0.000000000    0.00

This run demonstrates exceptionally good agreement between SSSWIN.EXE

simulation results for bmtype=1 and rntype=2 with theoretical normal

calculations.  In fact, the 5% lower-tail significance point of the

chi-square distribution with 47 degrees-of-freedom is 32.3 (while its

5% upper-tail significance point is 64.0).  Thus the observed

lack-of-fit is TOO SMALL to be attributable to mere chance at the 5%

significance level!!!

My experience with the pseudo-random number algorithms provided with

SSSWIN is as follows:

SSSWIN.EXE simulation results with bmtype=0 tend to be somewhat more

"short tailed" than a theoretical normal distribution.

SSSWIN.EXE simulation results with rntype = 3 are less smoothly

uniform in the long-run than are those with the rntype = 1 or 2.

Apparently, the 32-bit L'Ecuyer and Wichman-Hill algorithms are

capable of producing (uniform) variate values much closer to 0.0

and 1.0 than rntype = 3 can.

Simulation of an entire doubly-unbounded distribution is a truly

difficult task.  Simulation results can be expected to be least

accurate in the tails.  But tails that are "too thin" also imply,

of course, that the center ends up being "too thick"...or vice

versa.  As a result, significant lack-of-fit to the parent can

occur anywhere, even near the "center" of the distribution.

==========

<<< REFERENCES >>>

==========

Carr, W. E. (1991). "A New Process Capability Index: Parts Per

Million."  Quality Progress 24, no.8 (August), page 152.

Chisholm, Jesse, and Thomborson, Clark. (1991).  Public domain C-

language source code (LECRAND/SETRAN/CHECKRAN) to generate,

seed and check the L'Ecuyer(1988) random number generation

algorithm.

Six-Sigma-Simulator for Microsoft Windows         Page 13 of 14

Harry, M. J. (1987). "The Nature of Six-Sigma Quality." (booklet)

Motorola Communications Sector, Schaumburg, IL.

Hoaglin, D. C. (1982).  "Generation of Random Variables."  Encyclopedia

of Statistical Sciences, (N.L.Johnson, S.Kotz & C.B.Read,

editors).  Vol.3, 376-382.  New York: John Wiley and Sons.

Johnson, N. L., Kotz, S. and Read, C. B. (1982).  "Box-Muller

Transformation"  Encyclopedia of Statistical Sciences,

Vol.1, 312-313.  New York: John Wiley and Sons.

Knuth, D. E. (1981).  The Art of Computer Programming - Vol.2:

Seminumerical Algorithms, 2nd ed., Sec.3.2-3.3.  Reading,

Mass:  Addison-Wesley.

Lancaster, H. O. (1982).  "Chi-Square Distribution."  Encyclopedia

of Statistical Sciences, (N.L.Johnson, S.Kotz & C.B.Read,

editors).  Vol.1, 439-442.  New York: John Wiley and Sons.

L'Ecuyer, P. (1988).  "Efficient and Portable Combined Random Number

Generators."  Communications of the ACM 31, 742-749,774.

McLeod, A. Ian (1985).  "Remark AS R58: A Remark on Algorithm AS 183.

An Efficient and Portable Pseudo-Random Number Generator."

Applied Statistics 34, 198-200.

Moore, D. S. (1986).  "Test of Chi-Squared Type."  Goodness-of-Fit

Techniques. (R.B.D'Agostino and M.A.Stephens, editors.)  63-95.

New York: Marcel Dekker.

Park, S. K. and Miller, K. W.  (1988).  "Random Number Generators: Good

Ones are Hard to Find."  Communications of the ACM 31, 1192-1201.

Press, William H., Flannery, B. P., Teukolsky, S.A., and Vetterling, W.

T.  (1988). Numerical Recipes in C: The Art of Scientific

Computing.  [especially Chapter 7:  Random Number Generation.]

Cambridge University Press. { Source Code Disk:  Copyright 1987

by Numerical Recipes Software, P.O.Box 243, Cambridge, MA 02238.}

Roscoe, J. T. and Byars, J. A. (1971).  "An investigation of the

restraints with respect to sample size commonly imposed on the

use of the chi-square statistic."  Journal of the American

Statistical Association, 66, 755-759.

Thomborson, Clark. (1991). "Mrandom (version 1): a uniform interface to

several random number generators."  Comp.Sources.Unix 25(23).

Revised: August 1992.  Internet: cthombor@ub.d.umn.edu

Wichman, B. A. and Hill, I. D. (1982). "Algorithm AS 183: An Efficient

and Portable Pseudo-Random Number Generator."  Applied Statistics

31, 188-190.

Six-Sigma-Simulator for Microsoft Windows         Page 14 of 14

Zeisel, H. (1986). "Remark AS R61: A Remark on Algorithm AS 183.  An

Efficient and Portable Pseudo-Random Number Generator."  Applied

Statistics 35, 89.

SSS Update History:

===================

9109 ...Accumulate using LONG integers; Correct Motorola Description

9208 ...Make MenuOn mode optional, except on the first KeyPress.

9302 ...Add L'Ecuyer(1988) 32-bit random number generator option.

9404 ...First Microsoft Windows version of Six-Sigma-Simulator.

...Major revision of this documentation file.