TIDES.DOC

DOCUMENTATION FOR TIDES - VERSION 2.07 - 1991 NOV 23

Edward P. Wallner

32 Barney Hill Road

Wayland, MA 01778-3602

(508)-358-7938

(Note: Recent changes and additions are underlined)

This is a program for predicting the height of the tide or the strength of

the tidal current at various stations. It runs on IBM compatible machines

under MS-DOS or IBM-DOS. The method and format followed in the predictions

are generally those used by the National Ocean Service (NOS) in the

standard tide and current tables.

This program may be freely copied and used for non-commercial purposes.

If you use the program a $15 registration fee is requested. In any case

feel free to try the program and pass it on without alteration and without

charge.

The author assumes no responsiblity for any errors in the program or the

predictions.

Detailed predictions are made for high and low water (or maximum ebb and

flood) at reference stations for which the tidal constants have been

derived from observations made over a long period of time. The tides for

subordinate stations are found by making corrections to the times and

heights (or speeds) at the reference stations. The predictions for

subordinate stations are less accurate than for reference stations,

particularly for the lower high and higher low water where tides are

nearly diurnal. Current floods/ebbs at subordinate stations may also be

less accurate. For stations where the current is rotary the current along

the principal axis is predicted. The times and speeds of flood and ebb in

these cases are approximate.

The TIDES program predicts the height of the tide or strength of the

current at any time. The values at reference stations are of the same

order of accuracy at any time as for the high/low or flood/ebb values. The

predictions for subordinate stations may be less accurate at intermediate

times, the more so the larger the differences from the reference station

PROGRAM OPERATION

Just enter TIDES and follow the prompts. When a date is to be input a

default date is displayed. It may be modified using the numerical keys and

Home, End, left and right arrows and backspace. In general entering "Q"

will stop any operation and return you to the menu.

Initially the start date is today and the end data is a week from today. To

run the next week's tides for the default station just enter "A".

If the start date is earlier than 1582/10/15 the Julian calendar is

assumed, otherwise the Gregorian calendar is assumed.

The main menu gives options for computing and printing highs and lows, for

computing and printing hourly heights, for plotting heights at a given

interval (0.5 hours generally works well), for computing the height at

specified dates and times, for changing the time interval and station, for

changing the default station, for creating additional data files, for

changing from standard to daylight time or vice-versa (in the U.S. Daylight

Saving Time extends from 2 a.m. on the first Sunday in April to 2 a.m. on

the last Sunday in October), for changing from metric to English units or

vice-versa and for printing the astronomical and constituent factors

entering the predictions (used mainly for verifying the program). The

equivalent quantities are computed for tidal currents.

The printed output can be redirected to a file instead of the printer. The

output will be an ASCII file which can be edited using any text editor.

When the "CHANGE STATION" or "CHANGE DEFAULT STATION" option is executed a

list of stations for which tidal constants have already been stored in the

TIDES format is shown, including the file names under which they are kept.

The first pages show a selected set of stations for a specific area. The

following pages generally give all of the U.S. reference stations except

those in Alaska. The next page is shown when M is pressed and the list is

restarted from the top when R is pressed.

When the desired station is on the screen press S to stop paging. You will

then be prompted to enter the station file name which is shown in

parentheses after the station coordinates.

The list can be edited using EDLIN or any other text editing program. For

example undesired stations can be eliminated and other station titles

moved to the desired line.

The station names and locations stored in file STALIST.T&C are displayed

by the program as a reminder, but the name of any file of data in the TIDES

format can be used whether it is on the screen or not. New stations for

which no data file exists can be created using menu option H.

CREATING ADDITIONAL DATA FILES

Option H is used to generate data files for the TIDES program both for

stations for which harmonic constants are available and for subordinate

stations, for which only differences from a reference station are

available.

First a title and file name for the station are entered. The prompt for

the title has places for the latitude and longitude of the station which

are optional inputs for better identification and do not affect the

computation. For stations for which harmonic constants are available you

are then prompted to enter the type of station (tide, current, or

hydraulic current) and the amplitude and epoch of each of 37 tidal

constituents. The constants are entered in sequence from the NOS form 444

of "Standard Harmonic Constants for Prediction", columns B (?.??xH) and D

(k'). If there is no data in column  B, use column H (Amplitude). If the

amplitude of a constituent is zero entering "0" or just a carriage return

will enter 0 for both the amplitude and epoch. The modified epoch D (k')

should be entered as a number from 0 to 360; ignore any minus sign in

column D.

Several minor constituents can optionally be inferred from the major

constituents if they are not entered directly. This generally gives a

slight improvement in accuracy at the cost of a slight increase in running

time.

For subordinate stations the corresponding reference station data must have

been entered. The list of stations for which data is already stored is

shown and the file name of the reference station entered. The time

differences between the reference and subordinate stations at high and low

(or maximum flood and ebb) are entered, then the height differences. In

some cases the range of the tide at both stations is requested, as is the

maximum flood and ebb for current stations. This data can be taken from

Table 2 of the NOS tide or current tables.

When Option H is run the title of the new station is added to the end of

the STALIST.T&C file. This file must be edited if it is desired to place

the station title in a different line.

REQUIREMENTS AND CHARACTERISTICS

TIDES should run on any IBM machine or compatible with 128K of free memory

and the text output should be OK on any printer. The plot of tide heights

on the monitor works on the Color Graphics Adapter (CGA) board and on the

printer uses codes for the EPSON (IBM extended ASCII.) Special codes to set

up NEC pinwriter printers are also available. TIDES.EXE, the executable

version of the prediction program, runs with or without an 80x87

coprocessor.

TIDES computes the highs and lows for Boston for a month in about 1

minutes on an IBM-XT with an 8087 coprocessor, and in about 38 minutes

without an 8087. (An 8087 coprocessor can be bought for less than $100 and

is a good investment if you do much number crunching, such as tidal

predictions. For systems with a 80286 or higher chip the program is

reasonably fast without a coprocessor.)

Predictions for Boston and New York for 1986 agreed with the standard

tables to the minute and tenth of a foot for most cases with some times off

by 2 or 3 minutes and an occasional difference of .1 or .2  foot. Currents

in Cape Cod Canal were generally within 0.1 knot. (If the tide level

remains nearly constant for an extended period the predicted time of high

of low can vary considerably but the height will still be accurate.)

Predictions should be accurate for any era within a few centuries of the

present unless the physical conditions at the station change. Accuracy

increases very slightly using short prediction intervals. Intervals much

greater than one year may lead to significant error and those greater than

480 days are rejected.

The source program TIDES.BAS is written in QuickBASIC. In order to fit a

good selection of station constants on the distribution disk, the source

program is omitted. It is available to registered users on request.

DATA SOURCES

A list of places for which I currently have harmonic constants is given at

the end of this document.

The harmonic constants for various North American reference stations are

published by:

National Oceanographic and Atmospheric Administration

National Ocean Service N/OMA132

Rockville, Maryland 20852

I have a complete set of this data for all U.S. reference stations

including Alaska, about 76 other U.S. stations and many stations in British

Columbia (see BCLIST.TXT for a list of the latter.) I will send a copy of a

limited number of station constants and subordinate station corrections to

registered users who let me know the area of interest. The stations for

which constants are available are listed below.

The International Hydrographic Organization maintains a data bank of tidal

constants for  4208 stations worldwide. If you wish constants for foreign

locations send me the latitudes and longitudes (to the nearest minute) and

I will eventually send the constants for the closest station in the IHO

data set if available. (The IHO data uses the symbols H for amplitude and G

for phase in place of k'.)

The tide tables published by the British Admiralty give data for use in

their Simplified Harmonic Method of Tidal Prediction. This data is given

for about 4400 tide and 148 current stations world wide. A PC program

implementing this method is also distributed. Harmonic constants are given

for M2, S2, K1 and O1 as well as data from which M4, M6, S4, S6, MS4, MSf,

Sa and SSa can be computed. Predictions of about the same accuracy can be

made with TIDES if inferred constants are included in the data file.

Differences for tides and currents for subordinate U.S. locations are given

in the annual tables published by the National Ocean Service. In 1989 the

datum to which height of tide is referred was changed from mean low water

to mean lower low water for all U.S. stations. Though the difference is

generally only a couple of tenths of a foot or less, data should be taken

from 1989 or later tables. The constants themselves do not change

appreciably over many years so any later table can be used.

The theory used in the TIDES program is given in:

Manual of Harmonic Analysis and Prediction of Tides

Paul Schureman

Coast and Geodetic Survey Special Publication No.98

U.S. Gov't. Printing Office, Washington D.C. 1941

This book has been reprinted recently and is available from NOS for $5.00.

A supplement to this document, "Computer Applications to Tides in the

National Ocean Survey" is also available from NOS at no cost.

MISCELLANEOUS

The present version of the program does not give predictions of slack

water at current stations and for rotary currents the north and east

components of velocity must be predicted separately at hourly intervals

and combined vectorially by the user. If there is a demand for these

features I may get around to adding them sometime.

Starting in the 1989 tables the datum was changed to mean lower low water

for all U. S. ports, which meant a change of a few tenths of a foot in Z0

(item 1, line 2 of -.TID data files) for ports on the Atlantic coast. The

new values have been used in the data on this disk.

If you have any suggestions, comments or problems please let me know.

HARMONIC CONSTANTS ARE AVAILABLE FOR THE STATIONS LISTED BELOW

REFERENCE STATIONS COVERED BY NATIONAL OCEAN SURVEY

TIDES - EAST COAST

Albany, New York                     Punta Palmas, Venezuela

Amuay, Venezuela                     Philadelphia, Pennsylvania

Baltimore, Maryland                  Portland, Maine

Boston, Massachusetts                Reedy Point, Delaware

Breakwater Harbor, Delaware          St. Marks River Entrance, Florida

Bridgeport, Connecticut              St. Petersburg, Florida

Caleta Percy, Magellan Strait, Chile***

Charleston, South Carolina           Sandy Hook, New Jersey

Cristobal, Panama                    San Juan, Puerto Rico

Eastport, Maine                      Savannah, Georgia

Galveston, Texas                     Savannah River Entrance, Georgia

Hampton Roads, Virginia              Suriname Rivier, Surinam

Isla Zapara, Venezuela               Tampico Harbor, Mexico

Key West, Florida                    Washington, D.C.

Mayport, Florida                     Willets Point, New York

Maimi Harbor Entrance, Florida       Wilmington, North Carolina

Mobile, Alabama

New London, Connecticut

Newport, Rhode Island

New York, New York

Pensacola, Florida

Punta Gorda, Venezuela

TIDAL CURRENTS - EAST COAST

Baltimore Harbor Approach, MD        Mobile Bay Entrance, FL

Boston Harbor, MA                    Pollock Rip Channel, MA

Cape Cod Canal, MA                   Portsmouth Harbor Entrance, NH

Charleston Harbor, SC                St. Johns River Entrance, FL

Chesapeake & Delaware Canal          Savannah River Entrance, GA

Chesapeake Bay Entrance              Tampa Bay Entrance, FL

Delaware Bay Entrance                The Narrows, New York Harbor, NY

Galveston Bay Entrance, TX           The Race, Long Island Sound

Hell Gate, East River, NY            Throgs Neck, Long Island Sound

Key West, FL                         Vieques Passage, Puerto Rico

Miami Harbor Entrance, FL

TIDES - WEST COAST

Aberdeen, Washington                 Matarani, Peru

Anchorage, Alaska ***

Astoria, Oregon

Balboa, Panama                       Nikishka, Alaska

Buenaventura, Columbia               Nushagak Bay, Alaska

Callao, Peru                         Port Townsend, Washington

Cordova, Alaska

Dutch Harbor, Alaska                 Puntarenas, Costa Rica

Guayaquil, Ecuador                   St. Michael, Alaska

Guyamas, Mexico                      Salina Cruz, Mexico

Honolulu, Hawaii                     San Diego, California

Humboldt Bay, California             San Francisco, California

Juneau, Alaska                       Seattle, Washington

Ketchikan, Alaska                    Seldovia, Alaska

Kodiak, Alaska                       Sitka, Alaska

Kwajelein Atoll, Marshall Is.        Sweeper Cove, Alaska

La Union, El Salvador

Los Angeles, California              Talara, Peru

Massacre Bay, Alaska                 Valdez, Alaska

TIDAL CURRENTS - PACIFIC COAST

Admiralty Inlet, WA                  Rosario Strait, WA

Akutan Pass, Aleutian Islands        San Diego Bay Entrance, CA

Ch'ang Chiang Entrance, China        San Francisco Bay Entrance, CA

Deception Pass, WA                   San Juan Channel (S. Entrance), WA

Grays Harbor Entrance, WA            Sergius Narrows, AK

Huang P'u Chiang, China              Strait of Juan de Fuca Entrance

Isanotski Strait, AK                 The Narrows, Puget Sound, WA

Kvichak Bay, AK                      Unimak Pass, Aleutian Islands

North Inian Pass, AK                 Wrangell Narrows, AK

*** Complete prediction requires more than 37 constituents.

OTHER TIDE STATIONS

Alameda, CA                          Apalachicola, FL

Apia, Samoa Islands

Aquia Creek, Potomac River, VA       Aransas Channel, TX

Arthur Harbor, Palmer Station, Antarctica

Atlantic City (Steel Pier), NJ       Avon, NC

Bar Harbor, ME                       Bayou Rigaud (Grand Isle), LA

Bayshore, L.I., NY                   Bay St. Louis, MS

Bears Bluff, N. Edisto R., SC        Block Island (Old Harbor), RI

Boca Congrejos, Isla Verde, PR       Buzzards Bay Entr, Cape Cod Canal, MA

Burntcoat Head, Nova Scotia          Canton Island, Phoenix Is., S. Pac.

Cape Cod Canal, E. Entrance, MA      Cape Hatteras (Fishing Pier), NC

Cape May (ferry terminal), NJ        Cape Steenboom, New Guinea

Carlos Point, Estero Bay, FL         Charleston, OR

Chesapeake Beach, MD                 Coconut Point, FL

Colonial Beach, VA                   Constantine Harbor, AK

Crescent City, CA                    Daytona Beach, Fl

Daytona Beach Shores (Sunglow Pier), FL

Elkhorn Slough, CA                   Esbjerg, Denmark

Friday Harbor, San Juan Island, WA

Galveston, Pleasure Pier, TX         Garibaldi, OR

Guam (Apra Harbor), Marianas         Hirtshals, Denmark

Indian River Inlet, DE

Lewisetta, Potomac River, VA         Turkey Point, FL

La Jolla, CA                         Eureka, CA

Point of Pines, SC                   Port San Luis, CA.

Neah Bay, WA                         Cross Island, AK

Nantucket, MA                        Malakal Harbor, Palau Is. Caroline Is.

Matanzas Pass, Estero Island, FL     McMurdo Sound, Antarctica

Mokuoloe Island, HI                  Montauk, Long Island, NY

Monterey Harbor, CA                  Morehead City, NC

Moss Landing, CA                     Myrtle Beach (Springmaid Pier), SC

Naples, FL                           New Bedford, MA

Newport Bay, CA                      Nikiska, Cook Inlet, AK

Old Saybrook Point, CT               Padre Island (South End), TX

Ponape Harbor, Caroline Islands      Port Aransas, TX

President Channel, Orcas Is., WA     Providence, RI

Puerto Soberania, Palmer Peninsula

Punta Tunas, PR                      Reykjavik, Iceland

Rincon Island, CA                    Rockland, ME

Sabine Pass, TX                      Saint John, New Brunswick

Seavey Island, ME                    Shell Point, FL

Southbeach, OR                       Stamford, CT

Tarawa, Gilbert Islands              Toke Point, WA

Virginia Beach, VA                   Wachapreague, VA

Wallops Island, VA                   Yakutat, AK

INTERNATIONAL HYDROGRAPHIC ORGANISATION DATA

Banyuls, France                      Socoa, France

Toulon, France                       Marseille, France

Saint Servan, France                 Cherbourg, France

Nice, France                         Venezia, Italy

Grado, Italy                         Esbjerg, Denmark

Aarhus, Denmark                      Hirtshals, Denmark

Saint John, New Brunswick, Canada    Burntcoat Head, Canada

Grand Turk, UK

Cabo San Lucas, B.C.S., Mexico       La Paz, B.C.S., Mexico

Magdalena Bay, Mexico                San Carlos, B.C.S., Mexico

Topolobampo, Sin., Mexico            San Juanico Bay, Mexico

Yavaros, Son., Mexico                Ballenas Bay, Mexico

Guaymas, Som., Mexico                Isla Guadalupe, B.C., Mexico

Bahia de los Angeles, B.C., Mexico   Puerto Penasco, Son., Mexico

Ensenada, B.C., Mexico               Salina Cruz, Oax., Mexico

Matavia, Tahiti, France              Motuoini, Tahiti, France

Nuku'alofa, Tonga                    Urangan Jetty, Australia

Brisbane Bar, Australia              Caloundra Heads, Australia

Brisbane (W. Inner Bar), Australia   Snapper Rocks(Danger Point), Australia

Ballina, Australia                   Iluka, Australia

Victoria Dock, Singapore             Horsburgh Lighthouse, Singapore

Tg. Segenting, Malaysia              Colombo, Sri Lanka

Hong Kong, UK                        Kao-Hsiung, China

Fujairah, Oman (U.A.E.)              Naha Ko, Japan

Kobe, Japan                          Nagoya, Japan

Sinko, Yokohama, Japan

CANADIAN DATA

Constants for all reference tide stations in the NOS tables for the East

and West coasts are available as well as many stations in British Columbia.

Tidal current constants for Grand Manan Channel (Bay of Fundy Entrance) are

also on hand.

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

Public (software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  PsL cannot debug pro-

programs over the telephone, though we can answer questions.

Disks in the PsL are updated  monthly,  so if you did not get

this disk directly from the PsL, you should be aware that the

files in this set may no longer be the current versions. Also,

if you got this disk from another vendor and are having prob-

lems,  be aware that  some files may have become corrupted or

lost by that vendor. Get a current, working disk from PsL.

For a copy of the latest monthly software library newsletter

and a list of the 4,000+ disks in the library, call or write

Public (software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

1-800-2424-PSL

MC/Visa/AmEx/Discover

Outside of U.S. or in Texas

or for general information,

Call 1-713-524-6394

PsL also has an outstanding

catalog for the Macintosh.