Spheric

=========

A Spherical Geometry Calculator and Navigation Aid

Version 1.20

by J. Andrzej Wrotniak

July 22,1994

Table of Contents

1.  Introduction

1.1.  What does Spheric do?

1.2.  PC-DOS and HP95/100LX mutations

1.3.  Extended (registered) version

1.4.  Disclaimer

2.  The Main Screen

2.1.  The History Log

2.2.  The Status Line

2.3.  The Fix Database Display

2.4.  The F-Key Menu

2.4.1.  Nested menus

2.4.2.  Nested F-key notation

2.4.3.  Help and Exit keys

2.4.4.  The Top Menu

2.5.  The Prompt Line

3.  The Fix Database

3.1.  Fix display format

3.2.  Moving around

3.3.  Finding a fix

3.4.  Inserting a fix

3.5.  Appending a fix

3.6.  Modifying a fix record

3.7.  High accuracy fix display

3.8.  Deleting a fix

3.9.  Saving a fix file

3.10.  Loading a fix file

3.11. Sorting fixes by name or category

3.12. Sorting fixes by proximity

3.13. Clearing the database

4.  Computational operations

4.1.  Distance between fixes

4.2.  Azimuth from a fix to another

4.3.  Midpoint between two fixes

4.4.  Fix at a given azimuth and distance

4.5.  Intersection of two great circles

4.6.  Projection of a fix on a great circle

4.7.  Distance from a fix to a great arc and its circle

4.8.  Distance between two great arcs

5.  Data entry

5.1.  Editing the input

5.2.  Special characters

5.3.  Entering a single fix name

5.3.1. Name completion

5.3.2. Recalling previous inputs

5.3.3. New and old fix names

5.4.  Entering two fix names

5.5.  Entering fix coordinates

5.6.  Entering an azimuth

5.7.  Entering a distance

6.  The History Log

6.1.  Display format

6.2.  Scrolling through the log

6.3.  Deleting a log entry

6.4.  Clearing the log

7.  Program settings

7.1.  Distance units

7.2.  Distance display precision

7.3.  Angular modes

7.4.  Sign convention

7.5.  Fix category display

7.6.  Earth model

7.6.1.  Spherical Earth

7.6.2.  Elliptical Earth

7.6.3.  Accounting for the ellipticity

7.7.  Sound alerts

8.  Miscellaneous operations

8.1.  On-line help

8.2.  Clearing history and data

8.3.  Exiting the program

9.  Program installation and use

9.1.  The packing list

9.2.  Installing Spheric on your computer

9.3.  Character set compatibility

9.3.1.  Screen (PC)

9.3.2.  Printer (HP)

9.4.  Running Spheric from DOS

9.5.  Running from the Filer (HP)

9.6.  Making a loader file (HP)

9.7.  Saving disk space (HP)

10. Registration and distribution

10.1. How to register

10.2. The extra goodies

10.3. Registration fees

10.4. Distribution

10.5. Support

10.6. Improvements and new features

10.7. Credits

10.8. About the author

1. Introduction

1.1. What does Spheric do?

Spheric  allows you to define a number of points (fixes) on the  Earth

sphere and then to perform various calculations involving these fixes:

distance, azimuth, arc intersections etc.

1.2. PC-DOS and HP95LX/HP100LX mutations

Spheric  has  been  originally written for  the  Hewlett-Packard  95LX

palmtop computer. The HP95 version will also run on the HP100LX. It is

denoted with the letters "HP" in the version number, e.g., "1.20/HP".

Although  the HP palmtop version will also function on  "regular"  DOS

machines,  there is a version of the program compiled specifically for

them.  It  centers the Spheric display in the screen,  uses color  (if

applicable)  and  provides  function  key  labels.   This  version  is

designated  as "PC",  as in "1.20/PC".  Otherwise both  mutations  are

identical,  and the HP version will run on a full screen - but without

color and F-key labels.

The PC version of Spheric will run on virtually all PC-DOS clones with

any  display.  A math coprocessor,  if present,  will be detected  and

used.

1.3. Extended (registered) version

Spheric is shareware; the details on its distribution and registration

are given in Chapter 10.

***  Some program features may be available only in registered  copies

of  Spheric,  distributed  exclusively  to  those  who  paid  the

registration  fee.  These are designated with letter "R"  in  the

version number (e.g., 1.20R/HP).

***  Parts  of this document referring specifically to the  registered

version are marked with a "***" string.

***  In  the  current version,  there is only one  difference  between

registered and non-registered copies of the program:  the size of

the fix database and of the history log.  As a  calculator,  both

versions are equally capable.  The registered  version,  however,

comes with two fix data files included (see Section 10.2).

1.4. Disclaimer

Although  the author has extensively tested the software and  reviewed

the documentation,  he makes no warrant or representation, explicit or

implied,  with  respect to this  package,  its  quality,  performance,

merchantability or fitness for a particular purpose,  in any  hardware

or software environment.

In no event shall the author be liable for direct,  indirect, special,

incidental or consequential damages arising out of the use,  misuse or

inability to use the software or documentation.

2. The Main Screen

When  you run Spheric,  you will first find yourself in the  program's

Main Screen, which consists of four parts:

��������������������������������������Ŀ

�Fix at distance 1200mi and            �

�azimuth -75�19' from HONOLULU,HI      �

�                      24�42'N 176�23'W�

��������������������������������������Ĵ

� 4� NE � mi � DMS �Ellip� N = 32  �","�

��������������������������������������Ĵ

�  12� HONOLULU  HI   21�19'N 157�52'W �

�  13� HOUSTON   TX   29�46'N  95�22'W �

�  14� INDIANAPO IN   39�46'N  86�09'W �

�  15� LASVEGAS  NV   36�10'N 115�09'W �

�  16� LOSANGELE CA   34�04'N 118�15'W �

�  17� MEMPHIS   TN   35�08'N  90�03'W �

�  18� MIAMI     FL   25�47'N  80�11'W �

����������������������������������������

CAL FFd Azi Mid Aim FAd Prj AAd Int Top

These are,  from top to bottom,  the History Log, the Status Line, the

Fix Database and the F-Key Menu.  The last one is sometimes taken over

by prompts asking for some input.

2.1. The History Log

This  is the long,  three-line area near the  top.  Every  calculation

performed by Spheric is displayed here.  The first two lines show  the

input  data,  the  third  one - the result.  For more  details  -  see

Chapter 6.

2.2. The Status Line

This  line  is  located below the Expression  Line  and  contains  the

following information (left to right).

-   The  number  of the History Log entry  currently  displayed  (see

Chapter 6).

-   The default direction indicator,  showing whether North or  South

(East  or  West)  will be assumed when  latitude  (longitude)  is

entered  without the direction letter N or S (E or W).  See  also

Section 7.4.

-   Unit indicator,  showing the units in which distance is displayed

or entered (see Section 7.1).

-   Angular mode: DMS (degrees and minutes) or Deg (decimal degrees).

This  is  the mode used for display of  latitude,  longitude  and

azimuth. See Section 7.2 for more.

-   Earth model used in calculations: elliptical or spherical.

-   Number of fixes currently in the database.

-   The comma mode indicator,  ",",  or an empty space (see  Sections

6.1 and 7.4).

2.3. The Fix Database Display

Most of the Main Screen is taken by the Fix Database Display.  The fix

number  is shown at the left,  followed by the fix name and  category,

latitude  and longitude.  The display shows up to seven fixes  at  the

time.

The Fix Database is described in detail in Chapter 3.

2.4. The F-Key Menu

The  bottom line is taken by the Function Key Menu with mnemonics  for

the function keys F1..F10.  We will be referring to function keys with

these abbreviations in square brackets, e.g., [Set] instead of F2.

2.4.1. Nested menus

Some of the function keys do not cause any immediate action.  Pressing

such  a  key  will,  instead,  bring  up a  new  menu  with  different

operations assigned to the F-keys.

For example,  F3, marked as [Cal], will bring up the Calculation Menu;

while that one is active,  F2,  denoted as [Azi], performs the azimuth

calculation.

The name of the currently active menu is always displayed in uppercase

letters above the F1 key.

2.4.2. Nested F-Key notation

Nested  function  key sequences are denoted in this  manual  with  key

mnemonics  separated by a vertical bar;  thus the operation  described

above would be referred to as [Cal|Azi].

As  the current menu is indicated with the key mnemonic in the  Status

Line,  [Cal|Azi] may be also interpreted as "Press the [Azi] key while

the CAL menu is active,  i.e.  'CAL' is being displayed at the  bottom

left of the screen".

2.4.3. Help and Exit keys

In all function key menus, F1 activates the on-line help. Because this

is true for every menu, the space above the F1 key is not used for the

[Hlp] mnemonic (like in previous versions of the program), but for the

menu name, displayed in capital letters, e.g. "SET".

In all menus F10 is used to return to the next higher menu level (when

shown as [Top]) or,  in case of the top menu, to leave the program (in

this case it shows as [Exit]).

2.4.4. The Top Menu

This is the F-key menu visible first when the program is executed.  In

addition  to [Hlp] and [Exit] functions,  it allows for  switching  to

other,   more   specialized  menus,   containing  groups  of   related

operations:

[Set]     brings  up the Settings Menu,  from where different  program

settings can be adjusted, as described in Chapter 7

[Cal]     switches  to  the Calculations Menu,  with all  the  numeric

operations described in Chapter 4

[Dat]     calls  the  Database  Menu,  with  the  database  operations

(mostly those from Chapter 3)

While in any of these three menus, the [Top] function key will take us

back to the Top Menu.

2.5. The Prompt Line

Some  times  Spheric will ask you for a numeric or  textual  input  by

displaying a prompt at the bottom of the screen, temporarily replacing

the F-key menu.

Data input at prompts is discussed in detail in Chapter 5.

3. The Fix Database

Spheric  is  capable  of storing up to 32  geographic  locations  (for

short: fixes).

***  The  registered  version  stores up to 9999 fixes  (if  there  is

enough memory in the system).

One  of  the fixes (or,  sometimes,  an empty record) will  always  be

selected,  i.e.  highlighted.  This  will be always a fix  within  the

currently displayed range.  Up to eight fixes will be shown in the Fix

Database Display at one time.

3.1. Fix display format

Each fix is shown as one line of text. It consists of the following:

*   Fix  name (up to eleven characters starting from  a  letter;  all

letters will be in uppercase)

*   Fix category,  if present;  up to ten characters (denoting state,

country or any other information you wish)

*   The  latitude  in  degrees and minutes  or  in  decimal  degrees,

depending on the current angular mode

*   The longitude, as above

Some  examples  of this format could be seen in  the  illustration  in

Chapter 2.

Individual fixes can be displayed with higher accuracy as described in

Section 3.7.

3.2. Moving around

At most seven fixes are visible at a time in the Fix Database Display.

Others may be viewed by scrolling the display up or down.

The  up-  and  down-arrow keys select the  previous  (next)  fix  with

respect to the currently selected one. If the latter is already at the

top (bottom) of the display,  the displayed range will be scrolled  by

one record.

Obviously,  it  is impossible to scroll above the first record in  the

database or more than one record beyond the last one.

The  Page Up and Page Down keys (Shift-Up and Shift-Down on the  95LX)

will take you to the top or to the bottom of the current page  or,  if

you are already there, will scroll one page up or down, respectively.

The  Home and End keys (on the Palmtop:  Shift-Left  and  Shift-Right)

will cause a jump to the top or to the bottom of the whole database.

3.3.  Finding a fix

In  order to find a fix by its name,  just start typing this  name.  A

one-second  pause  (or pressing the Return key) marks the end  of  the

input.  Spheric  will then jump to the first record matching what  you

have typed in. After that, pressing Tab will take you to the next such

record (if it exists).  The display will be scrolled so that the newly

selected fix will be in the displayed range.

You  can  also search for fixes with specified  category  abbreviation

(for example,  to find fixes in California,  denoted with "CA" in  the

small database US.FIX,  enclosed with the program). To do this, type a

comma and then the category, e.g. ",CA".

3.4. Inserting a fix

To insert a new fix into the database, press the Insert key (Shift-Del

on the HP). You will be prompted, first for the new fix name, and then

for its latitude and longitude. (See Chapter 5 for the details on data

entry).  If  your answers do not contain errors,  the new fix will  be

inserted into the database before the currently selected one.

If the name you entered already exists, Ex will warn you about it. You

will be then able to cancel the operation or to overwrite the old  fix

of that name.

3.5. Appending a fix

Another way to add a fix to the database is,  instead of pressing  the

Insert  key,  to  go to the bottom of the database (the  empty  record

following the last one) and press Enter.

The  remaining part of the procedure is as described in  the  previous

section,  except that the new fix will then be appended at the end  of

the database.

3.6.  Modifying a fix record

To  change  the name and/or the coordinates of a  fix,  select  it  as

described in Section 3.2,  and then press the Enter key.  You will  be

prompted  for  the  new  name,  and then  for  the  new  latitude  and

longitude.

If you are renaming a fix,  the new name has to be different than that

of  any  other fix already in the database.  (Entering a fix  name  is

described in Section 5.3.)

3.7. High accuracy fix display

Although  fix coordinates are displayed,  depending on  the  [Set|Ang]

setting, with an accuracy of 1' or 0.01�, which is high enough for all

practical  purposes,  sometimes you may need to view a fix  in  higher

precision.

Pressing  the Escape key will show the currently selected fix  at  the

bottom of the screen, with an accuracy of 1'' or 0.0001�. Any key will

then bring the menu display (and functionality) back.

3.8. Deleting a fix

Pressing the Delete key will remove the currently selected fix,  after

asking for a confirmation.

3.9. Saving a fix file

Spheric will save the fix database as an ASCII (plain text) file  with

fix  names  and coordinates.  This file can be then  used  by  another

program or by Spheric itself.

To save a fix file, press [Dat|Sav]. You will be prompted for the file

name which may contain up to eight characters.  Do NOT enter the  path

(which will default to the current Spheric path) or the extension (the

program will append ".FIX" to the name you enter).

If  the  name  thus created is not a legal DOS  file  name,  an  error

message will be displayed.

If a .FIX file with this name already exists,  it will be  overwritten

without a warning.

The output file format is simple:  each fix takes one line, consisting

of  the  fix  name and category (separated with  a  comma),  a  space,

latitude,  a space again,  and then longitude. The angle notation will

depend on the current angular mode (DMS or Deg,  see Section 7.2). For

example,  the last of the fixes from the screen shot in Chapter 2 will

be written as

NEWARK,NJ 40�44'N 74�10'W

or as

NEWARK,NJ 40.73�N 74.17�W

Note  that  both forms are not exactly  equivalent,  as  the  rounding

errors  due to the limited number of digits will be different in  both

cases.

3.10. Loading a fix file

A fix file can be loaded into the program with [Dat|Loa].  Again,  you

will  be prompted for the .FIX file name without path or extension  as

described above.

The program will also ask,  whether you want to clear the fix database

first; if not, the new fixes will be appended to the database.

The  input  file standard is the same as the output one  described  in

Section  3.8,  except  that  in the Deg mode the degree  sign  is  not

required,  and the coordinate values can be preceded by a sign instead

of being followed by a letter.

In  other  words,  in  addition to the two  formats  specified  above,

Spheric will also recognize a more simple notation as in

NEWARK,NJ 40.73N 74.17W

or simply

NEWARK,NJ 40.73 -74.17

The sign convention (what directions correspond to "+" or "-") can  be

changed by [Set|+/-] (see Section 7.3), so it is also possible to have

the last record interpreted as (35.13S,  90.05E).  To avoid confusion,

do not switch between various sign conventions unless you really  have

to.

3.11. Sorting fixes by name or category

Pressing [Dat|Nam] will sort the fixes in ascending alphabetical order

(as  per  the ASCII codes of the characters).  This is  quite  useful,

especially if you intend to search for fixes by name,  as described in

Section 3.2.

[Dat|Cat]   will   sort  the  database  alphabetically   by   category

abbreviations;  within  the  same category fixes will be  arranged  by

names.

3.12. Sorting fixes by proximity

Fixes can be also be sorted in the increasing order of their distances

from a given fix.  To do this,  select a fix by highlighting it in the

database  window and press [Dat|Prx].  The fix,  distances from  which

have  been used as the basis for the sort will be shown at the top  of

the database.

3.13. Clearing the database

Pressing [Dat|Clr] will bring up the CLR menu.  From here,  [Fix] will

clear  the fix database,  [His] - the history log,  while  [All]  will

clear both.

4. Computational operations

Here we come to what Spheric actually does.  All these operations  are

performed from the Calculations Menu.  To get here, press [Cal] in the

Top Menu.  (There is also a shortcut:  just press the space bar at any

time, and you will jump right here.)

4.1. Distance between fixes

[Cal|FFd]  ("Fix-to-Fix distance") will compute the  distance  between

two fixes. You will be prompted for both fix names.

The  distance is defined as the length of the  great  arc,  connecting

both fixes on the Earth sphere.

Instead of the first fix name you may enter just a "+" (without  quote

marks).  Spheric will then replace it with the name of the second  fix

from the previous distance calculation, compute the arc length, add it

to the previous result and display the total.

This way,  a total length of a route defined by a series of fixes  can

be easily computed.

4.2. Azimuth from one fix to another

Pressing  [Cal|Azi] will compute the azimuth from one fix to  another.

You will have to identify both fixes by their names.

The  azimuth from fix A to fix B is defined as the angle (measured  at

A) between two great arcs - one running from A to the North Pole,  and

the other from A to B.  The clockwise direction is defined as positive

("+").

The result will be always between -180 and 180 degrees.

If  both  points  have the same coordinates,  or  if  one  is  exactly

opposite on the sphere to the other, an error will be reported.

The azimuth from the North Pole to anywhere else is 180 degrees;  from

the South Pole, 0 degrees. Similarly, the azimuth from anywhere to the

North Pole is 0, to the South Pole, 180 degrees.

4.3. Midpoint between two fixes

This point is computed with [Cal|Mid].  A midpoint between fixes A and

B  is defined as a fix C,  located on the great arc from A to  B,  and

such that the distances A-C and B-C are equal.

You  will be prompted for the names of the original two fixes  (A  and

B), and then you will be able to assign the midpoint a name. If you do

that, a new fix will be added to the database.

An error will be reported if one of the points is exactly opposite  on

the sphere to the other.

4.4. Fix at a given azimuth and distance

This operation is invoked with [Cal|Aim]. Given a fix A, azimuth angle

a,  and distance d, it will compute a new fix B, such that the azimuth

from A to B will equal a, and the distance - d.

4.5. Intersection of two great circles

Two  great arcs may or may not intersect (within their  lengths),  but

their great circles always do. This operation, activated by [Com|Int],

will  find  the intersection between the great circles  to  which  two

given great arcs belong.

First  you  will be prompted for the names of two fixes  defining  the

first  great arc (A,B),  then for those defining the other arc  (C,D).

You will be also given an option to assign a name to the  intersection

point E; in this case the fix will me added to the database.

Two great circles (different from each other) intersect in two points.

Out of these two possible solutions,  Spheric will chose the one which

is  closer  to the arcs involved (more  accurately:  to  the  midpoint

between their respective midpoints).

The  result  displayed  in the History Log will  be  followed  by  two

characters;  each of them denotes the relationship of the intersection

E to the first or the second arc:

"*"  means  that the intersection belongs to (falls within the  length

of) the given arc.

"-"  denotes the intersection preceding the arc's first fix.

"+"  means that the intersection follows the arc's second fix.

For  example,  let us consider two great arcs:  the first one  running

from  (5N,0) to (10N,0) and the second from (0,10W) to (0,5W).  It  is

not  difficult  to predict,  that the intersection point  will  be  at

(0,0). It will be marked with "+-", as it follows the end of the first

arc, but precedes the start of the second one.

On  the other hand,  with the first arc from (5S,0) to (5N,0) and  the

second from (0,5W) to (0,5E), the intersection point will be (0,0) but

it will be denoted as "**", falling within the spans of both arcs.

4.6. Projection of a fix on a great circle

The  projection  can  be defined as the point on  the  circle  at  the

closest distance from the arc. For example, the projection of (10N,5E)

on the Equator will be (0,5E).

We  may  be,  for  instance,  interested in finding the  point  on  an

aircraft's  route  (a  great arc) from Warsaw  to  Chicago,  which  is

closest  to London.  This will be exactly the projection of London  on

the great arc (or its circle) Warsaw-Chicago.

This  computation  is activated by pressing  [Cal|Prj].  You  will  be

prompted: first for the names of the fix being projected, then for the

names of two fixes defining the great arc.

The displayed result will be followed with a character,  "*",  "-"  or

"+"  denoting  whether  the  projection falls  within  the  arc  span,

precedes its start,  or follows its end, respectively. For example, if

our fix is (10N,0) and the great arc runs from (0,5E) to (0,10E), then

the projection will be (0,0) and it will precede the arc's start  fix,

so the solution will be marked with a "-".

4.7. Distance from a fix to a great arc and its circle

Pressing  [Com|FAd] ("Fix-to-Arc distance") will compute  and  display

two results:

-   The  "passing distance":  this is the distance between a fix  and

its projection on a great circle (see the preceding section).

-   The  "closest distance",  i.e.  the distance from the fix to  the

closest point of the arc.

The  prompts  and  data  input will be the same  as  in  the  case  of

[Com|Prj],  and the character flag following the result value has  the

same meaning as well.

If that character is a "*" (i.e.  the projection falls within the  arc

length), both results will be the same. For a "-" or a "+", the second

result  (greater than the first one) will be the distance between  the

fix and the start or the end of our arc, respectively.

4.8. Distance between two great arcs

[Com|AAd]  ("Arc-to-Arc  distance")  will prompt  you  for  fix  names

defining  two arcs,  and then the distance between those arcs will  be

computed.  This is the shortest distance between any point on one  arc

and any point on the other.

Obviously,  if  the arcs intersect (within their lengths) or  overlap,

the computed result will be zero.

5. Data entry

At certain times, Spheric will prompt you for input from the keyboard.

Sometimes it will be a simple Yes/No question, sometimes you will have

to answer with a textual or numeric input.

The  prompt  is  always displayed  in  the  Prompt  Line,  temporarily

replacing the F-Key Menu at the bottom of the screen. Your answer will

be also echoed here.

The Yes/No questions require you to press just the Y or N key. Instead

of "Y" you may press again the same key which caused the prompt to  be

displayed,  and  pressing Escape is equivalent to  "N".  For  example,

[Dat|Clr|All]  will prompt you with "Clear all?".  Instead  of  moving

your finger to the Y key,  just press [All] again,  and the  clear-all

operation will be performed.

The  following  general  rules are applicable to all  other  kinds  of

keyboard input.

5.1. Editing the input

Sometimes  a default (or just the previous) answer will  be  displayed

after the prompt.  The answer can be edited with use of the  customary

line-editing keys:  left- and right-arrow, Delete, Backspace, Home and

End.

Pressing  Escape  when any input is present on the  Prompt  Line  will

clear it. The same key with no input present will exit the prompt, and

the  operation  of  which  the prompted input  was  a  part,  will  be

abandoned.

After having the appropriate input typed in,  press Enter to continue.

The  same effect can be achieved by pressing again the same  F-key  in

response to which the prompt has been displayed.

5.2. Special characters

In  all  inputs involving angles,  the degrees character  "�"  can  be

entered by pressing Alt+D (hit the D key, while holding Alt down). The

minutes symbol can be entered as an apostrophe, but also as Alt+M, and

seconds - as double quote or Alt+S.  (Note that seconds are not  shown

in the display, but the input data may use them if you wish.)

5.3. Entering a single fix name

The  fix name can be typed in either upper- or lowercase;  it will  be

converted  to  uppercase  anyway.  Leading  and  trailing  spaces  are

discarded;  spaces inside the name are not allowed.  The maximum  name

length is eleven characters.

The  name  can  be  immediately followed by a  comma  and  a  category

abbreviation (up to ten characters), as e.g. in "PARIS,FRA". The total

length of name+category is limited to 11 characters.

If  these limits are exceeded on input,  name and/or category will  be

truncated  appropriately and redisplayed.  Pressing Enter  again  will

accept the truncated version.

For  example,  "STOCKHOLM,SWEDEN" will be truncated to  "STOCK,SWEDE",

while "NEWBIGPOINT,X" - to "NEWBIGPOIN,X".

5.3.1. Name completion

Instead of entering the whole name (or name+category), you may type in

just  one or more of its first characters and press the Tab  key.  The

program  will then display the first matching name (as ordered in  the

database) on the Prompt Line, together with the category abbreviation.

If  more  names  match,  subsequent uses of the Tab  key  will  scroll

through all matching names,  finally returning to the first one again.

If  there  are  no  matching names,  the entered  text  will  be  just

reformatted to uppercase.

5.3.2. Recalling previous inputs

If the Tab key is pressed when the prompt line contains no input,  the

last used (and accepted) fix name will be displayed there. Press Enter

to accept this input.

Alternatively,  the  up-  and down-arrow keys can be  used  to  browse

through the three most recent inputs.

*** In  the registered version,  last ten inputs (not three)  can  be

recalled.

5.3.3. New and old fix names

In  some  situations  Spheric  expects you to enter  the  name  of  an

existing fix (i.e.  one already in the database).  If this is not  the

case, an error message will be shown.

An opposite case is also possible,  when Spheric waits for a name of a

new fix (different than any already in the database), but you enter an

"old"  one.  If  this happens,  the program will alert  you  that  the

operation will overwrite the existing fix and you can proceed or not.

5.4. Entering two fix names

This  case  is very similar to the one described  above,  except  that

Spheric  expects two existing fix names,  separated with one  or  more

spaces.  Each name can be immediately followed by a comma and category

abbreviation.

Pressing the Tab key will,  obviously, complete just one fix name: the

last one in the Prompt Line.

For  example,  if  the input is just "lo",  it may be  redisplayed  as

"LONDON,GBR ",  but if it is "LO PA",  then pressing Tab will bring up

"LO PARIS,FRA".  To get "LONDON,GBR PARIS,FRA" enter "lo",  press Tab,

then add "pa" and press Tab again. It can't be more simple than this.

If the Tab key is pressed when the prompt line contains no input,  the

last used and accepted pair of fix names will be displayed there.  The

up-  and  down-arrow  keys  allow to  scroll  through  three  (in  the

registered version: ten) most recently entered fix name pairs.

5.5. Entering fix coordinates

At  many points Spheric will expect you to enter the coordinates of  a

fix. It will then display a prompt

Lat, Lon:

followed  by the current coordinates of this fix,  if the fix  already

exists in the database.

Now  the  program  will expect you to enter  values  of  latitude  and

longitude  (in  this particular order),  separated with  one  or  more

spaces (but no spaces within values are allowed!).

The program will understand quite a variety of input formats here, and

they can be mixed in almost any way you wish (the current angular mode

setting does not affect this).

For example, the same coordinates can be entered in any of the formats

shown below:

13�30'N 5�15'W

13.5�N -5�15'

13.5 -5.25

(Note  that the meaning of "-" in coordinate values can be changed  as

described in Section 7.4.)

Any illegal input (and there are quite a many possibilities here) will

cause  an error sound (no message,  though!) and you will be  able  to

correct the mistake.

5.6. Entering an azimuth

When Spheric prompts you for an angular value of an azimuth,  it  will

be able to recognize input in the DMS or Deg format (see Section 7.2),

regardless of the current angular mode.

Azimuth  is measured clockwise from North,  usually from -180 to  +180

degrees,  although  values  between  -360 and  +360  degrees  will  be

interpreted properly.

For example, an azimuth of -90 degrees denotes the direction due West.

The same can be entered as +270 degrees.

5.7. Entering a distance

Every  prompt for value of a distance explicitly states the  units  in

which  the input is expected.  This depends on the current setting  of

distance units (see Section 7.1).

The entered value can (but does not have to) contain a decimal point.

6. The History Log

Every  computational  operation performed by Spheric  is  entered,  if

successful,  into the History Log.  The most recent entry is displayed

in the three-line area at the top of the Main Screen.

Up to 12 recent entries (in the non-registered version) are stored and

can be reviewed as described in Section 6.2.

*** The registered version stores up to 60 log entries.

When the full capacity of the history log has been reached,  each next

entry  will cause the oldest one to be removed from the log,  and  all

others will be pushed up by one number.

6.1. Display format

A  log  entry  consists  of three lines:  the  first  two  define  the

computational problem, the third shows the result.

Coordinates  and  distances  are displayed according  to  the  current

settings  (see Sections 7.1 and 7.2).  Changes to these settings  will

not  affect the existing log entries,  only those which will be  added

afterwards.

Display  of  fix  categories  in the history log  can  be  enabled  or

disabled.  Switching  between  these  two formats  is  done  with  the

[Set|Com] function key sequence,  and if the second mode is active,  a

"," symbol will be shown on the Status Line.

6.2. Scrolling through the log

The left- and right-arrow keys can be used to move from one log  entry

to another.

The pressing these keys while Ctrl is held down will jump to the first

or to the last entry, respectively.

6.3. Deleting a log entry

The  currently  displayed entry can be (after  a  confirmation  query)

deleted by pressing the Backspace key.

6.4. Clearing the log

[Dat|Clr|His] will clear the history log,  while [Dat|Clr|All]  clears

also the fix database.

7. Program settings

To get to the Settings Menu,  press [Set] while in the Top Menu. There

is  also  a shortcut:  while in the Calculations  Menu,  you  can  get

directly  to the Settings Menu by just pressing the Space  key  (press

Space again to get back to the Calculations Menu).

7.1. Distance units

This  setting affects in what units Spheric displays  distance  values

and in what units it expects them on input.

Pressing  the [Set|Uni] function key repeatedly will  change  distance

units  between kilometers (km),  statute miles  (mi),  nautical  miles

(nm), minutes of arc (') and degrees of arc (�).

The  last  two  settings  are  useful  primarily,  but  not  only,  in

astronomical  applications  (and you thought Spheric dealt  just  with

Earth?).

One statute mile is defined as 1.609344 km, and one nautical mile - as

1.852 km.

The  relationship between degrees/minutes of arc and the  other  units

depends,  obviously,  on  the  assumed  Earth  radius.  This  will  be

described in Section 7.6.

7.2. Distance display precision

Depending on the [Set|Uni] setting,  the distance results displayed in

the  History  Log are rounded to the  nearest  km,  mi,  nm,  arc'  or

0.01 arc�.

This  accuracy  is  sufficient for most practical purposes  but  if  a

higher display precision is needed, [Set|Acc] can be used to switch to

a mode where one more decimal digit is displayed.

7.3. Angular modes

This  setting decides how Spheric displays angular  values:  latitude,

longitude  and azimuth.  It does not affect the data input (either  of

the two formats will be always accepted).

Pressing  the [Set|Ang] function key toggles you between the  DMS  and

Deg angular modes.

In  the  DMS mode angles are displayed as degrees  and  minutes,  e.g.

23�15',  in the Deg mode - as degrees with decimal fraction,  like  in

23.25�

When  angular mode is changed,  all entries shown in the Fix  Database

Display will be reformatted.

7.4. Sign convention

Some of us prefer to use "-" and (optionally) "+" signs instead of the

N, S, W and E letters when entering latitude and longitude values.

The  usual convention seems to be that positive values  correspond  to

North  and East.  Spheric allows this to be tailored to your  personal

preferences.  Pressing  the [Set|+/-] key repeatedly will  change  the

sign  convention from NE (where North and East are "+") to  NW  (North

and  West,  often  used  in the United States),  SE  (useful  for  our

Australian friends) and SW.

One has to remember, that if a fix file using signs instead of letters

is  read  by  Spheric  (see Section  3.10),  it  will  be  interpreted

according  to the current setting.  This is important only  for  files

generated  by other programs,  as Spheric always uses the letter  (not

sign) convention when saving fix files.

7.5. Fix category display

The  [Set|Cat]  function key toggles between the two  formats  of  fix

name/category display in the History Log, as described in Section 6.1.

When  the category display is enabled,  a "," symbol will be shown  on

the Status Line,  and an arrow sign will be pointing to the [Cat] menu

abbreviation.

7.6. Earth model

Spheric allows,  in a somewhat limited way, to use either spherical or

ellipsoidal  model of Earth in its  calculations.  Pressing  [Set|Ert]

toggles between these two models.

This may cause some delay,  especially for large fix databases, as the

program  has  to recompute the internal representation of  all  stored

fixes.

7.6.1. Spherical Earth

This model assumes that Earth is a sphere with the radius of 6367.4357

kilometers.

The   spherical  model  is  useful  when  comparing  the  results   of

calculations by Spheric with those of other programs;  it also  should

be  used  when  Spheric  is  used  for  astronomical  (as  opposed  to

geographic/navigational) calculations. In the latter case the distance

units should be set to degrees or minutes of arc (miles or  kilometers

do not make much sense on the sky!).

7.6.2. Elliptical Earth

This model,  often referred to as GRS 1980,  assumes that Earth is  an

ellipsoid with the polar axis of 6356.7523 and the equatorial axis  of

6378.1370 km.

The geometric average of these two values is equal to the  "spherical"

Earth radius as shown in Section 7.6.1.

7.6.3.  Accounting for the ellipticity

For  most  purposes you may just skip this section and  remember  only

that the elliptical model is more accurate than the spherical one. Now

more explanations for those who have to know.

In  the first approximation Earth is a sphere.  It  can  be,  however,

represented  more  accurately as an ellipsoid with  the  shorter  axis

about 1/300 less than the longer one.  (Better approximations are also

available).

When  dealing with the elliptical Earth,  one may distinguish  between

two different definitions of latitude.

The geocentric latitude of a fix P can be defined as the angle between

the Equator plane and the line connecting P with the Earth's center.

The  geographic (geodetic) latitude is the angle between  the  Equator

plane and the line perpendicular to the ellipsoid at P. It is slightly

greater than the geocentric latitude (except at 0 and 90 degrees S  or

N,  where  it is the same).  This is the latitude usually given for  a

point.

Most of navigational and cartographic applications account for Earth's

ellipticity just by performing their operations in three steps:

(1)  Translating  the  geographic  latitude into  geocentric  for  any

points on input;

(2)  Doing all calculations on a sphere with some "average" radius;

(3)  Translating  the  geocentric  latitude into  geographic  for  any

points on output.

Spheric also follows this method,  with the average radius assumed  as

6367.4357 km (the same as the one for the spherical model).

Such  a  procedure  accounts for most,  if not  all,  of  the  Earth's

ellipticity. In principle, the shortest path length between two points

can be computed on an ellipsoid with the full accuracy (a  cumbersome,

iterative  procedure),  but it would be no longer a distance  along  a

great  arc:  there is no such thing like a great arc on an  elliptical

Earth! All other concepts used in our program are based on the concept

of great arc - and this, obviously, makes sense only on a sphere.

7.7. Sound alerts

Spheric uses two kinds of sound alerts:  one to signal a completion of

a computation and one to alert you to an error.

These  sounds  can  be disabled (or enabled  again)  by  pressing  the

[Set|Snd]  function  key.  Enabled  sound is  denoted  with  an  arrow

preceding the [Snd] menu entry.

8. Miscellaneous operations

Here we describe all remaining features of Spheric,  which did not fit

into any of the preceding chapters.

8.1. On-line help

Spheric has a simple on-line help, activated by pressing the F1 key in

any  menu.  The  Help  Screen contains a  list  of  topics  available.

Entering  the topic's first one or two letters (enough to  distinguish

it  from  others) at the prompt and pressing Return,  will  bring  the

proper help page.  Some help pages will offer you a further choice  of

topics - the help is structured in a tree-like fashion.

Pressing  Return,  Escape  or F10 at a topic prompt (or any key  in  a

prompt-less screen) will take you back to the next higher level or  to

the Main Screen.

8.2. Clearing history and data

[Dat|Clr] will bring up a new menu,  denoted as CLR,  from where three

operations can be performed:

[Fix] - clear the fix database

[His] - clear the history log

[All] - clear all (i.e. both of the above).

8.3. Exiting the program

To  leave the program,  press [Exit] in the Top Menu.  Before  exiting

Spheric  will allow you to save its current status  (including  fixes,

history and settings) to disk file named SPH.ENV.  There will be  also

the final request for confirmation,  with the exit prompt showing  the

program version and (for registered copies) the serial number.

To  exit  quickly without saving (and with no  extra  queries),  press

Shift+F10; going to the Top Menu is not necessary to do this.

The  size  of  SPH.ENV depends on the number of fixes  stored  in  the

database and on the number of entries in the history log.

SPH.ENV is automatically read in as Spheric is executed,  so that  you

can exit the program, do something else, and reenter it without losing

continuity of your work.

9. Program installation and use

Users of the PC-DOS mutation of Spheric will be interested only in the

first  three  sections of this chapter;  the  remaining  ones  contain

information specific to the HP95LX.

9.1. The packing list

Whatever  was the source from which you obtained Spheric,  you  should

have the following files:

SPH.EXE - the program itself

SPH.HLP - the text file used by on-line help

SPH.ERR - the text file with error messages

SPH.DOC - the manual (you are reading it now)

US.FIX - a fix file with coordinates of 30 major U.S. cities

WD.FIX - a fix file with 30 major world cities

WHATS.NEW - a brief list of changes since the last version

READ_ME.1ST - the introductory file

***  Registered  copies  of  Spheric contain a  number  of  additional

files:

SPHTUT.DOC - a text file with tutorials for some of the features

USA.FIX - a file with coordinates of more than 200 U.S. cities

WRD.FIX - same, for more than 200 cities of the world

9.2. Installing Spheric on your computer

This  is  a simple task.  Just copy the  files  SPH.EXE,  SPH.HLP  and

SPH.ERR to one directory, e.g., C:\SPH.

If  that  directory  is  included  in  the  PATH  definition  in  your

AUTOEXEC.BAT file,  you will be able to run Spheric while anywhere  in

the system, as explained in Section 9.4.

9.3. Character set compatibility

9.3.1. Screen

The only non-standard ASCII characters used by Spheric are the  degree

symbol (�),  corresponding to the value of 248 in the ASCII table, and

the graphic symbols used to draw the Main Screen.

These  symbols are available on the HP Palmtop as well as  on  desktop

machines, at least these using English or International character sets

(code page 437 or 850, respectively).

9.3.2. Printer

Not all printers may print the above symbols correctly; this may cause

minor  problems  in  a  few places  of  this  documentation;  a  minor

annoyance.

On  the  HP  LaserJet III (and perhaps some other  models)  all  these

problems  can  be  avoided  by  setting  the  character  set  to   850

Multilingual.

If  you run into printout problems,  you may request a neatly  printed

copy of this manual (but only at the time of program registration!).

9.4. Running Spheric from DOS

To run Spheric we must assure that

(i)  DOS will find the program file, SPH.EXE, and

(ii) Spheric will find its related files SPH.HLP, SPH.ERR, SPH.UNI and

SPH.ENV.

The first is done either by moving (with use of the CD command) to the

directory where Spheric resides before running it,  or, preferably, by

specifying  this directory in the PATH statement in your  AUTOEXEC.BAT

file.

The  second is done by keeping all files listed in (ii) above  in  the

same directory where SPH.EXE is.

9.5. Running from the Filer (HP)

As  distributed,  Spheric for the HP Palmtop does not work  under  the

System Manager.

You  may run it from DOS as explained above,  or from  the  Filer,  by

highlighting SPH.EXE and pressing F4.

9.6. Making a loader (HP)

The  Loader  Maker shareware utility by David Goodman can be  used  to

make an SPH.EXM file, allowing to run Spheric from the built-in HP95LX

applications  by  pressing  a "hot key"  (on  the  other  hand,  these

applications will not be accessible until you exit Spheric).

The process is described in the Loader Maker's instructions.

9.7. Saving disk space (HP)

The RAMdisk space on the HP Palmtop is quite limited;  every  kilobyte

counts, and the program itself is quite large (about 55k).

You  may save some space by shrinking SPH.EXE with the shareware  Diet

program  by Teddy Matsumoto.  For the price of a few second  delay  in

program loading, you will save almost 20k!

After you become familiar with Spheric, you can get rid of the SPH.HLP

file.  The on-line help will become disabled,  but you will gain about

9k of extra space on your disk.

Removing the SPH.ERR file is also possible,  although not recommended:

the  gain  is much smaller,  and error diagnostic will be  reduced  to

cryptic error numbers - not worth the trouble.

10. Registration and distribution

Spheric is a copyrighted program,  released as shareware. You are free

to use any evaluation (non-registered) copy of it on a trial basis for

a month or so. If you decide to use it afterwards, you are expected to

register it (as described below); otherwise please delete it from your

system.

10.1. How to register

To register,  send your name and address (including GEnie,  CompuServe

or Internet, if applicable) together with the appropriate registration

fee - a check or money order (U.S. funds, drawn on a U.S. bank) to

J.A.Wrotniak

2057 Happy Lane

Crofton, MD 21114-1917

Please state your current version number and computer type (HP palmtop

versus any other DOS machine).

The basic registration fee is $30,  and the complete price list can be

found in Section 10.3.

10.2. The extra goodies

"Yeah,  yeah,  I'm going to pay you some time later this year, but not

now; I've spent too much for Christmas!"

To increase chances that you keep your end of the deal,  those who pay

the  registration  fee  will  receive  a  package  of  extra  goodies,

including:

*   The newest registered (enhanced) version of Spheric on a 3.5"  DD

or HD floppy disk

*   A file with step-by-step tutorials

*   A printout of this documentation (if requested)

***  Registered copies of Spheric,  denoted with an "R" in the version

number  (e.g.,  1.20R/HP  or 1.20R/PC),  cannot  be  distributed:

resold,  leased, given away, placed on electronic bulletin boards

etc.

***  The owner may make backup copies only for his/her own use, safety

and convenience.

***  Every  registered  copy  of Spheric has a  unique  serial  number

embedded inside and displayed at the program exit.  Please  refer

to this number in all related correspondence.

10.3. Registration fees

Here is the complete price list:

-   A single copy of Spheric (PC or HP): $30

-   Both versions at the same time: $36

-   The other mutation ordered later: $12

-   A single update to a new version: $10

-   Both updates at the same time: $12

Institutions may order multi-user site licenses at prices obtained  by

multiplying the above rates by square root of the number of users. For

example,  a site license for both versions of Spheric and for 20 users

will be $40*sqrt(20) = $40*4.472 = $178.88.

10.4. Distribution

You are free to distribute Spheric (only evaluation copies, without an

"R" in the version number) to anyone,  as long as all original  files,

as in the Packing List above, are enclosed. This permission extends to

the electronic distribution.

Please do not distribute (give away,  sell,  lease, post on electronic

bulletin boards etc.) any registered versions of Spheric.

Public  Domain and shareware vendors are granted the right to  include

complete evaluation copies of Spheric into their libraries only if the

total  cost  to  the  user stays within the  limit  of  $6  per  disk;

otherwise my explicit permission is required.

(I  would,  however,  recommend  contacting me ahead of  time;  I  may

already have a newer version.)

10.5. Support

If you have any questions,  suggestions or problem reports,  I can  be

contacted on GEnie (J.WROTNIAK) or CompuServe (70611,2552).  My  email

address on the Internet is "wrotniak@umdhep.umd.edu".

If  you  do  not have an access to these  services,  write  under  the

address shown above. I read all letters and answer most of them.

10.2. Improvements and new features

Unavoidably,  the program reflects, to a large extent at least, my own

needs,  preferences and working habits.  In your own interest,  let me

know what you like and dislike about Spheric,  and how would you  like

it to evolve.

10.7. Credits

Spheric   has  been  developed  in  Turbo  Pascal  7.0  from   Borland

International, and contains library code copyrighted by Borland.

Some of the math algorithms used in Spheric have been borrowed from my

other  program,  Star Base,  running on the Atari ST series (the  real

computers!)  and distributed by Debonair Software.  I have  also  used

some  algorithms I developed for various Civil  Aviation  applications

(your tax money at work!).

10.6. About the Author

In  case  you  would  like to know who  wrote  this  program,  let  me

introduce myself.

I  am  a  physicist by  education,  with  M.Sc.  and  Ph.D.  from  the

University of Lodz, Poland, where I have spent 16 years doing research

in high-energy cosmic ray physics,  at the same time teaching physics,

numerical methods and computer science.

In the United States since 1984,  currently I am working as a software

systems  engineer  and  Chief Scientist  for  Aerospace  Research  and

Engineering Associates, Inc. in Maryland.

Outside of my regular job,  I still do physics. I am also active among

the  Atari ST computer enthusiasts,  programming quite a lot for  that

machine and writing a regular column for the Current Notes magazine.

In  the  PC-DOS  and HP Palmtop field I may be known to  some  as  the

author of Ex, a calculator, function plotter and expression processor.