______DSKCHK                                                            Ver. 3.00

Purpose: Batch file utility to detect existence and functioning of disk

drive; is there a drive, has it a formatted readable disk & (op-

tionally) can it be written to?

Format:     DSKCHK d[:] [/W]

_         d is any legal drive letter; it may not be omitted.

__         /W forces an attempt to write if the disk is readable.

Remarks: Result of test is reported in ERRORLEVEL:

0 = No error.  Drive exists and disk can be read.  If /W

switch was used, disk can also be written to.

1 = Disk cannot be read.

2 = Disk can be read but not written to.  This report can

appear only if /W switch is used.

255 = User has bungled the drive letter (by calling for

drive % or some such).

An empty command line or one with a question mark or a hopeless

mess in it gets a simple help screen.

If the command line contains a drive letter, DskChk tests the disk

for readability by calling DOS int 25 (absolute disk read) to read

sector 1 of the requested drive.

When /W switch is used, DskChk calls INT 21h, AH=3Ch.  If success-

ful, it creates a 0-byte file ISSUDERE.$$$ in the root directory

(which DskChk then deletes).  Such a failure produces ErrorLevel 2

and normally means means the disk is write-protected, but it could

also be produced by a full root directory.  A full disk wouldn't

produce the error.

If either of these efforts fails, a temporary Critical Error Hand-

ler speeds the return and prevent "Abort, Retry...."

Some words of caution: there are a lot of ways of hitching a disk

drive to a PC, and it's possible that some will not respond to

DskChk correctly.  On other drives, particularly floppies, DskChk

may take a very long time before it brings back a negative reply.

History: 1.00 (Jan, 1991) used DOS int 25h (absolute disk read) to check

for readability.  Gave false readings on SCSI drives, from which

int 25h always returns unsuccessful.

2.00 (Aug, 1992) readability test changed to operate correctly on

SCSI drives.  Uses int 4Eh (Find First) with a critical error

handler to head off "Abort, Retry..." 2.02 (Jan, 1994) corrects

bug in read check.  (Never encountered an actual error, but I

don't know why.)

______DSKCHK                                                               Page 2

3.00 (Feb, 1994) returns to DOS int 25h for better speed with old

floppy drives, but under DOS versions greater than 3.x, it now

uses the extended version of that interrupt.  It's not SCSI that

made a mess, it turns out, but the mere size of the drives I was

using.  DSKCHK2 is still in the packet, however, because it's

possible that it will do better on network and compressed drives

than version 3.

R. N. Wisan, Feb, 1994

37 Clinton St., Oneonta, NY 13820

Internet: wisanr@hartwick.edu