DialThat.com  v. 1.01

=====================

(C) Copyright 1990 Roger Schlafly.

All Rights Reserved.

How often do you look up a telephone number on your PC screen, and

then manually dial it on your telephone?  If so, you can automate

the process with DialThat.  DialThat is a resident program to read

phone numbers on the screen and dial them thru a modem.

Load DialThat by typing DIALTHAT at the DOS prompt.  When you wish to

dial a phone number on the screen, invoke DialThat by pressing Alt-D.

The cursor will fatten, and you can use the arrow keys to position

it on the screen.  When the cursor is at the beginning of a phone

number, press Enter.  You should hear the modem dialing.  When it

is finished dialing, pick up the telephone hand-set and press the

space key to shut down (hang up) the modem.  The DialThat then

relinquishes the phone line to your telephone and the PC to your

application.

Permission is hereby granted for free personal (non-commercial)

use of this program.  Others can register a license by sending

$10 to the author.  MasterCard and Visa are accepted.  Member ASP.

Technical Notes

---------------

You can repeat the last call with Alt-R.

These options are available when loading DialThat.

/C

Load with COMMAND.COM.  You can then unload DialThat by typing EXIT

at the DOS prompt.  This option could use as much as 8K of RAM,

depending on the DOS version.  Otherwise, DialThat only uses less than 2K.

/2

Load assuming modem is on COM2.  Likewise /3 is for COM3, etc.

The default port is COM1.

/S

Require the left shift key to be depressed while you press the hotkey.

This reduces conflicts with other programs.

/Pnumber

Defines a prefix to be dialed before the telephone number.  Enter still

just dials the number on the screen, but the Plus key dials the prefix

and the number.  For example, if your database shows 408-555-1212 but

you want to dial 1-408-555-1212, then put /P1 on the command line and

dial with Plus (+).

Assumes a Hayes-compatible modem is on a serial port.  Also, the

phone line must accept tone dialing.

You can also abort a call with the Escape key.  The Home, End, PgUp,

and PgDn keys move the cursor in reasonable ways.

For programs that use Alt-D, just press Alt-D twice and the second one

will be ignored by DialThat.  To pass Alt-R, press Alt-D followed by Alt-R.

If you insist on changing the hotkeys, you can use DEBUG or a similar

utility.  The Alt-D and Alt-R scan codes, 2000h and 1300h, are located

near the beginning of DialThat.com.

A program for showing scan codes, KEYCODE.COM, is included.

A phone number is a contiguous string drawn from "1234567890,-()*#".

Leading garbage is skipped.

Uses only interrupts 10h, 14h, 16h, and 21h.  Should be compatible with

most PC hardware and software.

History

-------

1.00: Initial version.

1.01: Added /S switch, and logic to pass second hotkey thru.  Inserted

hotkey codes near beginning for easier patching.  Added /P switch.

Author:

Roger Schlafly

Real Software

PO Box 1680

Soquel, CA  95073

USA

phone: 408-476-3550

CIS: 76646,323

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

Public (software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  PsL cannot debug pro-

programs over the telephone, though we can answer questions.

Disks in the PsL are updated  monthly,  so if you did not get

this disk directly from the PsL, you should be aware that the

files in this set may no longer be the current versions. Also,

if you got this disk from another vendor and are having prob-

lems,  be aware that  some files may have become corrupted or

lost by that vendor. Get a current, working disk from PsL.

For a copy of the latest monthly software library newsletter

and a list of the 2,000+ disks in the library, call or write

Public (software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

1-800-2424-PSL

MC/Visa/AmEx/Discover

Outside of U.S. or in Texas

or for general information,

Call 1-713-524-6394

PsL also has an outstanding

catalog for the Macintosh.