WATCHDOG timer handler, Version 1.0

Copyright 1988 by Ken Sykes.  All rights reserved.

This program may be used freely as long as the copyright notices are

preserved and it is not sold for profit.  The author makes no warranties,

expressed or implied, of the accuracy or fitness of this program.

DESCRIPTION

This program takes advantage of timer channel 3, the watchdog timer,

to avoid cold boots after a system crash.  The watchdog monitors channel

0 (system timer) and decrements its counter if a system clock tick occurs

while IRQ 0 is active.  In other words, if the system timer interrupt is

not available to service the tick, the watchdog counter is decremented.

If the watchdog counter reaches 0, an NMI is triggered.

WATCHDOG enables the watchdog timer and provides an NMI routine that

restores the machine stability to the point where the keyboard will be

serviced properly.  In particular, software interrupts are enabled and

the 8259A Mask register is set to enable keyboard interrupts.  Bus

arbitration, which was turned off for the NMI, is resumed again.

This timer is available ONLY on PS/2 Models 50 and above.

USING WATCHDOG

To install watchdog: watchdog

To uninstall watchdog: watchdog u  (must be lowercase 'u')

If the system hangs, press CTRL-ALT-DEL as always

KNOWN BUGS

- Only a portion of the used memory is reclaimed during an uninstall

- Once the NMI is triggered, the watchdog is disabled forever (?)

Originally the watchdog was re-enabled on exit from the NMI, but

system resets would not work properly.  The code as-is seems to

break out of VERY tight loops and out of multiple areas of disabled

interrupts.  I would welcome explanations of why this works (or doesn't

work) from people who have used this timer.

RESTRICTIONS

CodeView and Symdeb grab the NMI interrupt, thus knocking WATCHDOG out of

the picture.  Well almost.  If the system DOES hang, an NMI is still triggered

but it is the debugger's NMI routine that is activated.  Rebooting at this

point does not work.  This is due to the arbitration being turned off and the

watchdog timer still active.  Thus, it is a good idea to uninstall WATCHDOG

before using the debuggers.

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

The Public (Software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  The P(s)L cannot de-

bug programs over the telephone.

Disks in the P(s)L are updated monthly, so if you did not get

this disk  directly from the P(s)L,  you should be aware that

the files in this set may no  longer be the current versions.

For a copy of the latest monthly software library newsletter

and a list of the 1,400+ disks in the library, call or write

The Public (Software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

(713) 665-7017