DISK

All you wanted to know about your disks, but didn't know how to ask.

DISK provides information about your disk drives and disk controller.  DISK

gives you information that would take several other utilities including DEBUG to

duplicate.  DISK is for those times when you need to know something about the

physical characteristics of your drives as FDISK, DOS, and the BIOS understand

them.

Let's say you have carefully studied the choices and have decided which hard

disk to buy.  Now you need to know whether your disk controller will support

that drive.  DISK can help you determine what you have to do to install a new

drive and can give you the usage and formatting information on your current

drive.

You may discover that your controller is ready to support your new drive just by

changing the disk type through SETUP.  If your controller doesn't directly

support your new drive, you will at least have enough information to use the

DSKSET and DSKDRV utilities to configure your controller for the new drive.

SHAREWARE

DISK and its related products BOOT, DSKSET, DSKDRV, DXTSET, and DXTDRV are

shareware.  If you decide to use them after a reasonable trial period (1 month),

you are obligated to pay for them.  The fee for continued use is $10.  For this,

you may use the products indefinitely and I will send you notice of any updates

for one year.  For $25, I will send you the source code immediately and will

send you any updates for one year.  These fees apply to any use of the products

on a single computer including government agency and commercial use.  Volume

discounts and site licenses are available to reduce the cost for large users.

Send the fees and any inquiries to:

Ronald Q. Smith

11 Black Oak Road

North Oaks, Mn. 55127-6204

You may also contact me via CompuServ mail at userid 71620,514.  I will be

happy to respond to any problems and suggestions for future capabilities.

USING DISK

DISK displays information about both floppy and fixed disks and the fixed disk

controller.  You may request information about either a logical drive (d:) or a

physical drive (0, 1, 0F).

DISK d[:]

d may be any drive letter including floppy drives, fixed drives, and virtual

drives.  The allocation information associated with the drive is displayed as

shown in figure 1.

================================================================================

Figure 1.  Disk Allocation Information

DISK Version 2.00 - Copyright 1989, 1991 Ronald Q. Smith

Information for drive C: volume label VOLUME_1 created Jul 18, 1989 10:48

DISK  MEDIA   MAXIMUM          SECTORS/   HIDDEN  RESRVED  NUMBER  SEC/  ROOT

TYPE  DESCR  CYLINDERS  HEADS    TRACK   SECTORS  SECTORS    FATS  FAT    DIR

_____  _____  _________  _____  ________  _______  _______  ______  ____  ____

FIXED    F8         923      5        17       17        1        2   77   512

TOTAL         TOTAL      SECTORS/    BYTES/      AVAILABLE      AVAILABLE

BYTES        CLUSTERS    CLUSTER     SECTOR        BYTES        CLUSTERS

_____________  __________    ________    ______    _____________    _________

33,462,272      16,339           4       512        4,812,800        2,350

================================================================================

DISK n[F]

n is a drive number and may be 0 or 1.  If an F follows the number, information

about the floppy drive is displayed.  The BIOS parameter table is displayed for

both fixed and floppy drives.  The partition table (created by FDISK) is

displayed for fixed disks.  If any of the partitions are extended DOS partitions

(partition type 5), the logical drive table for that partition is also dis-

played.  See figure 2 for an example of the floppy disk display and figure 3 for

an example of a fixed disk display.

================================================================================

Figure 2.  Floppy Disk Parameter Table

DISK Version 2.00 - Copyright 1989, 1991 Ronald Q. Smith

Information for floppy disk 0

SECT      HEAD  HEAD   HEAD  STEP                             MOTOR MOTOR

SIZE MAX  LOAD SETTLE UNLOAD RATE NORMAL FORMAT FORMAT  DATA  START WAIT

BYTE SECT  MS    MS     MS    MS   GAP    GAP    FILL  LENGTH 1/8 S TICKS DMA

____ ____ ____ ______ ______ ____ ______ _____  _____   ____  _____ _____ ___

512    9    2     1    240     3   27     84     F6     255     8    37  YES

================================================================================

Figure 3.  Fixed Disk Drive Information

DISK Version 2.00 - Copyright 1989, 1991 Ronald Q. Smith

Information for fixed disk 0

LAND  REDUC  WRITE               TIMEOUT

TYPE   CYL   HD  SEC   ZONE  WRITE  PRECM  ECC  REC  STD  FMT  CHK  STP

____   ____  __  ___   ____  _____  _____  ___  ___  ___  ___  ___  ___

14     924   5   17   None      0      0  N/A  YES  N/A  N/A  N/A  N/A

PARTITION TABLE

PART. PART.        STARTING       ENDING      RELATIVE     TOTAL

NO.  TYPE  BOOT CYL.   HD SEC CYL.   HD SEC   SECTOR     SECTORS

1    X-DOS  NO   771    0   1  922    4  17    65,535      12,920

2    NONE   NO     0    0   0    0    0   0         0           0

3    NONE   NO     0    0   0    0    0   0         0           0

4    DOS16  YES    0    1   1  770    4  17        17      65,518

Press any key to see the logical device table for partition 1.

PARTITION 1 LOGICAL DEVICE TABLE

DEVICE PART.        STARTING       ENDING      RELATIVE     TOTAL

NO.  TYPE  BOOT CYL.   HD SEC CYL.   HD SEC   SECTOR     SECTORS

____  _____ ____ _____  __ ___ ____   __ ____  ______     ________

1    DOS12  NO   771    1   1  922    4  17       17      12,903

================================================================================

DISK TA[BLE]

The BIOS disk parameter table entries for all disk types supported by your

controller are displayed in edited form.  See figure 4 for a sample display and

the section "The Format of the DISK Parameter Table" for a discussion of the

contents.

================================================================================

Figure 4.  BIOS Fixed Disk Parameter Table

DISK Version 2.00 - Copyright 1989, 1991 Ronald Q. Smith

BIOS disk parameter table.

LAND  REDUC  WRITE               TIMEOUT

TYPE   CYL   HD  SEC   ZONE  WRITE  PRECM  ECC  REC  STD  FMT  CHK  STP

____   ____  __  ___   ____  _____  _____  ___  ___  ___  ___  ___  ___

1     610   4   17    686      0   None  N/A  YES  N/A  N/A  N/A  N/A

2     615   4   17    615      0    300  N/A  YES  N/A  N/A  N/A  N/A

3     615   6   17    615      0    300  N/A  YES  N/A  N/A  N/A  N/A

4     960   5   17   None      0    512  N/A  YES  N/A  N/A  N/A  N/A

5     920   9   17   None      0   None  N/A  YES  N/A  N/A  N/A  N/A

6   1,000   8   17   None      0    512  N/A  YES  N/A  N/A  N/A  N/A

7     900  15   17   None      0   None  N/A  YES  N/A  N/A  N/A  N/A

8     960   5   17   None      0   None  N/A  YES  N/A  N/A  N/A  N/A

9     600   4   17    656      0    128  N/A  YES  N/A  N/A  N/A  N/A

10     960   5   17    978      0    978  N/A  YES  N/A  N/A  N/A  N/A

11     614   4   17    665      0   None  N/A  YES  N/A  N/A  N/A  N/A

12   1,000   5   17   None      0    512  N/A  YES  N/A  N/A  N/A  N/A

13     600   4   17    665      0   None  N/A  YES  N/A  N/A  N/A  N/A

14     924   5   17   None      0      0  N/A  YES  N/A  N/A  N/A  N/A

================================================================================

DISK TAX

The BIOS disk parameter table entries for all disk types supported by your

controller are displayed as 16 hex values.  This information will help you

create your own entry using DSKSET and DSKDRV.  See figure 5 for a sample

display.

================================================================================

Figure 5.  Hex BIOS Fixed Disk Parameter Table

DISK Version 2.00 - Copyright 1989, 1991 Ronald Q. Smith

BIOS disk parameter table.

TYPE  HEX

1  62 02 04 00 00 FF FF 00 00 00 00 00 AE 02 11 00

2  67 02 04 00 00 2C 01 00 00 00 00 00 67 02 11 00

3  67 02 06 00 00 2C 01 00 00 00 00 00 67 02 11 00

4  C0 03 05 00 00 00 02 00 00 00 00 00 00 00 11 00

5  98 03 09 00 00 FF FF 00 08 00 00 00 00 00 11 00

6  E8 03 08 00 00 00 02 00 00 00 00 00 00 00 11 00

7  84 03 0F 00 00 FF FF 00 08 00 00 00 00 00 11 00

8  C0 03 05 00 00 FF FF 00 00 00 00 00 00 00 11 00

9  58 02 04 00 00 80 00 00 00 00 00 00 90 02 11 00

10  C0 03 05 00 00 D2 03 00 00 00 00 00 D2 03 11 00

11  66 02 04 00 00 FF FF 00 00 00 00 00 99 02 11 00

12  E8 03 05 00 00 00 02 00 00 00 00 00 00 00 11 00

13  58 02 04 00 00 FF FF 00 00 00 00 00 99 02 11 00

14  9C 03 05 00 00 00 00 00 00 00 00 00 00 00 11 00

================================================================================

FDISK and CHKDSK display some of the same information as DISK, but FDISK makes

you walk through a series of menus.  A display of the BIOS parameter tables is

not available with any of the common utilities.  DISK may only be an interesting

addition to your set of utilities if you are not changing disk drives or working

with multiple systems.  But, if you want to install a new drive or have to work

with PCs that you have not set up yourself, DISK can be invaluable.

THE FORMAT OF THE DISK PARAMETER TABLE

There are two disk parameter table formats, one for the PC/XT and one for the

PC/AT.  Both require essentially the same information.

PC/AT

The PC/AT disk controller BIOS and parameter tables are located in the system

BIOS.  The format of the PC/AT parameter table is:

BYTE         SIZE   DESCRIPTION

0           Word   Total number of cylinders

2           Byte   Number of read/write heads(surfaces)

3           Word   Unused

5           Word   Starting write precompensation cylinder

7           Byte   Unused

8           Byte   Control byte

Bit 7     No retry on disk errors

Bit 6     No retry on ECC errors

Bit 3     More than 8 heads

9           Byte   Not used

10           Byte   Not used

11           Byte   Not used

12           Word   Landing zone cylinder

14           Byte   Sectors/track

15           Byte   Reserved

Offset 0(word) - Contains the total number of cylinders.  The drive specifica-

tions will supply this number.  This number is 2 larger than the number that

will be returned to you when you make a request to retrieve drive parameters.

The innermost cylinder is reserved as a test cylinder and the returned drive

parameters give you the largest cylinder number (starting at 0) rather than the

number of cylinders.

Offset 2(byte) - Contains the total number of read/write heads.  This is also

the number of read/write surfaces.  Some disks have a clocking head/surface that

is not used for data and is not included in this count.  The number in this

field is 1 greater than the number returned by a request to retrieve drive

parameters.  Again, the returned drive parameters give the highest head number

(starting at 0) rather than the number of heads.

Offset 3(word) - Unused on a PC/AT.  Set to 0.

Offset 5(word) - Starting write precompensation cylinder.  Starting with this

cylinder, the controller applies modified logic for determining bit timing on

the disk surface.  Each cylinder on the disk contains the same number of bits

but the inner (higher numbered) cylinders are substantially smaller than the

outer cylinders thus packing the bits closer together.

Offset 7(byte) - Unused on a PC/AT.  Set to 0.

Offset 8(byte) - Control byte.  Bit 3 is set to a 1 if your disk has more than 8

read/write heads.  Bits 6 or 7 may be set if you have a special device driver

that does all its own device error recovery and does not want the controller to

attempt automatic recovery, but these bits are usually 0.

Offset 9(byte) - Unused on a PC/AT.  Set to 0.

Offset 10(byte) - Unused on a PC/AT.  Set to 0.

Offset 11(byte) - Unused on a PC/AT.  Set to 0.

Offset 12(word) - Landing zone.  Value to be used by park programs.  Often equal

to the total cylinders value at offset 0, but some drives permit seeking to

higher cylinder numbers than can be used to contain data.  The value may be zero

if the drive automatically retracts its heads at power off.

Offset 14(byte) - Sectors per track.  Usually 17.

Offset 15(byte) - Reserved.  Set to 0.

PC/XT

The PC/XT parameter table is located on the BIOS chip on the disk controller

card.  This allowed the addition of optional controllers to a PC that was not

originally designed to accommodate them.  Several of the fields are the same as

in the PC/AT parameter table.  Those fields will be identified but not explained

again.

BYTE         SIZE   DESCRIPTION

0           Word   Total number of cylinders

2           Byte   Number of read/write heads(surfaces)

3           Word   Reduced write current cylinder

5           Word   Starting write precompensation cylinder

7           Byte   Maximum ECC data burst length

8           Byte   Control byte

Bit 7     No retry on disk errors

Bit 6     No retry on ECC errors

Bit 3     More than 8 heads

Bits 0-2     Step rate

9           Byte   Timeout value for normal read/write operations

10           Byte   Timeout value for format operations

11           Byte   Timeout value for disk check operations

12           Word   Unused

14           Byte   Unused

15           Byte   Reserved

Offset 0(word) - Normally all of the cylinder numbers are stored in Intel 80xxx

order with the low-order byte first.  However, at least one XT controller stores

the values high-order byte first.  Only the word fields (offsets 0, 3, 5, and

12) are affected.  DISK attempts to compensate for this inversion in the edited

display.  The hex display will always show the bytes in the order the controller

wants them.  The hex display shows the bytes in the order they actually appear

in the table.

Offset 3(word) - Reduced write current cylinder.  Because bits are packed more

closely on inner cylinders, it is often necessary to reduce the current used to

write the bits so that nearby bits are not effected (the bits are actually

smaller on the inner cylinders).  The PC/AT controllers assume that this is the

same as the write precompensation cylinder and only use that value.

Offset 7(byte) - Maximum ECC data burst length.  The maximum number of bits that

the ECC can correct.  The first and last bits in error in any sector must be

within this length (inclusive) of each other or the ECC cannot correct the

error.

Offset 8(byte) - Control byte.  Bits 0-2 specify the head step rate on cylinder-

to-cylinder motion.

Offsets 9-11(byte) - Timeout values.  These values are used by the BIOS software

to timeout disk operations.

Offset 12(word)   - Unused on a PC/XT.

Offset 14(byte)   - Unused on a PC/XT.