User Documentation                 EZDIAG.EXE                        Page no.  1

USER DOCUMENTATION

WESTERN DIGITAL USER DIAGNOSTICS

This document is intended to accompany the utility EASYDIAG.EXE

placed on this bulletin board for your convenience.

The utility is a safe, non-destructive way for your customer

(or your self) to verify the configuration of your AT

compatible computer, and to specifically test the hard drive/

controller subsystem.

Upon loading the program, the low-order ram pointer bytes are

presented to the screen. The machine's power-on self tests

test installed peripherals, and if valid ones exsist, the

tables are updated.

NOTE : THIS UTILITY DOES NOT PERFORM THE SETUP OR FORMAT

FUNCTIONS. THESE FUNCTIONS MUST BE PERFORMED BY

THE MACHINE'S MANUFACTURER-SUPPLIED SETUP AND

FORMAT UTILITIES. IN THE EVENT A FORMATTER IS

REQUIRED, WE SUGGEST THE OBTAINING OF THE WD

UTILITY WDFMT.

A sample of the start-up screen follows:

User Documentation                 EZDIAG.EXE                        Page no.  2

System contains the following :

640K bytes of RAM

2 Floppy disk drive(s)

1 Parallel printer port(s)

2 Asynchronous serial port(s)

1 Hard disk drive(s) of the following configuration(s) ...

Drive    Cylinders    Heads    Precomp cyl    Landing zone    Sectors/track

0          820        10         65535           820              17

Total formatted capacity of drive  0  is  71,372,800 bytes

If the above information is not correct, run your system's setup utility

The following diagnostic will use the configuration information above

Do you want to do a hard drive / controller diagnostic ( y / n) ??

Note that the total count for all devices, as well as the

physical capacity of the hard drive is indicated. The useable

capacity is somewhat less, due to the partitioning method

that DOS uses.

You can proceed to the controller/drive diagnostic by typing

'y' now. No return is necessary. The following menu appears:

User Documentation                 EZDIAG.EXE                        Page no.  3

Select the diagnostic mode desired

(A)utomatic Quick Test ..........

(U)ser Selected Advanced Test ...

(E)xit to DOS / END TEST ........

...  ENTER YOUR OPTION  ...  ==>

Explanation of options on following page :

User Documentation                 EZDIAG.EXE                        Page no.  4

(A)utomatic Quick Test ... Operation

A really fast and thorough test of the controller/drive subsystem.

The following repetitive commands are issued :

1) Rezero (recalibrate) command is issued to the

controller. The purpose of this command is to

simply move the hard drives' data heads over

the very first cylinder. The drive must then

respond with  DRIVE READY, SEEK COMPLETE, and

TRACK ZERO signals all present. The following

errors can occur :

REZERO (RECALIBRATE) COMMAND ERROR LIST

NO SEEK COMPLETE FROM DRIVE ...... --- OR ---

DRIVE NOT READY ERROR  ........... The drive never

reported that it was ready, or that the requested

seek (to track zero) was complete. This is usually

a drive select jumper or termination option (see the

drive's installation manual for more details) if the

drive activity lamp never lit when the controller was

trying to access the drive. Can also be a wrong, mis-

installed, or a defective cable. If the drive never

came to full speed (due to mechanical or power supply

limitations) this is also the error you will see.

TRACK ZERO ERROR  ................ The drive reported

that it was READY, and failed for some reason to reach

the first cylinder. This is usually either a termination

option problem, or a defective hard drive.

CONTROLLER STAYED BUSY  .......... This error can be

caused by failure of the controller to receive INDEX

(revolution) pulses from the hard drive. This input

is necessary for the timeout mechanism to function

properly on the controller. Try a known, good drive

and cable set. If the error persists, the controller

is probably bad.

User Documentation                 EZDIAG.EXE                        Page no.  5

2) 256 RANDOM SEEKS are issued all over the drive.

The drive does not have to be formatted to pass

this test. This is a thorough test of the

mechanics of the hard drive. A failure here will

probably be reported as a NO SEEK COMPLETE or

a NO DRIVE READY error. If the power supply in

the system under test is not capable of supplying

sufficient current to operate the drive, it will

usually show up as a drive error. If the wrong

number of cylinders or are selected in the

SETUP program (supplied with the machine), an

error listed above or a WRITE FAULT error will

be displayed. The errors possible follow :

a) NO SEEK COMPLETE  .......... Covered above

b) DRIVE NOT READY  ........... Covered above

c) WRITE FAULT ................ Beyond maximum

cylinder number seek was attempted. Check your

drives' parameters and your machine's SETUP.

Could also be a defective drive.

User Documentation                 EZDIAG.EXE                        Page no.  6

3) Now to the meat of it. 256 RANDOM TWO SECTOR

READ commands are issued to the drive. The

randomness of the location selected combined

with the deliberate selection of 2 sectors

(blocks) of data requested was carefully chosen

to bring out the elusive "random" failures.

The controller is told to NOT RETRY any operation

that resulted in an error. The possible errors

that could be reported are :

RANDOM READ POSSIBLE ERROR DESCRIPTION

a) DRIVE NOT READY ............. Covered above

b) WRITE FAULT ................. Covered above

c) DATA REQUEST FAILURE ........ A severe and

total controller failure. The card either

never transferred data, or won't quit

transferring data.

d) BAD BLOCK ON DRIVE .......... This area was

previously marked on drive. This location

should appear on the drive's error log. This

error is displayed solely for confirmation.

No automatic or combination tests are aborted

if this error is encountered.

e) ID FIELD (address) NOT FOUND . The controller

was unable to find any understandable data

on the hard drive. Check to ensure the drive

has been FORMATTED, the SETUP is correct, and

the ERROR MAP on the drive does not have this

location listed as a BAD TRACK.

f) UNCORRECTABLE or CORRECTABLE DATA ERROR

Indicates this location should be marked as

a BAD TRACK with a LOW-LEVEL FORMAT utility.

Such a utility can be found on this BBS. It is

called WDFMT.EXE and will format, verify, and

perform an intensive surface scan of the drive.

Unless you format out marginal areas, the data

you store on the drive could eventually be

corrupted THE NEXT TIME IT IS READ. Play it

safe. THERE ARE VERY FEW DEFECT FREE DRIVES !!!

User Documentation                 EZDIAG.EXE                        Page no.  7

4) NOW WE REALLY GET TOUGH ! A random data pattern

WRITTEN / READ / and VERIFIED on the LAST PHYSICAL

SECTOR of the drive as defined in SETUP. This

location was chosen because operating systems

(and hence application programs) RARELY access the

last sector on the drive. Even if this sector was

able to be addressed in the machine's operating

system, you would have to be using EVERY BYTE on

the drive (within 512 bytes of full). This is rare.

Some errors that can exsist :

RANDOM DATA WRITE/READ/COMPARE ERROR DESCRIPTION

a) WRITE FAULT FROM DRIVE ...... Covered before

b) DATA REQUEST FAILURE ........ Covered before

c) DATA COMPARE ERROR .......... A difference

between the WRITE data and the READ data

was discovered. You need to replace this

controller.

d) ID FIELD (address) NOT FOUND . Covered above

e) UNCORRECTABLE/CORRECTABLE DATA ERROR

Covered above.

f) DRIVE NOT READY ..............   --- OR ---

NO SEEK COMPLETE ............. Covered above

g) BAD BLOCK on drive ........... Covered above

Sample screen follows ...

User Documentation                 EZDIAG.EXE                        Page no.  8

(A)utomatic quick test in progress ... please stand by ...

Now issuing the rezero (recal) command ......... command successful !

Now issuing 256 random seeks ................... command successful !

Now issuing 256 two sector random reads ........ command successful !

Now issuing 50 random data write/read verifies . data compared O.K. !

Automatic quick test completed flawlessly !

CONGRATULATIONS on a successful installation !

(U)ser Defined Advanced Test ... Next Section ...

User Documentation                 EZDIAG.EXE                        Page no.  9

(U)ser Defined Advanced Test ... Operation

This disk diagnostic is useful for a problem installation,

or to verify absolute disk addresses in the event of a disk

error later.

The user can directly enter any cylinder, sector, or head

he desires to test. The drive number can also be selected

if TWO drives are present. The amount of sectors to test

(sector count) is valid only for a (R)ead operation. If

no value is entered for any parameter, the program

automatically chooses:

drive        =   0

cylinder     =   0

head         =   0

sector no.   =   1

sector count =   1

command      =   re(Z)ero

The currently supported commands are :

re(Z)ero ....... recalibrates drive to cylinder 0

(S)eek   ....... positions drive to any cylinder

(R)ead   ....... transfers data from drive starting

at location chosen, and continues

for duration of sector count

(W)rite / read . transfers a random pattern of

data to the drive, then reads

it back, and checks for errors.

THIS IS DONE ONLY AT THE LAST

SECTOR OF THE DRIVE. YOU CANNOT

CHANGE THE LOCATION.

(A)ll .......... each of the preceeding commands

done in order. Any error will

terminate the sequence, and you

have the option of trying a

single command.

User Documentation                 EZDIAG.EXE                       Page no.  10

As you may notice, the sector number, sector count, and

head number have no bearing on re(Z)ero and (S)eek

commands.

IMPORTANT NOTE : Illegal disk addresses have been screened

for, and the following is how the program

handles them.

ILLEGAL SECTOR NUMBER : results when sector 0 or a sector number

greater than the actual sector per track

count is selected. Sector 0 is fixed

to 1, and the sector per track value

from the system BIOS is the maximum.

ILLEGAL HEAD NUMBER   : results when a head number greater

than ONE LESS THAN THE BIOS VALUE

is selected. Head numbers are truncated

at one less than the BIOS value.

ILLEGAL CYLINDER NO.  : results when a cylinder number less

than zero or greater than ONE LESS

than the BIOS table. Handled exactly

like the heads, above. Can also

result when a read operation will

span many cylinders. The actual

cylinder entered is tested against

the selected value PLUS the sector

count divided by the number of sectors

in a cylinder PLUS 1. If the result

would be greater, than the max cylinder

count of the drive, the difference is

truncated.

ILLEGAL SECTOR COUNT .. results when less than zero or

greater than 256 is entered. The

program makes >256 equal the value

entered AND 255, and makes 0 equal 1.

A sample session follows:

User Documentation                 EZDIAG.EXE                       Page no.  11

(U)ser Defined Advanced Test ... Sample Screen

Drive 0 selected automatically

Select a cylinder between           0  and     819   ? 819

Select a head (surface) between     0  and       9   ? 9

Select a starting sector between    1  and      17   ? 1

Select a number of sectors between  1  and     256   ? 17

The following commands are available :

re(Z)ero,  (S)eek,  (R)ead,  (W)rite/read verify,  (A)ll

ENTER YOUR OPTION      :      ? a

Now issuing the rezero (recal) command ......... command successful !

Now issuing the seek command ................... command successful !

Now issuing the read command ................... command successful !

For your protection, this part of our show will only do the LAST SECTOR

That will be ... cylinder   819  , head   9  , sector   17

Now issuing the write command .................. command successful !

Now issuing the read command to compare data ... command successful !

Now comparing data ............................. data compared O.K. !

Want another test  ( y / n )          ?

IMPORTANT NOTE : In the event of an error, the status and

error registers are presented to the screen.

An English interpretation of the HIGHEST

PRIORITY ERROR will follow. If there is a

real problem, such as an XT controller

installed in the system, or a timeout error

that "hangs" the controller, the status

and error register values may be the same.

The sector count, head number, and cylinder

number are also displayed.

ERROR CODE SUMMARY FOR ALL TESTS SAME AS IN PRECEEDING SECTION

IF PROBLEMS PERSIST ... 24 hour Technical Support number 1-800-777-4787

==============

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

Public (software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  PsL cannot debug pro-

programs over the telephone, though we can answer questions.

Disks in the PsL are updated  monthly,  so if you did not get

this disk directly from the PsL, you should be aware that the

files in this set may no longer be the current versions. Also,

if you got this disk from another vendor and are having prob-

lems,  be aware that  some files may have become corrupted or

lost by that vendor. Get a current, working disk from PsL.

For a copy of the latest monthly software library newsletter

and a list of the 2,000+ disks in the library, call or write

Public (software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

1-800-2424-PSL

MC/Visa/AmEx

Outside of U.S. or in Texas

or for general information,

Call 1-713-524-6394

PsL also has an outstanding

catalog for the Macintosh.