Extended Drive Description Tables for the IBM AT

When the IBM/PC was introduced,  there  was  no  fixed  disk

option  available  from  IBM.  Several  third  party vendors

produced add-on fixed drives, but there were no standards as

far as addressing,  interrupts used,  etc.  This led to much

confusion.  The  situation  got a little better when the IBM

PC/XT  was  introduced.   A  set  of   standards   on   BIOS

interfacing, interrupts used, addressing, etc.  was set. The

fixed disk description was stored in  the  fixed  disk  BIOS

contained on the fixed disk controller card.  There was not,

however a standard set of disk description  tables  defined.

Each  controller  BIOS  had  it's  own  set of tables.  Some

vendors OEM'd controllers and burned their own  eproms  with

undocumented  table  entries.  This has caused many problems

when adding or replacing a fixed drive.

Next came the IBM AT.  The AT BIOS contained support  for  a

fixed  disk  in  the  standard  BIOS.  A  setup  program was

supplied that  stored  the  hard  drive  type  in  the  CMOS

configuration  ram.  When  the machine was booted,  the BIOS

would read the drive type from cmos and use this as an index

into a table of drive descriptions.  This made  life  easier

when  adding a hard drive...or did it?  IBM supplied only 14

drive types in the original BIOS. When the newer AT 339 came

out the number of drive types was expanded to 47,  but  only

22  entries  were  usable,  with  the  other  entries marked

reserved and filled in with zeros.  Again your options  were

limited.

With  the  falling prices of fixed drives,  their popularity

increased,  and everyone wanted to add one or more to  their

system.   Many   users   found  the  32mb  size  restriction

unacceptable and wanted to attach very large drives to their

systems,  but no drive type was supplied in the BIOS and DOS

did not support them.

The  most  popular  solution to the problem of limited drive

types is through software. Device drivers are loaded at boot

time and redefine the fixed drives.  This solution has  been

very effective, but has some problems. You must boot off the

fixed drive or have the driver on any floppy use for booting

to  properly access the fixed drive.  Some operating systems

other than DOS may not be supported by the device driver.

Another option is by placing code in to eproms and using the

two extra sockets on the system  board  to  contain  a  BIOS

extension  that defines the fixed drives during the power on

self test (POST) sequence. This should remove the dependence

on software drivers for defining drive tables.

I chose the BIOS extension option as the preferred  solution

in  my case and began looking at some commercially available

products.  All were found to  be  unacceptable  for  various

reasons.  One vendor required I purchase two sets minimum. I

thought that was ridiculous,  I only had one  machine!  Some

were  priced much too high for a single set and others added

only a few  drive  type  entries  beyond  what  was  already

defined.  Being  a  hacker  at heart,  I decided if you want

something done,  you might as well do it  yourself.  I  then

began   work  in  ernest  on  what  I  call  Extended  Drive

Description Tables or EDDT.

I  purchased  the  IBM  AT  Technical  Reference  and  began

studying the BIOS source code listings. It soon became clear

that  it was not difficult to do what I wanted.  During POST

the BIOS searches absolute addresses E0000 through EFFFF  in

sixty  four kilobyte blocks for a valid system board rom.  A

valid rom is define as follows:

BYTE 0-1 55 AA

BYTE 2   NOT DEFINED

BYTE 3   ENTRY POINT FOR A FAR CALL

If the 55AA is located, a checksum is calculated, and, if it

is correct, a call is done to the entry point.  At this time

the  add  on rom is in control and may initialize itself.  A

closer look at the BIOS listings shows that the add  on  rom

code is called right after the fixed drives are initialized.

This  was  both  good  news  and bad news.  It was good news

because all the BIOS functions of the fixed  disk  interrupt

(interrupt 13H) would be available, but the bad news was the

BIOS  had already attempted to initialize the hard drive for

which it may have an inaccurate description, and get errors.

My next problem was where do I  get  a  setup  program  that

describes any new drive types I put in my roms?  You guessed

it - do it  yourself  time  again.  I  discovered  that  the

original  AT's  used CMOS byte 12H to contain the drive type

for both drive zero and drive one, using four bits for each.

When the number of drive types was  expanded,  if  a  F  was

stored  for  a  drive  type,  it  indicated that this was an

extended drive type.  Extended drive types  were  stored  in

CMOS  bytes  19H and 1AH.  this allowed for a maximum of 255

different drive types to be defined.

Closer  examination  of  the  BIOS listings revealed that if

CMOS byte 12H was non-zero,  the POST routine  would  invoke

the fixed disk initialization routines. These routines would

initialize  the  interrupt  vectors,  making the services of

interrupt 13H available,  and if the drive  type  for  drive

zero  was zero,  no initialization of the fixed drives would

be attempted. My setup program would the store a 01H in byte

12H and the EDDT drive types in bytes 19H and 1AH.

#HANGE FOR EDDTV400

After  testing  on  AWARD BIOS Version 2.09 and PHOENIX BIOS

version 3.03 showed  that  they  failed  to  initialize  the

Interrupt 13H vectors for hard disk support if Fixed Drive 0

was a 0 in CMOS,  a change was made in the way the types are

setup.  A dummy drive type is needed for these  BIOS's.  The

dummy  type is chosen by ESETUP40 according to the following

rules:

IN ORDER TO INITIALIZE INTERRUPT 13H ON  SOME

AT  COMPATIBLES FIXED DRIVE 0 MUST BE DEFINED

IN CMOS.  SINCE THE ACTUAL DRIVE MAY NOT HAVE

A  CORRESPONDING  IBM DRIVE TYPE WE MUST PICK

ONE THAT MEETS THE FOLLOWING CRITERIA:

(1) THE TYPE DEFINED MUST HAVE LESS  THAT  OR

EQUAL  TO  THE PHYSICAL DRIVE'S NUMBER OF

HEADS _AND_ NUMBER OF CYLINDERS.

(2) IF THE PHYSICAL DRIVE  HAS  MORE  THAN  8

HEADS  THE  TYPE USED MUST HAVE A CONTROL

BYTE VALUE OF 08H  OR  IT  MUST  NOT  USE

WRITE PRECOMPENSATION.

For a drive of 8 or less heads the dummy type is  1,  for  a

drive of more than 8 heads the dummy type is 6. EDDTV400 has

been  tested  successfully  with  the above mentioned BIOS's

after this change was implemented.

When the EDDT add on rom code got control,  it could look up

the drive types in it's internal table,  issue the set drive

characteristics interrupt 13H  function  and  then  run  the

diagnostics  on  the fixed drives.  As long as I was running

diagnostics I decided to output some information about  what

was  being  tested and the status codes returned.  I dislike

the standard IBM error code that doesn't tell  you  anything

about  what  failed  so  displayed is the BIOS interrupt 13H

function code,  a description of the service  requested  and

the  return code.  If the return code is non-zero the system

pauses until you press a key to continue so you may  make  a

note of the error information provided.  the diagnostics are

similar to, but more extensive than those in the IBM POST.

So far I have made over 140 drive type entries, and provided

a macro to allow for easy expansion of the table.  To add  a

drive type just code an additional EDDT macro and reassemble

the EDDTV400 rom module and the ESETUP40.EXE setup program.

To install EDDTV400 perform the following steps:

(1) Assemble EDDTV400 and ESETUP40

(2) Link EDDTV400 and ESETUP40

(3) Use EXE2BIN to produce EDDTV400.BIN

(4) Next use the software provided with most Eprom

burners to checksum EDDTV400.BIN and split it in

to odd and even bytes.

(5) Program the odd and even bytes into 2 27256

Eproms.

(6) Insert the Eproms into the empty sockets on the

system board

I   am   including   here   two   files,   EDDTV400.EVN  and

EDDTV400.ODD.  These  contain  the code required for the odd

and even files for the system as  coded  with  the  included

tables.  You can just burn the EPROMS and install them. Pick

your drive type out of the table and run ESETUP40.

Here are some common questions and answers.

EDDTV400 - Extended Drive Description Tables

WHAT IS EDDTV400 ?

EDDTV400  is  a  system  to allow many more hard drive types

than contained in the IBM AT rom set.  Currently  about  120

different  drive  types are defined.  This covers every hard

drive for which I have the data.

HOW  IS  EDDTV400  DIFFERENT   FROM   SPEEDSTOR,   VFEATURE,

DISKMANAGER, ETC. ?

The  packages  mentioned  are SOFTWARE device drivers.  They

must be loaded into RAM when you boot your system. They must

reside on the fixed disk or diskette you boot from. The main

portion of EDDTV400 resides in ROM.  EDDTV400 is active even

when you boot from a floppy.  There is  no  software  device

driver.

WILL EDDTV400 PARTITION MY LARGE HARD DRIVE ?

No,  but  with EDDTV400 and DOS 3.30's built-in partitioning

feature,  you can partition large drives with no  additional

software.

HOW DO I INSTALL EDDTV400 ?

First you assemble the source code for EDDTV400, then, using

an EPROM burner and associated software,  burn the code into

two 27256 (200ns reccommended).  These go into the two empty

sockets on the IBM AT motherboard.  Next boot from a  floppy

and  run  EDSETUP3 and define your hard drive types from the

EDDTV400 list of drive types.  Now re-boot and  run  FORMAT,

FDISK, etc. just like any hard drive you are installing.

DOES EDDTV400 REPLACE THE IBM BIOS ROUTINES ?

No,  EDDTV400  just initializes the hard drive table pointer

interrupt vectors to point to his tables.  He  does  do  the

diagnostics  like  the IBM BIOS,  but he tells you what he's

doing,  and displays the actual  status  codes  received  to

assist in diagnosing a drive/controller problem.

IS EDDTV400 COMPATIBLE WITH CLONES ?

If  they are really compatible,  it should be.  EDDTV400 was

tested on an IBM AT 339 with the 11/15/85 BIOS installed. An

examination of the source listings of  the  other  IBM  BIOS

releases  doesn't reveal any incompatibilities,  but I could

only test with what I had.  Additional testing  for  Version

4.00  indicates  compatibility  with  AWARD 2.09 and PHOENIX

3.03.

WHAT DID YOU HAVE ?

I have an IBM AT 339, Miniscribe 6053, and CMI CM-6426.  The

Miniscribe is a 40MB drive and the CMI is a 20MB.

WILL EDDTV400 HURT MY DRIVE ?

I don't think so,  but,  like all  free  software  and  most

commercial products, there is no warranty.

IS EDDTV400 PUBLIC DOMAIN ?

No,  EDDTV400  is  NOT  public  domain.  I  have  included a

COPYRIGHT notice to protect my work.  You may  use  EDDTV400

but DO NOT SELL IT. I doubt if there is any money to be made

with this,  but if there is, I want it.  I put many hours in

writing the code and don't want to lose too much control.

WHAT DO I NEED TO ASSEMBLE EDDTV400?

I used the Microsoft Macro Assembler  Version  5.00.  A  few

version specific directives were used.

WHERE DO I GET HELP ??

If  you call me (601 992 0577) at a decent hour (7pm to 11pm

CST) I'll try and answer questions. NO WARRANTY is provided.

I would like to know of problems however, and if I have time

I'll try and fix any problems IF I HAVE TIME!

WILL YOU BURN THE EPROMS FOR ME?

If you make it worth my while.  I am not a company. I do not

keep  vast  quantities  of  eproms  on  hand.  I'd  probably

consider  it  for  $75.00  or so.  I would have to order the

eproms and the burn them.  I would do this but  NO  WARRANTY

would be provided. I'm a hobbyist (the good kind of hacker).

I'm making this available AT NO COST.  If it doesn't work in

your system I can't be held liable.

Happy Hacking

Don  Gentry

(601  992  0577)

183 Bellegrove Circle

Brandon, MS 39042

#