486DLC/DRx2 Microprocessor Installation (V 1.1)

Written By: Mark Thompson

(407) 453-6488

Well first of all you will need a 386DX computer.  I strongly recommend

that if you have a 33MHz or less machine, you use the Cyrix 486DRx2

microprocessor.  They require no hardware modifications and is a direct

replacement for the 386DX chip.  It contains a clock doubler curcuit

which turns a 33MHz machine into a 486SX running at 66MHz internally!

Only use the DLC if you have no choice like myself having a 40MHz

machine.  My program is very useful for people running UNIX, WINDOWS NT,

or even MS-DOS.  It sets up the DLC/DRx2 CPU before any operating system

loads. Due to the fact that some time was spent on this program, I am

asking for donations for my research and development cost.

I have a Compuserve Address 70053,3702 but I won't guarentee that I

call very often.

Now to continue on.

You do not have to make any hardware modifications but you should.  You

will get alot more speed from the very simple modifications at only under

$5.00.  You may/will need the following:

One Multimeter (Resistance Readings)

One 7400 Quad 2-Input Positive Nand Gate

Three Feet of #30 wire-wrap wire

One Soldering Iron

Some Solder

Two 20K Ohm 1/4 watt Resistors

-----------------------------------------

To affix the 7400, I used a hot glue gun,

you may use the same or epoxy, or even

thin double-sided tape.  Using epoxy means

you won't be able to remove the chip once

it hardens.

Optional CPU Fan. (~$12.00)

Note:

------------------

The following list the instructions to follow.  Please take your time,

a good slow job will pay off within an hour or so.  The instructions

assume you are somewhat familiar with test equipment and electro-static-

electricity, and some electronics.

Step 1. Remove the computer case, remove all the boards on

the motherboard, then remove the motherboard it'self.

Step 2. Using a chip puller (if you have the right one)

or a thin-wide flat bladed screwdriver, gently pry out the

original CPU chip. Take Your Time!!!!  Be Gentle!!!!

If this step takes you under 2 minutes, your probably

going too fast.

Step 3. Refer to Figure 1, Locate all the pins labled below

and verify the resistance from Ground to the pin is nil (infinate),

and from Positive 5VDC to the pin is the same.  These pins can

be tied high through a resistor (usually 4.7K to 20K Ohms). Use

and fill out the sheet below at the same time.  There are two pins

which must be tied high, ADS# and LOCK. If these lines are not

tied high you must install a resistor across that pin to VCC (+5VDC).

NOTE: The pin labeled NC must NOT BE CONNECTED!

And HLDA Must not be altered!

Step 4. If the NC pin was connected, you must either cut the NC

pin on the 486DLC chip (Be careful!) or somehow disconnect that

pin from the motherboard.  I would recommend the first if you

must do either.

Step 5. You may install the curcuit anywhere you want but the

best place is as close to the CPU as possible.  I took the

IC chip and cut the leads down to where they were barely exposed,

just enough to solder to, and then mounted it upside-down on the

bottom of the motherboard, directly under the CPU socket.  Remember

that if you mount it upside-down, keep the pin numbers straight!

Step 6. Now you must wire the NAND gate.

First - solder from pin 6 of the NAND gate to the FLUSH# pin E13.

Second - solder from pin 5 of the NAND gate to the HLDA pin N14.

Third - solder from pin 3 to pin 4 on the NAND gate.

Fourth - solder from  pin 1 to pin 2 on the NAND gate.

Fifth - solder from pin 1 of the NAND gate to the -MEMW pin B11

(on the bottom of the motherboard)

Sixth - solder from pin 7 (GND) of the NAND gate to one of the

VSS points (GND)

Seventh - solder from pin 14 (+5 VDC) of the NAND gate to one

of the VCC points (+5 VDC)

Optional - If you positivly know which pin is the A20 Mask or

the Keyboard Controller A20 Gate line, you may wire

in the A20M# line.  If you are not possitive, don't

mess with it.  (You must enable the NC0 bit if you

don't connect it!)  This will not improve the cache

very much.

Step 7. Now you should re-verify all your connections.  If your

sure that they are correct, using the hot glue gun (or whatever),

secure the wires so they don't move (don't use tape).

Step 8. You are ready to install your new 486DLC CPU.  Align the

CPU with pin one oriented correctly.  Pin one has a mark in the

corner or is not square on the corner.  If you are unsure, don't

guess, call Cyrix Tech Support (800) GO CYRIX.

Step 9. You now have an option, although to me it was necessary

for me, install a small fan and/or heat sink if you don't have

good ventilation.  You can buy a fan from Radio Shack.  I put

some small flat-head screws through the mounting holes, creating

a stand-off and placed some hot glue on the heads to act as rubber

feet, which will eliminate any excess noise and abration against

the chip surface.  I then wired it directly to the power input

connector (Pin 3 = +12 VDC, Pin 5 = GND).  The fan is held in

place with a strong rubberband.

Step 10. Install your motherboard and cards, close up your machine.

Step 11. Turn on the power.  The Cyrix CPU will disable all cache

processing upon the reset signal.  You now perform the software

installation if your BIOS don't support the Cyrix Internal Cache.

Step 12. Copy this diskette and make it bootable by typing the

following:

To Copy the Diskette:   DISKCOPY A: A:

To Make it Bootable:    SYS A:

(you must be on the C: drive)

Step 13. Now you need to run my program which will set up the

CPU.  If at any time you change your CPU configuration, simply

run my program to update the boot code.  Run CX.EXE

This program will write the original boot code onto the floppy

should it not work properly.  If the system always hangs and

you can't get it to boot past the DLC Processor Setup message,

then you will have to run the RECOVER.EXE program to restore

the boot code to it's original state.  (This program will work

on IDE, MFM, and RLL drives, it has not been tested with SCSI

drives although it should work just fine)

Step 14. Ensure the new diskette is in the A: drive and reboot.

Step 15. First a DMA Test will run.  If this test fails then the

Flush curcuit is not working properly.  Recheck your curcuit!

Step 16. If the DMA passed, your good to go.  Enjoy your new CPU!

The only error message I ever encountered was when I enabled the

A20M# line and left the FAST A20 option in my BIOS setup On.  Turn

it off and try again.  Oh, the error was the HIMEM.SYS could not

gain controll of the A20 line.

For setups without hardware changes:

You must still complete steps all steps except 5, 6, and 7.

The CPU pins must be correct.  You may still need to install

a resistor to properly tie a line high.  If your motherboard

checks out and you follow the steps as written, you should be

up and running within an hour, but plaese take your time.

------------------------------------------------------------------------------

PIN SIDE VIEW

VCC = +5VDC

VSS = GND

A    B    C    D    E    F    G    H    J    K    L    M    N    P

1   X    X    X    X    X    X    X    X    X    X    X    X    X    X

2   X    X    X    X    X    X    X    X    X    X    X    X    X    X

3   X    X    X    X    X    X    X    X    X    X    X    X    X    X

4  SUSP SUSPA X                                            X    X    X

5   X    X    X                                            X    X    X

6   X    NC  RPLSET                                        X    X    X

7   X    X   RPLVAL                                        X    X    X

8   X    X    X              CYRIX 486DLC                  X    X    X

PIN SIDE VIEW

9   X    X    X                                            X    X    X

10   X    X   LOCK                                          X    X    X

11   X   VSS  VSS                                           X    X    X

12   X   KEN  VCC  VCC   X    X    X    X    X    X    X    X    X    X

13   X    X    X    X  FLUSH A20M  X    X    X    X    X    X    X    X

14   X    X    X   ADS   X    X    X    X    X    X    X   HLDA  X    X

(Do not test

HLDA pin)

------------------------------------------------------------------------------

(The A20M# line is not very important, if you can find it

readily, great, if not, it won't matter that much)

A20M# (from chipset or

A20GATE# from Keyboard Controller)             Cyrix DLC Processor

|

\----------------------------------------- Pin F13 (A20M#)

-------------------------------------------------------------------------------

(This curcuit is the most important thing)

-MEMW (from ISA Bus) (See Figure 3)

ISA Pin ___      ____

B11    \__|-|1   \*___           The NAND Gates are the symbols

|-|2___/ 3  \          shown below with pin numbers.

|

|              ____        Cyrix

\-------------|4   \*_____ Pin E13 (FLUSH#)

+5VDC--- Pin 14           /-------|5___/6

GND---- Pin 7           |                     Cyrix

\___________________ Pin N14 (HLDA)

-------------------------------------------------------------------------------

(ISA Bus Pinout)

A    B

______

1  | .  . |

2  | .  . |

3  | .  . |

4  | .  . |

5  | .  . |

6  | .  . |

7  | .  . |

8  | .  . |

9  | .  . |

10  | .  . |

11  | .  x |      (Shown from Bottom View)

12  | .  . |

13  | .  . |

| .  . |

| .  . |

| .  . |

| Etc..|

------------------------------------------------------------------------------

Pins Data Sheet

Verify these pins are Not Connected

Pin  |   Name    | Res. to VCC | Res. to VSS |  Should Be

-----|-----------|-------------|-------------|-------------------------

F13  |   A20M#   |             |             | Infinate

-----|-----------|-------------|-------------|-------------------------

E13  |   FLUSH#  |             |             | Infanate

-----|-----------|-------------|-------------|-------------------------

B12  |   KEN#    |             |             | Infinate

-----|-----------|-------------|-------------|-------------------------

A4   |   SUSP#   |             |             | Infinate

-----|-----------|-------------|-------------|-------------------------

B4   |   SUSPA#  |             |             | Infinate

-----------------------------------------------------------------------

C6   |   RPLSET  |             |             | Infinate

-----------------------------------------------------------------------

C7   |   RPLVAL# |             |             | Infinate

-----------------------------------------------------------------------

B6   |   NC      |             |             | Infinate

-----------------------------------------------------------------------

The following pins should be tied high.  To verify these pins, you can

connect the power supply to the motherboard and turn it on, then check

the voltage on these pins with respect to GND (VSS).  The voltage should

be almost exactly 5 VDC, not 4.85, but near 4.9 to 5.1 or so.

Pin  |   Name    | Voltage to VCC | Voltage to VSS |  Should Be

-----------------------------------------------------------------------

C10  |   LOCK#   |                |                | Tied High (VCC)

-----------------------------------------------------------------------

E14  |   ADS#    |                |                | Tied High (VCC)

-----------------------------------------------------------------------