#DIAGS.EXE

#Special Serial, Parallel, Video diagnostics for the PC

Written by Joan Riff for:

Computerwise Consulting Services

P.O. Box 813, McLean, VA 22101

(703) 280-2809

#

##Introduction#

DIAGS is a tool for the advanced PC  user,  period.  It  is  NOT  for  the

general  public.  This manual will not explain things in elementary terms.

If you know what a 6845 is,  and  what  an  8250  is,  and  what  parallel

handshaking lines are, then DIAGS will be extremely useful to you. If not,

then  you  may  have better things to do with your time than try to figure

out this program.

DIAGS started  out  as  a  testbed  for  our  new  interrupt-driven  RS232

libraries. The Serial Port Diagnostics were the first written, to test the

Microsoft version 3.0 'C' interface to these routines.

It  has  since  grown.  As  we  have  added more assembler routines to our

library, we have added corresponding DIAGS functions to test them.

Additionally,  DIAGS has been expanded as we experienced the need for  new

diagnostics capabilities. So the Parallel Port diagnostics were added when

we  had  to debug yet another unknown printer's handshaking.  And the 6845

diagnostics were added when we needed to test various  video  tricks  like

displaying 30 lines of text, smooth scrolling, etc.

As it now stands, DIAGS is a very powerful facility for:

1) Testing all sorts of asynch communications lines

2) Testing printer handshaking and cable wiring

3) Examining an unknown system's DOS environment, including:

a) Interrupt vectors

b) Resident device handlers

4) Directly manipulating the 6845.

#

##Main Menu#

The  main DIAGS menu displays information about the machine and about DOS,

and presents a list of the various diagnostics that  are  available.  When

you exit a particular diagnostic, you are returned to this main menu.

##(D)isk Info#

Let's describe the weakest and least important diagnostic first.

The DISK INFO selection from the main menu displays just about every-thing

that can be determined about a disk. Make sure that you have a diskette in

the indicated drive before selecting this option.

Since  there  are  other utilities which do just about everything that you

may want to do with a disk, we haven't done much with this diagnostic.  We

just display disk information as reported by DOS as well as by the  disk's

Boot Record.

Once in the Disk Info screen, the following keypresses are available to

you:

#ESC# - return to the main DIAGS menu

#ALT-V# - verify a diskette by reading every sector via DOS INT

025h. This is SLOW!!!! Think of this as a demo of just how

slow INT 025h can be.

#ALT-R# - reset drive. Doesn't do much, huh? That's what we

wanted to see.

#A thru Z# - select new drive for which statistics are to be

displayed. Again, be sure that you have a disk in whatever

drive you select.

That's it. Not much to it. If you need to do more, then use:

The Norton Utilities

Quaid's Disk Explorer

Central Point Software's Option Board

#

##(I)nterrupt Vector Display#

This selection from the main menu displays the system's Interrupt

Vectors. For each interrupt, the following information is displayed:

The interrupt number

The interrupt vector's address within segment zero

The Segment:Offset of its handler

The interrupt's name and/or use

If an interrupt points to an IRET, then that fact is reported by a

highlighted "#[IRET]#" displayed after the name.

Once in the Interrupt screen, the following keypresses are available to

you:

#ESC# - exit back to main menu

#Home# - scroll display back to Interrupt 0

#End# - scroll display to the end of the interrupts (F0 thru FF)

#PgDn# - scroll to the next page of vectors

#PgUp# - scroll up to the previous page of vectors

#Up Arrow# - scroll up 1 line

#Down Arrow# - scroll down 1 line

#

##(P)arallel Diagnostics#

This  is  a  more complicated set of diagnostics.  It allows you to do all

sorts of useful things with the parallel ports. We use these routines when

we're trying to figure out whether or not  some  new  printer  is  working

properly with the system.

At the top of the screen is displayed:

The number of parallel ports on the system (1 thru 4)

The number of the currently-selected printer (0 thru 3)

The base I/O address of the currently-selected printer

The trip character that is currently set (see below)

Whether interrupts are allowed or disallowed during sampling

The Parallel Port Diagnostics menu gives you the following choices:

#(I)nit printer# - does a ROM BIOS "init printer" call.

#(E)nable  Interrupts  during  sampling# - toggles the indicator on the

top line,  which indicates whether INTs are to be dis-allowed  during

sampling.

#(N)ew  Trip  character#  - allows you to specify a new character to be

sent to the printer to 'trip' it. This is initially a Linefeed.

#(P)rinter switch# - selects next printer (in sequence) as the  current

printer.

#(T)rip printer# - sends the current Trip character to the printer, and

then  captures  the  printer  Status port lines using a special high-

speed capture routine.  These lines are  dis-played  in  oscilloscope

format,  allowing  you  to  see at a glance the operation of the BUSY

line, ACK line, etc. Interrupts will be inhibited during the sampling

if the top line of the display shows "No Ints".  Press  ESC  to  exit

from the display of the status lines.

#(U)nique  Printer  setup# - sends custom codes to set various specific

printers into various modes. The printers currently supported are:

Data Products 8070 (and perhaps IDS Color Prism)

IBM Graphics Printer (and certain Epsons)

Fujitsu DL-2600 (and perhaps Epson JX color series)

Okidata Microline 93

Diable 630 (and C.ITOH Starwriter, F-10, etc)

C.ITOH Prowriter

#

Follow the prompts that are displayed when you make this

selection.

#(Q)uit parallel diagnostics# - to return to main menu

The following three selections are  invaluable  for  checking  proper

printer cabling and handshaking:

#(D)ata lines# - displays the parallel port  Data  I/O  port  lines  in

oscilloscope-like   format.   Press  ESC  to  exit  the  oscilloscope

display.

#(S)tatus lines# - as above,  but lines displayed are from the parallel

Status I/O port.

#(C)ontrol lines# - as above,  but lines displayed  are  from  parallel

Control I/O port.

The following selections send various strings to the printer:

#(F)ox message# - sends a continuous FOX message  to  the  printer  via

direct I/O (bypassing the ROM BIOS). Press ESC to stop it.

#(A)scii character set# - sends the complete ASCII character set to the

printer via direct I/O (bypassing the ROM BIOS).  No carriage returns

are sent. Press ESC to stop it.

#(H)oriz and vert alignment tests# - send various Formfeed,  Tab,  line

spacing  and  Backspacing  sequences  to printer.  This is useful for

testing the results of #(U)nique# printer setups,  and to  see  if  the

printer  supports  true  backspacing.  This  selec-tion  has  its own

submenu, which is exited via ESC.

#

##(S)erial Port Diagnostics#

If you do PC communications as much as we do,  then this part of DIAGS  is

for  you!  It  provides  a  very powerful look at the comm line.  Its most

intriguing feature is the ability to serve as a line monitor  between  two

serial devices.

When  you  select  "Serial  Diagnostics"  from the main menu,  your screen

becomes a terminal screen.  At the top of the screen is displayed  various

port information as follows:

Whether the port is open ("OP") or closed ("CL")

The current serial port number

The current baud rate

The number of data bits being used

The type of parity being used

The number of stop bits being used

Whether the existing 8250 parameters are being re-used (this

feature is only used by us, for internal testing)

What port is being wrapped to (more on this later)

Whether keypresses are echoed to the screen

Whether DEBUG mode (HEX display) is set

To the right of this information are various stats,  displayed only if the

port is open and stats have been enabled. These stats include:

The size of the RS232 output queue (in HEX)

The size of the RS232 input queue (in HEX)

The status of Carrier Detect and DSR

The number of times (in HEX) that the following have occurred:

RING

BREAK

Framing Error

Parity Error

Overrun Error

Once the port has been opened,  any "normal" keys that you press are  sent

over  the  comm line by being queued into the output queue.  Incoming data

appears on the screen.  Incoming and outgoing characters are displayed  in

different colors, so that you can tell what's what.

The RS232 routines are entirely interrupt-driven. The output queue will be

serviced  only  if  the  8250  is properly interrupting the 8088.  This in

itself is a valuable test of proper system operation.  If  the  output  or

input  queue  sizes  stay  "stuck" at a non-zero value,  then something is

wrong with your system.

#

Besides the "normal" keys, you may use the following keypresses:

#ALT-S# - switch display between port 0 and port 1 (COM1: and

COM2:).

#ALT-M# - bring up a menu of miscellaneous features:

HANGUP the phone by dropping DTR

PICKUP the phone by raising DTR

RESET the Hayes 1200b by OUT1 toggle

FLUSH input and output queues

CRLF control (how to display incoming CR and LF)

STATS control - turning the stats off can speed up the

terminal display considerable

TRAP - control what happens when you press ALT-T to trap

some 8250 register

#ALT-X# - initiate an XMODEM transfer. This we use to test the

accuracy of XMODEM as implemented in other software packages.

#ALT-P# - control port parameters. This includes:

port OPEN and CLOSE

BAUDRATE

DATA bit selection

PARITY setting

STOP bit selection

REUSE of the 8250 params (for our internal use)

ECHO (of local keypresses) on/off

WRAP of incoming data to another comm port

DEBUG display toggle

#ALT-T# - to trap the data lines from a previously-defined 8250

register

#ALT-R# - display various 8250 registers

#ALT-B# - to send a BREAK

#ALT-W# - wait (do nothing) till next keypress. This is useful

for pausing a fast-moving display.

#ALT-Q# - quit back to main menu

#ALT-F# - send a single FOX message over comm line

Most of these functions contain their own prompts,  and require no further

explanation. Some, however, need to be explained in detail.

The  continuous  updating  of  the stats on the top line can slow down the

terminal display.  Turning  stats  off  (ALT-M  S)  can  speed  things  up

considerably.

#

The  baudrate  is  continuously  variable,  from  2 to 38400.  You are not

limited to "standard" PC baud rates.

If you want to clear the screen and reset the stats,  try switching  ports

twice (ALT-S ALT-S).

To  trap and display an 8250 register in the middle of communication,  you

must first define the register to be trapped (ALT-M T)  and  whether  INTs

are to be allowed during the sampling that follows.

Having done that, you cause the trap by pressing ALT-T. At that instant, a

special  high-speed  assembler  routine  is  executed  which  samples  the

designated register once every 50 machine cycles or so until  420  samples

have  been taken.  These sample bytes are then displayed as a screenful of

HEX bytes.  You may highlight a particular byte via the  arrow  keys.  The

highlighted  byte's interpretation is displayed on the message line toward

the top of the screen. Press ESC to exit this trap display.

The "wrap" feature is very useful.  It may be used to allow DIAGS to serve

as a line monitor located between two serial devices,  among other things.

Every incoming character is "wrapped" as an output to  whatever  port  you

have  specified  as the "wrap" port.  If you select as the "wrap" port the

same port that you're using,  then you have in effect become a full-duplex

system. The serial device connected to that port will see its "keypresses"

echoed back to itself.

Assuming  that  you  have  two  serial ports,  you may use DIAGS as a line

monitor by wrapping each port's input to the  other  port.  You  can  then

switch  between  ports (ALT-S) and generally watch everything that goes in

either direction.  You can turn on the DEBUG  display  (ALT-P  G)  to  see

things in more detail.

#

##(R)esident device handler display#

This  main  menu selection allows you to see a list of all resident device

handlers.  As you highlight each handler's name (via the  left  and  right

arrow keys), the device's attributes are expanded on the screen. Press ESC

to exit this display.

Block devices are indicated by device names of the form "BLOCKnnn",  where

nnn is the number of devices supported by this block device handler.

#

##(V)ideo Manipulation#

If you want to do some fancy 6845 programming,  then the Video  diagnostic

will  be of great interest to you.  It allows you to examine and alter the

6845 registers in a most graphic fashion.

Toward the top of the screen is a two-column list of all  6845  registers,

and  one  or  two  of  the special IBM PC video registers (the MODE and/or

COLOR registers).  There is a "#>#" pointer,  showing you which register  is

being worked with.

For each 6845 register, two values are shown:

1) The original value, as set by the ROM BIOS

2) The current value as set by you

You  move  the  "#>#"  pointer to various 6845 registers via the up and down

arrow keys.  As you select each 6845 register, that register's bit pattern

is expanded on the bottom half of the screen.

You  can  change the currently-selected 6845 register's value by using the

"+" or "-" keypad keys to increment or decrement its value. + -

You can also use the left and right arrow keys to move the  "bit  pointer"

across  the  various  bits  of the expanded byte on the bottom half of the

screen.  Pressing "1" or "0" sets or clears the bit currently pointed  to.

This is an alternate way to change the current value of a 6845 register.

The  updated  6845  registers are loaded into the 6845 ONLY WHEN YOU PRESS

RETURN.  This allows you to make extensive changes  to  registers  without

having intermediate states foul up the display.

If you hose things up and get an unreadable screen, just press ESC to exit

the Video Diagnostics, then enter it again. The 6845 is reset on exit from

the Video Diagnostics.

If  you  are  using  a  Color/Graphics Adapter,  then you may press "S" to

toggle Snow on or off.  The upper-right corner of the  display  shows  the

current status of snow as follows:

"#Snow: YES#" means that no effort is being made to eliminate

snow, and that snow is present (although you won't see it if

you have a good Color/Graphics board)

"#Snow: NO#" means that snow is being eliminated by our video

software.

####################################################################################################################################################################################################################################################################################################################################################################################################