MAXDOS.COM and HIRUN.COM

These two programs were written to fake out the DOS memory map so that

more than 704k can be used to load programs in a 386 system running

a memory manager which allocates memory to high ram.

This version assumes a monochrome monitor with no graphics ability

taking up 4k at b000. The memory manager will have already loaded

DOS up to 704k and allocated space at b100 to efff so that it can

be used. This program will modify the DOS memory map and show a

dummy application using the 4k at b000.

Note: CHKDSK.COM does not check for total memory, just contiguous

memory. After MAXDOS is run CHKDSK will show 16 bytes less memory

available. This is the memory control block which is reserving the

4k video buffer.

HIRUN.COM will take up all low memory and execute a second command

processor which will execute in the high memory area. You can run

a program or make a program resident in high memory. Normally a program

will run in low memory, but if you 248k up in high memory you can run

a fairly large program.

Charlie,

Try loading all the memory residents in low memory and then

use the remaining memory to hirun PCBOARD. It might just work.

I can't say if any of this will work, but I have been running

programs in my video buffer all afternoon. I can't clear the screen

or I zap command.com, but you should be able to handle it.

Keith