PROGRAM: MOREDOS.COM

AUTHOR: Keith P. Graham

CONTACT: PC-Rockland BBS (914) 623-2176

DESCRIPTION: MOREDOS is another variation on a series of utilities to

increase the DOS limit above 640k. This works with ordinary memory

cards for PC/XT's. On PC's set the memory switches as low as possible

and then run MOREDOS. This will prevent the DOS memory count which

can take so long. This is not applicable to XT's and clones which

have no switches for memory. This will work on both PC's and XT's

which have memory expansion cards which are switch selectable to

address memory above 640K, Set the boards so that they will go up to

the graphics card buffer (B000 for mono, B800 for color). MOREDOS

will automatically check up to 704k (B800) for other than this default

the form of moredos is MOREDOS n where n is the number of 32k memory

areas to check. To check up to 640k only (for EGA users) enter

MOREDOS 0. For machines without displays and full memory up to C000

then enter MOREDOS 4. For CGA systems enter MOREDOS 3.

OPERATION: MOREDOS.COM turns of NMI and the does reads and writes

to memory until the read/write fails. It then updates the BIOS

memory area and the DOS memory allocation blocks to reflect the

new memory. If there is a command line parameter of 0 to 4 it will

limit its search to the number of 32k blocks above 640k as indicated

by the command line. MOREDOS is not memory resident and can be entered

at any time, although it should confuse the hell out of Doubledos and

similar programs if not run first in the autoexec.bat.