SPEED.COM

This program compares the microprocessor and memory

operations in your computer with the "standard" IBM/PC.

Peter Norton's Norton Utilities has a routine called SI

(SYSINFO.COM) that attempts to measure the performance of a

compatible against an IBM/PC.  This program expands upon that

idea and gives a more complete showing of what the actual

performance of a machine may be in "real world" applications.

For a fair check, you should boot from the DOS disk so that

no extra device drivers are installed (that use interrupts

and slow things down).  With the standard IBM/PC, each of the

operations indicated will show 100% (or nearly so).  Any

slight difference is due to asynchronism between the programs

operation and the system clock.

The response from my "turbo-XT" looks like this:

Checking memory block write   ticks: 129 compared to IBM/PC 391%

Checking register to memory   ticks: 262 compared to IBM/PC 187%

Checking memory to register   ticks: 279 compared to IBM/PC 175%

Checking register to register ticks: 326 compared to IBM/PC 169%

Checking divide by register   ticks: 102 compared to IBM/PC 501%

Checking divide by memory     ticks: 169 compared to IBM/PC 357%

Checking multiply by register ticks: 115 compared to IBM/PC 418%

Checking multiply by memory   ticks: 143 compared to IBM/PC 389%

Checking stack operations     ticks: 248 compared to IBM/PC 179%

Checking far jumps, far calls ticks: 309 compared to IBM/PC 167%

Total time is: 2082 clock ticks, (115 seconds) compared to IBM/PC 247%

This means that my system is 247 percent faster than an

IBM/PC.  Note that memory operations (stack, register to

memory, etc.) are not quite as fast as register only

operations.  This shows that the probable bottle-neck in my

system is memory access.  This "turbo" system runs a V-20 at

8MHz.

If you take this program to computer swap-meets, you can

compare the speed of various "clones" before you purchase or

update your system.  It can even help you negotiate for a

better price if the speed is not quite as fast as the saleman

says!

Cheers,

Dick Johnson

SYSTOP

Program Exchange

3603 Chestnut Street Apt. C

Lafayette, CA. 94549

(415) 283-3505  data

(415) 283-2809  voice