---------------------

COMMCHK (TM) and COMMSET (TM)

Version 90-2

---------------------

Copyright 1990 by John W. Roberts

All Rights Reserved

UTILITY PROGRAMS TO CHECK AND SET

COMMUNICATIONS ADAPTER SETTINGS

1. INTRODUCTION

1.1 The Problems that COMMCHK and COMMSET Solve:

A  friend or client has asked you to help solve a  communications

problem.   You  remember having difficulty  installing  a  serial

device on your own system but that was a long time ago.   As  you

gather  the stuff you think you'll need for the help session,  be

sure  to  put  COMMCHK and COMMSET in your  toolbox.   These  two

utility  programs are designed to help you install a  new  serial

device.   They were created to overcome the deficiencies in  most

versions of DOS in use today.

Option  3  of  the DOS MODE command lets  you  set  the  protocol

parameters  of an Asynchronous Communications Adapter  or  serial

port.   For example entering,

A> mode com1:24,n,8,1

is sets the mode of operation of COM1 to 2400 baud, no parity,  8

databits, and 1 stopbit.  This works for COM1 and COM2 but  there

are some problems.

First  of all the MODE command does not provide a way to  examine

the  current settings.  After entering "mode  com1:24,n,8,1"  you

can't tell if it took.  This can be frustrating when you  suspect

that  one of the programs that is supposed to talk to the  device

sets  up its own protocol without telling you.   COMMCHK  reduces

this  frustration.   COMMCHK reads and displays  the  baud,  data

bits,  stop  bits  and parity settings  for  four  communications

ports.

The second problem involves the number of comm ports supported by

the mode command -- and DOS.  Basically, PC hardware in use today

is  capable of supporting the use of seven or more serial  ports.

The  mode command in some versions of PC-DOS (3.10  for  example)

can only set the communications parameters for COM1 and COM2. You

can't  use  the MODE command to set the parameters  for  COM3  or

COM4, although programs such as ProComm do it all the time.  When

you  are helping a friend or client and get an  "illegal  device"

response to this command,

A> mode com3:24,n,8,1

the  problem may be with the installed operating system  and  not

the  communications  adapter.  Use COMMSET to configure  COM3  or

COM4.   After  doing  so,  use  COMMCHK  to  "double  check"  the

settings.

The third problem can occur when you are trying to use a PC as  a

terminal  for  a  larger  local  computer  and  want  to  use   a

transmission  rate  of  19,200 bps.  The mode  command  does  not

support 19,200.  COMMCHK and COMMSET do!

1.2.  How to Use COMMCHK

To use COMMCHK type

[d:][path]COMMCHK

where  [d:][path] before COMMCHK specify the drive and path  that

contains  the COMMCHK.EXE file.  In a moment or two  the  display

will look something like this:

------------------------------------------------------

C> A:COMMCHK

COMM PORT NUMBER 1:    mode com1:2400,N,8,1

Transmission speed:   2400

8 Data Bits     1 Stop Bit   Parity: NO

COMM PORT NUMBER 2:    mode com2:1200,N,8,1

Transmission speed:   1200

8 Data Bits     1 Stop Bit   Parity: NO

COMM PORT NUMBER 3

This port does not exist or is not

set up for communications

COMM PORT NUMBER 4:    mode com4:2400,E,8,1

Transmission speed:   2400

8 Data Bits     1 Stop Bit   Parity: Even

C>_

------------------------------------------------------

Simply read and record the parameters of the port (or ports)  you

are  investigating.  Later, when you want to see if any  programs

or  batch files have changed the settings, run COMMCHK again  and

compare the results.

COMMCHK  always  tries to read the communications  parameters  of

four  communications  ports  even though fewer  may  be  present.

Notice  the two-line error message for COM3 in  example  display.

This  is  how COMMCHK handles the situation where the  comm  port

does not exist, or is not set up for communications.  If you  are

trying to configure a comm port that displays such a message, you

can  determine which situation applies by attempting to  set  the

parameters  using COMMSET.  If you are successful,  COMMSET  will

display  the new settings.  If the port does not  exist,  COMMSET

will  report the same error message and advise you that the  port

is  missing.   You may determine the situation by first  using  a

MODE command such as

A> mode com3:24,n,8,1

instead  of  COMMSET.  Next run COMMCHK to see  if  the  settings

"took."   If  the settings "take" (as indicated by  COMMCHK)  the

port  exists.  If the same two-line error message appears  again,

the  port probably does not exist.  If DOS responds to  the  mode

command  with an "illegal device" message, you must  use  COMMSET

instead of the MODE command to set that port.

It  is  possible  for COMMCHK  to  display  bogus  communications

parameters instead of the two-line message shown for COM3.  Treat

the  bogus  data  as an error message and  proceed  as  described

above.

By  setting  switches or moving jumpers you  can  configure  most

communications  adapters  as COM1, COM2, COM3, or COM4.   If  the

port  you  need  seems to be missing, you may have  to  open  the

computer so you can examine the boards.

Suppose you suspect your program is changing communications  port

settings when it starts but does not restore the original  values

when  it finishes. Prove it by installing a unique  setup  before

running the program.  After the program stops use COMMCHK to  see

if the program changed any of your settings.

1.3.  How to Use COMMSET

To use COMMSET type

[d:][path]COMMSET

where  [d:][path] before COMMSET specify the drive and path  that

contains  the COMMSET.EXE file.  COMMSET uses  interactive  input

routines  that suggest an appropriate response to each  question.

The first "screenfull" of questions looks something like this:

------------------------------------------------------

*** Sets Comm Port Parameters ***

Set parameters on port number= 4            <--- a

COMM PORT NUMBER 4:  mode com1:2400,N,8,1   <--- b

Transmission speed:   2400

8 Data Bits  1 Stop Bit   Parity: NONE

Is this the one you want to change? YES     <--- c

------------------------------------------------------

After  the screen clears and the title appears, line "a"  appears

in  the form of a question: "Set parameters on port  number?  1."

You  may  accept  the suggested answer, "1"  by  simply  pressing

Enter.   The  example shows the result of selecting  a  different

port.   After overrstriking 1 with 4 and pressing Enter,  COMMSET

changes the question mark to an equal sign to indicate acceptance

of  your choice. Next the settings for port 4 appear starting  on

line  "b."  The question on line "c" is COMMSET's way  of  asking

you to confirm or approve your choice of ports to change.  Accept

the  "YES"  answer  by pressing ENTER and the  next  screen  will

appear.   Overstrike  the YES with "n", "no",  "x",  or  anything

other that "YES" you start over again with line "a."

------------------------------------------------------

*** Enter the New Settings ***

1       110   Baud                <--- d

2       150   Baud

3       300   Baud

4       600   Baud

5       1200  Baud

6       2400  Baud

7       4800  Baud

8       9600  Baud

9       19200 Baud

Your choice= 5                         <--- e

Data bits (7 or 8)= 7                <--- f

Stop bits (1 or 2)= 2                <--- g

Parity codes: N = None, O = Odd, E = Even

Set the parity code to? N_           <--- h

COMM PORT NUMBER 4:  mode com1:2400,N,8,1 <--- i

Transmission speed:   2400

8 Data Bits  1 Stop Bit   Parity: NONE

Shall we set another one? Ok          <--- j

------------------------------------------------------

An  indexed table of Baud choices appears beginning on line  "d."

On  line "e" enter the index or number of the baud rate you  want

to  use.  As a reminder or reference the old or  original  values

appear in the COMMCHK-like table that starts on line "i."   Enter

your  choices  for data bits, stop bits and  parity  settings  on

lines  "f", "g", and "h" respectively.  Line "h" (and  the  table

starting  on  line  "i")  represent  the  situation  just  before

pressing  ENTER in response to the parity question.   Just  after

pressing ENTER the color of the table changes and your new values

appear.  The question on line "j" appears.

If you accept the "Ok" answer to the question on line "j" COMMSET

takes  you back to the "Sets Comm Port Parameters" screen so  you

can  select  a  different  port to change.   If  you  answer  the

question by overstriking the "Ok" with anything other than  "Ok",

the COMMSET stops.  (You may also stop the program pressing ^C or

entering  the  word  "abort"  (or "ABORT")  in  response  to  any

question.)

COMMSET  checks  the validity of every entry, tells you  when  an

illegal  value  is  found  and waits for you  to  enter  a  valid

setting.

Here's  how  the  display looks after you've  tried  to  set  the

parameters of a comm port that does not exist:

------------------------------------------------------

*** Enter the New Settings ***

1       110   Baud               <--- d

2       150   Baud

3       300   Baud

4       600   Baud

5       1200  Baud

6       2400  Baud

7       4800  Baud

8       9600  Baud

9       19200 Baud

Your choice= 5                        <--- e

Data bits (7 or 8)= 7               <--- f

Stop bits (1 or 2)= 2               <--- g

Parity codes: N = None, O = Odd, E = Even

Set the parity code to= N           <--- h

COMM PORT NUMBER 3                       <--- i

This port does not exist or is not     <--- k

set up for communications.

Somebody bad took that communications adapter!!! <--- l

(If it can't be set, it does not exist.)         <--- m

Shall we set another one? Ok         <--- j

------------------------------------------------------

In  this  case, the two-line error message starting on  line  "k"

would appear when the port is first selected.  The content of the

COMMCHK  table does not change when you enter your parity  choice

in line "h."  Instead lines "l" and "m" appear to explain why the

settings did not change.

1.4.  Development

The  original version of COMMCHK was written in Turbo  Pascal  in

1989.   It  was  based on information found in  a  public  domain

program   named  DUMBTERM.PAS,  created  by  Jim   McCarthy   and

distributed  by the Turbo User Group, P.O. Box 1510, Poulsbo,  WA

98370.  The Turbo Pascal version was published in April/May issue

of  Tug Lines, the magazine of the Turbo User Group.  In  January

1990,  COMMCHK  was  rewritten using  Janus/Ada.   The  code  was

modified   to   eliminate  the  occasional   display   of   bogus

communications port settings.  The need for a COMMSET utility was

recognized.   Communications  routines  that  could  be  used  in

COMMCHK  and  COMMSET  were placed in  a  package  of  "reusable"

routines called COMMSTUF.  In February 1990, COMMSET and  COMMCHK

were modified to handle a baud rate of 19,200.

COMMSTUF depends on off-the-shelf library units (System and  Bit)

provided  by  R.R. Software, P.O. Box  1512,  Madison  Wisconsin,

developers  of the Janus/Ada compiler.  All Ada compiler  vendors

provide  a System package and most provide one or  more  packages

that include the types of functions and procedures found in  R.R.

Software's Bit package.

COMMCHK is a "plain vanilla" Ada program that depends on or needs

only  the COMMSTUF package and the standard TEXT_IO package.   It

is  small  and  does a simple job without  any  extra  frills  or

features.

COMMSET  depends  on  COMMSTUF and  a  few  powerful  interactive

Ada  IO  packages  designed for use with  color  terminals  (AG6,

PORTNUMS,  and  NUMOPS).  These packages add a variety  of  extra

frills and features.

Ada  source code is provided for COMMSTUF, COMMCHK  and  COMMSET.

Those  who use Janus/Ada should be able to modify COMMCHK  and/or

reuse  the routines found in COMMSTUF in other  programs  without

difficulty.  The packages used to add the frills and features  to

COMMSET  are not yet ready for release.  Without these  packages,

modifying  COMMSET  will be more difficult  but  not  impossible.

COMMSET  source  code  contains enough  information  for  you  to

identify the routines taken from AG6, PORTNUMS and NUMOPS.   Once

these are replaced with routines based on TEXT_IO you will have a

"plane  vanilla"  version  of  COMMSET.   A  comparison  of   the

Display_setup_of_port procedure found in both COMMCHK and COMMSET

would be a good place to start.

2.  Warranty

This  software  and  manual  are provided  "as  is"  and  without

warranties  as  to performance of merchantability  or  any  other

warranties whether expressed or implied.  Because of the  various

hardware and software environments into which these programs  may

be  put,  no  warranty of fitness for  a  particular  purpose  is

offered.

Good  data  processing procedures dictates that  any  program  be

thoroughly  tested with non-critical data before relying  on  it.

The user must assume the entire risk of using this program.   Any

liability  of the seller will be limited exclusively  to  product

replacement or refund of the purchase price.

3.  Shareware

These utility programs are distributed as shareware.  This  means

you  may test these programs on your system free of charge.   The

programs are not free.  They are copyrighted software provided to

you  to  evaluate  before  paying.  If  you  find  these  utility

programs useful, you are encouraged to fill out the  registration

form  (found in COMMREGS.TXT) and mail it together with  a  check

for $10.00 to

John W. Roberts

P.O. Box 68453

Virginia Beach, VA 23455-9998

These  utility programs are not the kind of tools you are  likely

to  use on a daily basis.  I really don't expect you to  register

just because you were able to run the programs on your  system...

or  when  you first use them to actually solve  a  communications

interface  problem.   Wait until the second time  you  use  these

programs to solve a problem and then register.

If  you  develop software, there is a place on  the  registration

form  for you to request more information about Ada  software  in

general  and the release of the interactive IO packages  used  in

developing COMMSET in particular.

4.  Distribution

You  are free to make copies of these programs for  yourself  and

your friends providing you do not charge a fee to do so.   Anyone

wishing to charge people a fee for giving them a copy of  COMMCHK

or COMMSET must have my written authorization to do so.   Without

this  authorization,  the  distributor  is  guilty  of  copyright

violation.   To  obtain authorization, write to me at  the  above

address  and  request a COMMCHK  COMMSET  distribution  licensing

agreement.

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

Public (software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  PsL cannot debug pro-

programs over the telephone, though we can answer questions.

Disks in the PsL are updated  monthly,  so if you did not get

this disk directly from the PsL, you should be aware that the

files in this set may no longer be the current versions. Also,

if you got this disk from another vendor and are having prob-

lems,  be aware that  some files may have become corrupted or

lost by that vendor. Get a current, working disk from PsL.

For a copy of the latest monthly software library newsletter

and a list of the 2,000+ disks in the library, call or write

Public (software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

1-800-2424-PSL

MC/Visa/AmEx

Outside of U.S. or in Texas

or for general information,

Call 1-713-524-6394

PsL also has an outstanding

catalog for the Macintosh.