CHEAP AND EASY REAL-WORLD CONTROL FUNCTIONS FROM YOUR PC

Here's a cheap and easy way to make an IBM-compatable PC perform

real-world control functions using NO special hardware, and the

software you are already familiar with.

LPTWRITE is a program that allows you to write directly to

individual pins of a parallel printer port, thereby using the

printer port as a control device for external equipment. It's

designed to be called from other programs by temporarily

returning to DOS, executing with parameters, then exiting back to

the calling program.

LPTWRITE can individually execute a data ON or data OFF state for

each of the eight data bits on the LPT port. These correspond to

pin 2 through pin 9 on the connector. Pin 17 is used as a +5vdc

supply, and the data state exists as a potential between each of

the eight data bits and pin 17.

LPTWRITE version 1.0 addresses printer ports 1 and 2, although

these may have different physical addresses on different

computers. Your LPT1 might be someone elses LPT2. You'll have to

experiment to see which is yours.

The calling protocol is this:

LPTWRITE n 00000000

where n  := printer port to write (1 or 2)

00000000 := output bite, binary-coded

The binary coded output byte contains one place for each of the

eight data pins on the LPT port. Entering a "0" in any of the

places turns OFF that pin, making it electrically neutral.

Entering a "1" in any of the places turns ON that pin, making it

electrically grounded.

As an example, the following command turns ON pins 2 and 4 on LPT

port 2, and turns OFF all others on that port:

LPTWRITE 2 01010000

LPTWRITE executes a data ON by connecting the pins to ground.

Placing a load between each data pin and pin 17 (see the

schematic) provides a way to make use of this state. There's

enough power on an average parallel printer board to run eight

jumbo 5-volt LEDs, eight low-power micro-relays, or to provide

input for any TTL level output driver circuit.

In the OFF state, each of the data pins is "floating" that is,

they are connected neither to ground nor to +5vdc. In a TTL

driver circuit, be sure to use 10K hold-down resistors between

each of the pins and LPT port ground, pins 19-25; AND be sure to

use pins 19-25 as common ground for your custom circuits.

When using micro-relays, be sure to place a surge-protection

diode across each relay to prevent damage to your computer by the

current generated in the relay coils when they de-energize.

Obviously, fiddling with your computer hardware is a little

risky. However, the LPT card is usually very resilient and

difficult to damage. Just be careful to observe all normal

electronic safeguards.

Initialize the LPT port you intend to use in your programs by

including the following command in your AUTOEXEC file:

LPTWRITE n 00000000

where n := the port number for your computer. This forces the

port to a binary zero and sets up the background task which

handles the port communications with your program.

Commercial versions of LPTWRITE will address all available pins

on all ports, and will allow for READING as well as writing

individual pins. These versions will be available as libraries

and OBJ modules for linking with your applications, and also as

small TSR's.

LPTWRITE is Copyright (c) 1991 by Rob Cosgrove and is distributed

by Precision Data Corporation in Memphis, Tennessee. You are

hereby granted a limited license to use and distribute LPTWRITE

and this documentation commercially or non-commercially, as long

as you 1) don't charge money for it, and 2) don't modify

LPTWRITE.COM to eliminate the copyright message.

If you send us a registration fee of $20.00 you will receive a

commercial version of LPTWRITE which writes to more than eight

pins on each port and has no sign-on message, as well as a demo

version of LPTREAD.COM (see the hype below).  We'll also put you

on our mailing list for future announcements of similar neat

stuff.

LPTREAD.COM is a useful little TSR which watches an LPT port and

executes up to eight different keystroke sequences through the

keyboard buffer - one for each of the eight data bits on the LPT

port. Close a data bit on the LPT port, and LPTREAD types the

associated key sequence through the keyboard - no matter what

software is running.

LPTREAD is great for adding real-world data acquisition functions

to ANY program without difficult real-world programming. LPTREAD

reads simple switch closures like in alarm systems, laboratory

monitoring, simple robotics, and automation. The commercial

version has programmable keyboard macros, up to 255 bytes long,

for each of the eight data bits PLUS some extra pins.

Send $35.00 and receive the commercial versions of both programs.

We at PDC are available for consulting and programming using this

and other inovative technology for industrial control, data

aquisition, telemetry, and QC testing.

Precision Data Corporation

654 South White Station Road

Memphis, TN  38117

(901) 682-0732

CONTROL HARDWARE SCHEMATIC

for use with

LPTWRITE.COM

Parallel Port

Connector

-----------------

Function    Pin

_________________

DATA 0    2 ----------------------+--VUVUVUVU--------+

|                  |

+------|<----------+

|

DATA 1    3 ----------------------+--VUVUVUVU--------+

|                  |

+------|<----------+

|

DATA 2    4 ----------------------+--VUVUVUVU--------+

|                  |

+------|<----------+

|

DATA 3    5 ----------------------+--VUVUVUVU--------+

|                  |

+------|<----------+

|

DATA 4    6 ----------------------+--VUVUVUVU--------+

|                  |

+------|<----------+

|

DATA 5    7 ----------------------+--VUVUVUVU--------+

|                  |

+------|<----------+

|

DATA 6    8 ----------------------+--VUVUVUVU--------+

|                  |

+------|<----------+

|

DATA 7    9 ----------------------+--VUVUVUVU--------+

|                  |

+------|<----------+

|

|

+ 5 VDC  17 -----------------------------------------+

VUVUVUVU   :=   Load (Relay, LED, or TTL level input)

---|<---   :=   1N4002 Silicone Rectifier

The Loads in this schematic are low-power 5 VDC relays. Solid

state relays work best. For testing you can use 5 volt LEDs. If

using an inductive load (relays), be sure to use the 1N4002

diodes to protect your computer equipment against surges from the

coils.

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

Public (software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  PsL cannot debug pro-

programs over the telephone, though we can answer questions.

Disks in the PsL are updated  monthly,  so if you did not get

this disk directly from the PsL, you should be aware that the

files in this set may no longer be the current versions. Also,

if you got this disk from another vendor and are having prob-

lems,  be aware that  some files may have become corrupted or

lost by that vendor. Get a current, working disk from PsL.

For a copy of the latest monthly software library newsletter

and a list of the 4,000+ disks in the library, call or write

Public (software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

1-800-2424-PSL

MC/Visa/AmEx/Discover

Outside of U.S. or in Texas

or for general information,

Call 1-713-524-6394

PsL also has an outstanding

catalog for the Macintosh.

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

Public (software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  PsL cannot debug pro-

programs over the telephone, though we can answer questions.

Disks in the PsL are updated  monthly,  so if you did not get

this disk directly from the PsL, you should be aware that the

files in this set may no longer be the current versions. Also,

if you got this disk from another vendor and are having prob-

lems,  be aware that  some files may have become corrupted or

lost by that vendor. Get a current, working disk from PsL.

For a copy of the latest monthly software library newsletter

and a list of the 4,000+ disks in the library, call or write

Public (software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

1-800-2424-PSL

MC/Visa/AmEx/Discover

Outside of U.S. or in Texas

or for general information,

Call 1-713-524-6394

PsL also has an outstanding

catalog for the Macintosh.