EGACHG.COM

Ver. 1.1, Feb 17, 1986

Part of the ENHANCER series of programs

Copyright (C)  I. B. Miller     1985, 1986

If you find this of value and use it please send $25 to

Enhancers, 231 Otonabee Ave., Willowdale, Ontario, Canada M2M-2S9

It is suggested that companies contribute more for use on more than

one machine.  Please contact me for a site license.  Phone (416)

226-9753

EGACHG is a program which provides a means to preset 12 palette

color combinations for EGACLR.COM a companion program which Controls

the seventeen palette registers on the enhanced color card (Enhanced

Graphics Adapter), 12 different user selected color combinations of

any 16 of the 64 colors available can be made active by use of the

Alt key at the same time as any of the number keys 1 to = at the top

of the qwerty keyboard.  EGACHG will change the colors in the

Palette registers so that you can see what is available but unless

you use EGACLR these will be lost at the next video reset called by

any program.

The actual colors available to you depend on your actual hardware

set up and so may be different.  As an example if you have only a

regular monitor you will have only standard colors though you will

be able to swap them around.  In my experience THE limiting factor

for text display is the monitor.  Though for graphic display memory

size is also a limiting factor.

General Information

-------------------

With the Enhanced Monitor color coding is as follows.

Bit                            Hex

---                            ---

1 is    Medium     Blue     - 01

2 is    Medium     Green    - 02

3 is    Medium     Red      - 04

4 is Low   intensity Blue   - 08

5 is Low   intensity Green  - 10

6 is Low   intensity Red    - 20

1+4 is High  intensity Blue   - 09

2+5 is High  intensity Green  - 12

3+6 is High  intensity Red    - 24

Low and medium add to give high - this is the basic principle the

card works on.

Following is the standard color palette combination.

----------------------------------------------------

B  B  G  C  R  M  B  W  G High intensity colors O

l  l  r  y  e  a  r  h  r --------------------- v

a  u  e  a  d  g  o  i  e  B  G  C  R  M  B  W  e

c  e  e  n     e  w  t  y  l  r  y  e  a  r  h  r

k     n        n  n  e     u  e  a  d  g  o  i  s

t           e  e  n     e  w  t  c

a              n        t  n  e  a

a        n

palette

register # = 0  1  2  3  4  5  6  7  8  9  A  B  C  D  E  F 10

EGACHG  page 2

"10" really cannot be used in Enhanced Mode because of a seeming bug

in the EGA.  I cannot believe IBM wanted it that way.  Try it & see.

How the program works

---------------------

The program works by loading in the entire EGACLR.COM file and then

selecting the color controlling bytes which are all near the start

as initial values for setting the colors.  It then follows the user

through the registersets updating them in memory and changing them

and the colors on display as the user dictates.

Once the user is finished changing colors to his liking and has

pressed the End key EGACHG first checks to see how high in the

sequence of possible EGACLR1.COM to EGACLR99.COM exists on the disk.

once found it increments this and writes out a temporary file.

After it has succeeded in the write it renames the original

EGACLR.COM to EGACLR(whatever #).COM and then renames the temporary

file to EGACLR.COM.  In theory you could change names and have

almost unlimited numbers of color combinations which can be made

active by using these variants of EGACLR.COM, but in practice I find

it adequate to have a few and use the command parameters with

EGACLR.COM to change <alt 1>

Instructions

------------

EGACHG has been written to work on the EGACLR.COM file on the same

drive as EGACHG.COM.  Make sure both are present on the same disk.

Important: Unless you use the EGACLR command line to turn off auto

reset, or have not yet activated EGACLR, then EGACLR will proceed to

not allow this program to show the colors to you.  You must

Deactivate (or not start) EGACLR before starting EGACHG.

The program will proceed to load into itself the EGACLR.COM file.

It will then paint a window on the top half of the screen showing

the palette colors currently in use with EGACLR use of <Alt 1> which

I call registerset # 1. The program will next paint the bottom

window which is the working area and will display the current

register of the 17 included in the current registerset as well as

the current value in that register displayed in both decimal and

Hex.  It will also show the RGB components of the color

corresponding to that value.

From then on you are in the part of the program that does all the

work and the following keys are the only ones which will do

anything.

PgUp  Moves Up one registerset from 1 to 12 then back to 1

PgDn  Moves Down one registerset from 12 to 1 then back to 12

Up    Moves up one Pallette register from 0 to 16 then back

Dn    Moves Down one Pallette register from 16 to 0 then back

+   Increases value in Pallette register from 0 to 63 then back

-   Decreases value in Pallette register from 63 to 0 then back

EGACHG  page 3

R  Cycles Red component of color off, low, med, high & back

G  Changes Green component off, low, med, high & back

B  Changes Blue component off, low, med, high & back

Q  Allows you to quit without creating an output file

S  Standardizes colors In current Registerset

W  writes out a .BAT file to reset colors in Alt 1.

A sample appears here:

EGACLR 02A,126,202,337,404,505,614,707,818,919,A1A,B1B,

C3C,D2D,E36,F3F,s00

End  Writes out all the values to an EGACLR.COM files while saving

the previous ones as EGACLR1.COM up to EGACLR99.COM

Use the End key when you have completed your selection. the program

will then update EGACLR.COM with your selected colors available from

<alt 1> to <alt =>.

You may then activate the colors by entering at the DOS prompt

EGACLR R

Disclaimer

----------

For obvious reasons the user is totally responsible for use of this

program and determining that it is suitable for use by him, and he

uses this program with no warranty from myself.

It should work properly with the EGA from IBM or any card which

later maintains compatibility with it

This program and EGACLR.COM are used by myself with LOTUS Develop-

ment programs and work well solving a colors shift bug (or less than

perfect EGA driver routine) but obviously LOTUS does not support it.

General

--------

Again please note that if you have the regular monitor or the

Enhanced Monitor operating in regular mode none of the non-standard

colors are available to you!

Of Course

----------

The program EGACHG.COM will allow you to work with many colors and

see them on screen interactively without any of the work involved in

remembering all the numbers and color coding schemes.  I believe it

will make color control easy.

If you have any comments or suggestions please contact

me either at Home in the evenings at           (416) 226-9753

or at the following BBS's   PCanada(pc-1266)   (416) 751-6337

SGV-BBS        (416) 673-0557

I am also on Compuserve as  72167,2563          Ivor B. Miller

------------------------------------------------------------

This disk copy provided as a service of

The Public (Software) Library

the software library of

The Houston Area League of PC Users

For a copy of the latest monthly software library newsletter

and a list of the 600+ disks in the library, call or write

The Public (Software) Library

P.O.Box 35705

Houston, TX 77235-5705

(713) 721-6104

Newsletter subscriptions are available for $12 a year.