Author's Documentation:                                 12/10/91

QCRT 2.1:  Glenn K. Smith

RADON Software

Command: QCRT  [1 optional command]

[?]  <-- Information about QCRT

[K]  <-- Remove QCRT from memory

[S]  <-- Enable CGA snow checking

[X]  <-- Enable XTRA-CGA snow checking

[V]  <-- Display current version of QCRT

QCRT is a small TSR program that speeds up certain BIOS INT

10h functions dealing with text modes.  The actual increase varies

according to your computer speed, video card, and video mode.

Speed increases vary from 10 to 80 percent (40x25 and 80x25

average 20%, extended text modes such as 80x50 and 132x44 average

60%).  See the chart below for more information.

QCRT will only enhance the video display of the computer it

is installed on.  If you want to use it on a network system,

you'll have to install the program on each computer.

QCRT has been extensively tested and found to be compatible

with all software and video cards (I hope).  Version 2.1 has been

enhanced to support the extended text modes of EGA and VGA cards.

These extended modes really benefit from QCRT.

QCRT is also compatible with memory management program such

as QRAM, QEMM, 386MAX.  You may load QCRT into high memory using

any of these programs.  QCRT can be removed from this high memory

if needed.  It is also compatible with  DR. DOS 4.0-6.0, and MS.

DOS 3.2-5.0 (also PC-DOS).

To install QCRT, type QCRT at the DOS prompt.  To check to

see if QCRT is helping your video display, try copying a text file

to the CON: device.  It should scroll by at a rate faster than

before QCRT was installed.

If you own a CGA card, you may notice some little white

lines appearing on the screen (called snow).  These are harmless,

but may be annoying.  You may be able to get rid of them by using

the S option.  This will enable basic flicker elimination.  This

option should take care of most flicker on all CGA cards.  If you

have an older card or the S option didn't work, use the X option.

Using this feature will slow down QCRT somewhat (actually, quite a

bit!), but it will still be faster than not using QCRT at all.

Most newer CGA cards, all MDA, EGA and VGA cards don't need either

option and will benefit greatly from QCRT.  The S and X options

are ignored on systems equipped with MDA, EGA and VGA cards.  If

QCRT is already installed, you'll need to remove it from memory

first then re-install it.

To remove QCRT, just type QCRT K.  If you have trouble with

QCRT, typing QCRT ? will display a small help screen, along with a

begging for money screen!

If you need to determine what version of QCRT is installed in

memory, type QCRT V.  Current versions are 1.2 (old) and 2.1

(new).  If any other versions appear, it's not my program and you

should get rid of it!

If you own a VGA card or dual video card system, don't switch

back and forth between monochrome and color modes.  QCRT does not

recognize the change and won't work correctly.  If you are using a

memory manager, you probably can't switch modes anyway.  Switching

between CGA, EGA, or VGA text modes is ok since they all use the

same video display memory.

If you have any compatibility problems with QCRT, please

contact me and let me know what's going on.  Due to the variety of

computers and programs out there, it's hard to make sure a program

works with everything.  I've tried, but I'm sure somebody

somewhere will find a problem.

For those of you interested in what kind of speed increase

you might get from QCRT, I've included the following chart showing

benchmark times from the program VBENCH, which I wrote to see if

QCRT was really worth all the trouble.

VBENCH tests the three most common video display methods and

reports how long it takes your video card to complete the tests.

All times are reported in seconds.  I've included results from

several different systems.  I recommend trying the program on your

system to see how it performs.  Depending on your system, you may

see a drastic increase in video text display speed.

256 Screen Fills

+----------------------------------------------------------------+

|                            BIOS         QCRT     % increase    |

|----------------------------------------------------------------|

| 10 Mhz XT with MDA        31.75         3.24         89.8      |

|----------------------------------------------------------------|

| 8 Mhz 286 with CGA         2.42         2.03         16.1      |

|----------------------------------------------------------------|

| PS/2 model 25 with MCGA    3.25         2.36         27.4      |

|----------------------------------------------------------------|

| 12 Mhz 286 with VGA        2.31         0.66         71.4      |

|----------------------------------------------------------------|

| 25 Mhz 386 with SVGA       0.83         0.55         33.7      |

+----------------------------------------------------------------+

256 strings using TTY write w/o scrolling

+----------------------------------------------------------------+

|                            BIOS         QCRT     % increase    |

|----------------------------------------------------------------|

| 10 Mhz XT with MDA        12.41         3.08         75.2      |

|----------------------------------------------------------------|

| 8 Mhz 286 with CGA         3.90         2.20         43.6      |

|----------------------------------------------------------------|

| PS/2 model 25 with MCGA    8.07         2.04         74.7      |

|----------------------------------------------------------------|

| 12 Mhz 286 with VGA        4.29         0.66         84.6      |

|----------------------------------------------------------------|

| 25 Mhz 386 with SVGA       1.15         0.82         28.7      |

+----------------------------------------------------------------+

256 strings using TTY write with scrolling

+----------------------------------------------------------------+

|                            BIOS         QCRT     % increase    |

|----------------------------------------------------------------|

| 10 Mhz XT with MDA        16.15         6.59         59.2      |

|----------------------------------------------------------------|

| 8 Mhz 286 with CGA         8.84         4.07         53.9      |

|----------------------------------------------------------------|

| PS/2 model 25 with MCGA   10.76         4.56         57.6      |

|----------------------------------------------------------------|

| 12 Mhz 286 with VGA        5.32         1.49         71.9      |

|----------------------------------------------------------------|

| 25 Mhz 386 with SVGA       1.70         1.38         18.8      |

+----------------------------------------------------------------+

If you find QCRT 2.1 a worthwhile program, a $5.00

registration fee would be greatly appreciated!  Sending $10.00 or

more will ensure that you will receive (when available) the next

version of QCRT.  Be sure to include your version number so I can

send you the next upgrade.

If you are registering more than one copy, use the following

guidelines:  $15.00 registers 5 copies.  $25.00 registers 10

copies, and $50.00 registers 30 copies, and more than $50.00 gives

you all the copies you want.  The next version (when available)

will be sent to you.

Revision History:

-----------------

1.0 -- 08/01/88      First attempt.  30% increase over normal

BIOS.  Never released due to major bugs.

1.1 -- 12/20/88      Second attempt. 10% increase over version 1.0

Released on several BBSs for testing.  Still

had a few bugs when working with programs

that used internal cursors or directly

modified 0040:0050.  Didn't work without

ANSI.SYS.

1.2 -- 05/28/89      General release.  Fixed several bugs dealing

with TTY output, cursor positioning and

set-cursor routine.  Did not work correctly

when writing to a text page other than the

active one.

1.3 -- 1.9           Never released.  These versions were used to

test various EGA/VGA functions.  I wanted to

make QCRT compatible with all extended text

modes.

2.0 -- 12/10/91      Newest version.  Support for extended text

modes.  Support for writing data to other

text pages that the current one are now

supported.  Also, a couple of new functions

were added to make sure QCRT is aware of what

the video card is doing.  QCRT version is now

returned as part of the identity check.

2.1 -- 12/21/91      Minor bug fix concerning TTY Bell.  May

cause computer lock-ups or other odd

things to occur.  I never noticed this

because nothing I have sounds the BELL.

QCRT Registration Form

Name:

-----------------------------------------------------------

Company:

--------------------------------------------------------

Address:

--------------------------------------------------------

--------------------------------------------------------

City:                              State:

----------------------------        -----

Zip:

-----------------

Phone:  (       )

------- -------------------

Country:

--------------------------

Electronic

Mail:

--------------------------------------------------------

Where did you get your copy of QCRT from:

-------------------------------------------------------------

-------------------------------------------------------------

QCRT version:

----------

QCRT registration: $5.00 for each copy (up to 4).

$15.00  --  5 copies

$25.00  --  10 copies

$50.00  --  30 copies

$50.00+ --  30+ copies          $

-----------

Checks must be drawn on a US bank, and payable in US funds.

Please make checks payable to:

Glenn K. Smith

RADON Software

809 South Missouri

Weslaco, Texas 78596

-- or --

Glenn K. Smith

RADON Software

1068 East Carolina

Fruita, Colorado 81521

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

Public (software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  PsL cannot debug pro-

programs over the telephone, though we can answer questions.

Disks in the PsL are updated  monthly,  so if you did not get

this disk directly from the PsL, you should be aware that the

files in this set may no longer be the current versions. Also,

if you got this disk from another vendor and are having prob-

lems,  be aware that  some files may have become corrupted or

lost by that vendor. Get a current, working disk from PsL.

For a copy of the latest monthly software library newsletter

and a list of the 4,000+ disks in the library, call or write

Public (software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

1-800-2424-PSL

MC/Visa/AmEx/Discover

Outside of U.S. or in Texas

or for general information,

Call 1-713-524-6394

PsL also has an outstanding

catalog for the Macintosh.