Welcome to tgif, the Two-toned GIF utility!

Tgif is a utility program to display color graphic images on a plasma,

LCD or monochrome graphics display.  The input format is the Compuserve

GIF format for color images.  Tgif unpacks these GIF files and performs

a halftone process on the image so that the various colors of the

palette are displayed using patterns of black and white pixels that

approximate the intensities of each color.  This is the same process

that allows images to be printed in black and white media such as books

and newspapers.  The algorithm used to perform the halftone

transformation is an error distribution process developed by R.  W.

Floyd and L. Steinburg.

Tgif starts by computing the intensity of each palette color by summing

the values of the red, green and blue intensities supplied in the GIF

image.  A threshold intensity level is computed to represent the average

intensity of all colors in the palette.  This threshold value is used to

make the decision as to whether a displayed pixel is black (pixel <=

threshold) or white (pixel > threshold).  Unless the pixel displayed is

exactly black or white, an intensity error is incurred.  This error

represents the difference between the pixel's intensity and what was

actually displayed.  When distributed among the neighboring pixels, this

error influences the threshold decision for those pixels and changes

their display values thus creating a distribution of black and white

pixels that approximates the color intensity.

The above algorithm not only produces excellent visual results, it also

provides a remarkable and unexpected feature when the image is enlarged.

Tgif provides option switches to enlarge the image beyond the size

supplied by the GIF file.  What this actually does is increase the

number of pixels in the drawing.  In the instance of a double size

image, when a single pixel is unpacked from the GIF file tgif creates

four pixels of the same color; a block of four pixels where there was

just one.  The result is the same image but with pixels twice the

original size.  When this "big bit" image is halftoned by tgif, the "big

bits" are resolved into an image that actually has more detail than the

smaller version of the same image.  In other words, the big pictures

look better, have more detail and look just as smooth as the smaller

images.  Images can be enlarged to triple size with the same results.

Only when enlarged to quadruple size did the resulting image start to

get blocky and degraded, although this might be cured by increasing the

fractional part of the error accumulators used in the algorithm.

Tgif was developed to run using Toshiba plasma type display screens with

640 by 400 resolution (T3100, T3200, T5100).  If no command line option

is used, tgif will attempt to set the video mode to the special mode

0x74 used by Toshiba plasma screens.  A command line option forces tgif

to use any CGA type adapter and another option will force inverse video

for CGA compatible LCD displays.  The CGA mode used by tgif is the 640

by 200 two color mode (mode 6).  Since the aspect ratio of the pixels in

this mode is not 1:1 (that is, CGA pixels are taller than they are wide)

it is recommended that the command line option to widen the image be

used.

The usage of tgif is as follows:

Usage:     tgif {/w} {/l} {/d} {/t} {/r} {/c} giffile{.ext}

Options:   /w == widen image

/l == lengthen image

/d == double size

/t == triple size

/r == reverse video (for LCD screens)

/c == force CGA mode

Example:   tgif/d/r picture

Would open picture.GIF and display a double size

image in reverse video.

Notes:

Command line options are stacked onto the program name and

separated by slashes.

If a filename extension is not included, ".GIF" is assumed.

Once invoked, tgif starts unpacking and halftoning the GIF image. If the

image is supplied in interlace mode, tgif uses two passes; one to unpack

and de-interlace the raster lines and a second to halftone the pixels.

For non-interlaced images, one pass performs both operations. As the

color image is halftoned, the black and white raster data is stored into

memory until the entire image has been completed. Tgif provides status

displays about the GIF file as it is being processed.

Once completed, tgif shifts into an image display mode where the user

may view the image.  If the completed image is larger than the selected

display size, the arrow keys may be used move the display window to any

part of the image. The active keys are:

key		usage

up-arrow	move window up

down-arrow	move window down

left-arrow	move window left

right-arrow	move window right

page-up		big move up

page-down	big move down

home		display top of image

end		display bottom of image

escape		exit tgif

The source code module tgif.c has been provided for a number of reasons.

Hopefully it will help others avoid some of the pitfalls encountered

while unpacking and displaying GIF file images.  The included tgif.exe

file was compiled directly from the included tgif.c source module using

MIX software's Power C compiler.  This and many other "c" projects have

convinced me that this compiler is one of the greatest values in the

world of computing.

You will also notice that the source code statements in tgif.c are

uncommented.  Since this is a shareware product, you are requested to

support this effort with a $10 shareware fee.  Fully commented listings

as well as future enhancements will be available to registered users of

tgif.  One enhancement that is foreseeable is the conversion of certain

procedures into assembly language to increase the speed of tgif's

processing.

The reporting of any bugs, anomalies or GIF files that tgif cannot

display would also be appreciated.

To all novice or beginning users, tgif is offered for your entertainment

and possibly educational uses.  To all of you experts I offer a friendly

challenge: now that you have the source code, make tgif better in some

way and let me know how you did it.

This code was written and will be supported by:

James Lark

6101 W. Courtyard Dr.

Austin, TX. 78730