HP LaserJet Tools

Rip Toren

POB 674

Columbia MD 21045

This is a collection of programs that have developed over time to assist

in working with the HP LaserJet printer.  My HPLJ has no font cartridges,

and I quickly learened it could not do the PC line drawing symbols.

HPFILTER

This is a TSR (terminate and stay resident) program that I use to produce

line drawing characters on my printer that does not have any FONT

cartridges.  It is tied into the interrupt to the printer (17).  It looks

for ASCII codes 179 through 223.  When these are found, a command string

is sent to the HPLJ to produce those line grawing characters.  It will

also BOLD (fatten) characters between an ESC 'G' and ESC 'H'. ESC 'G will

start it, while ESC'H' or ESC'E' will terminate it. It will definitely

screw up any output to anything other that the HPLJ if these characters

are sent.  To turn off the filtering, simply run it a second time.

>HPFILTER

HPXFER

While trying to develop the strings that HPFILTER would use, it became

apparent that it was a pain to modify and rec assemble.  This drove the

development of HPXFER.

This program will read in a standard Ascii file and send part of it to

the printer. The file name is the only argument. If it is not found, the

extension '.HPX' will be appended, and the open tried again. The data

found on each line is sent until a tilde ('~') is located. Every thing

following the tilde is treated as comment.  If the end of the line is

found without finding a tilde, the CRLF is also sent to the printer.

If a line is read that starts with '@' in character 1, the rest is

assumed to be a file name. That file is opened in BINARY mode, and its

entire content is sent to the printer.  This is usually a bit graphic

image.

>HPXFER LOGOSET.HPX

** WARNING **   It is generally a bad idea to send binary data to the

printer while HPFILTER is active. It will garble (transl-

ate) any binary values that fall within its range.

Probably with very poor results.

CPAINT

Now that there is a way to send single command lines, and binary files,

all that is needed is a way to obtain those binary files. CPAINT (See

PAINT) is one way to do that. CPAINT will read a PC Paintbrush

(Zsoft/Microsoft) file (PCX or PCC) and produce up to four data files.

Each data file corresponds to one level or plane in the image.  They are

usually RED, GREEN, BLUE, and INTENSITY.  Each output file starts with

the HPLJ code to begin graphics ('ESC *r1A'), with the current cursor

location as the left margin. The file ends with an end of graphic code

('ESC *rB').  Each graphic line is also setup with the proper HPLJ code

for the number of bytes of binary data.

The program does one special thing. As it is scanning the input, it does

not transfer any data to the output files until a scan line has been

located that contain a set bit.  This means that your resulting file will

not contain white space above the image.  You position the cursor prior

to printing for exact registration.

>CPAINT RTLOGO.PCX YNNN

Since the amount of information is rather massive that can be produced, a

second argument is checked for flags. This argument will allow you to

suppress the production of any of the planes.  The output files are named

"PLANEx.DAT" where 'x' is a number 1 through 8. If you wish to produce

only planes 2 & 3, the second argument would by "NYYN".  This says '(N)o'

to planes 1 & 4, and '(Y)es' to planes 2 & 3.  When using an EGA adaptor,

you currently get only 4 planes, so you needn't worry about the last

four. Only enough data file are use to cover the number of planes in the

file.

If you draw your image in Black lines on a White background, you will

find that all four planes conatin the same pattern.  If you use patterns,

are different colors, you may find data in some planes, and not in

others.

READY - SET - GO

Draw your image using PC Paintbrush. Crop, cut and save only the part you

want to appear on the page. Run CPAINT to extract the planes you want to

print. Build the rest of the commands that you want for positioning and

line drawing, as well as any included image files into a an Ascii file.

Run HPXFER to send that file to your HPLJ. Ta-Da. If the lines look OK,

but the image is real trash, check to insure that HPFILTER is de-

activated or not installed at all.

NOTES

HPFILTER was originally designed to work with WordPerfect 4.1. I have

been using it now for 3 months with no problems. I have my HPLJ attached

to LPT1.  I had to create my own character definition using the "PRINTER"

program supplied with WP 4.1. I copied one of the character sets into my

own name, the modified it to send the simpla Ascii for characters 179

through 223. I also was able to obtain "BOLD" ouput by doing it on a

second pass, and having the start string offset the printer cursor up and

over one dot. At the end of the pass, the cursor is offset back and down

one dot.

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

The Public (Software) Library

Disks in the library are updated monthly.

For a copy of the latest monthly software library newsletter

and a list of the 800+ disks in the library, call or write

The Public (Software) Library

P.O.Box 35705 K

Houston, TX 77235-5705

(713) 721-6104

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.