______________

/              \

/                \

|    o         o   |

|                  |

|         ^        |

|   *           *  |

|     \_______/    |

\                /

\______________/

DMPLAS

Written by

Martin L. Waldman

A program to convert

Dot Matrix Printer Controls

to Laser Printer Control Sequences

Version 1.1

Copyright 1991

All rights reserved

TABLE OF CONTENTS

Page

PART 1. OVERVIEW

1.1  Introduction ..................... 1

1.2  Installing DMPLAS ................ 1

1.3  Page Size and Printer Modes ...... 2

1.4  Laser Printer Controls ........... 2

1.5  Printer Reset..................... 3

1.6  Printing a File .................. 3

1.7  Testing DMPLAS ................... 4

PART 2. CONVERTED PRINTER CONTROLS

2.1  Pitch Control .................... 5

2.2  Print Style ...................... 5

2.3  Special Effects .................. 6

2.4  Line Spacing ..................... 6

2.5  Vertical Movement ................ 6

2.6  Margin Control ................... 7

2.7  Horizontal Movement .............. 7

2.8  Dot Graphics ..................... 7

PART 3. PRINTER CONTROL CODE TABLE           8

PART 4. SOME HINTS AND TIPS

4.1  Screen Dumps .................... 11

4.2  Line drawing characters ......... 11

4.3  Using DMPLAS with a spooler ..... 12

4.4  Using DMPLAS on a LAN ........... 12

4.5  Using DMPLAS with Windows 3 ..... 12

DMPLAS  Version 1.1                          1

======================================================================

PART 1. OVERVIEW

1.1  INTRODUCTION

Some of your software may have the capability of being installed for a

laser printer -- but what about the other stuff ?  The programs that

you've used so often that your fingers do the walking by themselves --

especially your favorite graphics programs designed for a dot matrix

printer ?

DON'T  THROW  THEM  AWAY  --  Smile  --  now you have DMPLAS !

Now you can make your laser printer hum along using software designed

for a dot matrix printer -- just install DMPLAS and activate it when

you need it.

DMPLAS is a program that converts dot matrix printer control

characters (00h to 1Fh) and printer control escape sequences (1Bh +

character string) to the equivalent printer control commands used by

HP LaserJet compatible printers.

The program is written in 8088/8086 assembly language and will run on

any computer system that operates using DOS 2.x or higher.

Part 2 of this manual lists the dot matrix printer controls, by

functional group, that are converted to equivalent laser printer

controls by DMPLAS.

Part 3 contains a complete list, in ASCII value order, of the control

codes and escape sequences that DMPLAS recognizes and evaluates. Those

that have no equivalents or have conflicting meanings are trapped so

that your documents don't get messed up and your laser printer doesn't

start to spit paper.

1.2  INSTALLING DMPLAS

DMPLAS can be used in two different ways -- as a permanently installed

TSR (less than 6K) that can be activated and deactivated, and to print

files containing printer control data and text (see paragraph 1.6).

To install DMPLAS as a TSR, the command line to be placed in your

AUTOEXEC.BAT file must contain the drive and pathname where DMPLAS is

stored plus one of the following:

"DMPLAS /A" -- installs and/or activates DMPLAS.

"DMPLAS /D" -- installs and/or deactivates DMPLAS.

The first time either command line is executed, DMPLAS will be

installed as a TSR and remains in memory until the system is rebooted.

Subsequent entry of a command line will only turn DMPLAS on or off --

it will not be removed from memory and not be installed a second time.

Using /A switch with additional parameters (see 1.3 below) changes the

parameters in the installed program. The /D switch only deactivates.

2                       DMPLAS  Version 1.1

======================================================================

1.3  PAGE SIZE AND PRINTER MODES

Laser printers print on only 10" (e.g. 60 lines at 6 lpi) of an 11"

letter size page. If your software expects to use all 11" (66 lines),

enter "DMPLAS /AF" (full) as an intitial or subsequent command line.

DMPLAS then adjusts the space between lines (leading) to accomadate

the extra lines -- the adjustment works with whatever line spacing you

have programmed (i.e. 8 lpi yields 88 lines, etc).

To return to a normal (10") page, enter the command  "DMPLAS /AN".

The page size adjustment is not effective until DMPLAS receives a line

spacing or printer reset command (ESC '@') --  see para. 1.5

Many dot matrix printers have a set of mechanical (DIP) switches or

setup panel buttons that set power-on defaults, some of which can be

changed by software commands -- see your printer manual.

Often one of the switches sets the printer to either the so-called

Standard (Epson) mode or the so-called IBM (Proprinter) mode and

cannot be changed by software. The exact way in which some of the

print controls from these printers work is different depending on how

the mode switch has been set -- again see your printer manual.

DMPLAS defaults to the Standard mode. Where no conflict exists between

the two modes, DMPLAS ignores the mode. In Parts 2 and 3 those

commands that depend on the mode are flagged.

You can set the IBM mode by an additional parameter to the /A switch

either at the initial command in your AUTOEXEC.BAT file or when

issuing subsequent activation command lines. The formats are:

"DMPLAS /ANI"   sets IBM mode and normal page (10")

"DMPLAS /AFI"   sets IBM mode and full page (11")

(The page size parameter must be included for the mode to be set)

On some printers setting the mode to IBM causes the printer to accept

FS (1Ch) as a substitute for ESC (1Bh) in some control sequences --

the remainder of the sequence is the same. DMPLAS therefore also

accepts FS as being the same as ESC without further ado.

1.4  LASER PRINTER CONTROLS

DMPLAS can send unaltered control sequences directly to the laser

printer. To do so send the leading ESC character (1Bh) twice followed

by the rest of the sequence.                           ~~~~~

For example, to set the end-of-line wrap around function you would

normally send ESC + the characters "&s0C" to the laser printer. If

DMPLAS is actvated you can send ESC + ESC + "&s0C" to achieve the same

result. See DLTEST-4 for other examples.

DMPLAS  Version 1.1                          3

======================================================================

1.5  PRINTER RESET

ESC  '@' (Reset printer) is used by most dot matrix printers to reset

the printer to the power-on status defined by the settings of

mechanical (DIP) switches.

Some word processors take advantage of this by sending ESC '@' and

other condition resets at the beginning of a document to establish a

known base from which to proceed with formatting commands.

To emulate ESC '@', DMPLAS sets the following default parameters:

Pitch                  Pica  (10 cpi)

Line Spacing           1/6"  (6 lpi)

Auto Linefeed          off

Left/Right margins     cleared

Superscript/Subscript  off

Horizontal Tabs        set to every 8 columns

and a carriage return (0Dh) is sent to the printer.

The mode (STD or IBM) and page factor (10" or 11") are NOT changed.

To avoid the ejection of an empty sheet of paper from the laser

printer, DMPLAS does NOT send the laser printer soft reset (ESC 'E').

You may eject the current page by entering the command "DMPLAS /F".

This will send a formfeed (0Ch) to the printer whether or not DMPLAS

is active.

1.6  PRINTING A FILE

Some programs can create ASCII files that contain both the text to be

printed and the printer control data to format the printout. Such

files can be printed on your laser printer even if DMPLAS has not been

installed or activated.

Enter the folllowing command (entries inside brackets are optional):

"[drive:][\pathname\]DMPLAS /P [drive:][\pathname\]filename.ext"

You may also use both additional parameters with the /P command to set

the page size and mode. Note however that if DMPLAS has been installed

and activated, the TSR page size and mode takes precedence.

Before printing the file a printer reset is performed and the laser

end-of-line wrap and perforation skip functions are activated. The

laser printer soft reset (ESC 'E') is sent at the end.

NOTE: DMPLAS does NOT check printer status, neither as a TSR nor as a

file printer, but does return a normal status byte. You should make

sure your printer is on line and ready.

4                       DMPLAS  Version 1.1

======================================================================

1.7  TESTING DMPLAS

So that you can see if DMPLAS works well with your computer and your

printers, we have included a series of files on the distribution

diskette that you can use to check it out.

DLTEST-W.WS4 is a document written in Wordstar Version 4 to be printed

on a dot matrix printer. DLTEST-W.PRN is the result of redirecting the

printer output to a file -- it contains both formatting commands and

document text.

The other test files are named "DLTEST-n" (n = a number or letter).

There are two files each with the same name. DLTEST-n.BAS is the Basic

program that was used to create DLTEST-n.PRN -- the file that contains

both the printer controls and the text.

Before you install DMPLAS you may want to try the following:

1. Put your dot matrix printer on line, and at the DOS prompt enter

~~~~~~~~~~                  "COPY DLTEST-n.PRN PRN"

2. Now put your laser printer on line, and at the DOS prompt enter

~~~~~                  "DMPLAS /P DLTEST-n.PRN"

If you have installed and activated the TSR, you can print to your

laser printer with the command   "COPY DLTEST-n.PRN PRN"

The test files contain examples of the more commonly used controls.

All use the normal page length and standard mode.

DLTEST-1     Print Pitch, Style, and Special Effects

DLTEST-2     The Master Print Mode

DLTEST-3     Line Spacing

DLTEST-4     Left and Right Margins

DLTEST-5     Vertical and Horizontal Movement

DLTEST-6     Horizontal Tab Commands and Backspace

DLTEST-F     Dot Graphics - grey scale patterns

DLTEST-G     Dot Graphics - cross hatch pattern

DMPLAS  Version 1.1                          5

======================================================================

PART 2. CONVERTED PRINTER CONTROLS

The following is a list of the dot matrix printer controls, by

functional group, that are converted to equivalent laser printer

controls by DMPLAS. The controls that depend on mode are indicated by

<S> or <I>. All others are available in both modes.

The test files listed demonstrate some of the controls available in

standard mode.

2.1  PITCH CONTROL

DMPLAS pitch controls attempt to use Courier fonts -- Pica translates

to 10 cpi/12 pts, Elite to 12cpi/10pts, and Condensed to 16cpi/8pts.

If you are not happy with the appearance provided by your current

laser printer fonts see FONT.DOC (in DMPLASFN.ZIP) for alternatives.

The standard print pitch controls are demonstrated in DLTEST-1

SO              Expanded print - one line

SI              Condensed print

DC2         <S> Cancel condensed

DC2         <I> Set 10 cpi (Pica)

DC4             Cancel one line expanded

ESC  SO         Expanded print - one line

ESC  SI         Condensed print

ESC  DC2        Cancel condensed

ESC  ':'        Set 12 cpi (Elite)

ESC  'M'        Set pitch to Elite (12 cpi)

ESC  'P'    <S> Set pitch to Pica (10 cpi)

ESC  'W' 1      Expanded print on

ESC  'W' 0      Expanded print off

ESC  'g'        Set 15 cpi

2.2  PRINT STYLE

The standard print style commands are demonstrated in DLTEST-1

ESC  '%G'   <I> Begin Italics

ESC  '%H'   <I> End Italics

ESC  '4'    <S> Select italic characters

ESC  '5'        Cancel italic characters

ESC  'E'        Emphasized printing on

ESC  'F'        Emphasized printing off

ESC  'G'        Boldface on

ESC  'H'        Boldface off

ESC  'P' 1  <I> Select proportional printing

ESC  'P' 0  <I> Cancel proportional printing

ESC  'p' 1      Select proportional printing

ESC  'p' 0      Cancel proportional printing

6                       DMPLAS  Version 1.1

======================================================================

2.3  SPECIAL EFFECTS

These special effect commands are demonstrated in DLTEST-1

ESC  '-' 1      Underline on

ESC  '-' 0      Underline off

ESC  'S' 0      Set superscript on

ESC  'S' 1      Set subscript on

ESC  'T'        Set superscript/subscript off

ESC  '!' n  <S> Set Master Print Mode n   command is explained and

demonstrated in DLTEST-2

Not demonstrated

ESC  'I' n  <I> Set Master print mode n

2.4  LINE SPACING

The standard line spacing commands are demonstrated in DLTEST-3

ESC  '%8' n   <I> Begin n/360 line spacing

ESC  '+' n    <I> Begin n/360 line spacing

ESC  '0'          Set line spacing to 1/8"

ESC  '1'          Set line spacing to 7/72"

ESC  '2'      <S> Set line spacing to 1/6"

ESC  '2'      <I> Execute  ESC 'A' n

ESC  '3' n        Set line spacing to n/216"

ESC  'A' n        Set line spacing to n/72"

ESC  'A' n    <S> Set line spacing n/72" immediate

ESC  'A' n    <I> Set line spacing n/72" for ESC '2'

Not demonstrated

ESC  '[' n    <S> Begin n/360 line spacing

2.5   VERTICAL MOVEMENT

These vertical movement commands are demonstrated in DLTEST-5

LF                Line feed

FF                Form feed

ESC  LF           Reverse Linefeed

ESC  FF           Reverse Formfeed

ESC  'J' n        Onetime linefeed of n/216"

ESC  'f' 1 n      Set print position to current line + n

Not demonstrated

ESC  '%4' n   <I> Onetime n/360 line feed

ESC  '5' 1        Auto Linefeed on

ESC  '5' 0        Auto Linefeed off

ESC  'j' n    <S> Send one reverse LF n/216"

DMPLAS  Version 1.1                          7

======================================================================

2.6   MARGIN CONTROL

These margin control commands are demonstrated in DLTEST-4

ESC  'l' n       Set left margin to column n

ESC  'Q' n   <S> Set right margin  col n

ESC  'X' l r     Set left & right margins

This commands is demonstrated in DLTEST-3

ESC  'N' n       Set bottom margin n lines

Not demonstrated

ESC  'O'         Cancel top & bottom margins

2.7  HORIZONTAL MOVEMENT

These horizontal commands are also demonstrated in DLTEST-5

ESC  '$' n1 n2       Absolute horizontal position

ESC  '\' n1 n2   <S> Relative horizontal position

ESC  'f' 0 n         Set print position to col n

These are demonstrated in DLTEST-6 (new in DMPLAS 1.1)

BS                   Backspace

HT                   Horizontal tab

ESC  'D' n1 n2 ...0  Set variable horizontal tabs at n1,n2,etc.

ESC  'e' 0 n         Set horizontal tabs at every n columns

Not demonstrated

ESC  'd' n1 n2   <I> Relative horizontal position

2.8  DOT GRAPHICS

Since dot matrix and laser printers differ in the way 8 bit dot

graphics (raster graphics) are printed, some adjustments are made by

DMPLAS. Horizontal spacing is maintained by adding dots to make it

equivalent. Vertical spacing of 8/72" per line is maintained by

duplicating dot rows on the laser printer.

These commands are demonstrated in DLTEST-F and DLTEST-G

ESC  'K' n1 n2 m1 m2 ...   Normal density (60 dpi)

ESC  'L' n1 n2 m1 m2 ...   Double density (120 dpi)

ESC  'Y' n1 n2 m1 m2 ...   Double density/double speed (120 dpi)

ESC  'Z' n1 n2 m1 m2 ...   Quadruple density (240 dpi)

ESC  '*' n n1 n2 m1 m2 ...   Select graphics mode n where

n = 0 = Normal density (60 dpi)

1 = Double density (120 dpi)

2 = Double density/double speed (120 dpi)

3 = Quadruple density (240 dpi)

4 = Semi-double density (80 dpi)

5 = Plotter graphics (72 dpi)

6 = CRT graphics (90 dpi)

7 = Double density Plotter (144 dpi)

8                       DMPLAS  Version 1.1

======================================================================

PART 3. PRINTER CONTROL CODE TABLE

The following table lists, in ASCII value order, all of the control

codes and escape sequences that are recognized by DMPLAS.

The symbols in the Flag column indicates what action is taken when

translating the code;  * = trapped or > = translated to equivalent

laser sequence. Actions dependent on the mode are prefixed with

S (Std) or I (IBM).

Code/Sequence     Flag  Printer Function Performed

~~~~~~~~~~~~~     ~~~~  ~~~~~~~~~~~~~~~~~~~~~~~~~~

BEL                 *   Sound printer bell

BS                  >   Backspace

HT                  >   Horizontal tab

LF                  >   Line feed

VT                  *   Vertical tab

FF                  >   Form feed

CR                  >   Carriage return

SO                  >   Expanded print - one line

SI                  >   Condensed print on

DC1                 *   Set printer on line

DC2                S>   Cancel condensed

DC2                I>   Set 10 cpi (Pica)

DC3                 *   Set printer off line

DC4                 >   Cancel one line expanded

CAN                 *   Cancel a line (clear buffer)

DEL                 *   Delete last character sent

ESC  LF             >   Reverse Linefeed

ESC  FF             >   Reverse Formfeed

ESC  SO             >   Expanded print - one line

ESC  SI             >   Condensed print on

ESC  DC2            >   Cancel condensed

ESC  EM  n          *   Sheet feeder commands

ESC  EM  'T' n      *   Set print start on sheet feeder

ESC  ESC            >   DMPLAS bypass - sends ESC once

ESC  US n           *   Double/Triple height printing

ESC  ' ' n          *   Add n dots between characters

ESC  '!' n         S>   Set master print mode

ESC  '!' n         I*   Select international char set

ESC  '#'            *   Accept bit 8 as is

ESC  '$' n1 n2      >   Absolute horizontal position

ESC  '%' n         S*   Download character set control

ESC  '%4' n        I>   Onetime n/360 line feed

ESC  '%8' n        I>   Begin n/360 line spacing

ESC  '%G'          I>   Begin Italics

ESC  '%H'          I>   End Italics

ESC  '&' 0 ...      *   Define download characters

ESC  '(' n          *   Select NLQ/HSD print quality

ESC  '*' n ...      >   Select 8 bit graphics mode n

ESC  '+' n1 .. .   S*   Macro instruction commands

ESC  '+' n         I>   Begin n/360 line spacing

ESC  '-' 1          >   Underline on

ESC  '-' 0          >   Underline off

ESC  '/' n          *   Select vertical tab channel n

DMPLAS  Version 1.1                          9

======================================================================

PART 3. PRINTER CONTROL CODES (continued)

Code/Sequence     Flag  Printer Function Performed

~~~~~~~~~~~~~     ~~~~  ~~~~~~~~~~~~~~~~~~~~~~~~~~

ESC  '0'            >   Set line spacing to 1/8"

ESC  '1'            >   Set line spacing to 7/72"

ESC  '2'           S>   Set 6 lpi

ESC  '2'           I>   Execute  ESC 'A' n

ESC  '3' n          >   Set line spacing to n/216"

ESC  '4'           S>   Select italic characters

ESC  '4'           I*   Set TOF here (IBM mode)

ESC  '5'            >   Cancel italic characters

ESC  '5' 1          >   Auto Linefeed on

ESC  '5' 0          >   Auto Linefeed off

ESC  '6'            *   Select IBM character set #2

ESC  '6'            *   128-159,255 = Italic Internat

ESC  '7'            *   Select IBM character set #1

ESC  '7'            *   128-159,255 = contol codes 0-32,127

ESC  '8'            *   Disable paper out detector

ESC  '9'            *   Enable paper out detector

ESC  ':'            >   Set 12 cpi (Elite)

ESC  ':' 0 n 0      *   Copy ROM font n into RAM

ESC  '<'            *   Set uni-directional print

ESC  '='            *   Set bit 8 to zero

ESC  '='n n 20 /1   *   Define download characters

ESC  '>'            *   Set bit 8 to one

ESC  '?' n0 n1      *   Redefine graphic mode

ESC  '@'            >   Reset printer

ESC  'A' n         S>   Set line spacing n/72" immediate

ESC  'A' n         I>   Set line spacing n/72" for ESC '2'

ESC  'B' n1 .. .0   *   Set vertical tab positions

ESC  'C' 0 n        *   Set page length n inches

ESC  'C' n          *   Set page length n lines

ESC  'D' n1 .. .0   >   set variable horizontal tab positions

ESC  'E'            >   Emphasized printing on (Bold)

ESC  'F'            >   Emphasized printing off (Bold)

ESC  'G'            >   Boldface Double strike on

ESC  'H'            >   Boldface Double strike off

ESC  'I' n         S*   Character set selection

ESC  'I' n         I>   Set Master print mode

ESC  'J' n          >   Onetime linefeed of n/216"

ESC  'K' n1 .. .    >   Normal density graphics 60 dpi

ESC  'L' n1 .. .    >   Double density graphics 120 dpi

ESC  'M'            >   Set 12 cpi (Elite)

ESC  'N' n          >   Set bot marg n lines = skip perf

ESC  'O'            >   Cancel top & bottom margins

ESC  'P'           S>   Set 10 cpi (Pica)

ESC  'P' 1/0       I>   Set Proportional on/off

ESC  'Q' n         S>   Set right margin  col n

ESC  'Q' 3,35, 36  I*   Deselect printer

ESC  'R'            *   Reset horiz and vert tabs

ESC  'R' n          *   Select character set n

ESC  'S' n          >   Set superscript/subscript on

ESC  'T'            >   Set superscript/subscript off

10                       DMPLAS  Version 1.1

======================================================================

PART 3. PRINTER CONTROL CODES (continued)

Code/Sequence     Flag  Printer Function Performed

~~~~~~~~~~~~~     ~~~~  ~~~~~~~~~~~~~~~~~~~~~~~~~~

ESC  'U' n          *   Unidirectional print control

ESC  'V' n dat a    *   Repeat data n times (LQ-1500)

ESC  'V' NUL        *   Cancel repeat data (LQ-1500)

ESC  'W' 1/0        >   Expanded print on/off

ESC  'X'            *   Cancel all WP modes except propor.

ESC  'X' l r        >   Set left & right margins

ESC  'Y' n1 .. .    >   Dble dens/dble speed graphics

ESC  'Z' n1 .. .    >   Quadruple density graphics

ESC  '[' n         S>   Begin n/360 line spacing

ESC  '[' n ...     I*   Print double high/wide

ESC  '\' n1 n2     S>   Relative horizontal position

ESC  '\' n1 n2     I*   Print all codes as characters

ESC  ']' n          >   Onetime linefeed of n/360"

ESC  '^' 0/1 . ..   *   60/120 dpi graphics

ESC  '^' n ...      *   9 pin graphics

ESC  '^' n          *   Print char(n) from all char chart

ESC  '_' 1/0        *   Overlining on/off

ESC  'a' n          *   Align or center printing

ESC  'b' n ... 0    *   Set vertical tabs in channel

ESC  'c' n          *   Set top margin n lines

ESC  'd' n1 n2      >   Relative horizontal position

ESC  'e' 0/1 n      *   Set H/V tabs every n chars

ESC  'e' 0 n        >   Set horizontal tabs every n columns

ESC  'e' 1 n        *   Set verical tabs every n lines

ESC  'f' 0 n        >   Set print position to col n

ESC  'f' 1 n        >   Printer to current line + n

ESC  'g'            >   Set pitch to 15 cpi (Micron)

ESC  'h' n          *   Enlarged characters on/off

ESC  'i' n          *   Set immediate printing on/off

ESC  'j' n         S>   Send one reverse LF n/216" (STD)

ESC  'j'           I*   Deselect printer

ESC  'k' n          *   Select font/typeface n

ESC  'l' n          >   Set left margin to column n

ESC  'm'            *   Select Triple width printing

ESC  'm' 1/0        *   Select/Cancel HX-20 graphic symbols

ESC  'p' 1          >   Select proportional printing

ESC  'p' 0          >   Cancel proportional printing

ESC  'q' n          *   Select character style n

ESC  'r' n          *   Select color to print

ESC  'r' n          *   Set top margin n lines

ESC  's' n          *   Set printing speed

ESC  't' n          *   Select graphic/italic chars set

ESC  'v' n          *   Custom character set n

ESC  'w' 0/1        *   Double high off/on

ESC  'x' 1/0        *   Set Near Letter Quality on/off

ESC  'y' n          *   Select quasi 8 bit graphics

ESC  '~' 0/1        *   Print normal/slashed zero

DMPLAS  Version 1.1                          11

======================================================================

PART 4. SOME HINTS AND TIPS

4.1  SCREEN DUMPS

Text screen prints (from pressing the Shift-PrtSc key) on some dot

matrix printers as well as laser printers have special characters

printed instead of true lines as shown on the screen. See 4.1 for a

solution to this problem.

The program GRAPHICS.COM included with DOS before version 5 worked

only with CGA screens and IBM/Epson dot matrix printers. DMPLAS /A

allows these screen dumps to be printed on HP LaserJet compatibles.

Unfortunately GRAPHICS.COM will still only work properly with CGA

graphic screens. There are other programs that handle EGA, VGA, and

Hercules graphics and DOS 5's GRAPHICS.COM accomidates these as well

as HP LaserJets.

For DMPLAS to be properly effective it should be loaded (DMPLAS /A)

before GRAPHICS.COM.

~~~~~~

4.2  IBM LINE DRAWING CHARACTERS

IBM Graphics and IBM Proprinters are capable of printing the IBM line

drawing characters. The Epson FX is effectively compatible with the

IBM Graphics printers except that italic characters are printed

instead. This is why text screen dumps have "e"s and other characters

instead of lines.

There are 2 solutions to this problem.

1) DOWNLOAD CO120RPN.SFP from the DMPLASFN font set pack.

2) With a HP LaserJet series II the following will change the

character set of the native font from Roman 8 (with italic characters)

to IBM-US (with line drawing characters).

Hold down the menu key until the display reads "SYM SET = Roman 8*"

Press the + key until the display reads "SYM SET = IBM-US"

Press the Enter/Reset Menu key to make the IBM-US the new default set.

(The display marks this with the asterisk from Roman 8)

Hold down the Continue/Reset key until the display reads "07 RESET"

You only need do this once. After this the LaserJet will power-on with

the IBM-US symbol set active.

12                       DMPLAS  Version 1.1

======================================================================

4.3  USING DMPLAS WITH A SPOOLER

Since a spooler can effectively hold up the physical printing of the

output until the laser printer can catch up, we recommend that you

load DMPLAS as a TSR after you load your spooler software into memory.

~~~~~

If your spooler works as a device driver (in CONFIG.SYS) there DMPLAS

should work fine also.

4.4  USING DMPLAS ON A LAN

Due to the way DMPLAS achieves activation and de-activation, DMPLAS

must be loaded on remote terminals (where the programs are actually

run) rather than on the file server (where the printer is attached).

DMPLAS will then translate the output to the HP LaserJet standard

before the LAN transfers the output over the wires to the printer.

4.5  USING DMPLAS WITH WINDOWS 3

DMPLAS works fine when used with DOS programs being run on the DOS

shell under Windows. DMPLAS can either be loaded before WIN is

executed or from the DOS shell after the DOS icon has been clicked.

DMPLAS can also be included in a batch file executed by a .PIF

routine. In this case there is no need to deactivate DMPLAS after the

program has executed.

Please remember that even in a multi-tasking situation, the activation

and deactivation of DMPLAS via   DMPLAS /A   and   DMPLAS /D    takes

place immediately. If there is print left in a spooler or being

outputted by another program running in the background the changes to

the status of DMPLAS might affect the final output to the printer.

Before issuing a DMPLAS command the system should be "quiet".

################################################