HPLOT.DOC

HPLOT V1.1 is a program which allows direct communication with

Hewlett Packard plotters, or with the many plotters which emulate

HP's, from the keyboard of an IBM-compatible computer.  Plotter

commands are typed in and carried out immediately.  If data is

requested from the plotter, it is also shown onscreen.

HPLOT is for anyone who wants or needs to understand HP-GL, the

graphics command set understood by HP plotters.  This includes

people who write plotter drivers, people who want to understand

how a plotter works, or curious people who find themselves

associated with a plotter.  However, a word of warning.  Even if

you do not think that I have just named YOU, when you find out

how easy it is to order your plotter around, you may change your

mind about what you want your drawing robot to do.

NOTES

Command terminator: HPLOT adds the standard terminator ';' to

most commands if you do not supply it.  Therefore, typing

the command terminator is optional.

Label Terminator: HPLOT automatically adds ^C to strings appended

to the 'LB' (write label) and 'BL' (buffer label) commands.

You cannot type this from the keyboard because ^C (ctrl C,

CHR(3)) is the MS-DOS "break" command.  Therefore, do not

use 'DT' (Define label Terminator) to redefine this

terminator, or you will inactivate commands which require a

label.

Serial Port: HPLOT allows you to interactively select the serial

port your plotter is on, and sets that port to 9600 baud,

even parity, a 7 bit data packet, and 2 stopbits.  This is

what my plotter likes.  But you can use the following table

with DEBUG, the Norton NU program, or another disk

modification program to change HPLOT for your own situation.

Address

(Hex)    Comment

BAUD         CS:2DEC   80 25, 9600D = 2580H, "back-words"

PARITY       CS:2DEE   'E'= even, 'O'= odd, 'N'= none

WORDLENGTH   CS:2DEF   07 or 08

STOPBITS     CS:2DF0   02 or 01

These addresses are as loaded, or as given by DEBUG.  If

your disk patching program counts bytes from the file

beginning, subtract 0100H from the offsets given.

Exit:  You can exit from HPLOT with the command 'SP 0'<cr>, which

caps the pens.  If you just want to think, not exit from

HPLOT, 'SP'<cr> or 'SP0'<cr> will cap pens without leaving

the program.

Plotters: HPLOT should work with HP7220C,S,T / HP7225A,B /

HP7470A / HP7475A / HP7550A, HP7580A,B / HP7585A,B / HP7586B

plotters.  However, it has not been tested in most of these

situations.  Please send me a note if you have problems.

Timing: HPLOT uses software handshaking to receive messages from

the plotter.  This is a matter of timing, to some extent.

Timings have been chosen to work with most computers and

plotters.  But if yours doesn't, send me a note.

Hangups: HPLOT strains out commands beginning with 'O' (Output)

which are not legal output commands.  But it must accept

commands which are legal HP-GL, but not implemented on a

particular model of plotter.  In this case, HPLOT can sit

indefinitely, listening for a plotter answer which will

never come.  The symptoms are an 'O' command in the enter

box, and apparent stasis.  In this case, just hit any key

and the program will give up on an answer from the plotter,

and continue as usual.

Here are some commands to experiment with, until you can get the

full story from your manual:

IN - initialize all parameters

PD - pen down ; PD - pen up

PR x,y... - plot relative at x,y

PA x,y... - plot absolute at x,y

LB abcd.. - write ASCII label

DI run, rise - direction for label

CI x - circle of radius x

OA - output pen position and status, 0/1=up/down

OI - tell me who you are

OP - output P1 and P2

HPLOT allows you to send commands which will result in bizarre

actions, or no actions.  A common example of the latter is moving

the pen outside the limits of the plotting area.

Check the manual.  And keep on plottin'.

January, 1987                                Mike Yarus

2231 16th Street

Boulder, CO 80302

Compuserve 73145,513