OverLay v3.0  (c)1988-91 Advance Systems Norwood, PA

OverLay Macro Utility for HP-LaserJet Compatible Laser Printers

OverLay converts HP-LaserJet PRINT files to a HP-LaserJet Macro files. Macros

are great for applications such as letterheads, forms, graphics, logos and

text.  OverLay will ONLY convert "PRINT" files to Macros.  A PRINT file being

one that when sent to the printer with COPY /B or the PRINT Command, the file

(text or graphics) will be output from the printer to paper.  An ASCII file

is a PRINT file.

OverLay produces Overlay Macros with the .OVL file extension by default.  An

Overlay Macro places the print image on every page of output until the

printer is reset.

Starting OverLay with the /M switch turns OFF the Overlay function and

produces a Macro file with the .MCR file extension.  A Macro must be Called

or Executed every time the image is printed.

To send output directly to the printer and suppress the creation of an output

file, start OverLay using the switch: /1 for LPT1 or /2 for LPT2.

To Use OVERLAY.EXE

1. CREATE A PRINT FILE

Use your favorite Word Processor or desk top publisher to produce the image

(text or Graphic or both) you want for an Overlay.  Most programs allow you

to send the output to a file instead of the printer it is this file that will

be converted to an HP-LJ Macro.  If your program does not support PRINT files

then try using a program that will redirect printer output to a file. You can

find one on most electronic bulletin boards.

In general, an Overlay Macro will continue to print the image on every page

until the reset command [ESC E] is sent to the printer or the page length,

page size, paper tray select or orientation command is issued.  SoftFont

management commands are not allowed in a macro, that is, fonts may not be

downloaded, deleted or made permanent in a macro.  Downloaded the fonts

separately.  If one of the above events occurs, the document contents will

print first, followed by the Overlay image on the next page.  Avoid these

commands in Macros.

2. CONVERT A PRINT FILE TO AN OVERLAY FILE

Run OVERLAY.EXE. You will be prompted for a source file name and ID number.

The file name will be the PRINT file created in step 1.  The ID number is any

number from 0 to 32767.  OverLay will create a Macro file with the same base

name as the source file and add the .OVL extension or .MCR if the /M switch

was used at start up.  If the /1 or /2 switch was used the output is directed

to parallel print port 1 or 2.

OR, name the file and ID# in the start up command.

C>OVERLAY MYLOGO.PRN,1         Creates MYLOGO.OVL with an ID of 1

C>OVERLAY MYLOGO.PRN,1 /1      Sends output to LPT1: with an ID of 1

C>OVERLAY MYLOGO.PRN,1 /M      Creates MYLOGO.MCR with an ID of 1

3. TO USE AN OVERLAY FILE

COPY /B the Overlay Macro file to the printer.

EXAMPLE:   C>COPY /B  MYLOGO.OVL  PRN

Every page printed from this point on will have my logo superimposed on it.

Some word processors reset the printer prior to sending the document.  This

will deactivate the Overlay but will not delete it from the printers memory.

In this case the printer reset command must be removed from the printer

driver or set-up string or try using a Macro that is not an Overlay, a .MCR

file created using the /M switch.

When using .MCR Macro files the Macro must be Called or Executed from the

document before the Overlay image will print on the page.  A printer command

sequence must be placed in the document you are printing.  Up to 32 Macros

may be Loaded into the printer at one time depending on the amount of printer

memory available.  The size of the Macro file(s) must not exceed the printers

memory.

Use the following printer commands in the document to control the Macro

actions.  Replace # with the Macro ID# used to create the Overlay.  Look in

the word processor manual if you need help on placing printer command

sequences into your documents.

Execute Macro  ESC&f#y2X

This command will activate the Macro.  Any changes made by the Macro to the

printing features listed below will be retained by the printer.  The cursor

remains where it was at the end of the Macro.

Call Macro ESC&f#y3X

This command will activate the Macro.  Any changes made by the Macro to the

printing features listed below will NOT be retained by the printer. The

cursor remains where it was at the end of the Macro.

Page Length    Orientation    Input Control       Copy Count

Margins        Perf Skip      Line Wrap           Font/Macro Id

Rule Size      Underline      Resolution          Graphics Mode

Font Attributes               HMI/VMI

Note: The Above two commands MUST be used on each page and will not activate

the Overlay feature.  Create the Macro file with OVERLAY /M switch.

Disable Macro Overlay  ESC&f#y5X

This command turns off an Overlay Macro, effective on the current page.

Executed and Called Macros are disabled when completed.  Use the ESC&f#y4X

command to Enable the Overlay again.

Delete Macro ESC&f#y7X

This command will delete the specified Macro from the printers memory.  Note

the difference from the Disable Macro Overlay command.

WordPerfect 5.x Notes

If you need to remove the printer reset command from the printer driver for

WordPerfect, use the PTR.EXE program to change the document initialization

string in your printer driver.

Enter PTR and the name of your printer driver file at the DOS command line.

C>PTR DRIVER.PRS

Choose Edit from the command menu.  Select Initialize and Reset then select

Initialize at Start of Print Job.  Remove [27]"E" from the command string,

this is the command that deactivates Overlay Macros.  F7 to EXIT and Save

when finished.

Next, set-up WordPerfect to select the desired Macro from the ALT F8 Styles

menu.

1.  Use ALT F8 3 to Create a new Style item.

2.  Enter a Name and Description for the new item

3.  Set the Type to 2(OPEN)

4.  Select 4(Codes) and enter the commands.

Shift F8 4 6 2 1 [27]"&f#y4X" F7 F7 F7 F7 6

Replace [27]"&f#y4X" with the Proper Macro Command from above.

Repeat this for each Macro you wish to add to the Styles menu. To select a

Macro after they have been loaded into the printer use ALT F8 and select

which Macro to use.

Troubleshooting Overlay Macros

With the release of version 3.0 of OverLay, Debugging utilities have been

added to the package.  If you are having trouble getting the Overlay Macro

to work, follow the steps below and you should be up and running in no time

at all.

The creation of the PRINT file is the most likely place for problems to be

introduced.  Follow the guidelines above and avoid problem commands.  It is

not always possible to eliminate the use of these commands.  OverLay

removes some of these commands when it creates the Overlay Macro file, some

unexpected commands may not be removed.

Find and remove these unwanted commands from the Overlay Macro and it will

work properly.  This brings us back to the Debugging utilities.

PCL2ENG.EXE  :This program written by Helamar B. Herman reads HP-LJ PRINT

files and translates the printer commands into English.

READPCL.BAT  :Batch file to redirect the output of PCL2ENG to a file named

PCLENG.TXT.  The file IN. is needed by READPCL.BAT.

PCLSTRIP.EXE :This program will remove an unwanted printer command from a

Macro file.

Test the PRINT file for the proper output.     COPY /B MYLOGO.PRN PRN

Create a empty PRINT file.  If a desk top publication document is printed to

a file and it is a blank page, the resulting PRINT file will contain only the

printer commands.  This will make it very easy to locate any unwanted printer

commands that are causing problems.  Convert the Print File to an Overlay

file.

C>READPCL MYLOGO.OVL entered at the DOS command line will create the

PCL2ENG.TXT file.  Read this file and look for unallowed commands.  The ESC

character is not displayed by PCL2ENG so you can print out the file contents.

Commands may vary depending on what options or settings your publication is

using.  Change the settings to eliminate these commands from your PRINT file

if you can.  IF there is still unwanted printer commands in the Overlay Macro

file use PCLSTRIP.EXE to remove them.

PCLSTRIP MYLOGO.OVL /ESC&l0O  This will remove the set orientation to

portrait command from the Overlay file.  It is important to note that even

though portrait mode is the default, if a set orientation to portrait mode

is encountered, the Overlay will fail and act as mentioned above.  Remember

PCL2ENG does not display the ESC character, don't forget to use it on the

command line for PCLSTRIP.

Repeat these steps until all the unwanted printer commands are removed.

COPY /B the Overlay Macro file to the printer then do a few form feeds.  If

your printer won't allow you to do a form feed from the control panel then

copy the FF file to the Printer a few times.  If the Overlay is on every page

and is correct then the Overlay Macro is ok.

If you are still having problems, the problem is with the word processor or

other method used to send the document to the printer.  The most common cause

of problems here is the word processor resetting the printer at the start of

the print job or the beginning of the individual document.

If you can send the printer output from your word processor to a print file,

follow the same steps used above to examine this file and look for unwanted

printer commands.  A program that redirects output for the printer to a file

is helpful here.  Converting a document to a DOS or ASCII file won't work.

Once the unwanted commands are located, check the setup options for the word

processor to see if they can be shut off.  The printer driver may need to be

modified to accomplish this.  See WordPerfect Notes above.

An alternate approach is to convert a word processor document to a ASCII

file.  All printer commands will be removed during conversion.  Copy the

ASCII file to the printer after the Overlay Macro is loaded.  If it works ok

then the problem is with the word processor.

The most common problems with softfonts not printing correctly is caused by

the word processor Downloading fonts with the same ID# or descriptor as the

fonts used in the Overlay Macro.  Remember, font management commands are  not

allowed.

If the program you are using to make the PRINT file has a option that will

suppress the Downloading of fonts, use it.  Download the fonts separately.

You may need to determine the font id# to assign the softfonts.  LoadFont

4.51 will create SoftModules from SoftFonts.  PCL2ENG can read SoftFont or

SoftModule files to help you with the font ID# numbers and descriptors.

Copy /B the SoftModule file to the printer followed by the OverLay file.

Download the fonts or load the SoftModule before the Overlay Macro.

Test the SoftFonts, files do get damaged.

Check the printer memory, chips can fail and go bad.  There must be enough

available memory to hold all the Macros and SoftFonts.

If you are still having problems, send me a copy of the PRINT file, the names

of the programs you are using to create and print the Overlay Macro and a

description of the problem.  Sorry, but I can only offer this level of

support to registered users.

When you send in your $20 to register the OverLay program, you will receive

the current version of OverLay and 2 Archived disks full of SoftFonts and

Laser-Jet Utilities FREE.

Jim Savoia

Advance Systems

215 W Amosland Rd

Norwood, PA 19074

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

Public (software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  PsL cannot debug pro-

programs over the telephone, though we can answer questions.

Disks in the PsL are updated  monthly,  so if you did not get

this disk directly from the PsL, you should be aware that the

files in this set may no longer be the current versions. Also,

if you got this disk from another vendor and are having prob-

lems,  be aware that  some files may have become corrupted or

lost by that vendor. Get a current, working disk from PsL.

For a copy of the latest monthly software library newsletter

and a list of the 3,000+ disks in the library, call or write

Public (software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

1-800-2424-PSL

MC/Visa/AmEx/Discover

Outside of U.S. or in Texas

or for general information,

Call 1-713-524-6394

PsL also has an outstanding

catalog for the Macintosh.