PC/370 help documentation

So you've successfully installed PC/370 using the BAT\INSTALL.BAT

command, have run the demo programs, and have read the documentation

files, but you still have some questions.  Well, here are some of the

most frequently asked questions and some answers which may help:

1.  Where can I get more help?

Send letter and/or diskette with problem to me at the address

listed below and I will reply.  Subscribe to Compu-Serve

electronic mail survice adn send me EASY mail message

using ID# 73047,1113.

2.  The A370.EXE cross assembler gets I/O error reading my source

ALC program file?

The file must be in ASCII text form with line feed (X'0A')

character ending each line of text.  This is standard PC format

used by most text editors including EDLIN and SEE.  If the ALC

file was downloaded from an IBM mainframe, it may need to be

translated from EBCDIC to ASCII.  Most mainframe to PC link

facilities have this translation option as a default.

3.  The A370.EXE cross assembler did not recognize some statements?

See USER.DOC for OS/VS assembler features not supported by

A370.EXE such as macros, and see the reference section for

additional texts on OS/VS assembler and ASSIST.  If the program

has macros, you must use the M370.COM macro preprocessor to

expand macros into basic assembler first.  If the program has

floating point E, D, or L type DC constants, an 80x87 math co-

processor must be installed for the assembler to correctly

assemble the floating point constants.

4.  The L370.EXE linkage editor keeps getting unresolved external

references even though I've checked that all subroutines have

been added to system or program LIB file with correct /B option

of MS-DOS copy command as shown in BAT\BLDLIB.BAT?

The linkage editor scans the program LIB file first and then the

system LIB file in sequential order attempting to resolve all

external references in one pass.  If any module selected for

inclusion calls a module previously scanned and not selected,

then the link will fail as a result of the backward reference.

You must sequence the libraries to eliminate backward references

or force loading of required modules with explicit external

address constants.  To allow unresolved references use option U.

5.  My first program will not run.  How do I debug it?

Specify an upper case T as only parameter when executing program

to start in debug mode or link it with option D specified.  At

the interactive debug prompt, type T to trace instructions up to

point of failure.  You can stop trace at any point by hitting

any key and then typing T again to restart.  If no entry point

name was specified on END statement, the program will start at

first byte of code.  If there are too many instructions before

failure to trace, enter Q for quiet mode execution up to point

of failure and then enter N for list of last 20 instructions

executed.  See debug chapter of USER.DOC for more information.

6.  My write to operator message printed garbage after message?.

Add $ character to tell MS-DOS where end of text is.

7.  My first file I/O operation caused an I/O error?

Make sure that file is in current directory or that complete

MS-DOS file path was specified in DCBDSN field of DCB with a

trailing null character (X'00').  Make sure that record length

(LRECL) includes room for carriage return and line feed

characters in each text record.  Text record processing on the

PC is quite different from the normal fixed length text record

processing on an IBM mainframe.  Text records on the PC created

by the SEE.COM full screen editor or most other PC editors such

as EDLIN store text records in variable length ASCII form with a

carriage return (X'0D') and line feed (X'0A') character

indicating the end of each record and an end of file (X'1F')

character after the last line feed.  If the file being read or

written has 80 data characters, then the record length needs to

be 82 bytes for text type files.  See SYSTEM.DOC for more

information on file DCB options including translation between

EBCDIC and ASCII.  The ASSIST extended instructions XREAD and

XPRNT make the translation between ASCII text file format and

EBCDIC fixed length record format automatically.  See the ASSIST

demo programs DEMOAST1.ALC through DEMOAST4.ALC and the ASSIST

section of USER.DOC for more information.

8.  The records read from my input file appear to be in ASCII

instead of EBCDIC as expected by the 370 program?

The default for DCB text file processing is not to translate

between ASCII and EBCDIC.  There is an option bit in the DCBFLG

byte named DBTRAN which can be set prior to open to request that

each record be translated such that file is in ASCII and record

processed by program is in EBCDIC.  The ASSIST extended

instructions XREAD and XPRNT automatically make this

translation.

9.  Why don't you have an option to let all character strings

default to ASCII instead of EBCDIC code in an assembler program?

Release 1 did have option E to specify either ASCII or EBCDIC

for DC character strings and self defining character strings

such as immediate data in CLI instruction.   This created non-

standard 370 code by default when the ASCII mode was selected

and was very confusing when mixing subprograms with different

options.  Release 2 removed the E option and added option to

define ASCII character strings using double quotes.   This

solution also allows both types of character strings to be

easily used in a single program.

10.  Can I download and use the MVS, VM, or VSE macro's?

No.  First they are copyright by IBM and licensed only on the

mainframe they reside on.  Second, they would not work without

changing them to use PC/370 supervisor calls and the limited

syntax of the macro pre-processor.  The common MVS macros

included such as DCB, OPEN, CLOSE, GET, and PUT have been coded

from scratch for the PC environment.  Several different users

are developing additional macros to provide compatible unit test

environments for each of the above operating systems.

11.  Can I download load modules from an IBM mainframe and run

them using PC/370?

Yes, but only if you also complete a PC/370 shell such as the

MVS demo shell to intercept all SVC's and supervisor state

instructions used.  The demo shell will support a native 370

mainframe MVS load module that only uses problem state

instructions and the WTO and WTOR svc's.  The actual load module

file would have to be processed to remove all the extraneous

fetch control records leaving just the 370 object code in a MOD

type file which could be loaded and executed by the shell

program.  The object code would have to be self relocating (i.e.

no relocation records in the load module file).

12.  Can I download fixed blocked files and read them directly with a

PC/370 program?

Yes, one of the DCB file processing options is fixed blocked.

This mode assumes that the file contains fixed length binary

and/or EBCDIC data records with no carriage returns or line

feeds.  Text files are not normally stored in this format on

PC's because it wastes so much space compared to the variable

length ASCII text format.

13.  Can I upload source, object, or load modules to the mainframe

and execute them?

Yes, with the following restrictions:

a. Source code must be translated back to EBCDIC and any SVC's

or macros used must be compatible with the mainframe

operating system and macro assembler.

b. Object code can be uploaded after it is converted from

compressed bit stream format (OBJ) to OS/VS linkage editor

80 byte fixed blocked format (370) using the utility

T370.COM which is included in both object and source form.

This is being used by some software developers who unit

test code on a PC and then upload tested object code to

mainframe.

c. Load module code can be uploaded but with the following

restrictions.  Only MOD format code would make sense to

upload since the loader code in COM format files is unique

to PC MS-DOS environments.  An application interface on

the mainframe would have to be written to read the MOD

file code into memory and link to it.  Of course, the

supervisor instructions would have to be compatible with

the mainframe operating environment.

Please send additional questions and answers you think would be

helpful to other users.  I will add them with appropriate credits.

This file was first added in release 4.2 at the suggestion of BIll

Earle.

Don Higgins

6365 - 32 Avenue North

St. Petersburg, Florida 33710