Docs for COMM.H C communications header file by

Mario Giannini - Compuserve ID: 76276,1576

New York, New York

Notes:

BUFSIZE is the size of the communications buffer, the default is 8K.

To change this ad the following line to your main source file:

# define BUFSIZE xxxxx

Where xxxxx is the size of the buffer in bytes.

port - When a port number is required, a 0 specifies COM1 and 1 specifies

COM2.

Commands:

The folowing functions are defined in COMM.H and can be called once they

have been combined with your source file using the following line, note that

functions that do not specify a COM port will use the port for the last

succesful setintr function.

#include <comm.h>

------------------------------------------------------------------------

setintr

summary:

#include <comm.h>

int setintr(port);

int port;            COM port on PC

description:

The setintr function installs the communications interrupt handler for

the specified COM port.  The port is deteremined by the following

variables:

port      COM?:

====      =====

0       COM1:

1       COM2:

Returns Value:

setintr returns a 0 if successful, or a -1 if the specified port was

not found.

example:

if (setintr(0))

printf("Unable to set interrupt\n");

------------------------------------------------------------------------

resetint

summary:

void resetint(port);

int port;                 COM port on PC

description:

The resetint function will remove the interrupt handler associated with

the com port specified by port.  Care must be taken not to call this

function if setintr() was not succesful, this might cause the machine

to lock up under other communications systems.

Return Value:

resetint does not return a value.

example:

resetint(0);

------------------------------------------------------------------------

seravl

summary:

int seravl();

The seravl function resturns a non-zero if there is a character in the

communications buffer.  This function should be called before calling

the getser function to avoid the buffer table pointers from collision

(resulting in 1 buffer full of garbage data.

Return Value:

seravl returns a non-zero if a character is available in the com buffer,

or a zero if not.

example:

if (seravl())

printf("A character is in the buffer\n");

------------------------------------------------------------------------

getser

summary:

#include <comm.h>

int getser();

description:

The getser function will get a character from the serial buffer.  Make

sure to check that a character is avaiable with the seravl() function

before getting it.

Return Value:

getser returns the ASCII value of the next character to be read from the

com buffer

example:

int ch;

if (seravl())

{

ch=getser();

printf("Character received: %c\n", ch);

}

------------------------------------------------------------------------

portinit

summary:

#include <comm.h>

int portinit(port, baud, parity, data, stop);

int port;    com port to initialize

int baud;    baud rate (tested upto 9600)

int parity;  parity: (0=none, 1=odd, 2=even)

int data;    data bits (6-8)

int stop;    stop bits (0-2)

description:

The portinit set the UART chip connect to the COM port specified

by port to the above mentioned parameters.

return value:

returns  0 if ok

-1 if port not there,

-2 if invalid parameter

example:

if (portinit(0, 1200, 0, 8, 1))

printf("Error initialising COMM port\n");

------------------------------------------------------------------------

is_carrier

summary:

#include <comm.h>

int is_carrier(port);

int port;     com port to check for carrier

description:

The is_carrier function returns a zero if there is no Carrier Detect (CD)

signal detected at the specified com port, a non-zero otherwise.

example:

if (is_carrier(0))

printf("Carrier detected at COM1:\n");

------------------------------------------------------------------------

sendstr

summary:

#include <comm.h>

void sendstr(str)

char *str;    string to be sent to modem.

description:

The sendstr function allows NULL-terminated strings to be sent to the

modem.  The following characters have a special meaning to sendstr when

used in the string being sent:

|  This character is replaced with a carriage return

~  This character causes the string to wait 1 second before

continuing to send the rest of the string.  Note:  This

function does not work the same way as the comma in the

Hayes ATDT command which sends the string and pauses the

function (dialing) for 2 seconds.

Return Value:

sendstr has no return value.

exmaple:

sendstr("||~+++~|ATH|");

The above example will send two CR's to the modem, then wait one

one second, it will then send a +++, and wait again for 1 second.

Once that wait is over it sends a CR, ATH, and another CR.  This

sequence will usually work as a software hangup method.

------------------------------------------------------------------------

xmit

summary:

void xmit(ch);

int ch;     character to send to COM port.

description:

the xmit function will transmit one character specified by ch out the

serail port last used by the setintr function.

Return Value:

xmit has no return value.

------------------------------------------------------------------------

clr_buf

summary:

void clr_buf();

description:

clr_buff clears out all character in the communications buffer.

Return Value:

clr_buf has no return value.

example:

clr_buf();

The buffer was just wiped clear.

------------------------------------------------------------------------

onesec

summary:

#include <comm.h>

void onesec();

description:

This function when called will create a one second time delay.  It used

by the snedstr function and may also prove useful for certain timed file

transfer protocols.

Return Value:

onesec has no return value.

example:

printf("Please wait one second...");

onesec();

printf("Thank you\n");

------------------------------------------------------------------------

hwhangup

summary:

#include <comm.h>

void hwhangup();

description:

The function inverts the signal from the DTR signal out of the COM port

to perform a hardware hangup for modems that support it.

Return Value:

hwhangup has no return value.

example:

sendstr("Disconnecting from system...");

hwhangup();

The above example would send the string "Disconnecting from system..." to

the modem (and through to any users connect) and the hangup the modem

via the DTR technique.

------------------------------------------------------------------------