SEEKTEST

A Hard Disk Benchmark In Turbo C

Ryle Design

P.O. Box 22, Mt. Pleasant, Michigan  48804

CompuServe 73047,1765

Introduction

SEEKTEST is a program written in Turbo C to very accurately measure the seek

time of a hard disk installed in an IBM compatible personal computer running

the MSDOS operating system.  By using BIOS interrupt services and a very

high resolution timing methodology, a very meaningful measurement of hard

disk seek performance can be made that reflects "real world" operating

conditions.

The test measurements are not destructive to the data on the device under

test.  However, it is essential that this test (or any other disk intensive

benchmark or diagnostic) should only be run on storage devices that are fully

backed up in case of failure.

Distribution

The distribution you have received should contain the following:

SEEKTEST.DOC    This document.

SEEKTEST.C      Fully commented Turbo C source code.

SEEKTEST.EXE    Small memory model executable.

NOT included in this distribution is the library containing the high

resolution timer functions, which is only required if you wish to modify

the source code and recompile the executable.

High Resolution Timer Toolbox

SEEKTEST uses TCHRT, the Turbo C High Resolution Timer Toolbox from Ryle

Design.  A shareware evaluation copy of TCHRT is available in DL 5 of

BPROGB on CompuServe as TCHRTV.ARC.  Various BBS systems may also have

TCHRTV.ARC or TCHRTV2.ARC, which are functionally identical.  These

archives hold version 2 of TCHRT, and contain files with datestamps from

1989.  Version 1 of TCHRT was distributed as TCHRT.ARC in 1988 and will

not link correctly with SEEKTEST.C.

Briefly, TCHRT gives the programmer 100 software timers each with one

microsecond accuracy.  The timer engine calibrates itself to the host PC

speed at run time, and can generate a detailed summary report of all timer

activity.  Precise timing of BIOS interrupts 10h (video services) and 13h

(disk services) is also available, and the timer calls can be used inside

user written interrupt handlers.  TCHRT is an ideal tool for both high

resolution execution profiling and embedded timer applications.  Registered

users receive complete source to the timer libraries and example programs

that illustrate execution profiling, BIOS interrupt timing, and hardware

interrupt triggered timer activation.  TCHRT registration is $7.50, and a

form to order TCHRT is provided as the last page of this document.

Seektest Usage

At the DOS prompt type SEEKTEST, and execution will begin.  The system is

checked for physical hard disks present, and the user is prompted to test

any disk found, or exit.  Measurements are made of single track and average

to all tracks seek time, as well as an estimate of software overhead (BIOS

interrupt and controller microcode) for each seek.  The test is completely

non-destructive to the data on the media.

-1-

TCHRT Registration Form

Name __________________________________________________________________

Address _______________________________________________________________

City/State/Zip ________________________________________________________

Daytime Telephone _____________________________________________________

CompuServ ID (for tech support) _______________________________________

Where did you get your copy of TCHRT?  ________________________________

What is your primary use of TCHRT? ____________________________________

Comments regarding this product _______________________________________

_______________________________________________________________________

_______________________________________________________________________

_______________________________________________________________________

Check Products desired:

____    TCHRT Registered Version (Turbo C V2.0) .................  7.50

____    TPHRT Registered Version (Turbo Pascal V5.0) ............  7.50

____    MSCHRT Registered Version (Microsoft C V5.1/MSDOS) ......  7.50

____    All three of the above (save 2.50) ...................... 20.00

Subtotal ................................................ _____

Michigan residents add 4% sales tax ..................... _____

Total enclosed (__check __money order) .................. _____

Remit to:   Ryle Design

P.O. Box 22

Mt. Pleasant, Michigan 48804

-2-