FM-PAK/A -- FB-01, Music Feature Editor Utilities

Copyright 1990,  Kevin Weiner

All rights reserved

Installation

------------

It is suggested that all the included files be copied into a subdirectory

called FMPAK, and that the subdirectory be added to the DOS search path for

ease of use.  If a separate working directory for data files is to be used,

the files CONFIG* and BANK* (27 files total) should be copied there.  These

are used by the FM-Edit program to avoid delays in loading rom data from the

sound module via MIDI.  The working directory name should then be entered on

the FM-Edit "Setup" screen and the setup file "Saved".

Hardware

--------

Requires: 260K of available ram; hard drive recommended; and either an

IBM Music Feature card (MFC), or an external Yamaha FB-01 connected via

one of the following MIDI interfaces: Roland MPU-401 compatible, Key

Electronics MIDIator, or Creative Labs SOUND BLASTER (with MIDI

adapter).  All video display types are supported.  A Microsoft comatible

mouse is optional.

FM-Edit supports 2 local Music Feature cards, and up to 14 remote units.

The remote units may be either Music Feature cards or Yamaha FB-01

synthesizers.  If a non-MFC MIDI interface is used, up to 16 remote

units may be controlled.  If only one MFC is in use, the unit selection

switch on the card may be set for either card 1 or 2.

If no music interface is present, some of the utilities will still

operate.  FM-Edit, for example, can be used for editing voice and

configuration files on disk.

Usage

-----

To run any of these programs in default mode, type one of the program

names listed below.  To select special MIDI device options, include the

-d option on the command line.  The -d option identifies the MIDI

interface(s) in use, and optionally, the IRQ and base I/O address:

-dmfc      selects IBM Music Feature

-dmpu      selects MPU-401 compatible

-dmidiator selects MIDIator

-dsbmidi   selects SOUND BLASTER MIDI

(The first 3 letters of the device name are sufficient.)

The MIDIator cannot be detected automatically and the -dmid option

must always be specified if one is in use.

Optional IRQ and I/O port address information can be added following

the device id if necessary:

-dxxx:<irq>:<ioaddr>

For example, to define an MPU-compatible interface using IRQ 5 and I/O

address 332 (hex), use:

-dMPU:5:332

If only the I/O address is required but the default IRQ is alright,

use -dMPU::332.

The irq and ioaddr are not relevant to the MIDIator, though the first

option number can be given to indicate use of the com port 2 instead of

the default com 1.  (ex: -dmid:2)

To set up a device before hand, define the "opt1" environment variable

to be the -d option that would be included on the command line.  For

example:

set opt1=-dmpu:5:332

This line would generally be added to the autoexec.bat file to

select the option permanently.

Other Command line optiions (interactive programs only):

-m  Forces monochrome display mode.  (Normally detected automatically)

-k  Forces keyboard mode if a mouse is present.

-b  Selects BIOS screen writes, rather than the default direct write

mode.  May be necessary for proper operation on certain machines.

All data entry within the interactive programs is self-prompting.  More

complete usage information is contained in the help files.  The names of

each interactive program's help files include the program name and have

the extension '.HLP'.  The help files may be viewed within the programs

via the F1 help option on any screen.  They may also be viewed as normal

text files, or printed.  If using an editor to view them, be careful not

to modify them, since they are specially formatted and may not be

viewable within the programs if modified.

Programs

--------

The following programs are included in the FM-PAK/A distribution.  Note

that where a file name is required, the current directory is assumed if

none is specified.  <options> refers to the "-" options described above.

They must follow any other command line items.

FMEDIT   - Configuration and voice editor.

Usage: fmedit <options>

FMTUNE   - Allows the FB-01/MFC fractional tuning feature to be used

by defining arbitrary note frequencies.

Usage: fmtune <options>

PLAYB    - Plays standard MIDI files.

Usage: play <file name> <mode> <options>

<mode> =    S   Scan mode - display file info, do not play.

X   Dump file contents to screen.

n   Channel number (force all output to same channel).

<mode> is optional; normal play mode is assume if not given.

LOADBANK - Loads a saved bank file to a specified user bank.

Usage: loadbank <file name> <bank number> <node number> <options>

Default for bank and node number is 1.

LOADCONF - Loads a saved config file to the specified user configuration.

Usage: loadconf <file name> <config number> <node number> <options>

Default for config and node number is 1.

SETCONF  - Selects the current synth configuration (1-20).

Usage: setconf <config number> <node number> <options>

Default for config and node number is 1

VOICES   - Lists the names of the voices contained in a bank file.

Usage: voices <file name>

DUMP     - Simple SYSEX dump program.  Sends a file containing MIDI system

exclusive messages to a device via the MIDI interface.

Usage: dump <file name> <options>

FM2MDX   - Exports FM-Edit bank files to raw bank dump format.

Usage: fm2mdx <FM bank file> <raw bank file>

CHECKMFC - Checks for proper operation of the Music Feature card.

Usage: checkmfc

RESETMFC - Resets the Music Feature card to power-up status.

Usage: resetmfc

RESETCNF - Resets all user configurations to factory defaults.  (batch file)

Usage: resetcnf <node number> <option>

RESETBNK - Resets user banks to factory defaults. (batch file)

Usage: resetbnk <nodenumber> <option>

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

Public (software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  PsL cannot debug pro-

programs over the telephone, though we can answer questions.

Disks in the PsL are updated  monthly,  so if you did not get

this disk directly from the PsL, you should be aware that the

files in this set may no longer be the current versions. Also,

if you got this disk from another vendor and are having prob-

lems,  be aware that  some files may have become corrupted or

lost by that vendor. Get a current, working disk from PsL.

For a copy of the latest monthly software library newsletter

and a list of the 2,000+ disks in the library, call or write

Public (software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

Orders only:

1-800-2424-PSL

MC/Visa/AmEx/Discover

Outside of U.S. or in Texas

or for general information,

Call 1-713-524-6394

PsL also has an outstanding

catalog for the Macintosh.