COSAN - Cost and Weight Analysis

How to Operate COSAN

COSAN is the cost and weight analysis module within the COPAN/II

system.   It is probably one of the easiest to operate and most

flexible  composite costing analysis programs that you will ever

come across.

Just below this opening screen information,  a small message will

remind  you to active the [NumLock] keys.   This is because COSAN

uses a menu pointer similar to the one in ACCESS.    But

practically, if you have loaded COSAN via ACCESS, you numeric

keypad  should already be activated.

------------------------------------------------

|COSAN  (1)                                    |

|- - - - - - - - - - - - - - - - - - - - - - - |

|                                              |

|    (2)  Parameter/Status Display Area        |

|                                              |

|                                              |

|- - - - - - - - - - - - - - - - - - - - - - - |

|    Main Menu  (3)     |   Communication (4)  |

|                       |                      |

|                       |                      |

|                       |                      |

|                       |                      |

|- - - - - - - - - - - (5) - - - - - - - - - - |

------------------------------------------------

Primary Screen in COSAN

Within COSAN,  there are five windows that are used to display

information and communicate with you.

WINDOW 1

The top window is a type of program header which keeps you

informed of vital information such as the time, date, which

program you are operating within,  default drive and active file.

The active file refers to the LAMCON file that you are analyzing.

WINDOW 2

The rectangular center window is used to keep you abreast of the

current calculation settings.   These are items such as panel

dimensions, resin type, internal or external components, syntac-

tic foam, cost and weight tolerance percentages and so on.

Each time that you change one of the parameters, the status board

is automatically updated.

#

WINDOW 3

The lower left hand window is the main menu which is controlled

by pressing the number keys.  Notice that I said NUMBER keys, not

with the arrow keys.   The reason for this is simple.  With other

programs, the arrow keys are used to move the menu pointer to the

appropriate position but then, the [NumLock] key has to be hit

before  values can be entered using the keypad.   With COSAN, we

have taken care of this shortcoming.   The [2], [4], [6] and  [8]

keys will cause the menu pointer to move one position with each

key depression.  When you choose a menu item that requires some

type of numerical data, the keypad will be ready to fire.   No

longer  will you be required to keep track of the [NumLock]  key

status.

An example would be the panel dimensions.   Simply press the  [2]

key once  when COSAN is first booted and the menu  pointer will

move down one space to the menu item [Panel Dimensions].  A press

of the <CR> will then produce a prompt in the Communication

window asking  you if you would like to change the current

dimensions.   After responding to this prompt, the keypad may then

be used to immediately to enter the proper panel dimensions.

What a breeze.

WINDOW 4

The lower right hand window is called the communication window

for obvious reasons.  It is through this window that many of the

COSAN prompts are produced.  In addition, small portions of

information are given through this area.

WINDOW 5

The 25th row of the screen is a single line window that is used

to provide some additional guidance during input prompts.   This

guidance is usually an extra sentence of explanation that helps

to clarify what input COSAN is waiting for.   During prompts that

require numeric values.   COSAN uses this line to display the

active units.

#

Where To Begin Each Analysis

Each analysis that you will run with COSAN will require a minimum

quantity of keystrokes from you.   The exact quantity may be more

depending upon your specific analysis requirements.

In order for a laminate file to be processed,  the following data

must be supplied by you the first time you run an analysis for

each COSAN session.

LAMCON file name

Panel Dimensions (length and width)

Resin Data  (Description, weight and cost)

Core Cost *

* If one was included in the LAMCON laminate and a C2BASE  file

did not provide the information.

Let's run through these steps as they will occur during program

operation:   (make sure you have already created at least one LCF

file using LAMCON before proceeding)

NOTE:

When a bracketed string of text occurs in this

documentation, it means that those words are taken exactly

from a program screen menu.    ie. [End  Session] is

considered a main menu COSAN option.  Within the program,

it appears just this way.

Specifying a LAMCON file

At the start of each COSAN session,  when the primary screen

appears, the menu pointer will be resting quietly upon the [Enter

LAMCON Files].   To activate that portion of the program, press

the <CR>.  The menu in window 3 will clear and a smaller one will

be displayed.

-------------------------------------

|      (( LAMCON Job Entry ))       |

|                                   |

|      Enter LAMCON job name(s)     |

|      Screen display of active     |

|      Save the list on disk        |

|      Read old batch from disk     |

|      Restart the list             |

|      Exit                         |

|                                   |

-------------------------------------

#

This new  menu, as shown above, will have the first choice

highlighted in reverse video assuming that you will want to enter

a LAMCON file name.   If this is the case,  which it is for  this

demonstration, press the <CR> again.   Now  the  communication

window  comes to life and produces a small ) with the number 1 in

front of it.

1>

-------------------------------------

This means that COSAN is waiting for the first LAMCON file to  be

entered.   If you do not wish to enter one, or if you have

forgotten  the file name, press the <CR> again with nothing

entered after the 1).   This will move you back one step to  the

previous menu.

After you enter a valid LAMCON file name (8 characters only, do

not include a file extension .LCF) control will return to the

sub-menu.

Immediately after the <CR> is pressed, the main menu  will  be

redisplayed.  Almost simultaneously, COSAN will go to the default

drive and read the LAMCON file that you have just specified.  In

the event that the file does not exist on the default drive,

LAMCON will tell you that the file was not found.   If you are

sure that it does exist, check to make sure that the default

drive is correct.   If not, use the [Change Drive] feature within

COSAN.  More on this later.

As it is possible to create solid or sandwich laminates with

LAMCON, it is also possible to analyze them with  COSAN.  Let's

say that we have loaded in a solid laminate for this example.

COSAN will read the laminate file and produce a summary in the

communication window.  This is so that you may confirm that the

file loaded is the correct one.  If it is not correct, you will

have to press the <CR> again and go back into the [Enter LAMCON

Files] in order that if may be respecified.

#

Entering Panel Dimensions

Now that the LAMCON file is loaded into memory, the panel

dimensions must be decided upon.   Usually a 12x12 inch panel is

used or a 1000x1000mm.   Your units will be based on what units

were defined in PCAT.    Move the menu pointer down one position

using the [2] key.  Then press the <CR>.

-------------------------------------

|      (( Panel Dimensions ))       |

|                                   |

|      Units: IN                    |

|      X  =  12                     |

|      Y  =  12                     |

|                                   |

|      I wish to change these       |

|        No     Yes                 |

-------------------------------------

Again, the communication window will be used to obtain the input

data.  COSAN will confirm before allowing you to enter the data.

Follow directions and enter the length and width.

Specifying the Resin Data

The only remaining necessity is the resin data.   As you  might

recall, all material weights were input within PCAT as the dry

fabric weight.   Also included within PCAT was a typical resin

percentage by weight.   Now in order for the weight calculations

to take place, a resin material must be specified.   The two

numerical items required are resin density and resin cost.  For

the US version, these are in pounds per cubic foot and US dollars

per pound.

Move the menu pointer down to [Specify Resin Data] and press the

<CR>.   This  will produce another sub-menu that you must use to

accomplish the entry task.

-------------------------------------

|      (( Resin Specification ))    |

|                                   |

|      Manually specify a Resin     |

|      Read a Resin File            |

|      Create a Resin File          |

|      Screen Listing               |

|      Exit                         |

|                                   |

-------------------------------------

#

If you have not entered a resin material prior to this, you will

have to enter it manually.   Press the <CR> again and the

communication window along with window 5 (the 25th  row) will

display the appropriate prompts.   When you are done entering the

data, be sure to use the [Create a Resin File] option so that

next time you will not have to re-enter all of the same data.

When all of this is complete, move the menu pointer down to the

last item and exit back to the main menu.

Let the Calculations Begin

With all of this behind us, the calculations may now begin.  Move

that menu pointer down to the [Analyze] option.   Press the <CR>.

COSAN will make sure that you wish to begin the calculations.

Respond accordingly.

---------------------------------------

|                                     |

|   I wish to begin calculations now  |

|             No     Yes              |

---------------------------------------

Before the calculations actually begin,  COSAN will perform a

basic check on the necessary data to ensure that there is enough

minimum information to process the laminate.   This only takes a

few nanoseconds but rest assured that if you do not have the

minimum data, COSAN will not run the analysis giving you invalid

results.

If the required amount of information has been supplied, COSAN

will tell you that the "Computations are in process."   After  a

two or three second delay, the screen will clear and a new output

menu will be visible on the bottom of the screen.

The Output Menu

The output menu allows you to screen view all of the results that

have been calculated as well as the calculation parameters such

as resin used, components included and cost/weight tolerances.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Lam.Schedule  Resin Data  Syntactic Foam  Components  Tolerances

Active Units    Ply Results    Laminate Totals    Exit

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

#

The  menu pointer is moved either left or right using the [4]  or

the [6] key.  When the pointer is resting on top of the item that

you would like to view,  press the <CR>.

After all of the data has been reviewed, move the pointer to

[Exit] and smack the <CR>.   This will redraw the primary screen

with the original five windows.  You will then have the option of

re-running the same laminate with different parameters or other

laminates using the same parameters.

Entering a New LAMCON File

After you have analyzed one file,  it usually happens that you

want to analyze another using the same parameters.   This is

common in instances where you or someone else is asking,  "How

does laminate A compare to laminate B in terms of cost and

weight?"

This is easily answered using COSAN.  After you have finished

with on laminate, exit the output menu and return to the primary

screen.  Move the pointer back up to [Enter LAMCON  files].  As

always, press the <CR>.  Now, rather than press <CR> immediately,

move  the  pointer  down in the sub-menu to [Restart  the  list].

When you have positioned the pointer here, press the <CR>.  COSAN

will erase the old file and reset the file counter.   To let you

know that this has been done,  COSAN will respond with a strange

noise that is rather hard to miss.

Now move the sub-menu pointer up to the first item in the  list

[Enter LAMCON job name(s)] and enter in another file.   You  will

notice that the prompt in window 4 is again 1>.

What COSAN did not do is erase any of the calculation parameters.

So now when you return to the primary menu, you will not have to

respecify the resin, panel dimensions or anything else.  All that

is necessary is that you submit the new LAMCON  by  moving the

pointer to the [Analyze] option.   Press <CR> and the rest should

now be familiar.

#

Writing Output Reports

COSAN, as you might have guessed, can output just about anywhere.

When the program is activated,  the default setting is to the

monitor.  However, the line printer and a disk file may be

specified as well.

Within the main menu, there are three options that allow you to

rapidly toggle the status of the option.

-----------------------

Output to Printer  |

Output to Monitor  |

Output to Disk     |

For example, to reverse the screen setting, move the menu pointer

to the [Output to Screen] option.  Press <CR>.  COSAN will assume

that you wish to reverse the current setting as this  would be

the logical choice.

-------------------------------------

|      (( Set Monitor Default ))    |

|                                   |

|                                   |

|    I would like all analysis      |

|    results output to the monitor  |

|                                   |

|            No      Yes            |

|                                   |

-------------------------------------

To confirm the assumption, press the <CR> a second time.  If you

wish to keep the current status, slide the sub-pointer to the

appropriate response and press the <CR>.  Simple.

All three options operate the same way.  Each will assume that

you wish to reverse the status but you will always have to

confirm the command before making any changes.

#

The Purpose of Disk Output Files

Why route output to a disk file?

We thought that there were two primary reasons for this.   The

first is when you are operating with a lap top machine and the

printer is back in the office.  By outputting to a disk file, you

will be creating an exact copy of the report that would normally

be line printed.   This means that when you do return to the

office, you can simply print the output files using the COPAN/II

LINER program.   Or if you wish,  an editor such as WORDSTAR can

be used to punch it up and spit it out.

The second reason was just as important.   Not everyone is

concerned with the information that we decided to present with

the COSAN output.   Additionally, not everyone feels that our

report format is what they want.  With a disk file of the report,

you have the option of modifying it before it hits the press.

This way, by using an editor,  you can customize the printer

output with your company name, remove certain data items that may

not be necessary in your report or add lines of explanation to

enhance the report presentation.   With disk files, this is all

possible.

Analyzing Cored Constructions

When using COSAN to analyze sandwich constructions, most

everything remains the same except that the cost of the core must

be given proper attention.

When using COSAN with PCAT,  the core records are created without

the inclusion of a core cost.  This is because within PCAT, there

is not a default thickness on which to base a cost.  What does

exist however, is the name of a C2BASE file which can be used to

supply the core costs.

When a C2BASE file does not exist

When analyzing a sandwich laminate with COSAN, it is not

necessary that you have a C2BASE file available.  COSAN will look

for this file if you have called out one in PCAT but will

continue cost and weight calculations even if one does not exist.

It will be necessary to assign a cost to the core before COSAN

will process the laminate.   Submitting a sandwich file without a

core cost will produce an error message and processing will halt.

#

To assign a core cost, use the [Core Options] menu item.   The

sub-menu will appear in window 3.

-------------------------------------

|      (( Core Options ))           |

|                                   |

|      Assign cost to active        |

|      Change thickness of active   |

|      Assign costs to queue        |

|      Change thickness to queue    |

|      List current values          |

|      Exit                         |

-------------------------------------

When this menu is produced, make sure that the pointer is resting

on top of the [Assign cost to active].  Then press the <CR>.

The core type,  thickness and density will be produced in the

communication window (window  4) so that you may assign the

correct cost.  If the laminate has more than one core layer, all

layers will be shown.   All core layers will have to then have a

cost assigned.   When the assignments(s) are complete, move the

pointer to [Exit] and press the <CR>.   This will cause the main

menu to reappear.

When a C2BASE file does exist

If you specified a C2BASE file in PCAT and you have created one

on disk, here is what happens when COSAN reads a sandwich file.

(1)  First the .STR file is read which provides the structure or

outline of the cost file.   This structure includes how many

different thicknesses, densities, styles and price catagories can

be held in the actual cost file.

(2)  Now the matching process begins.  COSAN, knowing the density

of the active sandwich, searches the .STR file for a matching

density.   If a match is made, then step (3) will occur because

the file possibly contains useful information.   Obviously, if a

match is not made, COSAN will not read the .CST file because it

does not contain costs relative to the density of the active

sandwich laminate file.

(3)   The .CST file is now read.  All costs within the file are

loaded into RAM.  Immediately, COSAN initiates a search for a

matching thickness within the correct density.  If the exact

thickness is found, the corresponding cost is assigned to the

active core in process.

#

(4)  Should the thickness match not occur, one of four things can

happen:

1-  No core costs have been entered for the density even

though the density was found in the .STR file.

2-  The active thickness is below the smallest thickness

value found in the .CST file

3-  The active thickness is above the greatest thickness

value found in the .CST file.

4-  The active thickness lies between two thicknesses found

in the .CST file.

In any event, the result is the same.   No exact thickness cost

exists for the active laminate.  Here, it becomes necessary to

make a decision.

These are your two choices:

1-  Have COSAN interpolate a value based on surrounding cost

information

2-  Enter a cost value for the active core manually.

COSAN can interpolate  using a simple linear method.  If the

active thickness is halfway between two thicknesses found, the

cost will also be halfway between the two.   This is useful when

you are looking at a design where the core thickness does not

fall on a manufacturers standard.   We should recommend that you

give the supplier a call and get a quote but in the event you're

just doing a "what if" COSAN will do some quick math to speed you

on your way.

If you select the second choice,  you will be required to assign

the core cost using the method prescribed earlier.   In case  you

forgot, you will have to use the [Core Options] from the main

menu.

#

What about Syntactic Foam?

Because many fabricators, especially within the marine industry,

are concerned with the integrity of the blind core to glass

interface, syntactic foam (a bedding compound) is being used to

raise confidence levels.

This bedding  material is usually some type of polyester resin

which incorporates a lightweight filler to produce a thixotropic

paste.  The paste is laid down against the partially cured

laminate and then the core is positioned on to, and hopefully,

into the material.   This method is most effective when using a

type of scored core material so that the surface bonding area is

increased.

COSAN can include this material so that your final laminate

report will closely approximate that which you are intending on

producing.

Select [Syntactic  Foam] from the main menu by positioning the

menu pointer and pressing the <CR>.   A smaller sub-menu will

appear and you will have to make another choice.

----------------------------------------

|  (( Syntactic Specification ))       |

|       [Esc]  to  clear               |

|                                      |

|  Manually specify a syntactic foam   |

|  Read a S/F file                     |

|  Save a S/F file                     |

|  Screen listing                      |

|  Exit                                |

----------------------------------------

If it is the first time that you are specifying a syntactic foam,

you will have to do so manually.   This involves responding to

five prompts that will ask for the following information:

1-  Name of manufacturer

2-  Description or product identification

3-  Cost of the product per weight unit ($/lb)

4-  Density of the product (lbs/ft^3)

5-  Lay-down thickness (inches)

After you have entered these once,  it is suggested that you

create a .SYN file so that next time you need to use this

product, you may call it from disk.   To accomplish this, enter

the above data manually and then position the menu pointer to the

third selection from the top [Save a S/F file]

#

NOTE:  The .SYN is the file extension that is automatically

appended to the file name that you will provide.   Do not include

a file extension with your response.

When you are done specifying the material,  and the data has been

saved on disk for next time, move the pointer down to [Exit] and

hit the <CR>.

Working with Components

A component is anything that is attached to or included within a

laminate.   An  example of an external component would be a hat

section or some other type of stringer.

----

_____      _____

-------------------------------

-------------------------------

External Component

Obviously, an internal component would be something that would be

found between the two laminate faces.   An example would be  some

type  of access channel or an extensional stiffener to provide a

bit of additional buckling resistance while not increasing the

overall laminate thickness.

-------------------------------

|     |

-------------------------------

Internal Component

This componentfeature allows you to analyze a more complete

panel.   As many as 10 of each may be incorporated in a single

panel calculation.   The three input items that will be required

are as follows.

1-  Component Description

2-  Weight in units per unit length

3-  Cost in units per unit length

4-  Component orientation

The lengths referred to here are the lengths of the individual

component, not the length of the panel on/in which it is

situated.

#

Overriding a Ply Cost

When a ply record is created with PCAT, a real value for cost is

included as a default for calculations within COSAN.  You are not

locked into this value at runtime should it be necessary to

update a ply cost.

Consider this example:

A  laminate  file has been created with LAMCON and it  is

now ready for cost and weight calculations.   At the last

minute,  just before submitting the file to  COSAN,  your

supplier  calls  and  tells  you  of  a  tremendous  cost

reduction on the XRP-240 unidirectional product.  So that

your  results  are reflective of actual  conditions,  the

need  is now created for the ability to adjust  the  cost

values within the LAMCON file.

The COSAN feature that would come into play at this point would

be:  [Ply cost override]

To activate PLY COST OVERRIDE from the main menu within COSAN,

mover the menu pointer down to that option and press the <CR>.