Geometric Traverse Calculation

TRAV

Version 1.06

February 9, 1990

Copyright 1989,1990 by Robert Jones -- All rights reserved.

TABLE OF CONTENTS

Chapter  1

Disclaimer . . . . . . . . . . . . . . . . . . . . . . . . .    1

Copyright Notice and Software License  . . . . . . . . . . .    1

Registered User License  . . . . . . . . . . . . . . . . . .    2

Trademarks . . . . . . . . . . . . . . . . . . . . . . . . .    2

Chapter  2

Introduction . . . . . . . . . . . . . . . . . . . . . . . .    3

Requirements . . . . . . . . . . . . . . . . . . . . . . . .    3

Installation . . . . . . . . . . . . . . . . . . . . . . . .    3

Chapter  3

Overview of TRAV . . . . . . . . . . . . . . . . . . . . . .    5

Entering a traverse  . . . . . . . . . . . . . . . . . . . .    6

. . . . . . . . . . . . . . . . . . . . . . . . . . .    6

beginning and ending coordinates  . . . . . . . . . . .    6

enter the courses . . . . . . . . . . . . . . . . . . .    7

Curves  . . . . . . . . . . . . . . . . . . . . . . . .    8

Options  . . . . . . . . . . . . . . . . . . . . . . . . . .    8

Run . . . . . . . . . . . . . . . . . . . . . . . . . .    8

Show  . . . . . . . . . . . . . . . . . . . . . . . . .    8

Modify  . . . . . . . . . . . . . . . . . . . . . . . .    8

File  . . . . . . . . . . . . . . . . . . . . . . . . .    8

Print . . . . . . . . . . . . . . . . . . . . . . . . .    8

rOtate  . . . . . . . . . . . . . . . . . . . . . . . .    8

aDjust  . . . . . . . . . . . . . . . . . . . . . . . .    9

Area  . . . . . . . . . . . . . . . . . . . . . . . . .    9

Chapter  4

Modifying Traverse Data  . . . . . . . . . . . . . . . . . .   10

Chapter  5

Saving Traverses . . . . . . . . . . . . . . . . . . . . . .   12

Retrieving Traverses . . . . . . . . . . . . . . . . . . . .   12

Changing the Default Directory . . . . . . . . . . . . . . .   12

Deleting a Saved Traverse  . . . . . . . . . . . . . . . . .   13

Chapter  6

Sample Traverses . . . . . . . . . . . . . . . . . . . . . .   14

Sample 1 -- A Square  . . . . . . . . . . . . . . . . .   14

Sample 2 -- A Circle  . . . . . . . . . . . . . . . . .   15

Sample 3 -- A Triangle  . . . . . . . . . . . . . . . .   16

Appendix A  --  TRAV Version 1.06 Registration Form

Appendix B  --  TRAV Version 1.06 Bug Report Form

i

Chapter  1

Disclaimer

No claim is made as to the applicability of TRAV for any given use

and  the  user  must determine  whether  or  not  the software  is

suitable for  any particular application.  No  warranty of fitness

for a particular purpose is offered.

This software and  documentation is  offered "AS  IS" and  without

warranties  as  to  performance of  merchantability  or  any other

warranties expressed or implied.  The  user must assume the entire

risk  of using  the software  and  the author  is  not liable  for

damages of any kind.  By using this software, you agree to this.

Every effort has been made by the TRAV author to make this product

bug-free.  However, the nature of  software development is that it

is  impossible  to  guarantee  bug-free   software.    Should  you

encounter a bug, please  report it to the TRAV author.  Registered

users  who are  the first to  report a  verifiable bug or  are the

first to  make an  enhancement suggestion that  is implemented  in

TRAV receive an automatic registration to the next version.

TRAV is  a  user-supported program,  also known  as a  "ShareWare"

program.  TRAV has  been tested on a variety of  PC-DOS and MS-DOS

versions,  ranging in  number from 2.1  to 3.3.   It has  not been

tested on other versions of DOS.

Copyright Notice and Software License

This document; other accompanying written and disk-based notes and

specifications; and all referenced and  related program files, and

demonstration  data accompanying this  document are copyrighted by

Robert Jones.

The copyright owner hereby licenses you to:

1)   Use  this software  for  a period  of  30  days in  order  to

evaluate the software;  This is not  free software.  For  any

use  past  the  30 day  period,  a  $50  registration fee  is

required.

2)   Make as  many exact copies of the  software and documentation

as you wish;

3)   Give exact copies of  the original to anyone;  and distribute

the software  and documentation  in its  unmodified form  via

electronic means.

You are specifically  prohibited from making any  modifications to

the TRAV program or documentation.  No copy of the software may be

distributed or given away without this accompanying documentation.

Under no  circumstances is  the copyright  notice embedded  in the

TRAV code to  be modified  or removed.   This notice  must not  be

removed.

1

You  are  specifically  prohibited  from  charging  or  requesting

donations for any copies, however made.  Exceptions may be granted

to organizations which charge a small fee for materials, handling,

postage and general  overhead.  See file "VENDOR.DOC" for details.

NO  ORGANIZATION   IS  AUTHORIZED   TO  CHARGE   ANY  AMOUNT   FOR

DISTRIBUTION  OF  THE SOFTWARE  OR  DOCUMENTATION UNDER  ANY OTHER

CONDITIONS.  Organizations which charge a fee for distribution  of

any  and all TRAV materials, except as  noted above or without the

express,  written  consent   of  the  copyright  owner,   will  be

considered in violation of this copyright.

Registered User License

TRAV is a user-supported program, also  known as "ShareWare."  You

are permitted under the terms of this license to use this software

for 30  days without any payments.    If you continue  to use this

software past this point, you must register your copy.

Upon receipt of the $50 registration fee, the TRAV copyright owner

will license the user to use the TRAV product.

TRAV may  be used on  a Local Area  Net (LAN) provided  a separate

license  is  obtained for  each  station  that will  be  using the

program.  This version  of TRAV does not specifically  provide LAN

support.

Site licenses are available.  Write for information.

Trademarks

IBM, IBM  PC,  PC-DOS, and  PS/2 are  trademarks of  International

Business  Machines   Corporation.     MS-DOS  and   Microsoft  are

trademarks of  Microsoft Corporation.   Other  terms suspected  of

being  trademarks   or  service  marks  have   been  appropriately

Capitalized.  I cannot attest to the accuracy of this information.

Use of a  term should not be regarded as affecting the validity of

any trademark or service mark.

2

Chapter  2

Introduction

TRAV  is a  PC  based program  which  performs geometric  traverse

calculations.  Traverse  calculations are most often  performed in

the  civil  engineering  and surveying  fields  but  are generally

applicable to  any field concerned  with the determination  of the

location of points on the earth.   This program uses principles of

plane surveying and does not incorporate geodetic corrections.  It

is not suitable  for traverses of  large areas or distances  where

the curvature of the earth must be taken into account.

A traverse is comprised of starting and ending coordinates and one

or  more  courses  which,  when  followed  sequentially  from  the

starting coordinate, lead to the ending coordinate.  TRAV  Version

1.06 allows traverses of  up to 80  courses.  Provisions are  made

for  allowing rotation,  compass rule  adjustment,  calculation of

area of closed  traverses, solution  of unknowns,  and saving  and

retrieving of traverse files.

Requirements

TRAV is designed  for use with IBM or true IBM compatible personal

computers  including the  IBM PS/2  series.   A hard  disk is  not

required.  A hard  disk or RAM disk will increase  performance and

simplify use, however.

TRAV version  1.06 runs  under PC-DOS  or MS-DOS  versions 2.1  or

greater and requires approximately 170K of RAM.

TRAV does not require the use of a math co-processor.

TRAV has been successfully  used on Monochrome, CGA, EGA,  and VGA

displays.  This program performs direct memory writes to the video

buffer.  PC systems that are not hardware compatible at this level

will probably not be capable of executing TRAV.

If you are using  a portable or  laptop LCD display, setting  MODE

BW80 before starting TRAV will  cause TRAV to use black  and white

instead of color screen displays.   This may improve legibility on

this type of display.  On the other hand, if you are using a color

display  and TRAV  is  displaying  black  and white  screens,  try

setting MODE CO80.   Some programs I have run into change the mode

setting and don't  clean up after  themselves leaving the mode  in

the wrong state.

Installation

Before installing TRAV, make a working copy of the program and all

of  its documentation  files.   Put the  original  away in  a safe

place.

TRAV is completely self-contained and does not require any special

installation procedure.  You can execute  it from diskette or copy

it to a hard drive as desired.

3

If  you  are  going to  install  TRAV  on a  hard  disk,  create a

directory where you would  like to keep your traverse  files. Copy

the TRAV files into  this directory.   If you want  to be able  to

execute TRAV from any  directory, include the TRAV directory  in a

PATH  statement.    Or, if  you  already  have  a general  utility

directory in a PATH statement,  you may copy the TRAV.EXE  file to

that directory instead.   Refer to  your DOS Reference manual  for

information about the PATH statement.

If TRAV is to  be run from  a diskette based PC  you may make  the

working  copy diskette bootable, if desired, by formatting it with

the FORMAT command using the /S  option prior to copying the  TRAV

program files onto it.  For example: FORMAT A: /S

If  the  working  diskette  is  made  bootable,  booting  from the

diskette will automatically  start the TRAV  command.  If this  is

not  desirable,  delete  the AUTOEXEC.BAT  file  from  the working

diskette.

4

Chapter  3

Overview of TRAV

TRAV  is a  PC  based program  which  performs geometric  traverse

calculations.  This program uses principles of plane surveying and

does not incorporate geodetic corrections.  It is not suitable for

traverses of large areas  or distances where the curvature  of the

earth must be taken into account.

A traverse is comprised of starting and ending coordinates and one

or  more  courses  which,  when  followed  sequentially  from  the

starting coordinate, lead to the ending coordinate.  A course is a

directed  line  segment  made up  of  a  bearing  and a  distance.

Bearings are angles measured from the  north and the south, toward

the east and  the west --  they can never  be greater than  ninety

degrees.  TRAV expects angles to be expressed in terms of degrees,

minutes, and seconds.  Thus, "N 40  0 0 E" would be interpreted by

TRAV as "North 40 degrees, 0 minutes, and 0 seconds East".

Traverses may be open or closed.  An open traverse is one that has

starting   coordinates  that   are  different   from  the   ending

coordinates.    A closed  traverse  begins  and ends  at  the same

coordinate.    In  a  closed traverse,  the  courses  describe the

boundary of a closed figure and  TRAV can optionally calculate the

area of that figure.

Traverses  are  normally  calculated  using  known  bearings   and

distances to determine the  coordinates at the end of  each course

and to  verify that the given  courses will in fact end  up at the

ending coordinates.  However, it is  possible to use a traverse as

a design tool to solve  for unknown distances and bearings.   TRAV

can solve various  combinations of unknown bearings  and distances

including combinations of  one unknown  (closure cannot be  forced

for one unknown) and  combinations of two unknowns.   In addition,

TRAV  makes  special  provision  for  solving reverse  curve  with

tangent  problems  by assuming  that for  three and  four unknowns

unknown bearings are at right angles.

Calculations   are  performed  in   floating  point   with  eleven

hexadecimal digits in  the mantissa.  This  roughly corresponds to

13 significant decimal digits.   This should be adequate  for most

uses.  However, due to the  large number of calculations involved,

truncation and significance errors  can occur if the starting  and

ending  coordinates  or the  distances are  very  large.   This is

especially true for  area calculations  and solution of  unknowns.

Users  attempting  to  use  actual  Lambert polyconic  coordinates

should be cautious and review the results carefully.

TRAV is menu  oriented.  Menu Items may be selected through use of

the "arrow" keys or by typing the capitalized letter which appears

in the menu  item you wish  to select.  Lower  level menus may  be

exited by either  selecting "Exit"  from the menu  or by  pressing

"ESC".

5

Data is entered in data entry fields.  Data entry fields appear in

reverse video to  distinguish them from non-modifiable  text which

may appear  on the screen.  Data may  be entered into these fields

using the standard  keyboard functions of "Insert",  "Delete", and

"Backspace".  The  left and right arrow  keys may also be  used to

position  the cursor  to a  given  position in  the field  so that

modifications may be  made to previously  typed information.   The

CNTL X key combination will clear  the field if you wish to  start

over fresh.   When  the field contains  the data  in the  form you

desire, pressing the "Enter" key or "Tab" key will accept the data

and proceed to  the next field (if  any).  In addition,  the "Esc"

key may be used to cancel entry.

Context sensitive help  can be obtained  for data entry fields  by

pressing key "F1" while the field is active.

TRAV  displays  results  in a  scrollable  format.    Scrolling is

controlled by  the up and  down arrow keys to  scroll a line  at a

time, or the PGUP and PGDN  keys to scroll a screen at a  time, or

the HOME and END keys to scroll to the top or bottom respectively.

Entering a traverse

To begin  a traverse,  select "Begin  a traverse"  on the  primary

selection menu.  The program will display an input screen with the

cursor positioned in a data entry  field for the title.  Type  the

desired title  and press "Enter"  and the cursor will  move to the

data entry  field for  subtitle.   Type the  desired subtitle  and

press  "Enter".   These  titles are  optional  but recommended  to

document your printout and/or saved traverses.

Next, you will  be positioned  at the rotation  data entry  field.

This  field is pre-set  to zero degrees  rotation.  If  you do not

want to rotate the traverse, press  "Enter".  Otherwise, enter the

desired rotation angle.

Next,  you  will  be  positioned  at   the  beginning  and  ending

coordinates data  entry fields.   The standard  form for  entering

coordinates is:   1234567.89 1234567.89 where the first  number is

the Northing and the second number is  the Easting.  Commas may be

used to  improve readability  and are  ignored (e.g.  1,234,567.89

1,234,567.89).  These fields are initialized to  an arbitrary zero

coordinate.    If this  is  satisfactory, press  "Enter" otherwise

enter the desired coordinates.

If more  than one  traverse is  run in  the same  session, on  the

second and  subsequent traverses,  if there  is no  change in  the

coordinates  you  may enter  "S" for  same.   Coordinates  used or

calculated in the immediately preceding traverse may also be used.

This is done  by recalling  the coordinate pair  in the  following

manner: "R#"  where "R"  stands for  RECALL and #  stands for  the

number of the course ending in the coordinate pair to be recalled.

The  starting  coordinates  may be  recalled  by  "R0",  while the

closing coordinates my be recalled by a number  1 greater than the

number of the last course.

6

Entering an "A"  for area under  closing coordinates will set  the

closing coordinates equal to the  beginning coordinates and select

the area calculation option.

After the starting  and ending coordinates have  been entered, you

will enter the  courses of the traverse.   Courses are entered one

at a  time until  either the  maximum number of  courses has  been

entered  (80)  or  you have  entered  a  "E" or  pressed  "ESC" to

indicate that the end has been reached.  If you have made an error

in input and wish to  go directly to the modification menu,  press

"alt-m".   "Alt-m"  is  equivalent to  pressing "ESC"  followed by

selection  of the  modification option  on  the primary  run menu.

After correcting the errors,  you may return to the  initial input

mode by pressing  "alt-i" while still in  the modification option.

The basic input format  for courses is Bearing (deg.,  min., sec.)

and distance. For example: N 33 15 15 E 66.04 represents a bearing

of N 33 15  15 E for a distance  of 66.04 feet.  Please  note that

the  "N" and "E"  to denote North  and East may  be entered in any

position and  need not  surround the  bearing.   Thus "NE  30 0  0

100.55", "30 0 0 NE 100.55", and "N30 0 0E 100.55" are equivalent.

However, order is significant.  Entering "EN" instead of "NE" will

convert the bearing to the standard  "NE" by taking the complement

of the angle.

TRAV can  solve for various  combinations of unknown  bearings and

distances.  To indicate  an unknown bearing, either enter  "??" or

omit the NSEW reference and enter 0 0 0 .   To indicate an unknown

distance, enter "?" or 0.

It is also possible to go RIGHT  (R) of, LEFT (L) of, OPPOSITE (O)

of, or SAME (S) as a previous course.  This is accomplished by use

of "R"  "L" "O" "S" followed by the  number of the course followed

by  an  optional deflection  angle (in  deg.,  min., and  sec.) --

options  "R" and  "L" only as  "O" and  "S" do not  use deflection

angles --  and an optional distance.   If the deflection  angle is

omitted, the default  of 90 degrees is  used.  If the  distance is

omitted, the distance  will be the  same as the course  specified.

For example: R2 30 0 0 100 means turn 30 Deg. 0 min. 0 sec. to the

right of course  2 and  go 100 ft.;  L2 58.56  means turn left  90

degrees from course 2 and go a distance of 58.56 ft.

DO NOT use R, L,  O, or S to define a course based  on an unknown.

TRAV does not copy the status of unknowns when using R, L, O, or S

and  the defined course will  not be considered  an unknown.  This

was  done  to  allow  conversion  of   unknowns  to  knowns  after

calculation.    The  procedure  is  simple.    Enter  the  courses

indicating unknowns where  necessary.   Select "Run" to  calculate

the unknowns.  Select "Modify item".   Move the cursor to the line

containing the  unknown you  wish to  convert to  known and  press

"ENTER".  Press "CNTL X" to clear the data entry field and enter S

followed by the  line number  and press "ENTER".   The  calculated

value of the unknowns will be copied and the course will no longer

contain unknowns.

7

Curves are indicated by  placing a "+" or "-" on  the course which

begins at  the point  of curvature  and terminates  at the  radius

point.   If  area calculations  are  selected, the  sign indicates

whether the sector area  is to be added to or  subtracted from the

area of the traverse.  If area calculations are not  selected, the

sign may be either "+" or "-" but it is good practice  to indicate

a  sign appropriate  to the geometry.   If an  adjustment is made,

radial courses are held.

Specifying  a  curve  through  the  use   of  two  radial  courses

introduces  a problem.    Two radial  courses actually  define two

curves.   One has  a subtended  angle less  than or  equal to  180

degrees and the  other has a subtended angle 360 degrees minus the

first angle.  Which curve is used?  The majority of the time,  the

smaller angle is  the correct one to  use and TRAV always  selects

this curve.  If you do have  a situation where the angle subtended

is greater than 180 degrees, you will need to break the curve into

two or more  segments each of which  is less than or  equal to 180

degrees.

It  is  also possible  to enter  an  optional description  for the

endpoint of a course.  To enter the description, enter a semicolon

after the bearing and distance and follow with the description.

After all  the courses  have been  entered, enter  a "E"  or press

"ESC" to indicate that the end  has been reached.  An option  menu

will be  displayed.  Proceed  by selecting an  appropriate option.

Pressing "alt-m" instead of "E" or "ESC" will also terminate input

and directly select option "Modify".

Options

The following options are available after entering or retrieving a

traverse:

Option "Run" will  calculate the traverse and  display the results

in a scrollable format.

Option "Show"  will display  the current traverse  input data  for

review or printing.

Option "Modify" will display the  current traverse input data  and

allow you to make changes.  See chapter 4 for details.

Option "File" will allow you  to save, retrieve, delete traverses,

or change the default directory.  See chapter 5 for details.

Option "Print" will allow you to print the displayed traverse on a

local PC attached printer.

Option "rOtate" will allow you  to specify an angle by  which, the

traverse will be rotated about the starting coordinate.  The angle

is  expressed as  deg. min. and  sec. and  a sign to  indicate "+"

clockwise rotation, or "-" counter clockwise rotation.

8

Option "aDjust"  will perform  a simple  compass rule  adjustment.

Radial courses are  held.  If the  traverse is closed,  any course

may be used as the basis of bearings -- You  may select the number

of the course you want.  Determination of whether an adjustment is

warranted or appropriate is left  to the professional judgement of

the user.

Option "Area"  will select area calculation for a closed traverse.

If area  is selected,  the menu  item will  be shown  with a  "+".

Selecting area again will turn off the option.  Area is calculated

by the "Double  Meridian Distance"  method and is  subject to  the

limitations of that method.   Courses or curves must  not cross or

intersect each other as this would cause some portions of the area

to  be  subtracted and  result  in  an incorrect  answer.   Radial

courses  are  exempt  from  this   restriction  as  they  do   not

participate in area calculations --  area calculations go directly

from  one curve endpoint  to the other,  along the chord,  and the

area of the circular segment is  calculated separately.  Also, the

area may be incorrect if there is a error in closure.

9

Chapter  4

Modifying Traverse Data

After a traverse has been entered or retrieved, it may be modified

to correct errors or change the traverse.  To modify the traverse,

select  option "Modify"  from the menu.   A  modification sub-menu

will be shown, the current traverse  input data will be displayed,

and you will be allowed to make changes.

Traverse data  is displayed in  a line  oriented format  beginning

with   the  traverse  title  and  subtitle  and  ending  with  the

individual courses.   Use  the scroll  keys (PgUP, PgDn,  uparrow,

downarrow) to select  the line to be modified.   The selected line

is highlighted -- the scroll keys will move the highlighting up or

down.  When the line you wish  to work with is highlighted, select

the desired option ("Change",  "Insert", or "Delete") on the  sub-

menu and press "Enter".

If you selected "Change" or "Insert",  the modifiable data will be

displayed in  a  data entry  field and  you may  make any  desired

changes  or  additions.   Radius  point  indications  and endpoint

descriptions, if any,  must be left in  courses if they are  to be

retained.  Press "Enter" to accept  the change or "Esc" to  cancel

the change.  Pressing "F1" while the data entry field is displayed

will display context sensitive help information.

Data may  be entered  into data  entry fields  using the  standard

keyboard functions of  "Insert", "Delete",  and "Backspace".   The

left and right arrow keys may also be used to position  the cursor

to a given position in the field so that modifications may be made

to previously typed information.  The  CNTL X key combination will

clear the field if you wish to start over fresh.

Options  "Insert"  and  "Delete" apply  only  to  courses  and are

ignored for  other types  of data.   When  courses are  deleted or

inserted, subsequent courses are renumbered immediately after each

change.  A new course is  inserted following the highlighted line.

To insert  a course  in front of  the first  course, position  the

highlighted line on the descriptive heading  immediately preceding

the first course.   It is recommend that insertions  and deletions

be  done in  descending  order  from last  to  first  in order  to

minimize confusion and reduce errors.  You should also be cautious

when  using  the relative  command  forms that  reference specific

course numbers (S3, R5, L9 for example) to make sure that the line

number specified is indeed the intended course.

If you  wish  to insert  more than  one  line at  the end  of  the

traverse, you may press  "alt-i" to return to initial  input mode.

You  do  not  need to  be  positioned at  the  last  course before

pressing "alt-i".  Initial  input mode will resume ready  to input

courses following  the last previously entered course.   After all

additional courses have  been entered, exit  by entering "e",   or

"ESC" as usual.

10

Special mention needs to  be made regarding the way  in which TRAV

handles  circular curves.    Curves are  indicated  by entering  a

course  which  begins  on the  curve  (at  the  starting point  of

curvature)  and  proceeds  along  the  radial  to  the  center  of

curvature -- the length of the course  is the radius.  Enter a "+"

or "-" on this course to denote that this is the radius  point and

indicate  whether  the area  of  the  segment should  be  added or

subtracted if the  area of the traverse is to be calculated.  This

first radial course is followed by  a second radial course leading

from the  radius  point to  the  curve  (at the  ending  point  of

curvature).  Together,  these two  courses define a  segment of  a

circle.

Because the  radius must be  the same  for each course,  TRAV will

automatically set the length  of the second course when  the first

is entered.   When making modifications,  it doesn't matter  which

course is  modified first,  the other  will be  changed to  match.

There are two exceptions to this.

First, when  the second radial  course is  deleted, the  following

course is  not changed.   This allows  you to  delete one  or more

courses  without  losing  the  length  of  the  following  course.

Insertions immediately following the first radial course will have

their length set  however.   Deletion  of the first  radial course

has the effect of undefining the curve.

The other exception  occurs when  the first radial  course is  the

last course in  a closed traverse and the  second radial course is

the  first course  in the traverse.   This wrap  around is ignored

during  entry and update operations and a  change to one length is

not automatically reflected in the other.

Both  exceptions are taken  care of  in the  pre-calculation phase

when  "RUN" is selected.   At  this time, a  check is made  of all

courses and any discrepancies  in radius lengths are caught.   The

course numbers and lengths of the  two radial courses is displayed

and you must select which length to use.  Occasionally, due to the

inability  to  exactly  represent  decimal fractions  in  internal

binary format, TRAV may complain about courses that appear to have

identical lengths.  If this occurs, simply select either course to

proceed.

Specifying  a  curve  through  the  use  of  two  radial   courses

introduces a  problem.   Two radial  courses  actually define  two

curves.   One has  a subtended  angle less  than or  equal to  180

degrees and the  other has a subtended angle 360 degrees minus the

first angle.  Which curve is used?  The  majority of the time, the

smaller angle is  the correct  one to use  and TRAV  automatically

selects this curve.   If you do  have a situation where  the angle

subtended is greater than 180 degrees, you will need to break  the

curve into  two or  more segments each  of which  is less  than or

equal to 180 degrees.

11

Chapter  5

Saving Traverses

After a traverse  has been entered, it may be saved for later use.

Select  option  "File".   A  sub-menu  will be  displayed  and the

current directory will be identified.

Select "Save traverse".  TRAV will prompt for the name of the file

you  wish  to  create  --  enter  up  to eight  alpha  or  numeric

characters.   A file will be created  in the current directory and

the  traverse will  be saved.    If a  file of  that  name already

exists, you will be asked to verify that you intended to overwrite

the existing file.

If you are using a two diskette system with no  hard disk, you may

wish to run TRAV by  placing the TRAV diskette in drive A  and set

the default directory to drive  B.  This will minimize  the amount

of  diskette  switching and  provide  flexibility to  save related

files on the same diskette.

Retrieving Traverses

To  retrieve  a saved  file,  select  "Work with  files"  from the

startup menu or "File" from the main option menu.  A sub-menu will

be displayed and the current directory will be identified.  Select

"Retrieve traverse".  If any saved traverse files are found in the

directory, a  list of  files will be  displayed.   Use the  cursor

arrow keys to move the highlighted box to the name of the file you

wish  to retrieve.  Press "ENTER" to  select the file and retrieve

it or press "ESC" to cancel the request.

To provide for future compatibility,  TRAV maintains a saved  file

version number  which identifies the format of  the saved traverse

file.  If  you attempt to retrieve  a file created by  a different

version of  TRAV you may  get a  message indicating that  the file

cannot be retrieved because the file level is different.  Whenever

a  change in the file format  occurs, a conversion utility will be

provided with TRAV to  convert the older  format files to the  new

format.  As  of version 1.06, only  one file format exists  and no

conversion utility is necessary or provided.

Changing the Default Directory

If  you want  to  work with  a  directory other  than  the current

directory,  select  "Work with  files"  from the  startup  menu or

"File" from  the main option menu.   A sub-menu will  be displayed

and  the current  directory will  be  identified.   Select "Change

directory".   TRAV will prompt for  the name of the new directory.

Enter the name of the new directory and press "Enter" to change to

the new  directory or "ESC" to  cancel the change.   The directory

specified  must  already  exist.    TRAV  will not  create  a  new

directory.

TRAV will restore the original directory at program termination so

you will end up in the same directory as when TRAV was started.

12

Deleting a Saved Traverse

To erase or delete a previously saved traverse , select "Work with

files" from the startup menu or "File" from the  main option menu.

A sub-menu will  be displayed  and the current  directory will  be

identified.   Select "Delete  Traverse".   If  any saved  traverse

files are  found  in  the  directory,  a list  of  files  will  be

displayed.  Use the cursor arrow  keys to move the highlighted box

to the  name of  the file you  wish to delete.   Press  "ENTER" to

delete the file or press "ESC" to cancel the request.

13

Chapter  6

Sample Traverses

Sample 1 -- A Square

The  following  example shows  some of  the  basic concepts.   The

traverse  describes  a  square with  sides  ten  feet  long.   The

traverse  starts  at an  arbitrary  starting coordinate  (0,0) and

proceeds due north 10 feet then due east 10 feet then due south 10

feet then due west 10 feet  and ends up at the starting  location.

This is a  closed traverse.  The  input data, as displayed  by the

"Show" option, is as follows:

Title:    Demonstration Traverse

Subtitle: Square traversed by sides

Rotation: +  0 00 00.00

Start:             0.00           0.00

End  :             0.00           0.00

Bearing         Distance   RP  Endpoint Description

1)  N00 00 00.00E         10.00      ;

2)  N90 00 00.00E         10.00      ;

3)  S00 00 00.00W         10.00      ;

4)  N90 00 00.00W         10.00      ;

When this  traverse  is calculated  using  the "Run"  option,  the

result is displayed as follows:

Demonstration Traverse

Square traversed by sides

Tuesday, Jan. 9, 1990

BEARING          DISTANCE             NORTH            EAST

STARTING COORDINATES=              0.00            0.00

CO.( 1)  N00 00 00.00E           10.00

10.00            0.00

CO.( 2)  N90 00 00.00E           10.00

10.00           10.00

CO.( 3)  S00 00 00.00W           10.00

0.00           10.00

CO.( 4)  N90 00 00.00W           10.00

-0.00            0.00

CLOSING COORDINATES=              0.00            0.00

CLOSURE=             -0.00            0.00

The  numbers under the  NORTH and  EAST headings  are coordinates.

They begin with the given starting coordinates and are  calculated

at the end  of each course.  The closure is the difference between

the  calculated  ending  coordinate and  the  actual  given ending

coordinate.  In  this example, the  closure is (-0.00,0.00).   The

minus sign indicates that the difference  is greater than zero but

14

less  then the rounded value 0.00.  In other words, the difference

is less than 0.005 feet.

Since  all  the   courses  in  this  sample  were  along  cardinal

directions, it  is a clear  example of  how rotation  works.   The

following  display  was  generated from  the  previous  example by

selecting option  "rotate" and entering +  45 0 0  degrees.  Also,

the "Area" option was selected.

Demonstration Traverse

Square traversed by sides

Tuesday, Jan. 9, 1990

ROTATION: + 45 00 00.00

BEARING          DISTANCE             NORTH            EAST

STARTING COORDINATES=              0.00            0.00

CO.( 1)  N45 00 00.00E           10.00

7.07            7.07

CO.( 2)  S45 00 00.00E           10.00

0.00           14.14

CO.( 3)  S45 00 00.00W           10.00

-7.07            7.07

CO.( 4)  N45 00 00.00W           10.00

-0.00            0.00

CLOSING COORDINATES=              0.00            0.00

CLOSURE=             -0.00            0.00

THE TRAVERSE HAS AN AREA OF         100 SQ FT, OR     0.0023 ACRES.

Sample 2 -- A Circle

The  following  example shows  more of  the  basic concepts.   The

traverse describes a  circle with  radius 10,000 feet  long.   The

traverse  starts  at  an  arbitrary  starting  coordinate  on  the

circumference of the circle and proceeds  due south 10,000 feet to

the center of the circle.  Then, it goes due south 10,000 feet  to

a point on the  circumference.  This describes one  semicircle, we

must now  describe the other semicircle.   We go due  north 10,000

feet to the center  of the circle and then again  due north 10,000

feet and  end  up at  the starting  location.   This  is a  closed

traverse.  The input  data, as displayed by the  "Show" option, is

as follows:

Title:    Demonstration Traverse

Subtitle: Area of circle divided into semicircles

Rotation: +  0 00 00.00

Start:        10,000.00           0.00

End  :        10,000.00           0.00

15

Bearing         Distance   RP  Endpoint Description

1)  S00 00 00.00W     10,000.00   +  ;Center of circle

2)  S00 00 00.00W     10,000.00      ;

3)  N00 00 00.00E     10,000.00   +  ;Center of circle

4)  N00 00 00.00E     10,000.00      ;

Please note the  use of the  endpoint descriptions on the  courses

ending at the center  of the circle.  Also note the use of the "+"

sign to indicate  that the course  terminates at the radius  point

and  that  the  area of  the  sector  is  to  be  added  for  area

calculations.   When this  traverse is calculated  using the "Run"

and "Area" options, the result is displayed as follows:

Demonstration Traverse

Area of circle divided into semicircles

Tuesday, Jan. 9, 1990

BEARING          DISTANCE             NORTH            EAST

STARTING COORDINATES=         10,000.00            0.00

CO.( 1)  S00 00 00.00W       10,000.00

Center of circle                                    0.00            0.00

CO.( 2)  S00 00 00.00W       10,000.00

-10,000.00            0.00

CO.( 3)  N00 00 00.00E       10,000.00

Center of circle                                    0.00            0.00

CO.( 4)  N00 00 00.00E       10,000.00

10,000.00            0.00

CLOSING COORDINATES=         10,000.00            0.00

CLOSURE=              0.00            0.00

PT ( 1) + R=  10,000.00  DELTA=180 00 00.00  T=************  L=31,415.93

PT ( 3) + R=  10,000.00  DELTA=180 00 00.00  T=************  L=31,415.93

THE TRAVERSE HAS AN AREA OF 314,159,265 SQ FT, OR 7,212.1043 ACRES.

Please note the location  of the description text in  the display.

You  will  also  note  that  additional  information  is  provided

describing  the  curve.   "DELTA" is  the  angle subtended  by the

curve. "T" is the tangent  distance (i.e. if lines tangent to  the

circle  at  the  points  of  curvature  are  extended  until  they

intersect, the tangent distance is the  distance from the point of

intersection to  the point  of curvature).   And  "L"  is the  arc

length.  In  this example, the curve is a  semicircle.  Therefore,

the tangent  lines are  parallel and will  intersect at  infinity.

This is too  great a distance  to print, so,  the field is  filled

with asterisks to indicate a value too large to print.

Sample 3 -- A Triangle

The following is an example of how  TRAV can be used to solve  for

unknown values.  The traverse describes a right triangle which has

16

an unknown bearing for the hypotenuse  and an unknown distance for

one side.  The traverse starts at an arbitrary starting coordinate

(0,0) and proceeds  north for 300 feet.   A ninety degree  turn to

the right is made and we proceed an unknown distance.  We now turn

an follow an unknown bearing for 500 feet along the hypotenuse and

arrive  at the starting  point.  This  is a closed  traverse.  The

input data, as displayed by the "Show" option, is as follows:

Title:    Demonstration Traverse

Subtitle: Triangle with two unknowns

Rotation: +  0 00 00.00

Start:             0.00           0.00

End  :             0.00           0.00

Bearing         Distance   RP  Endpoint Description

1)  N00 00 00.00E        300.00      ;

2)  N90 00 00.00E           ?        ;

3)        ??             500.00      ;

Note the use of ?? and ? to indicate the unknown items.  When this

traverse  is calculated  using  the "Run"  option,  the result  is

displayed as follows:

Demonstration Traverse

Triangle with two unknowns

Tuesday, Jan. 9, 1990

BEARING          DISTANCE             NORTH            EAST

STARTING COORDINATES=              0.00            0.00

CO.( 1)  N00 00 00.00E          300.00

300.00            0.00

CO.( 2)  N90 00 00.00E          400.00**

300.00          400.00

CO.( 3)  S53*07*48.37W          500.00

-0.00            0.00

CLOSING COORDINATES=              0.00            0.00

CLOSURE=             -0.00            0.00

The unknown  values  in  the  traverse have  been  calculated  and

flagged with asterisks.  The unknown distance in course 2 has been

calculated  and flagged  with "**"  while the  unknown  bearing in

course 3 has been calculated and flagged with imbeded "*".

17

Appendix A  --  TRAV Version 1.06 Registration Form

Name:          ______________________________________

Address:       ______________________________________

______________________________________

City:          ______________________________________

State:         __________________________ Zip: ______

Telephone:     ______________________________________

PC TYPE:       ______________________________________

DOS Version:   ______________________________________

Where Did You

Obtain TRAV: ______________________________________

Item                                   Quantity      Total

-----------------------------------------------------------

TRAV Registration ($50 each)  ________|__________|_________

TRAV Trial Disk ($10 each)    ________|__________|_________

(does not include registration)

-----------------------------------------------------------

Total Due:                                       |_________

Preferred Disk Size:        3.5" _N/A_        5.25" ____

(3.5" Disks not available at this time)

____________________________________________  Date:____________

Signature

Mail Completed Form to:

Robert Jones

3534 Hawkwood Rd.

Diamond Bar, CA 91765

18

Appendix B  --  TRAV Version 1.06 Bug Report Form

Name:          ______________________________________  Date:_______

Address:       ______________________________________

______________________________________

City:          ______________________________________

State:         __________________________ Zip: ______

Telephone:     ______________________________________

PC TYPE:       ______________________________________

DOS Version:   ______________________________________

Bug Description:___________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

Mail Completed Form to:

Robert Jones

3534 Hawkwood Rd.

Diamond Bar, CA 91765

19

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

Public (software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  PsL cannot debug pro-

programs over the telephone, though we can answer questions.

Disks in the PsL are updated  monthly,  so if you did not get

this disk directly from the PsL, you should be aware that the

files in this set may no longer be the current versions. Also,

if you got this disk from another vendor and are having prob-

lems,  be aware that  some files may have become corrupted or

lost by that vendor. Get a current, working disk from PsL.

For a copy of the latest monthly software library newsletter

and a list of the 2,000+ disks in the library, call or write

Public (software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

1-800-2424-PSL

MC/Visa/AmEx

Outside of U.S. or in Texas

or for general information,

Call 1-713-524-6394

PsL also has an outstanding

catalog for the Macintosh.