OILDECL & OILPROJ

Copyright (c) 1990, H L Hendrick , all rights reserved.

H L Hendrick is a member of  the  Association  of  Shareware

Professionals  (ASP).  ASP  wants to make sure that the shareware

principle  works  for  you.  If  you  are  unable  to  resolve  a

shareware-related  problem  with  an ASP member by contacting the

member directly,  ASP may be able to help.  The ASP Ombudsman can

help,  but  does  not  provide  technical  support  for  members'

products.   Please write to the ASP Ombudsman at  P.O.  Box  5786

Bellevue,  WA  98006 or send a CompuServe message via easyplex to

ASP Ombudsman 70007,3536

OILDECL is a shareware microcomputer  program  which  allows  the

user to analyze declining production from producing oil wells and

to make projections of future production,  net cash flow and dis-

counted net cash flow based on that analysis.

OILPROJ is a microcomputer  program  which  allows  estimates  of

production  from new prospects which are not in production at the

time of the estimate by using known factors  from  wells  in  the

area.

Use OILDECL to determine intercept and decline factors from wells

in  the area then use OILPROJ and substitute these factors in the

program to determine production,  cash flow and  discounted  cash

flow.

Minimum requirements are a one disk drive microcomputer with 256K

memory  and an 8086/8088 processor using MSDOS 2.xx or compatible

computer.  A printer is optional.

In order to use the program,  format a new blank disk and install

your operating system on it.  Insert the program disk in drive a:

and  copy  the  program  to  the  new formatted disk in drive b:.

Store the program disk furnished by H L Hendrick and use the disk

you have created as your program disk.

Insert your program disk in drive a: and boot  up.    Follow  the

menu prompts.

The following entry is required:

1) Enter Operator:

2) Enter well name:

3) Enter month of 1st data point entry:

This entry must be a three letter month designator and may be

entered in lower or upper case letters. i.e. January is

entered as jan or JAN.

1

4) Enter year of 1st data point entry:

This entry must be a two number entry of the last two numbers

of the year of the 1st entry. i.e. 1984 is entered as 84.

5) Enter gross price of oil:

Enter price in dollars. i.e. 16.66.

6) Enter annual change in price:

This entry is the annual percentage of change of price

expressed in decimal fraction.

7) Enter state severance tax:

Must be the per-cent entered in decimal fraction.

8) Enter monthly operating expense:

Must be entered in dollars. i.e. 2000.00.

9) Enter annual change in operating expense:

Must be annual per-cent change in operating expense entered as

a  decimal fraction.

10) Enter division order interest or revenue interest:

Must be entered as a decimal fraction.

11) Enter working interest:

Must be entered as a decimal fraction.

12) Enter annual interest rate to be used for discounting:

Must be entered in a decimal fraction.

The  screen  will  clear  and  the  prompt  "Enter number of data

points" will be displayed.

This entry is the total number of monthly production data  points

which  you are going to input to generate the decline curve equa-

tion.

You will be prompted to enter the  monthly  production  for  each

data  point entry until you have entered all data points.  A zero

is not an acceptable entry.  If a zero production month is in the

sequence  of  data points,  adjust the sequence and eliminate the

zero month entry.

The method of least squares is used to derive equations for hyper-

bolic and exponential decline for the entered data.

The selection menu will be displayed as follows:

Selection Menu

0-Exit

2

1-Hyperbolic

2-Exponential

3-Economic Limit

Make your selection based on the type of decline exibited by your

well.

The number of projections is requested.   Select the  number  you

want.  The first projection displayed will be the month after the

last data point entry.

Each  month,  year,  barrels,  net revenue and discounted revenue

will be displayed.  Annual totals, cumulative totals and the equa-

tion used in the calculation are displayed.

A print out option is requested.

If the economic limit option is selected,  the type of decline is

requested.   The program will run until the net  revenue  becomes

zero.

A print out option is requested.

Once the economic limit begins running,  the program must run un-

til it finishes or you enter Control break.

All calculations are based on monthly production using  end-month

values.

If the per-cent change of any requested value is negative a minus

sign must precede the numerical entry. These values are converted

from the annual entry to monthly values for use in the program.

The discounting and price change is done on an end-month basis.

Recursive operation of the program is supported with a menu selec-

tion option to change any or several of the input values.

3