TeleShare Quick Start and Quick Fixes      Page 1

UPDATED: November, 1994

This Product is Shareware, if you continue to use it

Please Legalise your usage through Registration.

_______

____|__     |               (R)

--|       |    |-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

-----|   |   |---------------------

|___|___|    Member

Quick Start

Getting the System Up and Running:

You may have already used TSSETUP to configure the host or

remote, and may prefer to bypass this paragraph.  The first

thing is to ensure that the modem works with the software that

came with the modem.  They are basically guaranteed to be

compatible.  If/Once you know that they work together, steal

the settings from the original software (write them down or

print screen them) and transfer the values to TeleShare.

There is a useful tool called PORTEST that you should run to

verify what ports you are using.  In this (November, 1994)

release of TeleShare, simply put the initialization string in

a file called INIT, then run SCOMMUPD to update SCOMM's

values.  Start up tssetup.exe and set it to match your other

recorded values.  Then start up SCOMM, select Directory and

enter a test number.  Then call the number to ensure that the

port setup is correct.  If it fails consider changing from

port 1 to 2 or vice versa.  If it still fails after that check

out the quick fixes below.

Quick Fixes

The most basic and common problems that you are likely to bump

TeleShare Quick Start and Quick Fixes      Page 2

into are related to the modem.  The following advice should be

considered like a sandwich, with general advice like this

above and below and various likely materials in between.

First, some simple tests:

Run PORTEST and record what it yields.  If it locks up your

PC, turn the the machine off for 15 minutes (YES).  Some of

the internal settings are zonked and need time and 'cooling'

to self reset.

Second, run some batch files, or run these commands directly

from DOS.  Lets assume portest reports your modem is using

port 1 (COM1).

REM  Reset the modem.  Watch its lights.

ECHO ATZ > COM1

REM  Tell it to answer.  Listen and watch its lights.

ECHO ATA > COM1

REM  Tell it to hang up.  You might set this up after a

REM  pause in the same batch file as the ATA command.

ECHO ATH0 >com1

If you are getting responses, the COM port is correct and the

modem is responding.  If you aren't getting responses, try

changing COM ports.  If you're getting responses and you know

the port is correct turn the machine off for awhile (a longer

while if necessary).

Dvorak's Guide to PC Telecommunications has a list of the most

common problems, which has been interpreted and amplified

below:

1) Call waiting has not been cancelled.  You can often dial

70# or *70 or try setting both modems to ATS10=255 for a long

wait to bypass call waiting.

2) An external modem cable is bad or loose.

3) The phone line is disconnected at the wall or the modem.

Test push the connections together.

4) The communications software is incorrectly setup.  Try

N81.  It has to be trying to work with the correct port,

preferably COM1 or COM2.  Try changing ports.  A fair number

of P.C.s cannot use COM3 or COM4 for communications (or

anything else that matters).

TeleShare Quick Start and Quick Fixes      Page 3

5) Modem switches or jumpers are incorrectly setup.  Is the

modem switched on?  (Our most common problem is leaving the

external modem turned off for the first call of the day).

6) The modem is competing with another device for a COM port.

No you cannot physically attach a mouse and a modem to the

same port, but you can do so with software.  Test by

deactivating the mouse, or scanner, or whatever SOFTware it

might be.

7) Its the wrong type of cable.  Sorry you cannot tell

visually.  Direct connection and modem cables look alike, but

are wired differently.

8) Switches inside the computer are set incorrectly.

Conflicting computer interrupts can cause significant hair

loss in people trying to solve the problem.  We supply the

RS232OFF.EXE to reset interrupts that have gone nuts (You just

type in "RS232OFF").  Yes, the original settings can all be

fine, but some other software zonks the interrupts.

Apparently, a common example is Procomm which apparently

resets more ports than just the one it is using.

9) There is a BIOS problem.  AMI bios's have problems with

16550 chips.  P. C. Softsmith has found common problems with

16550 chips user inserted into cards.  Many cards are really

designed to work with older chips and do not let a 16550 chip

activate properly.  Normal diagnostics (and technicians) will

pass everything, but a separate test will find the 16550

acting like an older chip.  If you buy the card with a factory

inserted 16550, they should match and work fine.  We can

supply, or you can download, software from a BBS that 'fixes'

the AMI bios to 16550 problem.  A common name is

550RESET.exe.

10) The software is incompatible with the modem.  Their two

features sets fail to overlap in some crucial area.  Try some

other sofware, even a simple dialer, to make sure you have

correctly isolated the problem.

11) The modem is broken or defective.  We have found some

modems that overheat then gradually fail are very good tools

for detecting problems with telephone lines.  Voice checking a

phone line does not indicate its condition for a modem, but

your 'phone company can likely check it remotely from their

office.

Try to get anything working, then steal its

settings/information for the other things that you want to get

running.  If you have the system basically working, then keep

TeleShare Quick Start and Quick Fixes      Page 4

changing small portions out from the working core until they

work.  As an example, we often just erase the TSCONFIG.CFG

setup file and restart when a problem continues, because it is

the most basic setup file.  Many problems disappear once

everything is reset to original defaults.