DOCUMENTATION HANDBOOK

VK3UM AUTOTRACK

Version 5:10 Nov 1993

INDEX

Para  Section                                    Page

1.0   Introduction ............................... 2

2.0   Hardware requirements ...................... 3

3.0   Software requirements ...................... 3

4.0   Main Menu .................................. 4

5.0   Main Menu Options .......................... 4

6.0   Tracking Modes ............................. 4

6.1  Real Time Track ....................... 4

6.2  Auto Track ............................ 5

6.3  Graph Track ........................... 6

7.0   General Set Up ............................. 10

7.1  Home Location Change .................. 11

7.2  Summer/Winter Time .................... 11

7.3  Operating Frequency ................... 11

7.4  UTC/Local Computer Time ............... 11

7.5  Display Source Target ................. 11

7.6  Transmit Sequence ..................... 11

7.7  Period of Transmission ................ 11

7.8  Warning Sound On/Off................... 11

7.9  Save Set Up parameters ................ 11

7.10 Change Computer Time .................. 11

8.0   Auto Track Set Up .......................... 11

8.1  Mode Selection Analog/Digital/Pulse ... 11

8.2  Digital Set Up Parameters ............. 12

8.3  Analog Set Up Parameters .............. 12

Motor Run On .......................... 12

8.4  Pulse Set Up Parameters ............... 12

8.5  Home (Park) Position of Antennae ...... 14

8.6  Azimuth Motor Hysteresis .............. 14

8.7  Elevation Motor Hysteresis ............ 14

8.8  Drive Update Period ................... 14

8.9  End Stops of Antennae ................. 14

8.10 Communication Port .................... 15

8.11 Antennae Mount ........................ 15

8.12 Motor Speed Control ................... 15

8.13 Save Parameters to Disk ............... 15

9.0  Printed Circuit Board Interconnect .......... 16

10.0  Component List and Sources .................. 18

11.0  Check Out Procedure ......................... 20

12.0  Summary ..................................... 22

12.1  Printed Circuit Board Changes ......... 23

13.0  ADDENDUM OF FUNCTION KEYS USED .............. 23

13.1 Real Time Track Screen ................. 23

13.2 Auto Track Screen ...................... 23

13.3 Graphic Screen ......................... 24

13.4 Target Screen Options via [F2] ......... 24

1.0 INTRODUCTION

The Software was developed primarily as an aid for Amateur

Radio Operators wishing to communicate by reflecting

signals off the Moon. This communication mode is commonly

referred to as EME. (Earth -Moon - Earth).

The facilities provided by this software, when used with

the VK3UM Serial Interface controller attached to your

Computer, will allow you to automatically track the Moon,

(or any other Quiet or Noise Source).

The Software and Hardware has been designed to suit Analog,

Digital and Pulse Tracking Systems and may be used in both

Hemispheres.

Facilities are provided to Graph Noise, and Moon Echos. You

can also store these graphs on disk and recall them at a

latter date. Moreover the graphs may be overlaid for

comparison purposes. Printing of these graphs has also been

provided. Help instructions are available for graphic

facilities via [F1] within the Software.

The Software is quite complex and is an on going project by

the author. Not all the Authors' proposed facilities have

been included at this stage (Automatic Polarisation

Tracking for example). Accordingly when revisions become

available they will be announced and distributed at a

moderate cost. All options described in this document apply

to this version of the software.

It should be clearly understood that this Project is for

Amateur purposes and should any bugs be found or you have

suggestions for improvement please convey your

correspondence to the following :

Doug McArthur (VK3UM)

30 Rolloway Rise

Chirnside Park

Victoria 3116

AUSTRALIA

Home Phone        (61 3 7264484)

Work FAX - 24Hrs  (61 3 7286464)  ** Inwards Only **

Note. I am +10 hours ahead of UTC.

Again I stress that this is an Amateur project developed on

a cost covered basis for the advancement of our Hobby.

Unlike my EME PLANNER SOFTWARE that has been distributed

free for many years, the cost of the very complex interface

board has to be covered.

2.0 HARDWARE REQUIREMENTS

This Software will only run with the associated Interface

board. The board is complex and contains 20 "standard

IC's", 8 miniature relays and transistors plus an

assortment of capacitors and resistors. Because of the

complexity it was decided to produce the board as a plated

through hole (980!) fiber glass identity. A parts layout is

provided to clearly identify placement of components. A

circuit has not been provided as such is deemed unnecessary

due to the simplistic nature of construction. It is

recommended, but not essential, that IC sockets be used.

Input and output connections are via standard IDC

connectors. It is left to the user to package the unit as

desired. A +5v regulator has been provided on the board (as

well as reverse supply protection) for those that wish to

feed the unit with an unregulated +7 - +10 volt supply.

Total current, which is dependent upon the number of relays

operated at the time, is approximately 350mA.

3.0 SOFTWARE REQUIREMENTS

IBM or a compatible XT/AT/286/386/486 configuration is

necessary. The Graphic options used for graphing requires a

EGA or VGA driven monitor. Auto Track is provided for

monochrome, Hercules and CGA monitors however the extra

graphing facilities are not available with these monitors.

The software is "downward compatible" to EGA for higher

resolution screens. (350x640 16 Colors). The program

requires approximately 330k RAM to run.

NOTE  IF YOU DO NOT HAVE THE INTERFACE CONNECTED AND YOU WISH TO

RUN THE SOFTWARE THEN SELECT THE REAL TIME TRACK OPTION.

THEN PRESS THE HOME KEY AND THE SOFTWARE WILL BE CONFIGURED

TO THE DEMONSTRATION MODE.

This will select the Pulse Mode with internally generated

Pulses. To return to interface control exit the program and

re-boot.

3.1.1  Software/Hardware Debugging Functions.

These can only be selected from the Auto Track Screen but

are used in the graphics screen.

3.1.2  [F1]. When in the AutoTrack Screen and when using the Pulse

Mode, this will enable the display of Azimuth and Elevation

Pulse Counts. Pressing [F1] again will disable this

display. This is a useful option when calibrating your

Pulse System if you are unsure of the number of pulses per

degree. (pulses per 360 degrees is required to configure

your input for the pulse mode). This is achieved by first

turning on the pulse counter and running your drive over a

known number of degrees of travel. The pulse count will

indicate the number of pulses for that travel. Multiply the

number of pulses by the number of degrees and you have the

pulses per 360 degrees required in section 8.4. Note you

can input decimal degrees.

3.1.3  Control[F1] will reset the Pulse Counter.

3.1.4  Shift[F1]. When in the AutoTrack Screen and when using the

Pulse Mode, this facility will generate simulated pulses

from within the Software without the need for an external

pulse input. The one hundredth digit will turn red when

internally generated pulses are selected. Pressing

Shift[F1] again will return the control to the external

pulse input. This facility can be used as a training aid to

become familiar with the software and all the options

provided.

4.0 MAIN MENU.

After the opening Logo Screen the Main Menu Headings are

displayed. The following sections provide the user with the

information necessary to configure and interface your

system to the hardware. Please read this carefully as the

software is most comprehensive and abounds with many user

options. The [F1] key provides the Version Number of the

Software.

5.0 MAIN MENU OPTIONS.

Selection of these options may be achieved by either moving

the highlighted bar with the cursor keys (arrow keys) or by

typing the first letter of that option required. For

detailed description please refer to the following

sections:-

6.0 Tracking Modes.     7.0 General Set Up.

8.0  Auto Track Set Up.

6.0 TRACKING MODES.

Before running the various tracking modes please set all

your parameters to suit your Station as detailed in

Sections 7.0 and 8.0. This Sub Menu enables selection of

the Tracking Mode desired. The Real Time Track option

provides the location of the Tracking Source commensurate

with your Computer Clock. The Auto Track Modes, in

association with the Serial Interface, enable the automatic

or manual control of the antennae.

6.1 Real Time Track.

This option provides real time (Computer Set Time),

calculations of Azimuth, Elevation, Doppler Shift for any

"Tracking Source" (Moon - Sun - Noise), which may be chosen

from General Set Up Sub Menu or [F2] from within this

screen. This mode does not allow for automatic Antennae

control as is provided in Auto Track. It does provide

Azimuth/Elevation and Doppler Shift of the "Tracking

Source" relative to your location and Computer Time. The

screen will indicate both the Transmit (red) and Receive

(blue) periods and sound a warning prior to the change over

sequence. The Tx Sequence and Period as well as the bell

may be set as desired in the General Set Up Menu. (refer

7.6 and 7.7).

6.1.1  [F2] key will allow you to select any of the available

Target Sources available. The [TAB] key allows you to

select additional noise sources.

6.1.2  [F3] key will allow you to input a DX Station's Grid

Square. This will then provide a display, in the centre of

the screen, of the Azimuth, Elevation, Doppler Shift and

Spatial Offset of the distant station. The latter two

displays will only be present if you both have coincident Moon.

6.2 Auto Track.

This option provides Auto Tracking of any Target Source

provided in the EME Software. The Auto Track Setup Menu

allows you to configure the Software to suit both the

Hardware Interface and your own specific Station

requirement. The Antenna will then track the Target Source

or you may manually steer the Array as desired.

6.2.1  [F1] will this will enable the display of Azimuth and

Elevation Pulse Counts. This is a useful option when

calibrating your Pulse System if you are unsure of the

number of pulses per degree. This is achieved by first

turning on the pulse counter and running your drive over a

known number of degrees of travel. The pulse count will

indicate the number of pulses for that travel. Multiply the

number of pulses by the number of degrees and you have the

pulses per degree and then translate this to the number of

pulses per 360 degrees as required in section 8.4. Note you

can input decimal degrees but the greater the travel will

provide a higher order of accuracy. Pressing [F1] again

will disable this display. (Pulse Mode only).

6.2.2  Shift[F1] will generate simulated pulses from within the

Software without the need for an external pulse input. The

one hundredth digit will turn red when internally generated

pulses are selected. Pressing Shift[F1] again will return

the control to the external pulse input. This facility is

most useful as a training aid to become familiar with the

software and all the options provided. (available only for

Pulse Mode of operation).

6.2.3  Control[F1] will reset the Pulse Counter. (Pulse Mode only)

6.2.4  [F2] key will allow you to select any of the available

Target Sources available or input a Azimuth Bearing. The

positions of all the Targets are available on screen.

This facility will also permit using a beacon as a source and

plot a beam pattern or position the array to a known Az / El

for Auroa or FAI transmissions. In addition you may input a Grid

Square for terrestrial purposes. The software provides you

with the Azimuth bearing and distance.

6.2.5  [F3] key initiates the Automatic Tracking functions. When

in the automatic tracking mode ANY key will disable the

automatic tracking and return you to Manual Tracking.

6.2.6  Shift[F3] allows you to input a DX Station's Grid Square.

This will then provide a display, in the top left hand

corner of the screen, of the Azimuth, Elevation, Doppler

Shift and Spatial Offset of the distant station. The latter

two displays will only be present if you both have

coincident Moon.

6.2.7  [F4] key will automatically park your antennae to the home

position as defined by you in the Auto Track Set Up Menu.

(refer 8.5).

6.2.8  [F5] key allows you to re-calibrate your antennae readout.

Simply enter the new headings to change your display.

(Pulse Mode only).

6.2.9  [F6] key allows you to transfer your Target source bearings

into your Azimuth and Elevation antennae headings. This can

be very useful should you be unsure of your headings. If

for example, you are tracking the Sun and find that the

peak Noise heading is different to your beam heading you

can then transfer the Target to your own beam headings

(Pulse Mode only).

6.2.10 [F7] key allows you to save your current indicator Azimuth

and Elevation readouts. This allows you to leave your array

where you would prefer without driving it to the set park

position. You may then turn off your computer and recall

your previous position by the Shift[F7] key. (Pulse Mode

only).

6.2.11 Shift[F7] key allows you to recall your previously stored

Azimuth and Elevation Indicator position. When initiating

the program enter into the Auto Track Screen and hit

Shift[F7]. The programed Park position will be changed to

the "Temporary Parked" position as previously stored.

(Pulse Mode only).

6.3 Graph Track. (Requires EGA/VGA Monitor)

This is the real heart of the Software and provides

advanced facilities to check your system and to perform

accurate and or scientific measurements. It is this area

the author has concentrated his efforts.

The options provided by Graph Track allow you to select

many sources and:

(a) automatically position the array and then graph

either E or H Plane polar patterns.

(b) calculate the Antennae Gain and beam angle directly

from the screen. You can calculate gains either

separately or as a composite gain figure.

(c) graph your Moon echos.

(d) obtain a time domain/amplitude plot of received

signals.

(e) Utilize the received signals to perform rudimentary

Digital Signal Processing and recover the actual CW

signal and play it on the internal computer speaker.

(f) Sample a noise source and display level variations

on an expanded graph.

As with all the above facilities the Graphs may be stored

on disk and later retrieved (overlaid on top of another

graph and/or printed) for evaluation.

Whilst in the Graphic Screen help is provided by hitting

[F1] and stepping through the function key designations.

(Shift,Alt,Ctrl). The key functions are displayed on the

right hand side of the screen.

6.3.1  Graphic Screen Options.

To utilise the Graph Track facilities you must have a

monitor capable of displaying EGA or VGA. The software is

"downward compatible" to EGA and will result in a 640x350

resolution graphics screen. The options are now described

in function key order.

6.3.2  [F1]. Step through the Function keys accessed from the

Graphics Option. (Shift,Alt,Ctrl).

6.3.2  [F2]. Select a Target Source. You may select any of the

Menu displayed options provided. They include the Sun and

other Noise Sources as well as the Cold Sky areas. The

Software will then Track your selection. You may also input

a Azimuth Bearing (set in Autotrack Screen) which will permit

using a beacon as a source and then plot a beam pattern.

The [TAB] key allows you to select additional noise sources.

6.3.3  Shift[F2]. This is the DSP option. First load the receiver

file you wish to analyse with Shift[F10] before hitting

Shift[F2]. The displayed waveform will change to grey and a

yellow horizontal line will appear across the screen. This

line (set level), may be moved to a position representing

the DSP point. A counter on the left hand side of the

screen indicates its position.

[F1] will then translate the signal into a square wave

representing the level chosen. This will be displayed

in white.

[F2] will provide the CW output of this signal via the

computer speaker.

[F7] will return you to the set level condition so you may

alter the level as desired and repeat [F1] and [F2] as

often as desired.

[Esc] will return you to the main Graphics menu where you

may load another file (Shift[F10]).

6.3.4 Alt[F2]. Scan and Track a noise source (eg Sun) and record

the variations in level over a period of time. After

hitting this key, if the source chosen is above the

horizon, the software will position your array at the

source. If the source is below the horizon plotting will

commence immediately on the top half of the screen. The

sampling time is calculated commensurate with your computer

and is displayed at the bottom of the screen. On the right

hand side of the receiver input display the peak input

level is displayed. Your input level should be adjusted to

indicate 0.0dB (or as near as you can make it), and under

these conditions the level plotted on the top half of the

screen (yellow) will be referenced to 0dB. You will notice

that the display is linear and has a range of +/- 6dB. You

may check this by placing an attenuator in line with your

receiver. The input level is in this case the peak level as

seen by the A/D. Levels greater than or less than 6dB with

respect to the reference level will clip the plot. If you

are expecting a variation greater than 6dB then the level

can be set accordingly (-3 or -6dB for example). At the

commencement of the scan the time will be displayed on the

screen. When the scan reaches the far right hand side of

the display (or you hit any other key to stop the function)

the end date and time will be shown. The graph display may

be stored on disk by [F10] or the whole procedure repeated

with the Alt[F2] key.

6.3.5  Control[F2]. This option allows you to vary the time

between updating the Antennae heading with the Source when

using the Scan function. Alt[F2]. The default setting is 30

counts before updating. You will see the count reducing at

the end of each scan. When it reaches 0 it will update.

This function allows the user to input a value commensurate

with their Antennae Gain (Beamwidth). The maximum value is

499 which is the full screen width of the display. If you

choose such a value you will infact plot the beam width

slope of your array.

6.3.6  [F3]. Position your array for graphing Moon Echos. (Auto

Track the target in the Graphics Mode). The array will be

then set to track the Moon. After obtaining Lock you are

ready to plot your Echos by hitting Shift[F3].

6.3.7  Shift[F3]. Plot Moon Echos. When Lock is obtained with [F3]

hitting this key will send seven 200mS dashes which

activate the PTT Relay. Following the last dash and two

seconds after the rising edge of the first pulse, the graph

will plot the returned echo on the screen for the next two

seconds. You will also note that if you wish to plot

another Echo this will be done in another colour or if you

chose, you may clear the screen first with [F7]. There are

4 alternating colours selected automatically by the

software. This applies to all plotting options except for

the Receiver Scan functions where only green has been

provided.

6.3.8  [F4]. Park Antennae. You may Park your Antennae to the

Azimuth and Elevation positions as chosen and set in the

Auto Track Setup Menu.

6.3.9  Shift[F4]. Target to Indicator. If you are using the Pulse

mode of operation this facility allows you to re-calibrate

your indicator. Manually peak your array on a noise source

or your Moon Echos and then by hitting Shift[F4] you will

transfer the Target source Az and El positions to that of

your Indicator readout. This may upset your Park position.

6.3.10 [F5]. Position for Azimuth. This will position your array

the number of degrees counter clock wise from your Tracking

Target (eg the Sun).  The number of degrees offset will be

that as chosen from the four available scales provided in

the Change Degree Scale under Shift[F8].

6.3.11 Shift[F5]. Graph Azimuth Noise. After positioning your

array and obtaining "Offset Lock" with [F5], hitting this

key will start the Az Drive and plot the Noise on the

Graphics Screen. The motors will drive through the Target

and stop at the corresponding Azimuth on the other side of

the source. Again hitting Shift[F5] (after completing a

plot), will repeat the plot in the opposite direction in

another colour.

6.3.12 Alt[F5]. Graph Azimuth Noise. This is the same as Shift[F5]

except it provides for Non-Linear readouts. If your indicators

do not produce a linear step (increment in 0, 1, 3, 5, 7, 9

or similar) then this option should be used to Graph your Noise.

6.3.13 [F6]. Position for Elevation. This option functions the

same way as for Azimuth as previously explained in option

[F5] above noting that the array will initially be

positioned below the Target.

6.3.14 Shift[F6]. Graph Elevation Noise. Again as explained in

option Shift[F5] the Elevation Noise will be plotted in

this option.

6.3.15 Alt[F6]. Graph Elevation Noise. Again as explained in

option Alt[F5] the Elevation Noise will be plotted in

this option.

6.3.16 [F7]. Clear Graphics Screen. After plotting a graph you may

clear the screen by hitting this key or continue to "paint"

another graph, in another colour, over your previous

graphs.

6.3.17 Shift[F7]. This will calculate Antennae gain and Beam Angle

after plotting a graph. The 3dB calculation points will be

shown on the screen as small crosses.

6.3.18 [F8]. Change Degree Scale. This allows the user to select

the offset Azimuth and Elevation headings from where the

array will be positioned before commencing to graph noise.

(Options [F5] and [F6]). The offsets provided are +/- 5,

12.5, and 25 degrees. In other words if you select +/- 5

degrees and plot a Elevation noise response the antennae

will first be positioned 5 degrees below the Target and

then run through the target and stop 5 degrees above the

Target. The Azimuth will track correctly.  Hitting [F6]

again will drive your array downwards and provide another

plot in a different colour from where you came.  (less

Target change of position ofcourse!)

6.3.19 Shift[F8]. Change (invert) the dB Scale. Depending upon the

measurements you are making it is sometimes useful to

present the display in an inverted format. This option

provides this function where the dB Scale can be set for

either 0-30 or 30-0 dB.

6.3.20 Control[F8]. Calibrate input Voltage to the Receiver Input

A/D. This function will enable you to display your receiver

input voltage to the A/D converter. It will be displayed as

a open bar and vary in level commensurate to your input

voltage. The Maximum voltage will ofcourse correspond to +5

volts. (0dB reference for noise measurements or +x dB for

receiver S/N+N level). Press any key to clear screen and

resume.

6.3.21 [F9] Receiver Scan 10 Seconds. This option allows the user

to use the graph option to plot the receiver output. This

is a Time domain/amplitude display referenced to the levels

as set in Shift[F8] above. The graph will plot the receiver

for 10 seconds and then stop. Hitting the [F9] key again

will clear the graph screen and repeat the operation.

During the scan if you hit any key it will stop the

operation. You can store this graph and print it out later,

for inclusion with your QSL of the station you worked!!

6.3.22 Shift[F9]. Receiver Scan 50 Seconds. This option is exactly

the same as explained in [F9] except the scan period has

been extended to 50 seconds.

6.3.23 [F10]. Store the Graph to Disk. The default file extension

has been chosen for you depending upon the graph you have

just plotted. The program asks you the file name. Remember

to keep the file name down to 8 letters. The file is then

stored with the extension associated with the graph.

Azimuth         xxxxxxxx.AZM

Elevation       xxxxxxxx.ELV

Moon Echo       xxxxxxxx.ECO

Receiver        xxxxxxxx.REC

Scan            xxxxxxxx.SCN

xxxxxxxx = user chosen file name.

The file size is approximately 63kB long so make sure you

have enough room on your disk to store the file! The Graph

Plot provided in all the above options also Date and Time

stamps your plot. This is saved and will be displayed on

the screen when recalled. It also saves the dB and Degree

Scales. Note You don't type the extension as this is added

automatically).

6.3.24 Shift[F10]. Load a graph from disk. This option allows you

to recall and overlay any file, on any other, or on a blank

screen. The colours and scales will be the same as those

stored. The Software will step you through the types of

files available to which you respond to the one required

with a "Y". A [CR] or any other key will be a default "no"

response! After selecting the file type, respond with the

file name and this will be displayed on the screen! If the

file does not exist it will tell you so! A Directory of

files, (in the associated colours), appears on LHS of the

screen. (in place of the Function help keys).

6.3.25 Alternate[F10]. Print a Graph. This option will print the

graphics screen on any standard printer with a graphics

option. You may load from disk the display(s) you wish or

print direct from the screen.

6.3.26 Control[F10]. This option allows you to Erase a stored

Graph file. Choose the file type you wish to erase (this is

colour coded), and simply type in the File Name. The file

extension is NOT required.

7.0 GENERAL SET UP.

This Sub Menu provides the default parameters that are

stored as your own Home Location and are used on start up.

You may also set the Computer Time from within this Menu.

Remember to save any changes when you are finished.

7.1 Home Location Change.

This option provides the ability to input your Home

Location Data that is used for all calculations and is

called when initiating the Program.

7.2 Local Winter/Summer Time.

This option allows you to select either Summer (+1 hour),

or Normal Time.

7.3 Operating Frequency.

This option allows you to select any frequency from 50 MHz

to 24 GHz. It is used for doppler calculations.

7.4 UTC/Local Computer Time.

This option allows the user to run your Computer Time as

UTC if desired.

7.5 Display Source Target.

The user may select either the Moon, Sun or any Noise

Source for use in the Tracking Mode Routines. It is also

provided in the other screens.

7.6 Transmit Sequence.

This option allows the user to select either First or

Second TX sequencing.

7.7 Period of Transmission.

The user may select Transmission periods of 1, 2 or 2 1/2),

Minutes as desired as used in the Real Track Mode.

7.8 Warning Sound On/Off.

The user may turn off the Warning Alarm as used in the Real

Time Track Mode.

7.9 Save Set up Parameters.

This option allows you to save all options to Disk.

7.10 Change Computer Time.

You may correct your Computer Time with this option. Select

the highlighted figure you wish to change, with the [arrow]

keys, and use the [+/-] keys to increment or decrement the

value as required. [Esc] to exit when finished.

8.0 AUTO TRACK SET UP.

This Sub Menu provides all the facilities required to

utilise the Auto Tracking options of the Software. As your

Station will most likely be different from all others,

adjust the parameters to suit your particular situation.

8.1 Mode Selection ... Analog / Digital / Pulse.

This Option allows the user to select the method of

Computer Interface that is most adaptable to existing

Azimuth and Elevation Displays.  The available Modes are as

follows.

(a) The Analog facility requires a voltage input varying

from 0v to +5 Volts [Maximum] for both the Azimuth and

Elevation, (G.H.A. / Dec) displays. The user should

arrange their system to provide this maximum variation

for the best resolution. Calibration and scaling of

the input voltage is achieved from the "Analog Set Up

Parameters" Menu. Refer connection details P3 Section

9.0.

(b) The Digital Inputs [4 per digit] are connected in

parallel to the Azimuth 000.0 and Elevation 00.0

displays.  Normal logic levels are required. Refer

connection details P4 and P5 Section 9.0. Used as

input for the VK3UM/ZL3AAD Gray Code position

indicator hardware.

(c) The Pulse input requires +5v to 0 Volt pulses (50%

Duty Cycle - use a D Flipflop) corresponding to the

drive movement of the antennae. The reference starting

point is the Park position of the Array. The "Pulse

Set Up Parameter" Menu allows the user to specify the

number of pulses per degree which then provides

automatic calibration the display. Key [F5] allows the

user to Re-calibrate Az and El if required. Refer

connection details P3 Section 9.0.

8.2 Digital Setup.

This mode is for Operators that may already have a Digital

Display attached to some form of decoder. The interface

allows such a display to be utilised as input to the "VK3UM

EMETRAK" Software. The Connection for each group of BCD

lines, (four for each digit) and is designated Hun Ten Uni

and Dec). The maximum accuracy is 0.1 degree with overall

Software capable of 0.05 degs. Unused Decimal Degree inputs

should be tied Low if not required. (Refer Section 9

Connector P4).

8.3 Analog Setup Parameters.

This option has been designed to utilise potentiometers

attached to the Az and El of the Array. It is suggested

that quality 10 turn potentiometers be employed. It is left

to the user to design a system to allow a 0v - +5 volt

variation to be presented at the interface. The 0 and +5v

should correspond to the maximum range of movement. For

example should you wish to track from 90 to 270 degrees

then the maximum displayed resolution is (270-90)/255 or

0.7 degrees. Accordingly, set both the Az and El to suit

your needs. The 0v always corresponds to the smaller value.

Tracking in both hemispheres is automatic from your Home

Station information and the resolution is computed and

displayed from your designated range values.

8.4 Pulse Setup Parameters.

This option has been designed for those who choose to

employ an optical or a mechanical shaft encoder to generate

pulses during rotation. The software will count these

pulses and display the respective heading. Due to the speed

of the computer there is naturally a speed limitation. This

is approximately 25 pulses per SECOND for a 4 MHz machine.

THE PULSES MUST BE 50% DUTY CYCLE FOR MAXIMUM SPEED. LESS

THAN 50% DUTY CYCLE WILL SEVERELY LIMIT THE SPEED. Note I

am talking speed of rotation or the number of 50% duty

cycle pulses sent to the interface from your shaft encoder

in one second. Do not confuse this with pulses per degree.

With higher speed machines the figure naturally increases.

I use the 4MHz machine as a worst case scenario. Always

check this out first (assuming you are presenting sharply

defined pulses to the interface) to make sure you are not

"dropping or missing pulses". The resolution is a function

of pulses per 360 degrees as defined in this Menu by the

user. The reference point is set by the Home Position of

the Antennae as defined by the user. Re-setting is provided

by [F5] but will void the Home Position. Built into this

function is the ability to only Track in Azimuth or

Elevation. The drive you wish disabled should be set to 0

pulses per 360 degrees. This will inhibit that drive for

all modes. (Pulse, Digital and Analog). Since February 1993

Version 2:02, the software has been changed to permit

Pulses per 360 degrees to reduce possible cumulative

errors.

This now means that:

360 = 1.0 pulse per degree,

3600 = 0.1 pulses per degree or

36000 = 0.01 pulses per degree.

Other values will permit exact scaling to suit your system. Note

that the input pulse must be negative going as fed to the tracking

interface. A pull up resistor to +5v may be required to achieve

this depending upon the encoder you employ. ie. the pulse shall go

from +5v to 0v to count and held at +5v whilst idle. (RF immunity

design).

Azimuth and Elevation Motor Run-on.

These options allow for the correction of added pulses

that are not counted after the motor off command is

given.  You may input the number of run on pulses by

implementing the internal pulse counter [F1] (reset by

[ShiftF1]) and viewing the extra pulses after you stop

the motor. The extra pulses are added as soon as the

motor starts and as such will stop prior to reaching

the true heading. The resultant operation will provide

for correct tracking commensurate with any motor over-

run. The maximum number of added pulses is 99. A

substitute for stopping the motor dead !!  Note.. If

you require this motor run-on facility then the

hysteresis factor will have to be increased

commensurate to the distance travelled. This generally

means that a hysteresis factor of 3 or more is

required. Check the hysteresis window, as displayed in

degrees, with respect to the added run on pulses and

derive a factor suitable for your needs. (refer 8.6).

Again problems will be experienced as a result of the

time the motor is run and the time it will take to

stop! (an "un-programable" variable!)

8.5 Home (Park) Position of Antennae.

This facility provides for the parking of the Array

automatically by the [F4] key. Both Az and El will drive at

the same time.  Should you be in the tracking Mode this

will be disabled. The values input here are also used as

the reference starting point for those using the Pulse

Input Mode. You may enter a negative value for the Park

Elevation/Declination. In Auto Track this value will be

displayed in red to indicate a negative number.

8.6 Az Motor Hysteresis.

This facility allows the user to control the "window"

within which tracking will be maintained. If you have a

motor drive that runs on, (does not stop quickly enough),

then you may extend the "window" by a multiplication

factor. This factor, (normally set to 1), is used to

multiply your tracking accuracy. Set this factor to suit

your system.

examples:

a. If you are running an analog position indicator and

you are spanning 359 degrees (0 volts = 0 degrees and

+5 volts = 359 degrees), then your tracking

resolution = 359/255 or 1.41 degrees. If you choose

to use a Az Hysteresis Factor of 2 then you will then

have a window of +/- 2.82 degrees.

b. If you are running a pulse position system and you

are using 1 pulse per degree then your tracking

accuracy is 1.0 degree. Should you choose a Az

Hysteresis Factor of 5 then your tracking window will

be +/- 5.0 degrees.

c. If you are running a digital input positioning system

with 0.1 degree resolution and then using a

Hysteresis Factor of 10 the window becomes +/- 1.0

degrees.

The factor is stored as part of your start up file in

EMETRAK.DAT.

8.7 El Motor Hysteresis.

The functions are identical to that of the Azimuth

Hysteresis function.

8.8 Drive Update Period in Minutes.

This option allows the user to select how often you wish

the Az / El Motors to operate. If you enter 0 then the

motors will drive every 15 seconds as this is the update

Target calculation period. When using the Pulse Mode, the

time and Target is frozen until Lock has been achieved. The

period starts again after Lock has been achieved.

8.9 End Stops of Antennae.

This option allows the user to set Software End Stops to

prevent over run of their Array. Two, for each Azimuth and

Elevation, (G.H.A. / Dec) have been provided. Always set #1

as the lower value. You may enter a negative value for the

Elevation/Declination. In Auto Track this value will be

displayed in red to indicate a negative number.

UNDER NO CIRCUMSTANCES SHOULD THE USER RELY UPON THE

SOFTWARE ENDSTOPS, BUT AS A MATTER OF SAFETY, PROVIDE THEIR

OWN FAIL SAFE ELECTRICAL INTERLOCK PROTECTION.

8.10 Communication Port [9600/8/1/N].

This facility allows the user to choose either COM1 or

COM2 Serial Ports as defined by the IBM (or compatible)

configuration. Variation of the speed, number of data and

stop bits as well as parity can not be changed in Software.

8.11 Antennae Mount. Az/El or G.H.A./Dec.

This option allows the user to select either of the

Antennae mounts and track the Sources as required. The

respective displays will change according to the type of

mount selected.

8.12 Motor Speed Control

This option allows the user to control the speed of the

drive motor. The "Spare" relay is used for this purpose. As

only one relay is available, you only have the option of

controlling one motor. This is selected from the sub menu

from within this option. You may select either the Azimuth

or Elevation drives and input a "window" within which, the

"Spare" relay will NOT be actuated. That is, the relay will

operate for FAST speed and NOT operated for SLOW speed. The

figure you input sets a +/- value for the window. If you do

not require this option then make both the Azimuth and

Elevation windows equal to 0.

8.13 Save Set Up Parameters to Disk.

This option allows the user to save all Auto Track Setup

parameters as well as the General Set Up Parameters to

Disk. They are stored in the Data File EMETRAK.DAT. These

parameters are used on start up so remember to save all

your changes prior to exiting the Program!

9.0 PRINTED CIRCUIT BOARD CONNECTIONS.

Connector P1  (RS232).

1   Protective Ground

2   Transmit Data

3   Receive Data

4   Request to Send

5   Clear to Send

6   Data Set Ready

7   Signal Ground

8   Received Line Signal Detect

20  Data Terminal Ready

At the Computer end of the cable join 4-5 and join 6-8-20.

Connector P2 (Relay Output).

1   Azimuth - Clockwise.

2   Azimuth - Counter Clockwise.

3   Elevation - Down.

4   Elevation - Up.

5   PTT - Echos.

6   Polarity CW  - not used in the current software.

7   Polarity CCW - not used in the current software.

8   Spare now designated for Motor Speed Control.

9         Earth

10        Earth

11        Earth

12        Earth

13        Earth

14        Earth

15        Earth

16        Earth

The "Relay Outputs" may be Ground or +5v as selected by the

Interface Link.

Connector P3 (Analog and Pulse Input).

1         Not Used

2         Not Used

3         Not Used

4         Not Used

5         Not Used

6         Receiver  Audio Input [0 - +5v]

7         Elevation Pulse Input [0 - +5v]

8         Azimuth   Pulse Input [0 - +5v]

9          +5v      (available for external use)

10          +5v      (available for external use)

11         Earth

12         Earth

13         Earth

14         Earth

15         Earth

16         Earth

Connector P4 (Azimuth Digital Input). External 4511 Pin

1         Az Ten     0              7   (A)

2         Az Ten     2              1   (B)

3         Az Ten     4              2   (C)

4         Az Ten     8              6   (D)

5         Az Unit    0              7   (A)

6         Az Unit    2              1   (B)

7         Az Unit    4              2   (C)

8         Az Unit    8              6   (D)

9         Az Decimal 0              7   (A)

10         Az Decimal 2              1   (B)

11         Az Decimal 4              2   (C)

12         Az Decimal 8              6   (D)

13         Az Hundred 0              7   (A)

14         Az Hundred 2              1   (B)

15         Az Hundred 4              2   (C)

16         Az Hundred 8              6   (D)

Connector P5 (Elev. Digital Input). External 4511 Pin

1         El Ten     0              7   (A)

2         El Ten     2              1   (B)

3         El Ten     4              2   (C)

4         El Ten     8              6   (D)

5         El Unit    0              7   (A)

6         El Unit    2              1   (B)

7         El Unit    4              2   (C)

8         El Unit    8              6   (D)

9         El Decimal 0              7   (A)

10         El Decimal 2              1   (B)

11         El Decimal 4              2   (C)

12         El Decimal 8              6   (D)

13         Earth

14         Earth

15         Earth

16         Earth

10.0  COMPONENT LIST - VK3UM EME AUTOTRACK INTERFACE.

10.1  Integrated Circuits.

IC1  AY-3-1015D               IC11 74LS08

IC2  ICL232 (MAX232)          IC12 74LS04

IC3  74LS123                  IC13 74LS05

IC4  74LS155                  IC14 74LS00

IC5  74LS175                  IC15 74LS374

IC6  74LS175                  IC16 74LS374

IC7  74LS00                   IC17 74LS374

IC8  74LS04                   IC18 74LS374

IC9  74LS00                   IC19 74HC4060P

IC10 74LS08                   IC20 ADC0809CCN

10.2  Capacitors.

C1 - C2   27 pf  Ceramic

C3        22 nf  Green Cap         (223K)

C4       1.5 mfd Tantalum 16vW     (Watch Polarity!)

C5       2.2 nf  Green Cap         (222K)

C6       4.7 nf  Green Cap         (472K)

C7        22 nf  Green Cap         (223K)

C8       2.2 nf  Green Cap         (222K)

C9       0.1 mfd Green Cap         (104K)

C10       10 nf   Green Cap         (103K)

C11 - C17 10 mfd  Tantalum 16VW     (Watch Polarity!)

C18 - C21 22 mfd  Tantalum 16VW     (Watch Polarity!)

Green Cap = polycarbonate or similar capacitor.

10.3  Resistors.

R1  - R8  1.5k ohm 1/4 Watt

R9  - R16  390 ohm 1/4 Watt (180 ohm parallel LEDs used)

R17 - R18  22k ohm 1/4 Watt

R19        10M ohm 1/4 Watt

R20        10k ohm 1/4 Watt

R21       2.2k ohm 1/4 Watt

R22        10k ohm 1/4 Watt

R23       100k ohm 1/4 Watt

R24        10k ohm 1/4 Watt

R25       1.2k ohm 1/4 Watt

R26        10k ohm 1/4 Watt

10.4  Transistors.

T1  -  T8  BC547

T9         7805 (Voltage Regulator)

Use Link if not required.

10.5  Crystal.

X1   2.4576 MHz

10.6  Diodes.

D1  - D11  1N914

D12        1N4004

D13 - D20  5.6v Zeners

10.7  LED's.

1 - 8  Miniature Red - note these can be mounted on the board

plus remote LEDs on the front panel. If used in parallel

reduce R9-R16 from 390 ohms to 180 ohms.

10.8  Relays.

RL1 - RL8  NEC MR62-5S (5v DPDT)

10.9  Fuse.

F1 - 1A (Off Board Mounting - bridge if not required)

10.10 Sockets.

P1       26 pin IDC  Vertical Non Clip Mounting

P2 - P5  16 pin IDC  Vertical Non Clip Mounting

10.11 Alternative IC Component Designations.

IC1  AY-3-1015D TR1865PL (UART)

IC2  MAX232  MAX232CPE  AD232JN  DS14C232CN

IC19 74HC4060 MM74HC4060N M74HC4060B1N PC74HC4060P

PCF74HCT4060P CD74HCT4060E

RL1 - R8  Relays.

Also known in USA as Potter & Brumfield T82NI D113-05

The pin configuration is as follows:

1-2 = 5 volt coil.

C = Common. NC = Normally Closed Contacts

NO = Normally Open Contacts

1    C  NC NO

o    o  o  o

o    o  o  o

2    C  NC NO

10.12 Component Sources. To the best of my knowledge they

stock all the above components.

USA.

Jameco Electronics            Ph 415-5928121

1355 Shoreway Rd              Ph 415-5928097

Belmont CA 94002              FAX 415-5922503

EUROPE

Export Department             Ph +44-532-636311

Farnell Electronic

Components Ltd.            FAX +44-532-633411

Canal Road

Leeds  LS12 2TU

United Kingdom

On the order, write "Please charge credit card (say

Visa or MasterCard) number xxxx, expiry date xx/xx"

and sign the order. I am told that the card will be

charged in local currency in most countries at a fair

exchange rate.

11.0 CHECK OUT PROCEDURE.

As most of you that will be undertaking this project

are highly competent and experienced in the EME field

you will find this a breeze!!! Be assured that, if it

does not work, it is due to something YOU have done

wrong!  Refer to File "BOARDFIX.DOC" for additional

information before applying power to the board.

11.1 Assemble the board as per the component layout paying

careful attention to the orientation of the components.

I recommend the use of IC sockets but this ofcourse is

not necessary. Just be sure that the IC's are aligned

properly before soldering!! You may mount the LED's on

the interface card or externally.  Additionally you can

do both providing you reduce the resistor values as

shown.

11.2 Apply power be it 7 - 9 volts DC or +5 volts if you

choose to bridge out the onboard regulator. Measure the

current. It should be in the vicinity of 100 - 200 mA

depending upon your voltage source. All relays should

not be operated as indicated by the LED's.

11.3 Interconnect the board to your Computer (refer Section 9

for connection details). Plug the interface into COM1

(default Serial Port) or COM2 should you desire. In the

latter case remember to change to this Port in the

AutoTrack Set up options.

11.4 Run the EMETRAK Software and select the Auto Track

Option under Tracking Modes from the Main Menu.

11.5 Using the Cursor Keys (Arrow Keys), select in turn the

Right Arrow, (Clockwise) Left Arrow (Counter Clockwise)

and Up Arrow (Elevation Up) keys. The respective LED's

should light! I would advise you not to connect your

drive motors yet until you become familiar with the

software. Remember that the Space Bar or any other "Text

Key" will stop the motor(s) and restore you to Manual

Track if you happen to be in Auto Track.

11.6 You can now go back to the other Screens and set you QTH

and all other parameters you will be using. To detail

how to do this here is difficult as you have so many

combinations to chose from. It is very straight forward

and logical! The explanations within this document will

point you in the right direction. It all depends upon

just what you will be using for your position read outs.

11.7 Calibration of your readouts can be done directly from

the screen if you are using Analog Potentiometers.

Remember that the maximum count is equal to +5 volts and

should not be exceeded.  Zener diode protection is

provided but the board is not "Goof Proof".  When using

Digital inputs their connection is as previously

indicated. As a check you will mimic your readout directly on

the screen. If you chose to utilise the Pulse option then

provide a +5 to 0 volt pulse to the interface. I would

suggest that you use a Schmitt trigger driving a monostable.

A 74LS123 would be suitable and this would give you the

option to double the resolution of your pulse system. eg. If

you feed the output from your pulse encoder, (via a schmitt

trigger), to the respective inputs of both halves of the

74LS123 and configure them to trigger on +ve and -ve pulse

transitions and then "OR" their output you will provide two

pulses to the interface instead of one! This will make a one

pulse per degree encoder capable of 0.5 degree resolution.

Don't forget to set your pulses/360 degrees to 720!. Remember

that 3600 pulses per 360 degrees will give you a 0.1 degree

readout and anything less will be scaled accordingly. The

Pulse input will allow for greater input but, to be sure, and

this depends upon your speed of rotation, use a variable

frequency pulse generator to simulate what you will be using.

11.8 The relays can handle up to 2.5A but it is recommended that

you don't use them to directly control your motors. One of

each is provided for UP/DOWN/LEFT/RIGHT as well as a PTT Line

for Transmitter control when using the Echo Graphing Option.

Two are currently not used (CW and CCW Polarity) whilst the

"Spare" can be used for the motor speed option as provided.

You do not need to mount these components at this time should

you desire. (relays can cost up to $5!).

11.8 The Analog input from your receiver may be interfaced to the

Auto Track board via a suitable audio detector and DC

amplifier. An interface should be calibrated to provide the

level commensurate with the dB scale provided in the graphics

option. Again the reference voltage should be set to +5 volts

peak relative to the scale chosen.

11.9 Now that you have had a play with the options you can get

serious! Go to the Auto Track Options Menu and select the

Pulse Mode. You may not intend to use this mode but it is fun

to play with and you can get the feel of the software!!! Next

select the Auto Track mode and energize the internal Pulse

generator with Shift[F1]. Now you can now play with the

options!! Simulate Auto Tracking, Parking, testing your End

Stops etc. You can also check out your Graphic facilities

from within Graph Track. Watch your respective control LEDs

light. (alternatively select REAL TIME TRACK and hit the HOME

key which will place it in the Demonstration Mode).

NOW ALL YOU HAVE TO DO IS INTERFACE IT TO YOUR REAL SYSTEM!!!!

12.0 SUMMARY

As you can see the interface options are quite profound. I

leave it up to you to select which mode suits you best. In

general, the concept of this system has been conceived to

cover nearly all the current readouts being used today. The

accuracy is commensurate to the information you present to

the interface!

Its no good striving for 0.01 degree accuracy when you have

backlash in your drive, the readout system or the antennae

moves in a strong breeze! The more attention you place on

YOUR input the more accurate will be your tracking!

Similarly for those that promote 12 bit A/D converters and

claim a higher degree of resolution should consider the

potentiometers they hang across it!! Even with the most

expensive available 0.5% you cannot realize the accuracy

provided by the A/D!!

A final point with respect to the Software. As you can see

it is highly complex! There are well over 14,000 lines of

Pascal code. It is most difficult to be 100% sure that

there is not a bug. I have gone to extreme lengths to check

out as much as is practical. Should you have a problem

please document, as best you can, the nature, effect, and

events that caused the difficulty.

I use the total system (with digital Gray code position

indicators) all the time so that most problems are quickly

revealed, but one cannot be absolutely sure! I have based

the options from practical experience and from input from

other users. If you have any suggestions for improvement

please do not hesitate to write. (Don't ask me about

Satellite tracking facilities or CW options please ... I am

not interested! This is for those interested in EME and

Radio Astronomy operators, not telephone users!!). The

Software was written and developed as an Amateur project to

fill a need. (My need!) It is not my full time profession

nor do I expect to benefit financially! The cost has been

kept to the barest minimum. Accordingly I would appreciate

your help to cover costs of any future correspondence or

software updates.

12.1 Changes to Printed Circuit Board May 1991.

On Printed Circuit Board (Version May 91) there are two minor

changes to the Artwork. Both are on the solder side of the

board.

(a) Near 7805 Regulator - a small section of track has been

removed.

(b) IC 6 track to pin 10 has been re-routed from pin 11.

This is the input to the Spare relay. The error has been

corrected. In addition, the hole sizes for the 7805 are

too small. Bend ends of the leads over and solder direct

to the top side of the board.

12.2 Changes to Printed Circuit Board May 1992.

On some Printed Circuit Boards (Version May 92) there is a

solder link in a large track near the centre of the board.

The board manufacturer forgot a hole during the drilling

process!

12.3 Finally!! The board has been put together by some 70 Stations

so far, (1st Sept 1992) and the ONLY PROBLEMS HAVE BEEN

FAULTY IC's, solder bridges (please don't use a 300 watt

soldering iron!) and the odd wrong wiring of the serial

interface cable. The faulty IC's have been the inexpensive

ones like the 74LS00, 74LS04, 74LS05 and 74LS08. Make sure

you get them from a good source!!

13.0 FUNCTION KEYS

[F1] Main Screen .. Software Version Number.

13.1 Real Time Track Screen.

[F2] Select Target Sources.

[F3] Input a Grid Square.

13.2 Auto Track Screen.

[F1] Enable/Disable display of Az and El pulse counts

Shift[F1] Enable/Disable internal Pulse Generator.

Control[F1] Reset Pulse Counter (refer [F1]).

[F2] Select Target Sources.  [TAB] More Sources.

[F3] Initiate Auto Tracking Mode.

Shift[F3] Input a Grid Square.

[F4] Park Antennae.

[F5] Recalibrate Az/El indicators.

[F6] Transfer Target Az/El to Indicator Az/El.

[F7] Save Indicator Az/El.

Shift[F7] Recall Indicator Az/El.

13.3 Graph Track.

[F1] Help Screen of Graphic Function Keys.

[F2] Select Target Sources.  [TAB] More Sources.

Shift[F2] Digital Signal Processing (Rudimentary)

Load Receiver file with Shift[F10] first.

[F1] Set Level with arrow keys first then perform DSP.

[F2] Replay CW on Computer Speaker.

[F7] Repeat Set Level Procedure.

[Esc] Returns to Menu.

Alt[F2] Long Term Scan and signal variation Graph option.

Ctrl[F2] Change Tracking Update Scan Count. (Max 499).

[F3] Auto Track Target (Position for Echos).

Shift[F3] Plot Moon Echos.

[F4] Park Antennae.

Shift[F4] Transfer Target Az/El to Indicator Az/El. (Pulse Mode).

[F5] Position for Azimuth E Plot.

Shift[F5] Plot Azimuth E graph.  Linear increment Indicator mode.

Alt[F5] Plot Azimuth E graph.  Non-linear increment Indicator mode.

[F6] Position for Elevation H Plot.

Shift[F6] Plot Elevation H graph.  Linear increment Indicator mode.

Alt[F6] Plot Elevation H graph.  Non-linear increment Indicator mode.

[F7] Clear Graphics Screen;

Shift[F7] Calculate Antennae Gain and Beam angle.

[F8] Change offset Degree Scale. +/- 5 , 12.5 or 25 degrees.

Shift[F8] Invert dB Scale.

Ctrl[F8] Calibrate A/D input voltage.

[F9] Receiver scan plot 10 Seconds.

Shift[F9] Receiver scan plot 50 Seconds.

[F10] Store Graph Plot to Disk.

Shift[F10] Load Graph Plot from Disk.

Ctrl[F10] Erase a Graph File.

Alt[F10] Print Graph.

13.4 [F2] Target Source Options  ( plus [TAB] for more)

[F1] Moon.                 Shift[F1]   P1641-45

[F2] Sun.                  Shift[F2]   P1831-03

[F3] Aquarius.             Shift[F3]   P1858+03

[F4] Leo.                  Shift[F4]   P1354-62

[F5] Pictor.               Shift[F5]   Orion M42

[F6] Cassiopeia.           Shift[F6]   Taurus A

[F7] Sagittarius.          Shift[F7]   Centurus A

[F8] Cygnus.               Shift[F8]   Virgo

[F9] Omega M17.            Shift[F9]   Perseus A

[F10] User Input Az / El.  Autotrack and Graphics only.

Shift[F10] Input a Grid Square. Autotrack and Graphics only.

Doug McArthur (VK3UM)

22nd November 1993