CAMFORCE.EXE

This program calculates the loading on a cam follower.  Two standard cam

profiles are included for comparison.  The program neglects the effects of

friction and pressure angle.  However, for pressure angles less than 30 degrees

and for a coefficient of friction of less than 0.1, minimal error should be

introduced.

INPUT DATA:

SELECT CAM PROFILE:  Two cam profiles are available.

Cycloidal

Modified Sine

For a discussion of the advantages of each profile, see a mechanical

engineering text or reference book.  By running the program with the

same data for both profiles and observing the results, one can see the

effects of the two profiles.

After selecting the cam profile, the following data is entered:

CAM SPEED:       Enter the speed in revolutions per minute

PERIOD OF RISE:  Enter the number of degrees of cam rotation in which the

rise occurs.

CAM RISE:        Enter the rise of the cam lobe in inches.

INERTIA WEIGHT:  Enter the weight, in pounds, of the cam follower, valve and

any additional mechanism that will be moved by the cam.

SPRING PRELOAD:  Enter the initial spring loading that exists just prior to

starting the rise.  Load must be entered in pounds.

SPRING RATE:     Enter the spring rate, expressed in pounds per inch.  It is

assumed that the spring follows Hookes law, i.e. it remains in the

elastic range.

GRAVITY EFFECT:  If the inertia weight is significant or if the acceleration is

low, the effect of gravity may not be insignificant on the loading.  If

the centerline of the inertia weight is in-line with gravity, enter Y.

If the centerline of the inertia weight is perpendicular to gravity,

enter N.  If it is at an angle, a best-guess approximation must be

made after running the program with and without gravity affects.

RESULTS:

If the loading becomes negative during any part of the cycle, this is an indication

that the follower is "floating" away from the cam lobe.  Damage may result if

operated under this condition.  To eliminate float, increase the pre-load and/or the

spring rate.

The companion program, CAMDESGN.EXE, will calculate the acceleration of the

follower and attachments.  Cam loading is directly proportional to mass and

acceleration.

ACKNOWLEDGEMENTS:

The basis for this program was derived from an article in Machine

Design magazine on November 26, 1987.  The author of the article was

Chris S. Pedicini of the Eveready Battery Company of Brunswick, Ohio.

DISCLAIMER:

Results derived from the use of this program are believed to be accurate.

However, the author will not be responsible for any actual or

consequential damages that may result from the use of this program.

Your sole remedy is to recover any payment that you may have made

directly to the author for this program.  If you find an error, please

notify the author via Compuserve and a revision will be posted promptly.

SHAREWARE NOTICE:

If you find this program of value, you are requested to register the

program using the file, REGISTER.TXT, included with this distribution.

Bill R. Wittner PE                     Compuserve:  72167,3172

1705 Skyline Drive

Russellville, AR  72801

USA