CAMDESGN.EXE

This program calculates the displacement, velocity and acceleration of a

cam follower.  Two standard cam profiles are included for comparison.

The results may be either sent to the screen or to a printer.

INPUT DATA:

1.  Select Motion Profile:  Two cam profiles are available.

Cycloidal

Modified Sine

For a discussion of the advantages of each profile, see a mechanical

engineering text or reference book.  By running the program with the

same data for both profiles and looking at the displacement results,

one can see the physical difference between the two profiles.

2.  Select Motion Parameters:

a) Enter Cam Zone (1-4) -- The total 360 degree cam profile can be

divided into as many as four zones.  Use as many of the four

zones as required.  It is not necessary to use all four.  The data

described in items b) through f) below must be entered for each

zone.

b) Enter Motion (1=Dwell  2=Rise or Fall) -- Within the cam zone

selected, tell the program if it will be a dwell (a period of no rise

or fall) or if the cam follower will be rising or falling in this zone

of the cam profile.

c) Enter Initial Displacement (inches)  -- This is the radius of the cam

at the beginning angle of the zone for which the data is being

entered.  Data MUST be entered in inches.

d) Enter Final Displacement (inches) -- This is the radius of the cam

at the final angle of the zone being entered.  Data MUST be

entered in inches.

e) Enter Initial Angle (degrees) -- Enter the starting angle of the cam

profile for the selected zone.

f) Enter Final Angle (degrees) -- Enter the angle that defines the end

of the zone.

An example:  Assume that a cam has one dwell section that is 180

degrees in length with a radius of 1.5 inches (the first zone). It has a rise

section that is 90 degrees long with an initial radius of 1.5 inches and a

final radius of 2.2 inches (the second zone.)  The fall section is 90

degrees in length, with the radius then falling from 2.2 to 1.5 inches (the

third zone.)  The data entry would be as follows:

Cam Zone              1              2            3

Motion Type           1 (dwell)      2 (rise)     2 (fall)

Initial Displ         1.5            1.5          2.2

Final Displacement    1.5            2.2          1.5

Initial Angle         0              180          270

Final Angle           179            269          359

RESULTS:

Two parameters may be varied before or after you have entered the cam

physical profile data -- the speed of rotation and the motion profile

(cycloidal and modified sine).  Therefore, you can examine various

combinations of rotation speed and motion profile with a given physical

cam profile without re-entering the cam profile data.

The cam follower displacement, velocity and acceleration versus cam

angle may be viewed on screen graphs (item 3 on the menu).  If you

wish to capture this information, use the "print screen" function.

Additionally, you may print the displacement vs. cam angle, in one

degree increments, on your printer by selecting menu item 4.

The companion program, CAMFORCE.EXE, will calculate the forces

on the cam surface that result from the acceleration of the driven load.

ACKNOWLEDGEMENTS:

The basis for this program was derived from an article in Machine

Design magazine on November 21, 1985.  The author of the article was

Chris S. Pedicini of the Eveready Battery Company of Brunswick, Ohio.

DISCLAIMER:

Results derived from the use of this program are believed to be accurate.

However, the author will not be responsible for any actual or

consequential damages that may result from the use of this program.

Your sole remedy is to recover any payment that you may have made

directly to the author for this program.  If you find an error, please

notify the author via Compuserve and a revision will be posted promptly.

SHAREWARE NOTICE:

If you find this program of value, you are requested to register the

program using the file, REGISTER.TXT, included with this distribution.

Bill R. Wittner PE                     Compuserve:  72167,3172

1705 Skyline Drive

Russellville, AR  72801

USA